3GPP TSG RAN WG1 #109-e	R1-2203907

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.4.1.2

Source:	Samsung

Title:	On physical channel design framework for sidelink on FR1 unlicensed spectrum

Document for:	Discussion and Decision

# Introduction

In the WID [1], the following objective is included regarding the support of sidelink on FR1 unlicensed spectrum:

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4]

- Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation

  - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation

    - No specific enhancements for Rel-17 resource allocation mechanisms

    - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval.

- Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum

  - The existing NR sidelink and NR-U channel structure shall be reused as the baseline.

- No specific enhancements for existing NR SL feature

- The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

This contribution discusses aspects related to the physical channel design framework for supporting sidelink on FR1 unlicensed spectrum.

# Motivation and Working Scope

For operation with shared spectrum channel access in FR1, the required changes to sidelink signal/channel shall comply with the regulation requirement. More precisely, the following three aspects are required by regulation of FR1 unlicensed spectrum, and need to be considered when designing the physical channel design framework to support sidelink on unlicensed band:

- Energy detection (ED) based channel access procedure, including dynamic channel access mode and semi-static channel access mode, wherein the dynamic channel access procedure could further include Type1 channel access procedure (e.g. random sensing duration) and Type 2 channel access procedure (e.g. deterministic sensing duration). There are also features of LBT bandwidth, ED threshold adaptation, and channel occupancy sharing associated with the channel access procedure.

- Occupied channel bandwidth (OCB) requirement. Transmissions on a channel need to satisfy 80% of nominal channel bandwidth for FR1 unlicensed band.

- Power spectral density (PSD) requirement. Transmissions on a channel need to satisfy 10 dBm/MHz PSD requirement for FR1 unlicensed band.

The requirement from regulation may motivate the enhancement to sidelink signal/channel in frequency domain (e.g. to satisfy OCB and PSD requirement) and/or in time domain (e.g. to mitigate impact from channel access failure), and the details of the enhancement are described in the remaining of this contribution.

Meanwhile, based on the description of working scope for Rel-18 sidelink evolution, carrier aggregation with at least one carrier operated on unlicensed spectrum is not considered, hence, the development scenario for supporting Rel-18 sidelink on unlicensed spectrum (SL-U) shall at least include the standalone operation of all sidelink signal/channel for unlicensed spectrum.

Proposal 1: The study shall include investigating the following sidelink signals and channels to operate on unlicensed spectrum, subject to the regulation requirements:

- PSSCH

- PSCCH

- PSFCH

- S-SS/PSBCH block

- sidelink reference signals

# Frequency Domain Enhancement

The enhancements in the frequency domain are mainly motivated from the OCB and PSD requirement for unlicensed spectrum.

## Interlace based sub-channel and resource pool

In legacy sidelink (e.g. Rel-16 and Rel-17), resource allocation for sidelink transmission and reception (other than S-SS/PSBCH block) is based on the (pre-)configured resource pool, wherein the frequency domain unit for constructing the resource pool is a sub-channel. The sub-channel is a set of contiguous RBs, with the number of contiguous RBs in the set (pre-)configured. Typically, the bandwidth of a sub-channel may not be as large as or even comparable to the channel bandwidth, and reusing the legacy sidelink sub-channel and resource pool may require a large number of sub-channels allocated for a sidelink transmission to meet the OCB and PSD requirements, which may not be efficient or practical in real implementation.

Similar issue was observed and discussed in Rel-16 NR-U (or back to Rel-14 eLAA), and interlace based resource allocation was supported for uplink operating on unlicensed spectrum. In Rel-16 NR-U, an interlace is a set of non-contiguous and uniformly distributed RBs, with the interval size and number of RBs in the set specified for 15 kHz and 30 kHz. It has been justified that the interlace based uplink resource allocation can satisfy the OCB and PSD requirements on unlicensed spectrum.

For Rel-18 SL-U, similar approach to Rel-16 NR-U can be considered for sidelink sub-channel and resource pool. For example, a sub-channel can be defined based on at least one interlace, wherein the definition of interlace can use the one in Rel-16 NR-U as a baseline, e.g. fixed interval size for a given subcarrier spacing. RAN1 can further study whether enhancement to the Rel-16 NR-U interlace is needed for sidelink operation, including sub-RB interlace.

Moreover, a resource pool can be constructed based on the sub-channel, wherein the sub-channel is defined based on interlace, then naturally, the resource pool is also defined based on interlace. An illustration of the interlace based sub-channel and resource pool is shown in Figure 1. Then, the sidelink transmission and reception in the interlace based resource pool, e.g. PSSCH, PSCCH, PSFCH, and sidelink RS, can be further investigated, including potential enhancement to the indication of the resources (e.g. interlace index(es)), and the transmission and reception procedures.

Figure 1 Illustration of interlace based sub-channel and resource pool.

Proposal 2: To satisfy OCB and PSD requirements, it’s beneficial to support interlace based sub-channel and resource pool for sidelink on unlicensed spectrum.

- The structure of the interlace can use Rel-16 NR-U as a baseline.

- Further study whether enhancement for the structure of the interlace is needed (e.g. sub-RB interlace).

- Further study the resource allocation of PSSCH, PSCCH, PSFCH, and sidelink RS in the interlace based resource pool, and associated indication method and transmission/reception procedures.

## Frequency domain enhancement for S-SS/PSBCH block

In legacy sidelink (e.g. Rel-16 and Rel-17), a S-SS/PSBCH block (shown in Figure 2) has 132 contiguous subcarriers (e.g. 11 RB) in the frequency domain, which corresponds to 1.98 MHz for 15 kHz and 3.96 MHz for 30 kHz, and is much smaller than the nominal bandwidth of 20 MHz. Hence, the standalone transmission of a legacy S-SS/PSBCH block cannot meet the OCB and PSD requirements for unlicensed spectrum.

Figure 2 Illustration of legacy S-SS/PSBCH block structure.

In Rel-16 NR-U, similar issue was discussed for downlink SS/PBCH block, wherein a downlink SS/PBCH block has 20 RBs in the frequency domain and its standalone transmission cannot meet the OCB and PSD requirements for unlicensed spectrum as well. However, in downlink, the transmission of SS/PBCH block can be multiplexed with other signals (e.g. CSI-RS) or channels (e.g. PDCCH and/or PDSCH), and the multiplexed transmission can meet the OCB and PSD requirements for unlicensed spectrum. In particular, concept of “discovery burst” was supported for Rel-16 NR-U, wherein the “discovery burst” is a set of signals/channels multiplexed without any gap in time domain, and components of “discovery burst” must include SS/PBCH block as the basic one, and optionally include PDCCH/PDSCH of RMSI and CSI-RS, up to configuration. Such mechanism can be also considered for SL-U, but may need to further study how to multiplex sidelink signal/channel with S-SS/PBCH block, and its potential impact to the resource pool.

Other than multiplexing S-SS/PSBCH block with other signal/channel (e.g. similar approach in Rel-16 NR-U), RAN1 can also investigate direct enhancements to the S-SS/PSBCH block structure for SL-U, including a new S-SS/PSBCH block structure with a larger bandwidth, S-SS/PSBCH block repetition in the frequency domain, and interlace based S-SS/PSBCH block, in order to satisfy the OCB and PSD requirements.

An illustration of above mentioned approaches is shown in Figure 3.

Figure 3 Illustration of enhancements to S-SS/PSBCH block to meet OCB and PSD requirements.

Proposal 3: To satisfy OCB and PSD requirements, it’s beneficial to support frequency domain enhancement to S-SS/PSBCH block, and study at least the following approaches:

- Multiplexing of S-SS/PSBCH block with other sidelink signal/channel;

- New S-SS/PSBCH block structure with a wider bandwidth;

- S-SS/PSBCH block repetition in the frequency domain;

- Interlace based S-SS/PSBCH block structure.

# Time Domain Enhancement

The enhancements in the time domain are mainly motivated from the energy detection based channel access procedure for unlicensed spectrum.

## Enhancement to reduce the number of channel access procedures

Based on the requirement in regulation, if the transmission is non-contiguous in time domain, and the time domain gap between the transmissions is larger than a threshold (e.g. 16 us), a channel access procedure including channel sensing is required to be performed within the gap; while if the transmission is contiguous (e.g. no gap symbol) or the time domain gap in non-contiguous transmission is no larger than the threshold, the transmitter is allowed to continue transmission without performing any sensing of the channel. Hence, to guarantee efficiently using the channel occupancy, it’s beneficial to maintain the transmission burst without or with small time domain gap.

However, for legacy sidelink, there are natural time domain gaps, such as reserved gap symbol at the end of each slot, and the gap symbol between PSSCH symbols and PSFCH symbols in a slot (as shown in Figure 4). The duration of the gap is at least one symbol, which corresponds to 66.7 us for 15 kHz SCS, 33.3 us for 30 kHz, and 16.7 us for 60 kHz. It can be observed that these values are much larger than the threshold of gap to be exempt from channel sensing, which implies channel access procedure with channel sensing is required in the gap if no enhancement is supported. This would be a significant overhead on the number of channel access procedures, and place SL-U at a disadvantage comparing to other coexisting RAT on the unlicensed spectrum.

Figure 4 Illustration of gap symbols in legacy sidelink.

For Rel-18 SL-U, one direction to resolve or mitigate the above issue is trying to eliminate or shorten the symbol gaps, such that the sidelink transmissions on unlicensed channels could be contiguous or non-contiguous but with short gap duration, respectively, such that channel access procedure with channel sensing can be exempted. To eliminate the gap, symbol repetition from the last transmission before the gap or from the first transmission after the gap can be investigated. To shorten the gap, symbol duration adjustment such as CP extension performed for the first transmission after the gap can be investigated. For both approaches, their applicable cases for sidelink transmission should be considered. For example, there are cases that the gap is reserved for switching between transmission and reception, such that a minimum value of the gap shall be guaranteed for the delay from switching. An illustration of the approaches is shown in Figure 5.

Figure 5 Illustration of approaches to eliminate or shorten the symbol gaps.

Moreover, another direction to resolve or mitigate the above issue is trying to support larger granularity for resource allocation in time domain, such that once a channel access procedure succeeds, a UE can perform a longer transmission burst without sensing within the transmission burst. For example, multi-slot based PSSCH and PSCCH transmission can be investigated, wherein no gap symbol is needed within the multi-slot span of PSSCH and PSCCH, and the number of channel access procedures can also be saved. An illustration of this direction is shown in Figure 6.

Figure 6 Illustration of multi-slot based PSSCH/PSCCH transmission.

Based on above discussion, we have the following proposal.

Proposal 4: It’s beneficial to support mechanisms to reduce the number of channel access procedures for SL-U, and RAN1 shall investigate at least the following in the study:

- Mechanism to eliminate or shorten the time domain gap in sidelink transmissions, e.g. symbol repetition and CP extension.

- Mechanism to support multi-slot based PSSCH/PSCCH transmission.

## Enhancement to allow immediate transmission after successful channel access procedure

In legacy sidelink, the starting symbol for PSSCH/PSCCH transmission is (pre-)configured, and can be controlled in a semi-static manner. However, the channel access procedure is dynamic, in the sense that the channel could be sensed to be idle at any time. Then, for SL-U, there exist scenarios that a UE is able to complete the channel access procedure (e.g. Type 1 channel access procedure) before the allowed starting symbol for PSSCH/PSCCH transmission, and has to perform self-defer (e.g. delaying backoff counter decrement) to wait for the next allowed starting location. There also exist scenarios that the UE completes the channel access procedure (e.g. Type 1 channel access procedure) after the allowed starting symbol for PSSCH/PSCCH transmission, and has to drop the transmission in the corresponding slot. Both scenarios can happen frequently for SL-U, and they remarkably decrease the efficiency on channel utilization. In particular, for unlicensed spectrum, it’s very likely that the channel could be potentially accessed by other nodes or other RAT during the delay of waiting for next available allowed starting location, since there may not be such restriction on the transmission starting location by other nodes or other RAT. Hence, the study needs to consider enhancement to allow immediate transmission after successful channel access procedure.

In a first approach considered to resolve this issue, multiple starting locations for PSSCH/PSCCH transmission within a slot can be investigated. The UE can start the sidelink transmission according to the next allowed starting locations within the slot, which can reduce the latency for waiting for the single starting location of the next slot as in the legacy sidelink. The tradeoff between the number of starting locations in a slot and the complexity in searching the PSSCH/PSCCH within the slot should be studied, since a large number of starting locations typically implies better flexibility in starting sidelink transmission in the slot, but also leads to higher complexity in searching the transmission at the receiver side.

In a second approach considered to resolve this issue, a wake-up-signal (WUS) can be investigated for SL-U. For example, the UE can immediately transmit the WUS after successfully performing the channel access procedure such that no delay is required in contrast to the legacy sidelink. It could be further investigated that the number of symbols for WUS can be fixed as one or flexible. Other than the benefit of occupying the channel immediately by using a WUS, we also want to note that the second approach can be potentially combined with the first approach to resolve the issue of high blind detection complexity at the receiver side, when multiple starting locations for PSSCH/PSCCH transmission within a slot are supported.

An illustration of the two approaches are shown in Figure 7.

Figure 7 Illustration of approaches to allow immediate transmission after successful channel access procedure.

Proposal 5: It’s beneficial to support mechanisms to allow immediate transmission after successful channel access procedure for SL-U, and RAN1 shall investigate at least the following in the study:

- Mechanism to support multiple starting locations for PSSCH/PSCCH transmission in a slot.

- Mechanism to support sidelink wake-up-signal that can be transmitted immediately after successful channel access procedure.

## Enhancement to compensate transmission dropping due to failed channel access procedure

For operation with shared spectrum channel access, a transmission could be dropped due to failed channel access procedure. For those transmissions with high importance, e.g. S-SS/PSBCH block and PSFCH, mechanisms shall be considered to compensate the loss, e.g. allowing extra transmission in a later transmission occasion and/or associating the transmission occasions in a flexible way.

For one example, in legacy sidleink, the transmission occasions for PSFCH can periodically show up in the time domain within a resource pool, with a (pre-)configured period. A PSSCH resource with respect to a sub-channel in the resource pool and a slot in the period is mapped to one RB in the set of available RBs for PSFCH transmission. In this sense, one PSSCH transmission is only associated with a single transmission occasion for PSFCH, and if the channel access procedure fails before such transmission occasion, the HARQ-ACK feedback associated with the PSSCH cannot be transmitted and will be dropped. To mitigate the impact from channel access failure, the study can investigate flexible PSFCH transmission occasion mapping to allow multiple transmission occasions associated with a PSSCH resource, and the UE has flexibility to choose one of the multiple transmission occasions for PSFCH transmission subject to a successful channel access procedure. For example, if a channel access procedure before a PSFCH transmission occasion fails, the UE can select another transmission occasion, e.g. in the next period or in the next channel occupancy, to transmit the HARQ-ACK feedback. Potential collision issue of the transmission occasions with this approach may need to be further studied. An illustration of this example is shown in Figure 8.

Figure 8 Illustration of allowing multiple and flexible transmission occasions for PSFCH.

For another example, in legacy sidleink, the transmission occasions for S-SS/PSBCH block occur within a period according to a (pre-)configured pattern, wherein the pattern is given by an offset, an interval, and a number of transmitted S-SS/PSBCH blocks. When the interval is not taking a value of 0, the burst of S-SS/PSBCH blocks are not contiguous in the time domain, and very likely it requires to perform channel access procedure before the transmission for each of the S-SS/PSBCH blocks within the burst. If any of the channel access procedures fails, the corresponding S-SS/PSBCH block transmission is dropped, and the number of actually transmitted S-SS/PSBCH blocks are smaller than the (pre-)configured number. To address this issue, similar approach of discovery burst transmission window from Rel-16 NR-U can be investigated, and a number of slots more than the number of actually transmitted S-SS/PSBCH blocks can be reserved in the period, such that even a S-SS/PSBCH block is not transmitted due to channel access failure, it still has extra transmission opportunities in the later slots of the same period. An illustration of this example is shown in Figure 9.

Figure 9 Illustration of allowing extra transmission occasions for S-SS/PSBCH block.

Proposal 6: It’s beneficial to support mechanisms to compensate transmission dropping due to failed channel access procedure for SL-U, and RAN1 shall investigate at least the following in the study:

- Mechanism to support flexible association for PSFCH transmission occasions, and allow extra transmission occasion for a HARQ-ACK feedback if channel access procedure fails.

- Mechanism to support S-SS/PSBCH block transmission window with extra transmission occasions to mitigate the impact of channel access procedure failures.

# Conclusion

The proposals made in this contribution are summarized below:

Proposal 1: The study shall include investigating the following sidelink signals and channels to operate on unlicensed spectrum, subject to the regulation requirements:

- PSSCH

- PSCCH

- PSFCH

- S-SS/PSBCH block

- sidelink reference signals

Proposal 2: To satisfy OCB and PSD requirements, it’s beneficial to support interlace based sub-channel and resource pool for sidelink on unlicensed spectrum.

- The structure of the interlace can use Rel-16 NR-U as a baseline.

- Further study whether enhancement for the structure of the interlace is needed (e.g. sub-RB interlace).

- Further study the resource allocation of PSSCH, PSCCH, PSFCH, and sidelink RS in the interlace based resource pool, and associated indication method and transmission/reception procedures.

Proposal 3: To satisfy OCB and PSD requirements, it’s beneficial to support frequency domain enhancement to S-SS/PSBCH block, and study at least the following approaches:

- Multiplexing of S-SS/PSBCH block with other sidelink signal/channel;

- New S-SS/PSBCH block structure with a wider bandwidth;

- S-SS/PSBCH block repetition in the frequency domain;

- Interlace based S-SS/PSBCH block structure.

Proposal 4: It’s beneficial to support mechanisms to reduce the number of channel access procedures for SL-U, and RAN1 shall investigate at least the following in the study:

- Mechanism to eliminate or shorten the time domain gap in sidelink transmissions, e.g. symbol repetition and CP extension.

- Mechanism to support multi-slot based PSSCH/PSCCH transmission.

Proposal 5: It’s beneficial to support mechanisms to allow immediate transmission after successful channel access procedure for SL-U, and RAN1 shall investigate at least the following in the study:

- Mechanism to support multiple starting locations for PSSCH/PSCCH transmission in a slot.

- Mechanism to support sidelink wake-up-signal that can be transmitted immediately after successful channel access procedure.

Proposal 6: It’s beneficial to support mechanisms to compensate transmission dropping due to failed channel access procedure for SL-U, and RAN1 shall investigate at least the following in the study:

- Mechanism to support flexible association for PSFCH transmission occasions, and allow extra transmission occasion for a HARQ-ACK feedback if channel access procedure fails.

- Mechanism to support S-SS/PSBCH block transmission window with extra transmission occasions to mitigate the impact of channel access procedure failures.

# Reference

[1] RP-213678, New WID on NR sidelink evolution, OPPO, LG Electronics