3GPP TSG RAN WG1 #109-e	R1-2203909

# e-Meeting, May 9th – 20th, 2022

Agenda item:	8.9.1.1

Source: 	Samsung

Title: 	On SL Positioning Scenarios and Requirements

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss about SL positioning scenarios and requirements for Rel-18 study on expanded and improved NR positioning and provide our view.

- Discussion

In the SID [1], the following objective is included regarding the SL positioning scenarios and requirements:

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1]

According to the objective, all SL coverage scenarios (i.e. IC, PC, and OoC) and the use cases of V2X, public safety, commercial, and IIOT are considered for SL positioning study. In the study phase, other scenarios can be considered if necessary. For instance, as in our companion contribution [2], a scenario where SL UEs are equipped with multi-panels can be considered and the corresponding positioning measurement can be studied. However, the technical solutions for SL positioning should be agnostic over scenarios as long as measurement source(s) for positioning is available and the requirements can be met.

Proposal 1: Any scenario can be considered for SL positioning. Strive to determine consistent technical solutions for SL positioning over scenarios as long as measurement source(s) for positioning is available and the requirements can be met.

Next, the important issue is then to decide the target performance requirements to be considered for evaluation of SL positioning. Depending on evaluation scenarios, the target performance requirements for SL positioning would be different. Specifically, the number of measurement sources for positioning and its quality will have a great impact on positioning accuracy. However, these can be different SL coverage scenarios. For example, in OoC, a UE cannot utilize positioning signals from base stations. Also, depending on SL environments, the number of available measurement sources for positioning can be limited and its quality can be deteriorating from both reference and measurement UE’s moving. Therefore, it would be desirable to have multiple sets of the requirement which can be applied depending on the SL positioning environments. The legacy requirements for commercial and IIoT in Rel-17 can be a starting point to have two sets of the target performance requirements, respectively with the adjustment for Rel-18 SL positioning evaluation. Alternatively, the multiple sets identified in TR38.845 [3] can be a starting point as the target performance requirements for Rel-18 SL positioning evaluation as:

- Set 1: 10 – 50m with 68 – 95% confidence level.

- Set 2: 1 – 3m with 95 – 99 % confidence level.

- Set 3: 0.1 – 0.5m with 95 – 99 % confidence level.

We can decide the number of supported multiple sets and detailed values with further discussion. However, the defined multiple sets can be applicable for absolute positioning, relative positioning, and ranging for Rel-18 SL positioning.

Proposal 2: Support multiple sets of the target performance requirements for Rel-18 SL positioning evaluation as:

- Set 1: [10 – 50] m with [68 – 95] % confidence level.

- Set 2: [1 – 3] m with [95 – 99] % confidence level.

- Set 3: [0.1 – 0.5] m with [95 – 99] % confidence level.

- FFS for the number of supported multiple sets and detailed values in the brackets

- It is noted that all the three sets are applicable for absolute positioning, relative positioning, and ranging.

- Conclusions

This contribution discusses about SL positioning scenarios and requirements. Based on the discussion, the following proposals are provided:

Proposal 1: Any scenario can be considered for SL positioning. Strive to determine consistent technical solutions for SL positioning over scenarios as long as measurement source(s) for positioning is available and the requirements can be met.

Proposal 2: Support multiple sets of the target performance requirements for Rel-18 SL positioning evaluation as:

- Set 1: [10 – 50] m with [68 – 95] % confidence level.

- Set 2: [1 – 3] m with [95 – 99] % confidence level.

- Set 3: [0.1 – 0.5] m with [95 – 99] % confidence level.

- FFS for the number of supported multiple sets and detailed values in the brackets

- It is noted that all the three sets are applicable for absolute positioning, relative positioning, and ranging.

References

RP-213588, “Study on expanded and improved NR positioning,” RAN#94-e, Dec., 2021.

R1-2203916, “Discussion on expended and improved NR positioning,” Samsung.

TR38.845, “Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases,” V17.0.0