3GPP TSG RAN WG1 #109-e	R1-2203911

# e-Meeting, May 9th – 20th, 2022

Agenda item:	8.9.1.2

Source: 	Samsung

Title: 	Discussion on Potential Solutions for SL Positioning

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss about SL positioning solutions for Rel-18 study on expanded and improved NR positioning and provide our view.

- Discussion

In the SID [1], the following objective is included regarding the potential solutions for SL positioning:

- Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required]

In the following subsections, we discuss about resource configuration/allocation for SL positioning, SL positioning methods, and SL positioning reference signals in order.

  - Resource configuration and allocation for SL positioning

In this section, we discuss about resource configuration and allocation for SL positioning. In order to enable SL positioning, SL resources for positioning should be configured and allocated to UE at first. According to the existing SL operation (Rel-16), a resource pool is defined as a set of time-frequency resources used for SL transmission and reception and this is (pre-)configured.. When SL resources are configured for positioning, we can consider the following approaches as

- Approach 1: SL positioning resources are configured in the same resource pool with SL data transmission.

- Approach 2: SL positioning resources are configured in separate resource pools with SL data transmission.

Approach 1 is good for resource utilization aspect by sharing the resource pool for SL data transmission and SL positioning. However, SL positioning accuracy can be degraded from the interference occurred by co-located data transmissions. In Approach 2, SL positioning resources can be protected from the interference of data transmission but it has inefficiency in the aspect of resource utilization. Therefore, the pros can cons should be studied carefully when we configure resources for SL positioning.

Proposal 1: Study the following approaches for SL positioning resource configuration as

- Approach 1: SL positioning resources are configured in the same resource pool with SL data transmission.

- Approach 2: SL positioning resources are configured in separate resource pools with SL data transmission.

After the resource pool being configured, resource(s) used for actual transmission of SL positioning should be allocated within the pool. In Rel-16 SL, two kinds of resource allocation modes are supported for SL data transmission. The first one is Mode 1 where network allocates SL resources. This mode is especially for a case when a UE is in network coverage. In Mode 1, TX-UE can request SL resources to BS, e.g. via scheduling request (SR) and buffer status reporting (BSR) procedure. The BS then can allocate SL resource to TX-UE by configured grant or dynamic grant. The second one is Mode 2 where a UE performs resource allocation by itself based on sensing and resource selection mechanisms. Unlike Mode 1, the Mode 2 is available regardless of a UE is within network coverage or it is out-of-network coverage. We think that similar approach as Mode 1 and Mode 2 can be considered for SL positioning. Firstly, we need to study a network based resource allocation for SL positioning (similar to Mode 1). Specifically, network (i.e. gNB) can allocate resources for SL positioning reference signals. Similarly, network can allocate resources for UE to report SL measurement on positioning to BS.

Proposal 2: Study feasibility of network allocated resources for SL positioning including:

- Resources for SL positioning reference signals

- Resources for reporting SL positioning measurements.

Secondly, we need to study a UE based resource allocation for SL positioning (similar to Mode 2). Specifically, UE can allocate resources for SL positioning reference signals based on its own sensing and resource selection mechanisms. In a different way, another UE can allocate resources for SL positioning reference signals. This is a similar concept as inter-UE coordination introduced in Rel-17 SL.

Proposal 3: Study feasibility of UE allocated resources for SL positioning including:

- Sensing based resource allocation

- Resource allocation by another UE

  - SL positioning methods

According to the SID, we need to study positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements). In the study phase, all positioning methods can be investigated. For instance, carrier phase measurements can be studied not only for Uu but also for SL. However, it is important to note that SL environment is not the same as Uu and this should be considered when we apply potential positioning methods. Specifically, time offset between transmission points (TPs) who transmit positioning reference signals becomes a major problem for TDOA measurements. In SL, it would be difficult to assume a perfect synchronization between TPs because TP can be a UE and can move possibly with high speed. Therefore, the feasibility of TDOA for absolute and relative SL positioning should be studied considering synchronization error between UEs.

Proposal 4: Study the feasibility of TDOA for absolute and relative SL positioning considering synchronization error between UEs.

Moreover, depending on SL environments, the number of available measurement sources for positioning can be limited especially for OoC scenario and its quality can be deteriorating from both TP and measurement UEs’ moving. The evaluation results in our companion contribution [2] shows that measurement UE’s selection of measurement sources (i.e. TPs) is important for achieving high positioning accuracy. Similarly, SL UE may need to decide whether to perform absolute positioning or relative positioning or ranging depending on availability and quality of measurement source(s). It would be important to study this aspect for enabling SL positioning.

Proposal 5: Study a procedure for SL UE to decide whether to perform absolute positioning or relative positioning or ranging depending on availability and quality of measurement source(s).

  - SL positioning reference signals

In this section, we discuss about SL positioning reference signal (S-PRS) design. The positioning reference signal would be a key factor for providing positioning accuracy. Therefore, the target performance requirements for SL positioning should be defined at first [3]. According to the requirements to be considered for SL positioning, we can design S-PRS. Table 1 includes the candidate signals (i.e. DL PRS, Positioning SRS, and SL synchronization signal) which can be considered for S-PRS design and provide some details of signal patterns in frequency and time domain and applied sequence.

Table 1. Candidate signals considered for S-PRS design.

|  | DL PRS (Rel-16) | Positioning SRS (Rel-16) | SL synchronization signal (Rel-16) |
| --- | --- | --- | --- |
| Frequency | Max 273 PRBs (30KHz) Comb-2/4/6/12 patterns | Max 273 PRBs (30KHz) Comb-2/4/8 patterns | 127 subcarrier (S-PSS, S-SSS) 132 subcarrier (PSBCH, 1/4 DMRS REs) |
| Time | 2/4/6/12 symbols | 1/2/4/8/12 symbols | 2 symbols (S-PSS, S-SSS) 9 symbols (PSBCH, NCP) |
| Sequence | Gold sequence | ZC sequence | M-sequence (S-PSS) Gold sequence (S-SSS) |

As shown in Table 1, a big difference between DL PRS and positioning SRS can be the applied sequence. On the other hand, a big difference between Uu positioning reference signals (i.e. DL PRS, Positioning SRS) and SL synchronization signal would be available signal transmission bandwidth and symbol length which impacts on positioning accuracy. Uu positioning reference signals can provide flexible frequency and time domain patterns as shown in Table 1 and this would be good for different target performance requirements (i.e. for tight and relaxed positioning requirements). SL synchronization signal has some limitation for frequency and time domain patterns. However, it has an advantage to reuse the existing SL signal, e.g. SL synchronization signals. Therefore, the candidate signals in Table 1 can be a good starting point for S-PRS design.

Proposal 6: Study and evaluate performance of SL positioning reference signals considering the following

- Reusing SL synchronization signal in Rel-16

- Reusing DL PRS or positioning SRS design in Rel-16

Even though the existing candidate signals in Table 1 can be reused much as possible for S-PRS design, there would be potential issues when we apply this for SL positioning. Specifically, we need to study time and frequency domain behaviour for S-PRS in more details. In time domain, the supported S-PRS symbol length should be studied considering SL slot structure. Also, periodicity for S-PRS transmission should be investigated. We can study aperiodic S-PRS transmissions in a single slot or multiple slots. Also, semi-persistent or periodic S-PRS transmission needs to be considered.

Proposal 7: Study time domain behaviour for SL positioning reference signal as

- The number of supported symbols for SL positioning reference signal

- Aperiodic (one shot or N shots), semi-persistent or periodic

In frequency domain, the supported S-PRS bandwidth should be studied considering newly introduced band in n79 for SL [4] where the maximum transmission bandwidth of 100 MHz is available. In order to achieve high positioning accuracy, large S-PRS transmission bandwidth is very important and we need to study the number of supported sub-channels for S-PRS.

Proposal 8: Study frequency domain behaviour for SL positioning reference signal as

- The number of supported sub-channels for SL positioning reference signal

Conclusions

This contribution discusses about solutions for SL positioning. Based on the discussion, the following proposals are provided:

Proposal 1: Study the following approaches for SL positioning resource configuration as

- Approach 1: SL positioning resources are configured in the same resource pool with SL data transmission.

- Approach 2: SL positioning resources are configured in separate resource pools with SL data transmission.

Proposal 2: Study feasibility of network allocated resources for SL positioning including:

- Resources for SL positioning reference signals

- Resources for reporting SL positioning measurements.

Proposal 3: Study feasibility of UE allocated resources for SL positioning including:

- Sensing based resource allocation

- Resource allocation by another UE

Proposal 4: Study the feasibility of TDOA for absolute and relative SL positioning considering synchronization error between UEs.

Proposal 5: Study a procedure for SL UE to decide whether to perform absolute positioning or relative positioning or ranging depending on availability and quality of measurement source(s).

Proposal 6: Study and evaluate performance of SL positioning reference signals considering the following

- Reusing SL synchronization signal in Rel-16

- Reusing DL PRS or positioning SRS design in Rel-16

Proposal 7: Study time domain behaviour for SL positioning reference signal as

- The number of supported symbols for SL positioning reference signal

- Aperiodic (one shot or N shots), semi-persistent or periodic

Proposal 8: Study frequency domain behaviour for SL positioning reference signal as

- The number of supported sub-channels for SL positioning reference signal

References

RP-213588, “Study on expanded and improved NR positioning,” RAN#94-e, Dec., 2021.

R1-2203910, “Discussion on Evaluation for SL Positioning,” Samsung.

R1-2203909, “On SL Positioning Scenarios and Requirements,” Samsung.

TS 38.101-1, “NR; UE radio transmission and reception; Part 1: Range 1 Standalone,” V17.5.0.