3GPP TSG RAN WG1 #109-e	R1-2203914

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.5.2.3

Source:	Samsung

Title:	Discussion on LPHAP

Document for:	Discussion and Decision

# Introduction

In the SID [1], the following objective is included regarding the support of low power higher accuracy positioning (LPHAP):

- Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1]

  - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study.

  - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state

This contribution discusses aspects related to the support of LPHAP.

# Background

For industrial IoT devices, power efficiency is one of the most essential and critical considerations, especially for high accuracy positioning. On one hand, the development scenarios for industrial IoT devices may require high accuracy positioning, e.g. to avoid various utility sectors not proper for such devices; on the other hand, to achieve high accuracy positioning, industrial IoT devices may have to consume high power, e.g. frequency positioning fix to obtain real-time location information. Recharging these massive IoT devices would be a great burden for the industry operator, hence, low energy consumption per position fix is required for industrial IoT devices in order to sustain a long lifetime without changing the battery.

For this purpose, SA approved WI on Low Power High Accuracy Positioning (LPHAP) for industrial IoT scenarios [2], and specified requirements on low power high accuracy positioning service for industrial IoT scenarios, in particular for the following aspects:

- To add the requirements for low power high accuracy positioning for industrial IoT device into TS 22.104.

- To update the requirements related to low power high accuracy positioning in TS 22.261.

- To add informative texts for low power high accuracy positioning as Annex into TS 22.104.

RAN also approved Rel-18 SI [1] to study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements, and study potential enhancements to help address any limitations if needed.

# Clarification of Study Scope

TS 22.104 [3] has specified 8 use cases for LPHAP (quoted below) and the definitions of the corresponding service level are also quoted from TS 22.261 [4]. The targeted use case from SID [1] is use case 6, which includes both outdoor and indoor scenarios with the support of high UE movement velocity, and requires relatively high horizontal accuracy requirement (e.g. 1 m) and vertical accuracy (e.g. 2 m). The corresponding requirement on the positioning service availability is 99%, the positioning service latency is 1 s, the positioning interval is 15 to 30 s, and the targeted battery life time is 6 to 12 months. We believe use case 6 has covered a wide range of typical application scenarios industrial IoT devices, and if only single use case is determined for the study, use case 6 could be the best candidate.

Proposal 1: From RAN1 perspective, use case 6 as defined in TS 22.104 is sufficient for the study, and RAN1 can ask for RAN2’s further confirmation.

Table A.7.2-1: Low power high accuracy positioning use cases

| Use Case # | Horizontal accuracy | Corresponding service level (22.261) | Positioning interval/ duty cycle | battery life time/ minimum operation time |
| --- | --- | --- | --- | --- |
| 1 | 10 m | Service Level 1 | on request | 24 months |
| 2 | 2 m to 3 m | Service Level 2 | < 4 seconds | > 6 months |
| 3 | < 1 m | Service Level 3 | no indication | 1 work shift - 8 hours (up to 3 days, 1 month for inventory purposes) |
| 4 | < 1 m | Service Level 3 | 1 second | 6 - 8 years |
| 5 | < 1 m | Service Level 3 | 5 seconds - 15 minutes | 18 months |
| 6 | < 1 m | Service Level 3 | 15 s to 30 s | 6 - 12 months |
| 7 | 30 cm | Service Level 5 | 250 ms | 18 months |
| 8 | 30 cm | Service Level 5 | 1 second | 6 - 8 years (no strong limitation in battery size) |
| 9 | 10 m | Service Level 1 | 20 minutes | 12 years (@20mJ/position fix) |

Table 7.3.2.2-1 Performance requirements for Horizontal and Vertical positioning service levels

| Positioning service level | Absolute(A) or Relative(R) positioning | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Coverage, environment of use and UE velocity |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Horizontal Accuracy | Vertical Accuracy (note 1) |  |  | 5G positioning service area | 5G enhanced positioning service area (note 2) |  |
|  |  |  |  |  |  |  | Outdoor and tunnels | Indoor |
| 1 | A | 10 m | 3 m | 95 % | 1 s | Indoor - up to 30 km/h Outdoor (rural and urban) up to 250 km/h | NA | Indoor - up to 30 km/h |
| 2 | A | 3 m | 3 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 3 | A | 1 m | 2 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 4 | A | 1 m | 2 m | 99,9 % | 15 ms | NA | NA | Indoor - up to 30 km/h |
| 5 | A | 0,3 m | 2 m | 99 % | 1 s | Outdoor (rural) up to 250 km/h | Outdoor (dense urban) up to 60 km/h Along roads and along railways up to 250 km/h | Indoor - up to 30 km/h |

| 6 | A | 0,3 m | 2 m | 99,9 % | 10 ms | NA | Outdoor (dense urban) up to 60 km/h | Indoor - up to 30 km/h |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | R | 0,2 m | 0,2 m | 99 % | 1 s | Indoor and outdoor (rural, urban, dense urban) up to 30 km/h Relative positioning is between two UEs within 10 m of each other or between one UE and 5G positioning nodes within 10 m of each other (note 3) |  |  |
| NOTE 1: The objective for the vertical positioning requirement is to determine the floor for indoor use cases and to distinguish between superposed tracks for road and rail use cases (e.g. bridges). NOTE 2: Indoor includes location inside buildings such as offices, hospital, industrial buildings. NOTE 3: 5G positioning nodes are infrastructure equipment deployed in the service area to enhance positioning capabilities (e.g. beacons deployed on the perimeter of a rendezvous area or on the side of a warehouse). |  |  |  |  |  |  |  |  |

The study also intends to focus on RRC_INACTIVE and/or RRC_IDLE state, as described in the SID [1], to further alleviate the study load. However, RAN1 may need RAN2 to clarify whether positioning for RRC_IDLE state is within the scope of Rel-18 study since there was no support of positioning on RRC_IDLE state in Rel-16 and Rel-17. After the clarification of study scope from RAN2, RAN1 can further study whether enhancement is needed for RRC_INACTIVE and/or RRC_IDLE state to support LPHAP.

Proposal 2: RAN1 shall wait for RAN2’s clarification on the scope of the study. Especially, one of the following options shall be clarified:

- Option 1: The study investigates potential enhancement to positioning in RRC_INATIVE state to support LPHAP.

- Option 2: The study investigates supporting of positioning in RRC_IDLE state and potential enhancement to support LPHAP.

- Option 3: Option 1 + Option 2.

# Evaluation Methodology

In Rel-16 and Rel-17, RAN1 has already determined evaluation methodology for positioning accuracy, using either system level simulation or link level simulation. However, RAN1 has not studied the evaluation methodology for power consumption aspect of positioning. In order to evaluate whether existing RAN functionality can support these power consumption and positioning requirements in LPHAP, an evaluation methodology for the power consumption is needed, including the model to determine power consumption in positioning and key metric to evaluate the power consumption.

Proposal 3: RAN1 shall study the evaluation methodology for the power consumption in positioning, including the model to determine power consumption in positioning and key metric to evaluate the power consumption.

# Conclusion

The proposals made in this contribution are summarized below:

Proposal 1: From RAN1 perspective, use case 6 as defined in TS 22.104 is sufficient for the study, and RAN1 can ask for RAN2’s further confirmation.

Proposal 2: RAN1 shall wait for RAN2’s clarification on the scope of the study. Especially, one of the following options shall be clarified:

- Option 1: The study investigates potential enhancement to positioning in RRC_INATIVE state to support LPHAP.

- Option 2: The study investigates supporting of positioning in RRC_IDLE state and potential enhancement to support LPHAP.

- Option 3: Option 1 + Option 2.

Proposal 3: RAN1 shall study the evaluation methodology for the power consumption in positioning, including the model to determine power consumption in positioning and key metric to evaluate the power consumption.

# Reference

[1] RP-213588, Revised SID on Study on expanded and improved NR positioning, Intel

[2] SP-210216, New WID on Low Power High Accuracy Positioning for industrial IoT scenarios, SA WG1

[3] 3GPP TS 22.104, Service requirements for cyber-physical control applications in vertical domains

[4] 3GPP TS 22.261, Service requirements for the 5G system