3GPP TSG RAN WG1 #109-e	R1-2203915

e-Meeting, May 9th– 20th , 2022

Agenda item:	9.5.2.3

Source:	Samsung

Title:	Discussion on Positioning for RedCap UEs

Document for:	Discussion and Decision

# Introduction

In the SID [1], the following objective is included regarding the support of positioning for RedCap UEs

- Positioning support for RedCap UEs, considering the following:

  - Evaluate positioning performance of existing positioning procedures and measurements with RedCap UEs[RAN1]

  - Based on the evaluation, assess the necessity of enhancements and, if needed, identify enhancements to help address limitations associated with for RedCap UEs [RAN1, RAN2]

This contribution discusses aspects related to the support of positioning for RedCap UEs.

# Positioning accuracy requirement

Release-17 has specified support for RedCap UEs with reduced bandwidth support (i.e., 20MHz/100MHz RF BW for FR 1/FR2) and reduced complexity including reduced number of receive chains (i.e., reduced number of antennas at UE). Such UEs could support NR positioning functionality but there could be some positioning performance variation due to the change of UE capability. As indicated by [1], the performance requirements have not been specified for the positioning related measurements performed by RedCap UEs, and no evaluation was performed to see how the reduced capabilities of RedCap UEs might impact eventual position accuracy. Thus one important work for this study item is to check whether there is a problem to support Redcap UE to do RAT-dependent positioning with current specification.

Observation 1: the positioning accuracy (or latency) requirements are not clear.

Proposal 1: RAN1 is to study the requirements on positioning accuracy for RedCap UEs.

Following the logic that the RedCap UE is defined as not a high-end device, its required positioning accuracy shall not be as high as the that for IIoT use cases. On the other hand, the requirement defined for normal commercial use cases in Rel16 could be used as starting point as following.

-	Horizontal positioning error < 3m for 80% of UEs in indoor deployment scenarios

-	Vertical positioning error < 3m for 80% of UEs in indoor deployment scenarios

-	Horizontal positioning error < 10m for 80% of UEs in outdoor deployments scenarios

-	Vertical positioning error < 3m for 80% of UEs in outdoor deployment scenarios

Proposal 2: using positioning accuracy requirement for commercial use cases defined in R16 as starting point.

# Impact of reduced capability

As discussed above, two type of reduced capability have been introduced for RedCap UE:

- Reduced minimum RF bandwidth (100M 20M for FR1, 400M 100M for FR2);

- Reduced antenna (21)

Thus, from theoretical analysis, several potential impacts may be expected:

- Worse sequence detection performance. Reduced BW will lead shorter sequence applied for PRS or SRS, with Gold sequence for PRS or ZC-sequence for SRS used, the self-correlation will be degraded.

- Worse obtained RSRP. The reduced antenna will lead 3dB loss of the received power.

- Worse timing granularity. Reduced BW will lead smaller sampling rate compared to normal NR UE, then the time domain granularity will be doubled considering FFT size  reduced from 4096 to 2048;

- Higher power gain in UL based positioning. Reduced BW will lead to higher power per RE given the fixed power at UE side, this could benefit for UL reception at the gNB side.

A simulation on PRS sequence detection is used to check the actual impacts by reducing the BW. The results as shown in following figure, DL-TDOA is used and the ToA is estimated based on the peak found by cross/auto-correlation. A few observations as shown below:

- Reduced FFT size (corresponding to sampling rate) has obvious performance loss at 90% accuracy;

- Shortened sequence has not much difference at relative high SNR (e.g., at 0dB) case given same sampling rate (i.e., same FFS size)

- Shortened sequence has obvious loss at low SNR (e.g., at -20dB) case given same sampling rate (i.e., same FFS size) due to the reduced sequence length.

Observation 2: the reduced FFT size (in translated to reduced sampling rate) could obviously reduce the performance.

Observation 3: the shortened sequence length could obviously reduce performance at low SNR region.

Proposal 3: RAN1 to study the impacts to measurements caused by reduced BW and antenna number.

# Conclusion

The proposals made in this contribution are summarized below:

Observation 1: the positioning accuracy (or latency) requirements are not clear.

Proposal 1: RAN1 is to study the requirements on positioning accuracy for RedCap UEs.

Proposal 2: using positioning accuracy requirement for commercial use cases defined in R16 as starting point.

Observation 2: the reduced FFT size (in translated to reduced sampling rate) could obviously reduce the performance.

Observation 3: the reduced sequence length could obviously reduce performance at low SNR region.

Proposal 3: RAN1 to study the impacts to measurements caused by reduced BW and antenna number.

# Reference

[1] RP-213588, Revised SID on Study on expanded and improved NR positioning, Intel