3GPP TSG RAN WG1 #109-e	        	            		R1-2203919

e-Meeting, May 9th – 20th, 2022

Agenda item:		9.7.1

Source:			Samsung

Title:			NW Energy Savings Performance Evaluation

Document for:		Discussion and decision

- Introduction

In SID [1], the following objectives are specified regarding evaluation for NW energy savings:

- Definition of a base station energy consumption model [RAN1] - Adapt the framework of the power consumption modelling and evaluation methodology of TR38.840 to the base station side, including relative energy consumption for DL and UL (considering factors like PA efficiency, number of TxRU, base station load, etc), sleep states and the associated transition times, and one or more reference parameters/configurations. - Definition of an evaluation methodology and KPIs [RAN1] - The evaluation methodology should target for evaluating system-level network energy consumption and energy savings gains, as well as assessing/balancing impact to network and user performance (e.g. spectral efficiency, capacity, UPT, latency, handover performance, call drop rate, initial access performance, SLA assurance related KPIs), energy efficiency, and UE power consumption, complexity. The evaluation methodology should not focus on a single KPI, and should reuse existing KPIs whenever applicable; where existing KPIs are found to be insufficient new KPIs may be developed as needed. Note: WGs will decide KPIs to evaluate and how.

This contribution discusses aspects related to power consumption modelling and evaluation methodology for evaluating NW energy saving techniques.

- Power Consumption Modelling

As guidance in SID [1], the framework of power consumption modelling in [2] can be adapted for power consumption modelling to the BS side. Power consumption modelling for NW energy saving evaluation should provide BS power consumption model and corresponding scaling model for adaptation in different resource domains. To reuse the same principle from UE power saving evaluation, a BS power consumption model can be established first for a reference configuration, and power consumption scaling model can be further determined with respect to different adaptation aspects in different resource domains.

  - BS power consumption model

To establish a BS power consumption model, we need to determine average power consumption for a variety of BS power states (sleep or non-sleep). In practice, the power consumption amount varies with respect to occupied physical layer resources in different domains, such as carrier BW, and number of Tx/Rx antennas. Therefore, it’s necessary to provide a reference configuration for the BS power consumption model to be established. Table 1 provides our views on some key configuration parameters for both FR1 and FR2.

Observation 1: Average power consumption for a variety of power states shall be determined for a given BS reference configuration.

Table 1: BS Reference configuration

| Configuration parameter |  | Value |  |
| --- | --- | --- | --- |
|  |  | FR1 | FR2 |
| DL: TDD | Subcarrier spacing (SCS) | 30KHz | 120KHz |
|  | Number of carriers | [8] | [16] |
|  | Carrier BW | 100MHz | 400MHz |
|  | # of Tx antenna ports | 4 | 2 |
|  | Power level | [44] dBm | FFS |
| UL: TDD | Subcarrier spacing (SCS) | 30KHz | 120KHz |
|  | Number of carriers | [8] | [16] |
|  | Carrier BW | 100MHz | 400MHz |
|  | # of Rx antenna ports | 1 | 1 |
| Others | # of TxRU | [2] | [2] |

For the BS power consumption model, gNB can operate in a variety of power states. For the benefit of simplicity, it’s better to purse a smaller number of power saving states on gNB side. There is no need to determine power consumption for all L1 channels. For example, PDCCH-only transmission may rarely happen on gNB side. In our view, the power states on BS can be divided into three categories, including i) sleep states, ii) traffic processing, and iii) background activities.

For sleep states, three sleep types can be considered, wherein a sleep type can be associated with an energy saving (ES) mode. For the ES mode associated with deep sleep as described in our companion contribution [3], gNB doesn’t expect to perform any transmission or reception, including RS, SSB transmission. gNB may switch off RF completely in the ES mode associated with deep sleep. Light sleep can be associated with another ES mode, wherein no traffic processing is performed, but all RS, SSB, BCH will remain active or can be reconfigured with thin transmission. gNB may switch off most of the digital signal processing modules in the ES mode associated with light sleep. For both deep sleep state and light sleep state, the sleep duration should be larger than the total transition time entering and leaving the sleep state. Also, there will be additional transition energy for switching on/off corresponding RF components and digital signal processing modules. Lastly, a micro sleep state can be associated with a dormant mode of gNB, wherein gNB is idle for a short time duration. Immediate transition can be assumed for power saving study purpose from or to a non-sleep state. gNB may not be able to turn off any RF components or digital processing modules within a short idle time duration for micro sleep.

Observation 2: Multiple sleep types can be considered, wherein each sleep state is associated with an energy saving mode.

For traffic processing states, a unified power state can be considered for DL and UL traffic processing. Since both PDCCH and PDSCH/PUSCH are message based and includes channel encoding/decoding, there is no need to distinguish the case when a slot is for PDSCH/PUSCH only or for PDCCH + PDSCH/PUSCH. The average power for DL traffic processing can be larger than UL traffic processing, because TX involves PA operation which accounts for larger portion of power consumption.

Observation 3: A unified power state is sufficient to model power consumption for traffic processing in DL and UL, respectively.

For background activities, several power states for link quality or mobility maintenance can be considered. For DL background activities, periodic SSB and CSI-RS resources transmission should be considered. The average power for a slot of SSB can be same as DL slot for traffic as SSB includes both PSS/SSS and PBCH. The average power for a slot of CSI-RS resources should be lower than DL traffic slot, as the energy consumption for RS resources processing is lower than L1 channel processing based on channel encoding. For UL background activities, periodic CSI report, SR, and HARQ based on PUCCH reception and periodic SRS reception can be considered. For the benefit of simplicity, a long PUCCH can be assumed and corresponding average power can be same as UL traffic. The average power for SRS should be smaller than UL traffic as SRS can be detected based on correlations without channel decoding.

Given above, the average power for a variety power states are provided in Table 1, wherein the average is normalized relatively to the average power for deep sleep.

Proposal 1: Support BS power consumption model with power states in Table 2, given reference configuration in Table 1.

Table 2: BS power consumption model

| Power State |  | Characteristics | Relative Power per slot |  |
| --- | --- | --- | --- | --- |
|  |  |  | FR1 | FR2 |
| Sleep | Deep Sleep | - Time interval for the sleep should be larger than the total transition time entering and leaving this state - Cost additional transition energy to switch between associated energy saving mode (with no any transmission or reception, including RS, SSB transmission) and normal/active mode. | [1] | FFS |
|  | Light Sleep | - Time interval for the sleep should be larger than the total transition time entering and leaving this state - Cost additional transition energy to switch between associated energy saving mode (all RS, SSB, BCH will remain active or can be reconfigured with thin transmission) and normal/active mode. | [20] | FFS |
|  | Micro sleep | - Immediate transition is assumed for power saving study purpose from or to a non-sleep state | [45] | FFS |
| Traffic | DL | For any of: - PDSCH, or - PDCCH + PDSCH | FFS | FFS |
|  | UL | For any of: - PUSCH, or - PDCCH + PUSCH | FFS | FFS |
| Background activities (for link quality or mobility maintenance) | SSB | Two SSBs per slot | FFS | FFS |
|  | CSI-RS | 14 symbols per slot with TDMed CSI-RS resources | FFS | FFS |
|  | CSI report, SR, HARQ | 14 symbols of PUCCH per slot | FFS | FFS |
|  | SRS. | 14 symbols per slot with TDMed SRS resources | FFS | FFS |

For deep sleep and light sleep, there will be additional transition cost for switching between corresponding ES mode to normal mode.  As illustrated in Figure 1, the additional transition cost needs to take into consideration of additional transition energy and total transition time regarding ramping down and ramping up transitions. To determine the value of additional transition energy and transition time, the value from UE power consumption model in [2] can be reused or to be a starting point.

Figure 1: Illustration of BS power consumption at state transition

Proposal 2: Support transition time, Ttransition, and additional transition energy, Etransition, for switching between deep/light sleep state and non-sleep state

- Ttransition,, = [20]ms, Etransition, = [450], for deep sleep

- Ttransition,, = [6]ms, Etransition, = [100], for light sleep

  - BS power consumption scaling for adaptation

Another design aspect for BS power consumption modeling is scaling of average power when considering adaptation of resources in different domains.

For the adaptation in frequncy domain, scaling of averge power in terms of carrier BW and number of carriers can be considered. Similar as adaptation on BWP BW in [2], the scaling for adaptation on carrier BW, X MHz,  can be modeled as linear interpolation for intermediate bandwidths, such that scaling of X MHz = 0.4 + 0.6 * (X - 20) / 80. The valid carrier BW can be X = 10, 20, 40, 80, and 100 for FR1. For adaptation on number of carriers, a scaling factor can be predetermined for each applicable number of CCs. For example, 2CC is 1.7 *1CC, 4CC is 3.4*1CC.

For the adaptation in spatial domain, scaling of average power in terms of number of DL/UL antenna ports, and number of TxRU can be considered. To reuse the principle for UE power consumption scaling, a scaling factor can be predetermined for each applicable number of TX/RX antenna ports or number of TxRU. For example, 2TX power is 0.7x 4Tx  power, 1RX power is 0.7x 2Rx  power, and 1TxRU power is 0.7x 2TxRU power.

For the adaptation in time domain, scaling of average power in terms of number of symbols occupied per slot can be considered. The power for relaveant power states, such as CSI-RS or SRS resources, can be determined based on linear interpolation function with respect to number of occupied symbols per slot.

For the adaptation in Tx power domain, a linear interpolation function can be determined to determine power for a TX power within a range of [44] dBm to [x] dBm.

Given above, the scaling of average power for adaptation can be summaried in table 3.

Proposal 3: Support scaling of average power for adaptation based on BS power consumption scaling model in Table 3.

Table 3: BS power consumption scaling for adaptation

| Scaling |  | Proposal |
| --- | --- | --- |
| Frequency domain | Carrier Bandwidth, X | Scaling of X MHz = 0.4 + 0.6 * (X - 20) / 80. Valid only for X = 10, 20, 40, 80, and 100. |
|  | # of CCs | 2CC is 1.7 *1CC, 4CC is 3.4*1CC. |
| Spatial domain | Antenna scaling (DL) | 2TX power is 0.7x 4Tx power |
|  | Antenna scaling (UL) | 1RX power is 0.7x 2Rx power |
|  | # of TxRU | 1TxRU power is 0.7x 2TxRU power |
| Time domain | SSB | FFS for scaling w.r.t. # of SSBs [per slot] |
|  | CSI-RS | FFS for scaling w.r.t. # of symbols for CSI-RS |
|  | Short PUCCH | FFS for scaling w.r.t. # of symbols for PUCCH |
|  | SRS | FFS for scaling w.r.t. # of symbols for SRS |
| Power domain | TX power | FFS for scaling for reduced TX power from [44]dBm to X dBm |

- Evaluation methodology

  - KPIs

In SID [1], a variety of possible KPIs are provided, including system-level network energy consumption and energy savings gains, as well as assessing/balancing impact to network and user performance (e.g. spectral efficiency, capacity, UPT, latency, handover performance, call drop rate, initial access performance, SLA assurance related KPIs), energy efficiency, and UE power consumption, complexity. Among all the possible KPIs, we think energy saving gain (ESG) should be prioritized for evaluating the performance for an energy saving technique. The ESG can be defined as

ESG = (B-A)/B×100%,

where A is the energy consumption of gNB corresponding to an energy saving scheme, and B is the energy consumption of gNB corresponding to the baseline. The baseline can be defined as always-on case, wherein gNB is always available for scheduling and/or traffic processing.

In addition to ESG, latency loss (LL) on UE side should also be considered, such that

LL = Lenhancement - LbaselineLbaseline×100%, and L = tscheduled - tarrival.

where the latency, L, is calculated as the delay between the time when a packet arrivals and the time when the packet is scheduled. Meanwhile, throughput, UPT, can also be considered to reflect the impact on system performance.

When a new signal/channel is introduced, for example for triggering any adaptation. Evaluation metrics, including false alarm rate (FAR), miss-detection rate (MDR) should be considered to evaluate the reliability of the new signal/channel. Additionally, resource overhead or coexistence with legacy signal/channel should be further evaluated for the new signal/channel.

Proposal 4: Support the following evaluation metrics for evaluating a NW energy saving technique:

- Energy saving gain

- Latency loss

- UPT

- FAR, MDR, resources overhead, and coexistence with legacy signal/channel, if a new signal/channel is introduced

  - Traffic model

In TR 38.840, three types of traffic models, such as FTP, instant messaging, and VoIP application, are prioritized. The same traffic models can be reused for evaluating NW energy savings. One possible adaptation or modification is no need of C-DRX configuration. C-DRX is a legacy UE power saving mechanism, which is not necessary to be considered as baseline for evaluating NW energy consumption. Therefore, the following traffic models can be considered for evaluating NW energy saving techniques.

Proposal 5:  Reuse the traffic model for FTP traffic, instant messaging and VoIP from TS 38.840 except for DRX configuration.

Table 4: Traffic model

|  | FTP traffic | Instant messaging | VoIP |
| --- | --- | --- | --- |
| Model | FTP model 3 | FTP model 3 | As defined in R1-070674. Assume max two packets bundled. |
| Packet size | 0.5 Mbytes | 0.1 Mbytes |  |
| Mean inter-arrival time | 200 ms | 2 sec |  |

  - Simulation assumption

In this subsection, simulation assumptions regarding i) deployment scenarios, ii) link level simulation assumptions, and iii) system level simulation assumptions are provided.

According to SID [1], the following deployment scenarios are possible for simulation of NW energy saving techniques.

- a) Urban micro in FR1, including TDD massive MIMO (note: this scenario can also model small cells)

- b) FR2 beam-based scenarios (note: this scenario can also model small cells)

- c) Urban/Rural macro in FR1 with/without DSS (no impact to LTE expected in case of DSS)

- d) EN-DC/NR-DC macro with FDD PCell and TDD/Massive MIMO on higher FR1/FR2 frequency

The SID specifically says the study of NW energy savings should prioritize empty/idle or low/medium load scenario. In an attempt to reasonably manage the simulation efforts, b) and c) would be good enough to start for the simulation campaign.

Proposal 6: Prioritize the following deployment scenarios for evaluating NW energy saving techniques

- Urban/Rural macro in FR1 with/without DSS (no impact to LTE expected in case of DSS)

- FR2 beam-based scenarios (note: this scenario can also model small cells)

For link level simulation that may be used for evaluating any new signal/channel introduced, the following assumption in TR 38.840 can be reused.

-	Simulation assumptions as specified in Table A1.5-1 in TR38.802 [4] should be the basis for link-level simulation evaluation.

-	Antenna configuration may use IMT-2020 as reference.  The evaluation results should also include the antenna configuration if it is different from the reference antenna configuration,

-	For LLS simulation on power saving signal detection performance, at least the following SINR values should be evaluated: -6dB, 3dB, 20dB.

Proposal 7: Reuse the link level simulation assumption in TR 38.840.

For system level simulation on BS energy consumption evaluation, the following assumption in TR 38.840 can be reused.

-	Simulation assumptions as specified in Table A2.1-1 in TR38.802 [4] should be the basis for system-level simulation evaluation.

-	It is clarified that user throughput measured from system level simulation does not preclude user perceived throughput taking into account TCP slow start if applicable

-	For system level impact evaluation, IMT-2020 simulation assumptions can be adopted.

-	For reducing simulation time for system level simulation, scaling of the bandwidth and packet size of the traffic model can be considered (e.g. factor of 1/5). Companies to report the scale factor used, if any.

-	The same traffic model settings should be used across all users.

In addition, one adaptation needed is the assumption regarding beamforming and MIMO procedure. For UE power saving schemes, the evaluation focused on FR1 without consensus on assumption regarding beamforming or MIMO procedure. For evaluating the NW energy saving techniques applicable to FR2, the following assumption regarding beamforming and MIMO procedure can be reused.

-	beamforming and MIMO procedure to deal with AAS configuration may use configurations in RP-180524 (Summary of calibration results for IMT-2020 self-evaluation)

Proposal 8: For system level simulation assumption, reuse the assumption in TR 38.840, and the following assumption

- beamforming and MIMO procedure to deal with AAS configuration may use configurations in RP-180524 (Summary of calibration results for IMT-2020 self-evaluation)

- Conclusions

This contribution discusses aspects related to power consumption modelling and evaluation methodology for evaluating NW energy saving techniques. The proposals and observations made in this contribution are summarized as below:

Observation 1: Average power consumption for a variety of power states shall be determined for a given BS reference configuration.

Observation 2: Multiple sleep types can be considered, wherein each sleep state is associated with an energy saving mode.

Observation 3: A unified power state is sufficient to model power consumption for traffic processing in DL and UL, respectively.

Proposal 1: Support BS power consumption model with power states in Table 2, given reference configuration in Table 1.

Proposal 2: Support transition time, Ttransition, and additional transition energy, Etransition, for switching between deep/light sleep state and non-sleep state

- Ttransition,, = [20]ms, Etransition, = [450], for deep sleep

- Ttransition,, = [6]ms, Etransition, = [100], for light sleep

Proposal 3: Support scaling of average power for adaptation based on BS power consumption scaling model in Table 3.

Proposal 4: Support the following evaluation metrics for evaluating a NW energy saving technique:

- Energy saving gain

- Latency loss

- UPT

- FAR, MDR, resources overhead, and coexistence with legacy signal/channel, if a new signal/channel is introduced

Proposal 5:  Reuse the traffic model for FTP traffic, instant messaging and VoIP from TS 38.840 except for DRX configuration.

Proposal 6: Prioritize the following deployment scenarios for evaluating NW energy saving techniques

- Urban/Rural macro in FR1 with/without DSS (no impact to LTE expected in case of DSS)

- FR2 beam-based scenarios (note: this scenario can also model small cells)

Proposal 7: Reuse the link level simulation assumption in TR 38.840.

Proposal 8: For system level simulation assumption, reuse the assumption in TR 38.840, and the following assumption

- beamforming and MIMO procedure to deal with AAS configuration may use configurations in RP-180524 (Summary of calibration results for IMT-2020 self-evaluation)

- References

[1] RP-213554, “New SI: Study on network energy savings for NR”, RAN#94e, Huawei.

[2] 3GPP TR 38.840 V16.0.0 (2019-06), Study on User Equipment (UE) power saving in NR, Release 16

[3] R1-2203920, “Discussion on network energy saving techniques”. Samsung

[4] 3GPP TR 38.802 Study on New Radio Access Technology - Physical Layer Aspects

Annex:

Evaluation assumptions in RP-180524.

| Technical configuration Parameters | Reference |
| --- | --- |
| Frequency Range | FR1, FR2 |
| Channel model | According to the TR 38.901 |
| Antenna setup and port layouts at gNB | M, N, P, Mg, Ng; Mp, Np) – for example FR2: (4,8,2,2,2; 1,1) (dH, dV)=(0.5, 0.5)λ : FR1: (8,8,2,1,1;[2],[8]) (dH, dV)=(0.5, 0.8) - M: Number of vertical antenna elements within a panel, on one polarization - N: Number of horizontal antenna elements within a panel, on one polarization - P: Number of polarizations - Mg: Number of panels in a column; - Ng: Number of panels in a row; - Mp: Number of vertical TXRUs within a panel, on one polarization - Np: Number of horizontal TXRUs within a panel, on one polarization |
| Antenna setup and port layouts at UE | (M, N, P, Mg, Ng; Mp, Np) for example FR2 : (2,4,2,1,2; 1,2) (dH, dV) = (0.5, 0.5)λ FR1 : (1,2,2,1,1; 1,2) (dH, dV)=(0.5, N/A)λ - M: Number of vertical antenna elements within a panel, on one polarization - N: Number of horizontal antenna elements within a panel, on one polarization - P: Number of polarizations - Mg: Number of panels; - Ng: default: 1 - Mp: Number of vertical TXRUs within a panel, on one polarization - Np: Number of horizontal TXRUs within a panel, on one polarization] |
| BS Tx power | [44] dBm per TRP |
| MIMO scheme | SU-MIMO [optional: MU-MIMO] with rank adaptation is a baseline for overhead reduction. |
| MIMO layers | Provide the assumption on the maximum MU layers |
| CSI feedback | Feedback assumption “ CSI feedback periodicity (full CSI feedback) |
| Feedback assumption (Optional parameters) | Non-ideal CSI-RS/IMR channel/interference estimation (Number of CSI-RS ports = 16, 32 for (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1), (4, 4, 2, 1, 1), (4, 8, 2, 1, 1) |
| UE receiver | MMSE-IRC |
| Feedback assumption | Realistic |
| Channel estimation | Realistic |
| Evaluation Metric | TBD |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | (According to Zenith angle in "Beam set at TRxP") |