3GPP TSG RAN WG1 #109-e	R1-2203920

# e-Meeting, May 9th – 20th, 2022

Agenda item:	9.7.2

Source:	Samsung

Title:	Network energy saving techniques

Document for:	Discussion and decision

- Introduction

The study item on network energy saving was approved in RAN#94e and revised during RAN#95e [1]. As one of the objectives, network energy savings techniques were included as below:

- Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include:

- How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2]

- Information exchange/coordination over network interfaces [RAN3]

Note: Other techniques are not precluded

In this contribution, network energy savings techniques are discussed.

- Network states for energy saving

For network energy saving, a gNB can go to sleep when there is no traffic in the network. This scheme can be achieved by gNB’s implementation, however, a UE may still receive DL channels and transmit UL channels if the UE doesn’t know the gNB is sleeping. For DL, there will be power waste for the UE due to the attempts to detect the un-transmitted channels, e.g., SSB, PDCCH, SPS PDSCH. For UL, since the gNB may not always know whether the UE will transmit any UL signal, e.g., PRACH, SR, CG PUSCH, the gNB may miss these signals when it is sleeping and the UL transmission performance will degrade. On the other hand, if the gNB would like to ensure the reliability of UL transmissions, it cannot go to sleep if there is any UL transmission occasion. For the possible multiple UL transmission occasions, gNB has to do blind detection even if there is no UL transmission and therefore result in power waste for the gNB.

Based on above discussion, it’s beneficial to introduce the concept of network state for the study, and in our understanding, at least three network states need to be investigated, and there could be further refinement during the progress of the study:

- Non-energy-saving state (normal state): the gNB operates in a legacy way and no network energy saving technic is used, and this state can be the baseline for evaluating the energy saving gain.

- Energy-saving state 1: the gNB doesn’t transmit/receive any signal/channel, and this state is similar to the mechanism of DRX if analogue to a UE’s reception for PDCCH.

- Energy-saving state 2: the gNB only transmits/receives a particular set of signal/channel, for example, the particular set of signal/channel can be with high priority and prioritized in energy-saving state, such as URLLC traffic. In this state, a UE may only need to receive/transmit the particular set of signal/channel.

The UE behavior under each of the network state should also be investigated. For example, if a UE is aware of the network state as an energy-saving state, for DL, the UE can avoid detection of un-transmitted DL channels and can thus help reduce its power consumption; and for UL, the UE can transmit UL channels when the gNB is not in energy-saving state to help reduce network energy consumption.

It’s also beneficial to investigate the transition between network states, including at least the triggering mechanism and indication of the state. Moreover, it can be further investigated whether the network state transition applies for DL and UL jointly or separately.

Proposal 1:

- Consider to support at least the following three network states for the study of network energy saving:

  - Non-energy-saving state: the gNB operates in a legacy way and no network energy saving technic is used;

  - Energy-saving state 1: the gNB doesn’t transmit/receive any signal/channel;

  - Energy-saving state 2: the gNB only transmits/receives a particular set of signal/channel;

- The study shall further investigate the UE behavior in each of the network states.

- Network energy saving techniques in time domain

- 3.1 Network states transition in time domain

Due to the change of traffic at a gNB, the gNB may not always maintain the same network state, and it’s beneficial to support transition between network states in time domain. Moreover, the gNB may serve different traffic types for a same UE or different UEs in a cell. For periodic traffic, semi-static switching of network states can be a good choice, wherein the slot format configuration can be considered as a reference for designing semi-static switching of network states. An illustration of the examples for semi-static switching of network states is given in Figure 1, wherein “State 1” and “State 2” are both network states, either non-energy-saving state (normal state) or energy-saving state, and “Flexible State” can be referring a network state with similar functionality as flexible slot in the slot format configuration.

Figure 1 Illustration of semi-static switching of network states.

For aperiodic traffic, semi-static switching may not be applicable well, and hence dynamic switching of network states should also be studied, since it can provide more flexible states transition and can achieve larger network energy saving gain for random traffic. Joint operation of semi-static switching and dynamic switching can also be studied, including their relationship when both of them are supported. Still taking Figure 1 for an example, a dynamic indication of network state transition can indicate switching from State 1 to State 2 or switching from State 2 to State 1.

Proposal 2: Study semi-static switching and dynamic switching for network states transition.

- 3.2 Dynamic adaptation on periodic/semi-persistent gNB TX/RX

NR system supports periodic/semi-persistent (p/sp) physical layer resources in both UL and DL. The p/sp physical layer resources are essential for supporting link quality or mobility maintenance between gNB and UE. In the DL, a UE can be configured with periodic SSBs or p/sp CSI-RS resources for channel measurement or RRM measurement. In the UL, NR supports p/sp SRS resources for channel sounding. Moreover, the UE can be provided with p/sp physical layer resources, e.g. PUCCH for p/sp CSI report, and/or SR.

To avoid always-on signal/channel, current NR system supports adaptation on the p/sp physical layer resources triggered by higher layers. One legacy method to do the adaptation is reconfiguration of the p/sp physical layer resources via RRC signaling. Another legacy method for adaptation on the p/sp physical layer resources is (de)activation/release via MAC CE. For RRC signaling based reconfiguration, the signaling overhead can be large. Thus, the energy consumed by a gNB corresponding to the signaling overhead could be high, considering the large information exchanges not only in Layer 1. Furthermore, the legacy adaptation based on RRC or MAC CE is triggered per UE. It takes long time for gNB to provide a common adaptation to all UEs or large group of UEs for NW energy saving purpose via UE-specific signaling.

Observation 1: Current NR system requires large signaling overhead to adapt time domain resources for p/sp physical layer resources via RRC reconfiguration or semi-static (de)activation per UE.

To reduce the NW energy consumption for adaptation on p/sp physical layer resources, it’ beneficial to study cell-specific or UE group (UG) specific adaptation for the p/sp physical layer resources in both DL and UL. For example, the gNB may want to extend the periodicity for non-serving cell SSBs or all p/sp CSI-RS resources transmissions in DL in order to reduce energy consumption for background activities. So, one adaptation aspect to consider can be time domain configuration, such as periodicity. For another example, a group of p/sp CSI-RS resources can be configured to be measured by a group of UEs, and the gNB may want to deactivate the group of p/sp CSI-RS resources when the preferred beams from the group of UEs changes. Instead of deactivated each of the group of CSI-RS resources to each UE, it’s better to multicast a UE group specific deactivation indication for NW energy saving purpose. Thus, another adaptation aspect to consider can be the availability information for the p/sp physical layer resources.

Proposal 3: Study cell-specific/UG-specific dynamic adaptation on periodic/semi-persistent physical layer resources in DL or UL for NW energy savings.

- DL: CSI-RS resource, SSB, SPS PDSCH.

- UL: CSI-RS report, SR, SPS PDSCH HARQ-ACK, CG PUSCH.

- 3.3 UE-group specific dynamic adaptation on C-DRX

C-DRX is a legacy mechanism to reduce the power consumption in connected mode UE, adopted in both LTE and NR. For a UE configured with C-DRX, the UE is configured with a fixed ON duration within a C-DRX cycle. In practice, the gNB usually configure the same C-DRX cycle among UEs, and multiple ON durations to distributed UEs in time domain. From the gNB perspective, C-DRX can also achieve NW energy saving purpose if gNB can align the OFF periods from all UEs in a same location.

In R16, a WUS was introduced to dynamically wake-up UE for next ON duration. As illustrated in Figure 2, some ON durations can be cancelled by R16 WUS due to no traffic for the associated UEs, i.e. onDuration#2 and onDuration#4 are cancelled. In this scenario, gNB has to wake up multiple times for the noncontiguous ON durations, i.e. onDuration#1 and onDuration#3, which introduces additional transition energy to switch between ON and OFF on the gNB side.

Figure 2 Illustration of legacy C-DRX operation.

Observation 2: Legacy C-DRX results in large transition energy when gNB wakes up multiples times to process noncontiguous ON durations.

Moreover, C-DRX is configured per UE based on RRC signaling. For the configuration of C-DRX, there is a tradeoff between NW energy savings and UE performance, such as scheduling delay. For example, a large C-DRX cycle is preferred for NW energy savings, but it will increase latency. For NW energy saving purpose, it’s beneficial to provide the gNB the flexibility to dynamically adapt the C-DRX configuration, e.g. C-DRX cycle, onDurationTimer, and inAcitivityTimer, per group of UEs.

Observation 3: There is a tradeoff between NW energy savings and UE performance for C-DRX configuration. Current NR system requires large signaling overhead to adapt C-DRX configuration via RRC signaling per UE.

Based on above discussion, it’s beneficial to study dynamic adaptation on C-DRX per group of UEs or per cell for NW energy savings. One adaptation aspect to consider is dynamic start of DRX ON duration, such that the gNB may not need to have unnecessary transition duration within the C-DRX cycle and the ON durations for multiple UEs can be consecutive. Another adaptation aspect to consider can be the C-DRX configuration, e.g. C-DRX cycle, and onDuration.

Proposal 4: Study UG-specific dynamic adaptation of C-DRX for NW energy savings.

- Network energy saving techniques in frequency domain and power domain

- 4.1 Dedicated BWP for network energy saving

In the energy-saving state, a gNB can configure a dedicated BWP and perform transmission/reception, e.g. for a particular set of signal/channel, over the dedicated BWP for network energy saving. By doing so, the gNB can allocate all the serving UEs into the same dedicated BWP, e.g. in a dynamic or semi-static way, which can effectively reduce the operating bandwidth and save the gNB’s power consumption.

Proposal 5: Study supporting a dedicated BWP for gNB’s transmission/reception in the energy-saving state.

- 4.2 Combination of frequency and power domain adaptation

gNB transmission bandwidth and power spectral density can be jointly adapted. The following configurations can be possible:

- Configuration A (PSD reduction + keeping bandwidth): transmission bandwidth, BWA, and power spectral density, PSDA

- Configuration B (Keeping PSD + bandwidth reduction): transmission bandwidth, BWB, (BWB < BWA) and power spectral density, PSDB (PSDA < PSDB)

Figure 3 Joint adaptation of gNB transmission bandwidth and power spectral density.

Both configurations A and B can show gNB energy saving gains. For a given data rate (low to medium), configuration B would maintain coverage with reduced energy consumption. Configuration A would provide larger capacity over Configuration B while it may suffer from reduced coverage due to reduced PSD.

Observation 4: For a given data rate (low to medium), a combination of power and frequency domain adaptation would provide a balance between energy saving and system performance.

Proposal 6: Study supporting joint adaptation of gNB transmission bandwidth and power spectral density.

- 4.3 Cell switching on/off

For energy saving, a gNB can switch on/off one or multiple cells in the case of serving multiple cells and carrier aggregation (CA) as illustrated in Figure 4. As described in Figure 4, the macro & micro cells deployments (left), which consists of same frequency or different frequency, can be taken into account for cell switching on/off. Also, the macro cells overlaid with different frequency can be considered as a scenario for switching on/off cell.

Figure 4 cell switching off for macro & micro cells deployment (left) and macro cells deployment (right).

To switch off a cell, gNB should indicate semi-statically for UEs, which configured cell as PCell, to conduct handover. For more efficient UE handover, the gNB can support cell-specific/UG-specific signaling to indicate the handover information. Furthermore, for CA operation, SCell switching on/off also can be indicated by MAC signaling. It was operated as UE specific, so MAC signaling based SCell switching off results in the large signaling overhead. To resolve the signaling overhead, cell-specific/UG-specific signaling also can be studied to switch off the cell.

Proposal 7: Study supporting a cell-specific/UG-specific signaling for cell switching on/off in the energy-saving.

- Network energy saving techniques in spatial domain

Network energy saving could be achieved by focussing RF energy in spatial directions as per user distribution. In the spatial domain, large scale channel properties are governed by beams, while small scale parameters vary as per the antenna ports. Antenna ports can be associated with CSI-RS resources, i.e. UE identifies antenna ports by CSI-RS resource measurements and feedback the CSI information to the base-station. Mapping of antenna port or CSI-RS port to TxRU (transmitter unit) and TxP (transmit panel) are generally kept transparent to the specs. However, this mapping is important for the purposes of beamforming efficiently and realizing network energy savings.

A few spatial domain techniques are outlined in the following subsections:

- 5.1 Antenna port selection

The network configures multiple antenna ports or antenna port groups for a TRP. UEs share CSI feedback for the configured antenna ports. Based on the CSI feedback report shared by the UEs, gNB selects antenna ports or set of antenna ports for data channel. For example, as per Release 17 port selection codebook, if network configures X CSI ports then UE can select P out of X ports and feedback to the gNB. Based on the feedback received from all UEs, gNB can select a (super-)set of P’ ports for the data channel.

When not all X ports are selected, that would leave out some of the antenna ports from any subsequent data communication. Mechanism to switch off the non-selected antenna ports could be investigated. It should be noted that once CSI ports have switched off, they are not available for the CSI measurements, and therefore, will not be selected by UEs anymore. Mechanism should be investigated to switch on the non-selected ports again to make the same available for CSI measurements and ports selection.

Proposal 8: Study mechanism to dynamically switch off and switch on CSI ports for NW energy savings.

- 5.2 Beam adaptation

In order to have energy efficient physical resource allocation, network should create beams as per the user distribution and traffic requirements. gNB needs to adapt the beams dynamically with respect to the prevailing user distribution. Beam adaptation could be broadly categorized into two parts:

- Energy efficient beam management: Beam management procedure enables gNB transmit beams to be measured by the UEs and the measurement reports fed back to the gNB. As per the current procedure, number of beams for measurement are reduced in steps to identify transmit beams used for transmission with UE. Energy efficient mechanisms could be investigated to enhance beam management procedure.

- Energy efficient beamforming: As per current beamforming mechanism, antenna ports are selected at the UE and spatial domain (SD) coefficients are fed back from UEs to gNB. SD coefficient are used for power scaling for data channel, especially for type-II codebook. Energy efficient mechanism could be investigated to enhance beamforming operation.

Proposal 9: Study dynamic beam adaption mechanism for NW energy savings.

- 5.3 Extension to m-TRP

m-TRP operation has been defined using antenna port mechanism, and multiple NZP CSI-RS resources can be configured in a CSI-RS resource set, where a NZP CSI-RS resource corresponds to a TRP. In m-TRP deployment, UEs are served by multiple TRPs and port selection is performed by each UE for each TRP (except joint transmission).

Port selection could also be done across m-TRP and ports those are not selected by UEs could be switched off per TRP. Mechanisms should be investigated to completely switch off the TRP(s). As previously mentioned, once a TRP has been switched off, respective antenna ports will not be available for CSI measurements. Therefore, dynamic m-TRP switch off/on mechanism should be investigated for m-TRP energy saving operation.

Proposal 10: Study mechanism to dynamically switch off and switch on TRP for NW energy savings.

- gNB wake up signal and UE assistance information

- 6.1 gNB wake up signal (WUS)

If a gNB is in energy saving state, the UE may not be able to transmit UL channels. For UL latency sensitive traffic, the latency requirements may not be satisfied if the energy saving state is not properly configured/indicated. For example, the UE may wait a long time to transmit an SR or CG PUSCH after switching to the normal state. To reduce the UL latency, a gNB WUS can be used as an UL indication of network energy saving states transition. For example, if there arrives latency sensitive or high priority traffic at the UE, the UE can transmit a WUS and then transmit an SR or CG PUSCH. The gNB only needs to detect the WUS during the energy saving state, and once the gNB detects the WUS, the gNB will switch to the normal state to receive other UL transmission.

Proposal 11: Study gNB wake up signaling to trigger network energy saving states transition.

- 6.2 UE assistance information for requesting a DL channel transmission

gNB power consumption on transmitting periodic broadcast channel/signal is considerable, e.g., SSB, SIB1 and OSI. These broadcast channels/signals can be muted during network energy saving state. However, SSB muting may increase transmission latency which cannot be acceptable for some latency sensitive traffic. For example, DL synchronization may be lost when UL data is arrived at the UE side. In such case, the UE needs to first receive SSB to achieve synchronization before transmitting PRACH/SR/CG PUSCH. A UE assistance information can be used to request an SSB transmission to reduce the latency. After transmitting the UE assistance information, the UE can expect an SSB would be transmitted and receive the SSB to acquire DL synchronization. Here, the gNB needs monitor the UE assistance information during energy saving state, e.g., in an occasion before each muted SSB.

Proposal 12: Study UE assistance information for SSB request during network energy saving state.

- 6.3 UE assistance information for indicating an UL channel transmission

For UL, there are some unknown UL transmissions from a gNB's perspective and the gNB needs to do blind detection to receive these UL transmissions. Considering the number of UEs and the number of candidate UL transmissions, the blind detection can cause the network power consumption even if there is no UL transmission. The unknown UL transmissions include PRACH, SR and CG PUSCH.

For example, there can be up to 12 CG PUSCH configurations per UL BWP per UE and the minimum periodicity is 2 symbols. If there are multiple UEs in a cell, the gNB may need to do blind detection frequently. One enhancement is to transmit an UL transmission indication signaling before transmitting an UL channel. Only when the gNB receives the UL transmission indication signaling, gNB would do blind detection of the UL channels, otherwise, the gNB does not need to do blind detection of the other semi-static UL channels transmissions. The resource carrying the UL transmission indication signaling can be common or separate for all candidate UL transmissions. In addition, gNB can configure the common or different resources for the UL transmission indication signaling to different UEs. An example is given in Figure 4, instead of blind decoding all the candidate CG PUSCHs, the gNB only need to detect the UE assistance information which can be a sequence and shared by a group of UEs. Clearly, the UE assistance signaling can help gNB reduce blind detection and thus reduce the network power consumption.

Figure 4 Illustration of UE assistance information for indicating UL transmission.

Observation 5: UE assistance signaling for indicating semi-static UL channel transmissions is beneficial for network power consumption.

Proposal 13: Study UE assistance information for indicating semi-static UL channel transmissions during network energy saving state.

- Conclusions

This contribution discusses the network power saving techniques and the proposals are summarized below,

Proposal 1:

- Consider to support at least the following three network states for the study of network energy saving:

  - Non-energy-saving state: the gNB operates in a legacy way and no network energy saving technic is used;

  - Energy-saving state 1: the gNB doesn’t transmit/receive any signal/channel;

  - Energy-saving state 2: the gNB only transmits/receives a particular set of signal/channel;

- The study shall further investigate the UE behavior in each of the network states.

Proposal 2: Study semi-static switching and dynamic switching for network states transition.

Proposal 3: Study cell-specific/UG-specific dynamic adaptation on periodic/semi-persistent physical layer resources in DL or UL for NW energy savings.

- DL: CSI-RS resource, SSB, SPS PDSCH.

- UL: CSI-RS report, SR, SPS PDSCH HARQ-ACK, CG PUSCH.

Proposal 4: Study UG-specific dynamic adaptation of C-DRX for NW energy savings.

Proposal 5: Study supporting a dedicated BWP for gNB’s transmission/reception in the energy-saving state.

Proposal 6: Study supporting joint adaptation of gNB transmission bandwidth and power spectral density.

Proposal 7: Study supporting a cell-specific/UG-specific signaling for cell switching on/off in the energy-saving.

Proposal 8: Study mechanism to dynamically switch off and switch on CSI ports for NW energy savings.

Proposal 9: Study dynamic beam adaption mechanism for NW energy savings.

Proposal 10: Study mechanism to dynamically switch off and switch on TRP for NW energy savings.

Proposal 11: Study gNB wake up signaling to trigger network energy saving states transition.

Proposal 12: Study UE assistance information for SSB request during network energy saving state.

Proposal 13: Study UE assistance information for indicating semi-static UL channel transmissions during network energy saving state.

In addition, the following observations are made.

Observation 1: Current NR system requires large signaling overhead to adapt time domain resources for p/sp physical layer resources via RRC reconfiguration or semi-static (de)activation per UE.

Observation 2: Legacy C-DRX results in large transition energy when gNB wakes up multiples times to process noncontiguous ON durations.

Observation 3: There is a tradeoff between NW energy savings and UE performance for C-DRX configuration. Current NR system requires large signaling overhead to adapt C-DRX configuration via RRC signaling per UE.

Observation 4: For a given data rate (low to medium), a combination of power and frequency domain adaptation would provide a balance between energy saving and system performance.

Observation 5: UE assistance signaling for indicating semi-static UL channel transmissions is beneficial for network power consumption.

References:

- RP-213554, New SI: Study on network energy savings for NR, Huawei