3GPP TSG RAN WG1 #109-e			R1-2203921

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.8.1

Source:	Samsung

Title:	Side control information to enable NR network-controlled repeaters

Document for:	Discussion and Decision

- Introduction

In RAN#94-e, Rel-18 new study item on “Study on NR Network-controlled Repeaters” is endorsed. The objective of the study item is as follows.

The study on NR network-controlled repeaters is to focus on the following scenarios and assumptions: - Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands, while during the study FR2 deployments may be prioritized for both outdoor and O2I scenarios. - For only single hop stationary network-controlled repeaters - Network-controlled repeaters are transparent to UEs - Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously NOTE1: Cost efficiency is a key consideration point for network-controlled repeaters. Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1] - Beamforming information - Timing information to align transmission / reception boundaries of network-controlled repeater - Information on UL-DL TDD configuration - ON-OFF information for efficient interference management and improved energy efficiency - Power control information for efficient interference management (as the 2nd priority) Study and identify L1/L2 signaling (including its configuration) to carry the side control information [RAN1] Study the following aspects of network-controlled repeater management - Identification and authorization of network-controlled repeaters [RAN2, RAN3] NOTE2: Coordination with SA3 may be needed.

In this contribution, we firstly provide our general understanding on network-controlled repeater and then our views on the control information for network-controlled repeaters.

- General discussion

For the study of network-controlled repeaters (NCRs), one of the key issues is the frequency range. In our view, FR2 is preferred for the deployment of NCR as aligned with the approved SID [1]. In FR1, especially for FDD bands, there has been a commercial use of repeaters with solid performance. In comparison with FR1, the coverage issue is more critical in FR2 and thus more benefit is expected by the repeaters in this frequency range.

Proposal 1: FR2 is prioritized for the study of network-controlled repeaters.

- NCR control information (NCI)

In general, it is assumed that an NCR consists of two modules. As Fig.1 below, one is NCR mobile terminal (NCR-MT) and the other is NCR radio unit (NCR-RU). NCR-MT can receive NCR control information (NCI) and provide corresponding feedback to gNB; NCR-RU can amplify-and-forward RF signals to UEs or to a gNB. Moreover, the NCR-RU is controlled by the NCI received at the NCR-MT.

Figure 1. Functional assumption of network-controlled repeater

According to the SID [1], five NCI candidates to enable NR network-controlled repeaters were provided, including beamforming information, timing information, UL-DL TDD configuration information, on-off information, and power control information. In the following, the necessity and the design of these NCI will be discussed.

3.1 Beamforming information

In the case of FR2, signals experience more severe propagation loss in comparison with FR1 due to the higher carrier frequency. To compensate the loss, directional transmission using analog beamforming is necessary. One issue of directional transmission is that the effective coverage area will be significantly reduced if high analog beamforming gain is desired. In order to achieve high beamforming gain without sacrificing coverage area, beam sweeping/switching is essential and was introduced in NR system.

Similarly, for the deployment of repeater in FR2, the capability of beam sweeping/switching is one of the most essential features. To enable this feature, NCI for beamforming is necessary. With the capability to receive and process beamforming information, NCR is able to provide high beamforming gain towards different UEs within coverage hole or at cell edge without creating unexpected interference or noise to the UEs in neighbouring cells.

In the following, system level evaluation results are provided in terms of the signal to interference plus noise ratio (SINR). The evaluation assumptions of the system level simulation (SLS) are summarized in Table 1, wherein the parameters are taken from TR38.802. The deployment for the UMa scenario is illustrated in Figure 2, where 2-tier hexagonal grids with 3 repeaters per sector are assumed. To compare the impact of the NCR deployment, we consider the NR systems without repeaters, with legacy repeaters, with repeaters applying beamforming, and with repeaters applying beamforming and on/off management. The results are shown in Figure 3. The SINR gain is observed for the repeaters applying beamforming compare to the NR systems without repeaters and with legacy repeaters. The improved SINR gain of the repeaters applying beamforming are respectively 2.34 dB, 6.15 dB, and 6.53 dB at 5%-tile, 50%-tile, and 95%-tile CDFs of the SINR compared to the NR system without repeaters. In addition, 2.03 dB, 5.18 dB, and 6.53 dB gains at 5%-tile, 50%-tile, and 95%-tile CDFs of the SINR are observed compared to the NR system with legacy repeaters, respectively.

Table 1. System level simulation assumptions for NCR

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 30 GHz |
| Layout | UMa (ISD=500m, Outdoor (50%) + Indoor (50%)) |
| Channel Model | TR38.901 |
| BS Tx Power | 43 dBm |
| Repeater Tx Power | 33 dBm |
| BS antenna Array configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2), dH = dV = 0.5 λ |
| UE antenna Array configuration | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 1), dH = dV = 0.5 λ |
| Repeater antenna Array configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 2), dH = dV = 0.5 λ |
| Traffic model | Full buffer |
| Minimum inter site distance | BS-to-repeater: ISD/3m, Repeater-to-UE: 2m Repeater-to-repeater: 70m |

Figure 2. Deployment of repeaters in UMa

Figure 3. CDF of SINR

According to the discussion and the simulation results above, we have the following proposal.

Proposal 2: Support to specify beamforming information for network-controlled repeaters.

NCR-MT

For the design of beamforming information, both NCR-MT and NCR-RU should be considered. Similar to IAB-MT, NCR-MT only communicates with gNB. It is natural that NCR-MT can be regarded as a ‘UE’. This means that legacy beam indication design for UE can be reused as much as possible.

A further discussion point is which beam indication method can be used for NCR-MT. From Rel-15 to Rel-17, a large number of beam indication methods were introduced. With the assumption of stationary NCR, it is obvious that only a subset of them is necessary to for providing beamforming information to NCR-MT.

Proposal 3: For NCR-MT beam indication, legacy beam indication methods for UE can be reused as much as possible.

- FFS: which legacy beam indication methods, e.g., Rel-15/Rel-16/Rel-17.

For the convenience of the discussion on NCR-RU beam indication, the following terms are defined. As illustrated by Figure 4, an NCR-RU corresponds to two links: the link of gNB-side and the link of UE-side. For the gNB-side, the NCR-RU requires beamforming information for two directions (e.g., for DL reception from the gNB and UL forwarding to the gNB), which we refer to as DL Rx and UL Tx. For the UE-side, the NCR-RU also requires beamforming information for two directions (e.g., for DL forwarding to the UEs and UL reception from the UEs), which we refer to as DL Tx and UL Rx.

In the MIMO discussion, beam indication has been usually discussed using the source beam (the source RS ID), the target beam (the target RS ID), and the association between the source beam and the target beam (e.g., the TCI states indication). In the following, the beam indication of an NCR-RU will be discussed using the similar terms as mentioned above.

Figure 4. Beam indication for NCR-RU

3.1.1 The definition of the source beam for NCR-RU

Firstly, the definition of the source beam for an NCR-RU will be discussed. Specifically, gNB-side beams and UE-side beams will be separately discussed.

NCR-RU (gNB-side)

For the definition of the gNB-side source beam for an NCR-RU (DL Rx / UL Tx), a straight-forward way may be to use the source beam defined for the NCR-MT as the gNB-side source beam for the NCR-RU since they correspond to the same link. If the NCR-MT reuses the legacy beam indication method, TCI states ID and RS ID can be considered for the definition of the gNB-side source beams for the NCR-RU. In another word, the beam for reception/transmission of the RS corresponding to these IDs can be used for the gNB-side reception/transmission for the NCR-RU.

Proposal 4: For the definition of gNB-side source beam for the NCR-RU (DL Rx / UL Tx), reuse the source beam defined for the NCR-MT (e.g., TCI states ID, RS ID).

NCR-RU (UE-side)

The definition of the UE-side source beam for the NCR-RU (e.g., DL Tx) is different from the gNB-side. As mentioned in the SID assumptions [1], the presence of an NCR-RU should be transparent to UEs. Thus, the NCR is not expected to generate its own signals/channels towards UEs. In other words, the NCR may be able to provide several beams for DL-Tx, but it cannot generate its own reference signals for the corresponding beams. These assumptions impose an issue on how to define/identify the UE-side beam for the NCR-RU. In the following, two approaches are provided to address the issue.

Approach#1 (beam ID)

The first approach is to define the UE-side beam for the NCR-RU as a beam ID. For example, as illustrated in Figure 5, the NCR is able to generate four UE-side beams. The NCR associates the four beams with four different IDs so that each beam can be independently identified.

For this approach, the information associated with the beam ID (e.g., the number of beam ID) is needed by NCR. With this information, the NCR can perform one-to-one mapping between a UE-side beam and a beam ID. It is noted that, selection and labeling the UE-side beams for the NCR-RU is up to the NCR implementation. For example, it is up to the NCR implementation to call a first beam (e.g., the leftmost beam) as beam #1 and call a second beam (e.g., the rightmost beam) as beam #4, or vice versa. Such information need not be disclosed or exchanged with the gNB.

Figure 5. The definition of the UE-side source beams for an NCR-RU

Approach#2 (RS ID)

The second approach is to define the UE-side beam for the NCR-RU as the RS generated by the gNB but amplify-and-forward by the NCR-RU. For example, as illustrated in Figure 5, the NCR is able to generate four UE-side beams. For this case, the gNB generates four RSs (RS#4, RS#5, RS#6, RS#7) for the NCR-RU to perform beam sweeping of the UE-side beams. Note that the four RSs can correspond to the same DL spatial filter and can occupy different time domain resources. It is up to NCR implementation to associate the four beams with these four RSs. After the association, the four beams can be independently identified by the RS IDs (i.e., each DL-Tx beam corresponds to one RS ID).

For this approach, the information associated with RS IDs (e.g., the configuration related to the corresponding beam sweeping RSs) is needed by the NCR. With this information, the NCR can perform one-to-one mapping between a UE-side beam and a RS ID.

According to the discussion above, we have the following proposal.

Proposal 5: For the definition of UE-side source beams for the NCR-RU, at least the following approaches can be considered:

- Approach#1. A UE-side source beam for an NCR-RU is defined as a beam ID

- Approach#2. A UE-side source beam for an NCR-RU is defined as a RS ID

3.1.2 The definition of the target beam for NCR-RU

The issue for the definition of the target beam for the NCR-RU is very similar to the issue for defining the UE-side source beam for the NCR-RU. In order to define/identify the target beam for the NCR-RU, two approaches are provided as follows.

Approach#1 (time domain resource identifier)

The first approach is to define the target beam for the NCR-RU as a time domain resource identifier. This approach means that the time domain identifier can be regarded as the beam that used for NCR-RU transmission/reception in the corresponding time domain resource.

For this approach, the information associated with time domain resource identifier (e.g., the configuration of the corresponding time domain resource) is needed by the NCR.

Approach#2 (RS ID)

The second approach is to define the target beam for the NCR-RU as the RS generated by the gNB but amplify-and-forward by the NCR. This approach means that the RS ID can be regarded as the beam that used for NCR-RU transmission in the time domain duration of the RS.

For this approach, the information associated with RS ID (e.g., the RS configuration related to the RS ID) is needed to the NCR.

According to the discussion above, we have the following proposal.

Proposal 6: For the definition of the target beam for the NCR-RU, at least the following approaches can be considered:

- Approach#1. The target beam for NCR-RU is defined as a time domain resource identifier

- Approach#2. The target beam for NCR-RU is defined as a RS ID

3.1.3 The association of source beam and target beam for NCR-RU

For the association of the source beam and the target beam, at least the following combinations can be considered (source  target):

- #1. beam ID  time domain resource identifier

- #2. RS ID  time domain resource identifier

- #3. RS ID  RS ID

With the association of #1 or #2, the beam indication for NCR-RU at any time domain resource is possible. The association of #3 is similar to the association of source RS and target RS. For example, the association of SSB IDCSI-RS ID can be used to guarantee the same DL-Tx beam for an SSB and the corresponding TRS. For another example, the association of CSI-RS IDCSI-RS ID can be used to guarantee the same DL-Tx beam for multiple CSI-RSs.

Proposal 7: For the association of the source beam and the target beam for NCR-RU, at least the following combinations can be considered (source  target):

- #1. beam ID  time domain resource identifier

- #2. RS ID  time domain resource identifier

- #3. RS ID  RS ID

3.2 Timing information

Similar to the study of the IAB, the feasibility of over-the-air (OTA) synchronization and the impact of timing misalignment on NCR need to be studied. In the following, potential timing alignment assumptions between the gNB and the NCR-MT, and the NCR-RU are discussed.

For the NCR-MT, the same timing alignment procedure for the legacy UE (e.g., Rel-15) can be reused. Specifically, DL timing of the NCR-MT can be determined by the detection of SSB while UL timing of the NCR-MT can be determined by the timing advanced (TA) according to the RACH procedure.

Proposal 8: For the timing alignment for NCR-MT, the timing alignment procedure for the legacy UE can be reused.

For the NCR-RU, at least three different assumptions for the timing alignment can be considered (see Figure 6). For the common part of the three assumptions, the timing of the NCR-MT DL is determined by the SSB transmitted by the gNB which implicitly reflects the propagation delay between the gNB and the NCR-MT. The timing of the NCR-MT UL is determined by the timing advance (TTA), which is TTA earlier than the timing of the NCR-MT DL. Also, there are potentially four timings for different links and a UL/DL direction of the NCR-RU (DL-Rx, DL-Tx, UL-Rx and UL-Tx). Here, for the simplicity, the processing delay at the NCR-RU is assumed to be zero. Hence, the timing of DL-Rx and DL-Tx is aligned while the timing of UL-Rx and UL-Tx is aligned.

For timing alignment assumption#1, all the four timings of the NCR-RU are aligned with the timing of NCR-MT DL. This is straightforward for the time alignment of NCR-RU, where there is no additional timing information is needed other than the DL synchronization information provided to the NCR-MT.

For timing alignment assumption#2, the DL timings of the NCR-RU are aligned with the timing of NCR-MT DL, and the UL timings of NCR-RU are aligned with the timing of NCR-MT UL. This is also a straightforward way for the time alignment of the NCR-RU, where there is no additional timing information needed other than the DL synchronization information and the UL TA provided for the NCR-MT.

For timing alignment assumption#3, all the four timings of the NCR-RU are aligned with the timing of gNB. This assumption is similar to Rel-16 IAB case #1, where additional timing information is needed for the timing alignment between the gNB and the NCR-RU.

One question of timing alignment of the NCR-RU is, whether the gNB and the NCR-RU shall comply with the 3us timing requirement at DL. The answer to this question is very critical to the design of timing information of the NCR. If the answer is yes, timing alignment assumption#3 should be considered as a starting point.

Figure 6. The potential timing assumptions for NCR-RU

Proposal 9: For the timing alignment for the NCR-RU, study the following timing alignment assumptions:

- Assumption#1. The UL and DL timings of the NCR-RU are aligned with the DL timing of the NCR-MT.

- Assumption#2. The DL timings of the NCR-RU are aligned with the DL timings of the NCR-MT; and the UL timings of the NCR-RU are aligned with the UL timings of the NCR-MT.

- Assumption#3. The UL and DL timings of NCR-RU are aligned with the DL timing of gNB.

3.3 Information on UL-DL TDD configuration

The information on the UL-DL TDD configuration is one of the key information to enable NR network-controlled repeaters on the TDD bands. The UL-DL TDD configuration information provides flexibility for the gNB to adjust the proportion of the time domain resource for DL amplify-and-forward and UL amplify-and-forward.

Proposal 10: Support to specify the UL-DL TDD configuration for network-controlled repeaters.

For the design of the UL-DL TDD configuration for the NCR, one option is to reuse the semi-static UL-DL TDD configuration (e.g., the cell-specific TDD configuration and/or the UE specific TDD configuration). In this way, the UL/DL time domain resources for of the NCR-MT and the NCR-RU can be determined, respectively.

Proposal 11: For the UL-DL TDD configuration for the NCR, the legacy UL-DL TDD configuration can be reused as much as possible.

- E.g., the semi-static UL-DL TDD configuration

3.4 ON-OFF information

In section 3.1, the simulation results in terms of SINR are provided. It is assumed that the NCR turns off itself when there are no UEs to serve. In Figure 3, the gain is observed for the repeater applying beamforming and on/off management compare to the NR system with the repeater only applying beamforming. Hence, it is beneficial to have on-off information for NCR.

Proposal 12: Support to specify ON-OFF information for network-controlled repeaters.

In order for the gNB to dynamically determine that there are no UE(s) served through the NCR, and accordingly indicate an OFF command to the NCR, or to determine that a new candidate UE is starting to get served through the NCR, and accordingly indicate the ON command to the NCR, the gNB needs to be able to associate the UEs with the NCR. For example, the gNB needs to be able to determine whether a UE is being served by any of the beams that is associated with the NCR. The gNB also needs to keep track of any changes to the association of a UE with the NCR as the UE moves, such as whether the UE has moved in or out of the coverage area of the NCR.

Observation 1: Support for dynamic ON-OFF indication to the NCR requires gNB knowledge of whether a UE is or is not associated with the NCR.

The gNB may be able to make a basic/initial determination about whether a UE is or is not associated with the NCR using implementation-based techniques. For example, the gNB can configure the UE with DL RSs, including those associated with the NCR, and use beam/CSI reports from the UE to determine whether the UE is being served by a beam operated by the NCR. In addition, the gNB may use other measurements such as UE timing advanced (TA) values to improve the gNB knowledge about the UE association with the NCR. For example, in Figure 5 Approach #2, the UE #1 is associated with RS #5. There can be another UE, say UE #3, that is also associated with RS #5, but close to the gNB and facing away from the repeater. The gNB may determine that UE#1 is associated with the NCR, and that UE#3 is not associated with the NCR by comparing the TA values corresponding to UE #1 and UE #3 with the TA value corresponding to the NCR.

Observation 2: The gNB may be able to make an initial determination about whether a UE is or is not associated with the NCR using implementation-based techniques, such as by using beam/CSI reports from the UE or TA value corresponding to the UE.

However, such gNB implementation methods may not be sufficient to determine whether a UE is or is not associated with the NCR in all scenarios, for example, when beam/CSI report-based or TA-based determinations have ‘borderline’ values that cannot provide solid evidence for such determination. Therefore, further methods can be considered to improve the gNB determination about the UE association with the NCR. It is noted that, the NCR is in direct communication with the UE(s), and therefore has access to measurements that are not available to the gNB. For example, if the NCR is provided information of the resources for certain UE uplink transmissions, such as SRS, the NCR can measure those UE uplink transmissions and reports its measurements to the gNB. For example, for UE#1 in Figure 5 Approach #2, the NCR measurement of the UE#1’s SRS can have much better quality than the gNB measurement of the same SRS. This can be especially beneficial for FR2 operation with directional beamforming.

Proposal 13: Further study methods to facilitate the gNB determination about whether a UE is or is not associated with the NCR (and thereby support dynamic ON-OFF indication), including NCR reporting its measurements of the UE’s uplink transmissions to the gNB.

It is noted that methods for association of UE with NCR are not only essential for dynamic ON-OFF indication, but are also very beneficial for other NCIs as well, especially for beamforming information. For example, when a UE moves out of the coverage area of the NCR and is no longer associated with the NCR, the gNB needs to be able to determine such an event, and reflect that in the beamforming information provided to the NCR.

Observation 3: The gNB knowledge of the association (or not) of the UE(s) with the NCR is beneficial for other NCIs as well, especially for the beamforming information.

3.5 Power control information

For the RAN4 discussion for Rel-17 repeater, the gain of repeater is fixed. In our understanding, the situation is no different for the design of NCR. A fixed amplification gain can be assumed as baseline. Further enhancement to the power control of NCR needs justification.

Proposal 14: A fixed amplification gain can be assumed as baseline. Further enhancement to power control of NCR needs justification.

- Conclusion

The observations and proposals made in this contribution are summarized below.

Observation 1: Support for dynamic ON-OFF indication to the NCR requires gNB knowledge of whether a UE is or is not associated with the NCR.

Observation 2: The gNB may be able to make an initial determination about whether a UE is or is not associated with the NCR using implementation-based techniques, such as by using beam/CSI reports from the UE or TA value corresponding to the UE.

Observation 3: The gNB knowledge of the association (or not) of the UE(s) with the NCR is beneficial for other NCIs as well, especially for the beamforming information.

Proposal 1: FR2 is prioritized for the study of network-controlled repeaters.

Proposal 2: Support to specify beamforming information for network-controlled repeaters.

Proposal 3: For NCR-MT beam indication, legacy beam indication methods for UE can be reused as much as possible.

- FFS: which legacy beam indication methods, e.g., Rel-15/Rel-16/Rel-17.

Proposal 4: For the definition of gNB-side source beam for the NCR-RU (DL Rx / UL Tx), reuse the source beam defined for the NCR-MT (e.g., TCI states ID, RS ID).

Proposal 5: For the definition of UE-side source beams for the NCR-RU, at least the following approaches can be considered:

- Approach#1. A UE-side source beam for an NCR-RU is defined as a beam ID

- Approach#2. A UE-side source beam for an NCR-RU is defined as a RS ID

Proposal 6: For the definition of the target beam for the NCR-RU, at least the following approaches can be considered:

- Approach#1. The target beam for NCR-RU is defined as a time domain resource identifier

- Approach#2. The target beam for NCR-RU is defined as a RS ID

Proposal 7: For the association of the source beam and the target beam for NCR-RU, at least the following combinations can be considered (source  target):

- #1. beam ID  time domain resource identifier

- #2. RS ID  time domain resource identifier

- #3. RS ID  RS ID

Proposal 8: For the timing alignment for NCR-MT, the timing alignment procedure for the legacy UE can be reused.

Proposal 9: For the timing alignment for the NCR-RU, study the following timing alignment assumptions:

- Assumption#1. The UL and DL timings of the NCR-RU are aligned with the DL timing of the NCR-MT.

- Assumption#2. The DL timings of the NCR-RU are aligned with the DL timings of the NCR-MT; and the UL timings of the NCR-RU are aligned with the UL timings of the NCR-MT.

- Assumption#3. The UL and DL timings of NCR-RU are aligned with the DL timing of gNB.

Proposal 10: Support to specify the UL-DL TDD configuration for network-controlled repeaters.

Proposal 11: For the UL-DL TDD configuration for the NCR, the legacy UL-DL TDD configuration can be reused as much as possible.

- E.g., the semi-static UL-DL TDD configuration

Proposal 12: Support to specify ON-OFF information for network-controlled repeaters.

Proposal 13: Further study methods to facilitate the gNB determination about whether a UE is or is not associated with the NCR (and thereby support dynamic ON-OFF indication), including NCR reporting its measurements of the UE’s uplink transmissions to the gNB.

Proposal 14: A fixed amplification gain can be assumed as baseline. Further enhancement to power control of NCR needs justification.

References

- RP-213700, “New SI: Study on NR Network-controlled Repeaters”, ZTE Corporation.