3GPP TSG RAN WG1 #109-e	R1-2203922

# e-Meeting, May 9th – 20th, 2022

Agenda item:	9.8.2

Source:	Samsung

Title:	L1/L2 signaling for side control information

Document for:	Discussion and Decision

# Introduction

In RAN#94-e, a new study item on NR network-controlled repeaters was endorsed with the following objectives [1].

The study on NR network-controlled repeaters is to focus on the following scenarios and assumptions: - Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands, while during the study FR2 deployments may be prioritized for both outdoor and O2I scenarios. - For only single hop stationary network-controlled repeaters - Network-controlled repeaters are transparent to UEs - Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously NOTE1: Cost efficiency is a key consideration point for network-controlled repeaters. Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1] - Beamforming information - Timing information to align transmission / reception boundaries of network-controlled repeater - Information on UL-DL TDD configuration - ON-OFF information for efficient interference management and improved energy efficiency - Power control information for efficient interference management (as the 2nd priority) Study and identify L1/L2 signaling (including its configuration) to carry the side control information [RAN1] Study the following aspects of network-controlled repeater management - Identification and authorization of network-controlled repeaters [RAN2, RAN3] NOTE2: Coordination with SA3 may be needed.

In this contribution, we provide our view on architecture aspects and signalling design for network-controlled repeaters, referred to as “NCR” in the following.

# Functional architecture of NCR

The Rel-18 Network-controlled repeater (NCR) is an evloution of the Rel-17 NR repeater [2,3] for improving network coverage based on the amplify-and-forward (AF) operation. The Rel-17 NR repeater is an RF repeater that operates generally based on implementation (except for conforming to RF/EMC requirements) without any signaling exchange with the gNB. The Rel-18 NCR, however, performs the AF operation based on signaling exchange with the gNB, including beamforming information, TDD DL/UL information, ON/OFF indication, and so on. Such information is referred to as ‘side control information’ [1], or in this Tdoc, as the NCR control information (NCI). Details of NCI design are provided in our companion Tdoc [4].

Accordingly, the NCR consists of two functional modules, as shown in Figure 1: an RF unit (NCR-RU) that performs the AF operation similar to the Rel-17 NR repeater, and a mobile termination (NCR-MT) that facilitates the signaling exchange with the gNB.

Observation 1: An NCR consists of two functional modules: NCR-RU that performs the amplify-and-forward operation, and NCR-MT that facilitates the signaling exchange with the gNB.

Figure 1: functional architecture of the NCR

The NCR-RU performs the AF operation on the DL for gNB signals and channels received by the UE(s), and on the UL for UE(s)’ signals and channel transmitted to the gNB. The NCR-RU applies a power gain to the transmission/receptions it receives in the RF domain and relays them without any digital processing (e.g., unlike an IAB-DU that performs a decode-and-forward operation). In particular, the NCR-RU does not create a new cell, and operates in a transparent manner to the UE(s).

It is noted that, per WID, Rel-18 NCR is aimed at “extension of network coverage”, so the NCR is primary deployed in the cell edge or other coverage holes to provide coverage to UE(s) that are otherwise outside network coverage. The NCR is not intended to provide enhanced spectral efficiency or reliability benefits, via multiplexing or diversity schemes, to UE(s) that are already connected to the gNB. Therefore, it is expected that the NCR is the only path for the gNB to reach the UE(s) that are served through the NCR.

Observation 2: A Rel-18 NCR is aimed at network coverage extension; therefore, the NCR is expected to be the only path for the gNB to reach the UE(s) that are served through the NCR.

The NCR-MT is mainly tasked with reception of downlink NCR control information (DL NCI) from the gNB to apply to the AF operation of the NCR-RU. As mentioned before, such DL NCI includes the beamforming information, the TDD DL/UL information, the ON/OFF indication, and so on, based on which the NCR-RU performs the AF operation on the DL receptions by the UE(s) that are served through the NCR or the UL transmissions of such UE(s) to the gNB. In addition, it is expected that the NCR-MT is capable of transmitting uplink NCR control information (UL NCI) to the gNB, at least including confirmation (e.g., in the form of HARQ-ACK feedback) of the DL NCI, otherwise the repeater would be simply a network-informed repeater, rather than an NCR. The UL NCI can include other legacy UL UCI, such as CSI for the NCR-to-gNB link, or possibly also assistance information from the NCR to the gNB for better configuration or scheduling of (candidate) UE(s) that are served through the NCR, as discussed in [4].

Observation 3: The NCR-MT transmits uplink NCR control information (UL NCI) to the gNB, in addition to receiving downlink NCR control information (DL NCI) from the gNB; otherwise, the repeater would be simply a network-informed repeater, rather than a network-controlled repeater.

Based on the protocol architecture design for the NCR, as discussed in Section 3, it is expected that NCR-MT is capable of performing initial/random access procedures, and therefore, the NCR-MT is also responsible for the identification and authorization procedures for the NCR in the network. For example, the NCR-MT can establish the connection for NCR with the gNB, based on the NCR identity, and also establish an initial timing and beam for the NCR operation on the gNB-to-NCR link.

Observation 4: The NCR-MT handles the identification and authorization procedures for the NCR by performing the initial/random access procedure.

In order to perform the above functionalities, the Rel-18 NCR-MT can build on 5G NR Rel-17 UE features and functionalities, when necessary. But, it is expected that the NCR-MT may implement a reduced subset of such UE features and functionalities, due to NCR cost considerations.

In addition, in order for the NCR-RU and NCR-MT to achieve their individual tasks, as outlined above, the NCR design needs to consider the interactions of NCR-RU and NCR-MT, such as hardware interactions and procedural interactions.

Different designs for the NCR hardware implementation can be considered. A first type of NCR implementations considers a shared RF path/chain for both the NCR-RU and the NCR-MT, while a second type of NCR implementations consider two separate or independent RF paths/chains for the NCR-RU and NCR-MT. The first type is beneficial due to reduced cost (one less RF path), and also because such design would avoid the need for special handling for self-interference, such as inter-modulation issues, from NCR-MT to NCR-RU and vice versa, which is present for the second type of NCR implementation. On the other hand, the second type is beneficial, since the separate RF paths allow for a simpler and more flexible RF design. Figure 2 shows the first type of NCR implementation with shared RF chains. The NCR-MT receives the DL NCI by extracting and digital processing certain time/frequency resources from of the downlink RF path from the gNB, and transmits the UL NCI by inserting the corresponding information in the uplink RF path to the gNB.

Observation 5: An NCR hardware implementation can be based on a shared RF path/chain for both NCR-RU and NCR-MT, or two separate RF paths/chains for the NCR-RU and NCR-MT.

Figure 2: An NCR design based on shared RF path for NCR-MT and NCR-RU

Based on the SID [1], network-controlled repeaters are in-band RF repeaters, so that the NCR-RU and NCR-MT operate in a same frequency band. Accordingly, a single time/frequency allocation is available to the combination of NCR-RU and NCR-MT. Therefore, the next design question for NCR is the interaction of NCR-RU and NCR-MT with respect to time/frequency resource multiplexing. A general approach for resource multiplexing between NCR-RU and NCR-MT is time division multiplexing (TDM), where different slots/symbols are allocated to NCR-MT and NCR-RU. This approach is a low-complexity method, and possibly the only viable solution on the uplink direction. It appears that, it may be challenging for the NCR implementation to handle transmission of UL NCI from NCR-MT to the gNB simultaneously with (even when using different RB(s) or different beam(s) as for) the amplify-and-forwarded uplink transmissions from UE(s) to the gNB, for example, due to time/frequency synchronization or power setting issues. However, for the downlink direction, frequency division multiplexing (FDM) or spatial division multiplexing (SDM) may be also considered for more efficient operation and less signaling overhead. Therefore, it may be possible for the NCR-MT to receive DL NCI simultaneously with (but in different RB(s) or beam(s) as for) a downlink reception from the gNB that the NCR-RU is expected to AF to the UE(s). It is noted that the considerations described here for resource multiplexing between NCR-RU and NCR-MT apply to both types of NCR hardware implementation with shared or separate RF paths.

Observation 6: A baseline approach for resource multiplexing between NCR-RU and NCR-MT is TDM. It is also possible to consider FDM or SDM for the downlink direction.

The RAN1 study for NCR can include the functional architecture issues outline above, such as operations/tasks of the NCR-RU and NCR-MT, corresponding capabilities and physical layer procedures, and resource multiplexing aspects.

Proposal 1: Study the functional architecture of NCR, including operations/tasks of the NCR-RU and NCR-MT, corresponding capabilities and physical layer procedures, and resource multiplexing aspects.

# Protocol architecture of NCR

Design of the L1/L2 signaling for the NCR inherently depends on the protocol architecture supported by the NCR. Such design will have direct impact on how higher layer information is provided to the NCR-MT, and how L1/L2 signaling is exchanged between the NCR-MT and the gNB.

The NCR-RU supports only an L0/RF layer in the user plane (UP) without any component in the control plane (CP).

The NCR-MT also supports only an L0/RF layer in the UP, but supports one or multiple layers in the CP. Among various options for CP design for NCR-MT, supporting a full UE protocol stack, including support for RRC configuration, is a preferred solution, as it increases the system/operation flexibility and minimizes the specification impact, without material increase in the NCR implementation cost or complexity. In particualr, various exisiting designs in 5G NR can be re-used for configuration or L1/L2 signaling of the NCR. Details of the signaling mechanisms are discussed in Section 4.

Observation 7: Supporting a full UE protocol stack, including support for RRC configuration, increases the system/operation flexibility and minimizes the specification impact, without material increase in the NCR implementation cost or complexity.

Figures 3a and 3b show the proposed protocol architecture for the NCR in the user plane and control plane, repectively.

Figure 3-a: Proposed protocol architecture for the NCR – user plane (UP)

Figure 3-b: Proposed protocol architecture for the NCR – control plane (CP)

The RAN1 study on NCR should conclude on the protocol architecture aspects, including the supported protocol stack and accordingly the configuration method for the NCR-MT. It is preferred that RAN1 makes a conclusion on the method for signaling of higher layer information to the NCR within the RAN1#109-e meeting.

Proposal 2: Study the protocol architecture for the NCR, including the supported protocol stack for the NCR-MT, and the method for signalling the higher layer information to the NCR-MT.

# L1/L2 signaling design for NCR

Various options can be considered for the design of the L1/L2 signaling for NCR, as briefly described in this section. In the following, it is assumed that the UE is already provided higher layer information, e.g., via RRC configuration. Such higher layer information includes configuration of time/frequency resources for the L1/L2 signaling methods discussed in the following.

The following general categories can be considered as candidate L1/L2 signaling mechanisms for the NCR:

- Signaling based on control channel only, such as PDCCH only;

- Signaling based on data channel only, such as PDSCH only (e.g., SPS PDSCH with higher layer activation);

- Signaling based on both control and data channel, such as PDCCH + PDSCH (e.g., SPS PDSCH with DCI activation or dynamically scheduled PDSCH).

In addition, when PDSCH is selected as (part of) the NCR signaling method, a simple method is to treat the DL NCI as L2 control information which is included in the PDSCH in the form of a MAC CE. Corresponding methods can be used for signaling of UL NCI.

Observation 8: Candidate signalling methods for the NCR include: PDCCH only, PDSCH only; or PDCCH + PDSCH. When PDSCH is used as (part of) the NCR signalling method, the DL NCI can be L2 control information in the form of a MAC CE.

Selection of a method for L1/L2 signaling for NCR among the above candidates needs to consider a number of factors such as the following:

- a payload size of the NCI: smaller payload size for NCI better fits a PDCCH, while larger payload sizes are best communicated via a PDSCH;

- a periodicity or time-domain ‘density’ of the NCI: long-term and less frequent NCR signaling can be communicated on a PDCCH only or dynamically scheduled PDSCH, while short-term and more frequent NCR signaling is best exchanged via a SPS PDSCH with higher layer or DCI activation, for more efficient resource utilization;

- a complexity associated with NCI signaling: exchange of DL NCI with less need for PDCCH blind decoding, such as by SPS PDSCH, reduces the NCR implementation cost/complexity, while signaling of DL NCI based on frequent PDCCH blind decoding increases the NCR implementation cost/complexity;

- a supported protocol stack: an NCR-MT with a protocol stack supporting at least up to L2 can readily handle MAC-CEs, but an NCR-MT supporting only L1 needs to exchange ‘DL NCI format’ as L1 information on a PDCCH;

- specification efforts: specification of (new) MAC CEs for exchange of NCI is more straightforward (e.g., similar to MAC CEs for IAB), while specification of “DL NCI formats” may need additional efforts.

Observation 9: Selection of a signalling method for the NCR needs to consider at least: a payload size of the NCI, time-domain periodicity/density of the NCI, a complexity associated with NCI signaling, a protocol stack supported by the NCR-MT, and the required specification efforts.

The RAN1 study on NCR should study the various possible L1/L2 signaling methods considering impacts on signalling overhead, resource utilization, cost/complexity considerations, and specification efforts. Based on the discussion above, including DL NCI as MAC CEs on a SPS PDSCH appears to be an efficient approach.

Proposal 3: Study L1/L2 signalling methods for NCR-MT considering impacts on signalling overhead, resource utilization, cost/complexity considerations, and specification efforts.

# Conclusion

This contribution considered architecture aspects and signalling design for network-controlled repeaters (NCR), and proposed the following.

Proposals

Proposal 1: Study the functional architecture of NCR, including operations/tasks of the NCR-RU and NCR-MT, corresponding capabilities and physical layer procedures, and resource multiplexing aspects.

Proposal 2: Study the protocol architecture for the NCR, including the supported protocol stack for the NCR-MT, and the method for signalling the higher layer information to the NCR-MT.

Proposal 3: Study L1/L2 signalling methods for NCR-MT considering impacts on signalling overhead, resource utilization, cost/complexity considerations, and specification efforts.

In addition, the following observations were made.

Observations

Observation 1: An NCR consists of two functional modules: NCR-RU that performs the amplify-and-forward operation, and NCR-MT that facilitates the signaling exchange with the gNB.

Observation 2: A Rel-18 NCR is aimed at network coverage extension; therefore, the NCR is expected to be the only path for the gNB to reach the UE(s) that are served through the NCR.

Observation 3: The NCR-MT transmits uplink NCR control information (UL NCI) to the gNB, in addition to receiving downlink NCR control information (DL NCI) from the gNB; otherwise, the repeater would be simply a network-informed repeater, rather than a network-controlled repeater.

Observation 4: The NCR-MT handles the identification and authorization procedures for the NCR by performing the initial/random access procedure.

Observation 5: An NCR hardware implementation can be based on a shared RF path/chain for both NCR-RU and NCR-MT, or two separate RF paths/chains for the NCR-RU and NCR-MT.

Observation 6: A baseline approach for resource multiplexing between NCR-RU and NCR-MT is TDM. It is also possible to consider FDM or SDM for the downlink direction.

Observation 7: Supporting a full UE protocol stack, including support for RRC configuration, increases the system/operation flexibility and minimizes the specification impact, without material increase in the NCR implementation cost or complexity.

Observation 8: Candidate signalling methods for the NCR include: PDCCH only, PDSCH only; or PDCCH + PDSCH. When PDSCH is used as (part of) the NCR signalling method, the DL NCI can be L2 control information in the form of a MAC CE.

Observation 9: Selection of a signalling method for the NCR needs to consider at least: a payload size of the NCI, time-domain periodicity/density of the NCI, a complexity associated with NCI signaling, a protocol stack supported by the NCR-MT, and the required specification efforts.

# References

- RP-213700, “New SI: Study on NR Network-controlled Repeaters”, ZTE Corporation

- RP-210818, “New WID on NR Repeaters”, Qualcomm

- RP-220544, “WI summary for WI: NR Repeaters”, Qualcomm

- R1-2203921, “Side control information to enable NR network-controlled repeaters”, Samsung