3GPP TSG RAN WG1 #109-e	        		             R1-2203923

e-Meeting, May 9th – 20th, 2022

Agenda item:		9.9.1

Source:			Samsung

Title:			Considerations on PDCCH receptions in symbols with LTE CRS

Document for:		Discussion and decision

# Introduction

This contribution discusses aspects related to the support of PDCCH receptions in symbols with LTE CRS [1].

# Discussion

The motivation, for deployments with NR-LTE coexistence, to support PDCCH receptions in symbols with LTE CRS is to increase NR PDCCH capacity within the first 3 symbols of a slot (e.g. from 2 to 3 in case of 2 CRS ports, or from 1 to 2 or 3 in case of 4 CRS ports). The issue is a well known one and has been already addressed in Rel-15 by defining a UE capability to monitor PDCCH in CORESETs located after the first 3 symbols of a slot.

For NR UEs supporting CA, further enhancements were pursued in Rel-16 and Rel-17 to offload PDCCH monitoring from the PCell. As CA is a typical feature of NR UEs, the Rel-17 scheduling on the PCell from the sSCell is an effective mechanism to increase NR PDCCH capacity.

Observation 1: Current NR releases enable increased NR PDCCH capacity for LTE-NR coexistence.

If a UE capability to support PDCCH receptions in symbols with LTE CRS is defined in Rel-18, a network will still have to provide service to Rel-15/16/17 NR UEs which are likely to exist longer than LTE UEs (and once the latter UEs are phased out, the Rel-18 UE capability will not be of any use).

Observation 2: For a network that supports LTE-NR coexistence, NR UEs of current releases are likely to be present longer than LTE UEs.

CORESET#0 will need to avoid symbols with LTE CRS as it is used for PDCCH transmissions associated with UEs of all releases (for SI, RAR, paging) even if the CRS pattern was to be provided by SIB prior to dedicated RRC connection for Rel-18 UEs. The second CORESET that a UE supports will need to be different for Rel-15/16/17 UEs and for Rel-18 UEs and that can often lead to resource fragmentation and CORESET resources can be underutilized. Further, the second CORESET will need, in practice, to have same BW and location as CORESET#0 as otherwise scheduling of LTE UEs will be significantly constrained (LTE PDSCH cannot be scheduled in any symbol that includes RBs that are also included in any CORESET). That means that the second CORESET can only be over symbols that include CRS (cannot extend to the third symbol). Further, due to the presence of LTE UEs and the RRC-based configuration of CORESETs, the first symbol cannot be used (there is also PCFICH and PHICH which makes the first symbol practically unusable for NR PDCCH) unless the scheduler is willing to support significant further restrictions on when/how to schedule LTE UEs and NR UEs in both the DL and the UL, including for configured PUSCH in LTE due to the PHICH existence. A positive consequence from using only the second symbol is that the REG bundle will be over 6 RBs and a BLER degradation due to DM-RS puncturing will be contained.

Observation 3: A second CORESET for Rel-18 UEs can practically be only in the second symbol and is likely to have same BW and location as CORESET#0.

From a capacity/performance perspective and considering presence of the second CORESET only in the second symbol, the CRS will puncture 1/3 of the REs used for NR PDCCH of Rel-18 UEs. Ignoring the impact on channel estimation from puncturing DM-RS REs for interpolation in the frequency domain, the gain in NR PDCCH capacity will be 2/3 of the one available in a CORESET located in the third symbol. Due to the quantization of CCE aggregation levels and due to some additional performance loss from degraded channel estimation, the overall additional capacity would be closer to 50% of the one provided by a single-symbol CORESET that does not include CRS REs. For a 5/10 MHz BW, there will be the equivalent of 2/4 additional CCEs available.

Observation 4: The second CORESET for Rel-18 UEs can offer ~50% more capacity - that additional capacity cannot be utilized by UEs of current releases and would increase fragmentation of NR control resources. That additional capacity is only 2/4 CCEs for a CORESET BW of 5/10 MHz.

PDCCH decoding is based on resource granularity in terms of REGs and REG bundles. If specific REs, e.g. ones corresponding to CRS, are to be excluded from the PDCCH resources, that would not be consistent with existing UE implementation and would add to the UE complexity.

Observation 5: PDCCH decoding implemented by UEs of current releases does not remain applicable for PDCCH receptions in a CORESET that includes CRS REs and would need to be redesigned.

As the DM-RS spacing for NR PDCCH is 4 REs and the CRS spacing is 3 REs, collisions cannot be avoided for any CRS RE shift. A UE will then need to re-design the channel estimator in order to skip punctured REs and cannot apply mechanisms based on a uniform DM-RS pattern in the frequency domain. PDCCH channel estimation in current releases utilizes a uniform DM-RS pattern in the frequency domain (and also in the time domain although that is not relevant to the present case). When a DM-RS REs in each PRB is punctured from the CRS, non-uniform interpolation would be required for channel estimation. Although it can be arguable how complex it would be for a UE to implement an additional channel estimator (including potential configurability for a channel estimator), it is clear that the present one would not be sufficient when some DM-RS REs are punctured. The Rel-15 UE capability for performing channel estimation over 56 non-overlapping CCEs per slot at 15 kHz may also need to be revisited.

Observation 6: The channel estimator implemented by UEs of current releases would not be sufficient for PDCCH receptions in a CORESET that includes CRS REs and would need to be redesigned.

The above analysis did not consider that interference can be from more than one CRS as justified to often be the case for the other component of the present WI (use of two CRS patterns for rate matching). For such scenarios, having a CORESET include symbols with CRS REs would be meaningless. In practice, a similar situation would exist even for a single CRS pattern as neighbouring cells would not be perfectly synchronized and residual CRS interference would be more damaging to PDCCH (including the PDCCH DM-RS) than to PDSCH that benefits from larger BLER and HARQ (and CRS does not affect the PDSCH DM-RS).

Observation 7: There is no benefit from having a CORESET include symbols with CRS REs in cases that a UE needs to be configured two CRS patterns for rate matching/puncturing.

In conclusion, a benefit from having a CORESET include symbols with CRS REs would be limited (the equivalent of 2-4 more CCEs) or non-existent (e.g. more than one CRS pattern needs to apply for puncturing). It may have been possible to provide a larger benefit if that design was considered from Rel-15 but the alternative to have a UE capability for PDCCH receptions in CORESETs that include symbols after the first three ones in a slot was specified. Rel-17 also allows scheduling on the PCell from the sSCell. Both of those approaches offer substantially larger PDCCH capacity.

Observation 8: A benefit from having a CORESET include symbols with CRS REs would be limited or non-existent.

# Conclusions

This contribution considered potential support for CORESETs that include symbols with CRS REs. The following are observations are made.

Observation 1: Current NR releases enable increased NR PDCCH capacity for LTE-NR coexistence.

Observation 2: For a network that supports LTE-NR coexistence, NR UEs of current releases are likely to be present longer than LTE UEs.

Observation 3: A second CORESET for Rel-18 UEs can practically be only in the second symbol and is likely to have same BW and location as CORESET#0.

Observation 4: The second CORESET for Rel-18 UEs can offer ~50% more capacity - that additional capacity cannot be utilized by UEs of current releases and would increase fragmentation of NR control resources. That additional capacity is only 2/4 CCEs for a CORESET BW of 5/10 MHz.

Observation 5: PDCCH decoding implemented by UEs of current releases does not remain applicable for PDCCH receptions in a CORESET that includes CRS REs and would need to be redesigned.

Observation 6: The channel estimator implemented by UEs of current releases would not be sufficient for PDCCH receptions in a CORESET that includes CRS REs.

Observation 7: There is no benefit from having a CORESET include symbols with CRS REs in cases that a UE needs to be configured two CRS patterns for rate matching/puncturing.

Observation 8: A benefit from having a CORESET include symbols with CRS REs would be limited or non-existent.

# References

- RP-213575, “New WI: Enhancement of NR Dynamic spectrum sharing (DSS)”