3GPP TSG RAN WG1 #109-e	R1-2203925

# e-Meeting, May 9th – 20th, 2022

Agenda item:	9.10.1

Source:	Samsung

Title:	Multi-cell PUSCH/PDSCH scheduling with a single DCI

Document for:	Discussion and decision

# Introduction

This contribution discusses joint scheduling of PDSCHs/PUSCHs on multiple serving cells using a single DCI format, as considered in the following objective from the new Rel-18 WI for multi-carrier enhancements [1].

1. Specify a solution for multi-cell PUSCH/PDSCH scheduling (one PDSCH/PUSCH per cell) with a single DCI [RAN1] - Identify the maximum number of cells that can be scheduled simultaneously - Consider both intra-band and inter-band CA operation - Consider both FR1 and FR2 - The single DCI shall be optimized for 3 or more cells for the multi-cell PUSCH/PDSCH scheduling

Throughout this document, a set of serving cells that are jointly scheduled by a single DCI format are referred to as “co-scheduled” cells.

# Scenarios and general considerations

The WID in [1] considers a wide range of CA settings for multi-cell scheduling, including intra-band, inter-band, FR1, and FR2. The feasibility of a single DCI format with reasonable size for scheduling multiple PDSCHs/PUSCHs on multiple cells inherently depends on the commonalities of channel conditions and radio characteristics. The more common the scheduling parameters, the higher the savings in DCI format size. This was already observed in the simulation results provided during the Rel-17 study on two-cell scheduling [2]. It is evident that such commonalities are more available in intra-band, collocated CA, compared to inter-band or non-collocated CA scenarios. Therefore, the WI can focus on intra-band, collocated CA as a main scenario, although non-collocated or inter-band CA can/should be also considered to benefit operators with scattered spectrum in different frequency bands.

Observation 1: DCI overhead savings are more pronounced for intra-band, collocated CA scenario due to increased commonalities of channel conditions and radio characteristics.

Proposal 1: Prioritize the intra-band, collocated CA scenario in the multi-cell scheduling designs, while considering inter-band or non-collated CA as well.

One of the issues raised in the WID for multi-cell scheduling [1] is to “identify the maximum number of cells that can be scheduled simultaneously”. Selecting such maximum number needs to consider the trade-off among DCI size savings, scheduling flexibility, RRC overhead, and so on, as discussed in Section 3. In addition, the maximum number of co-scheduled cells also depends on the method used for multi-cell scheduling, as discussed in Section 4.

Observation 2: The maximum number of cells that can be jointly scheduled using a single DCI format depends on a trade-off among DCI format size, scheduling flexibility, RRC overhead, and the method used for multi-cell scheduling.

A maximum number of DL/UL co-scheduled cells, such as 8, can be concluded once the DL/UL DCI format design for multi-cell scheduling is progressed in order to determine the required payload for the DCI format and a resulting code rate and requirements for CCE aggregation levels.

Observation 3: A maximum number of DL/UL co-scheduled cells using a DL/UL DCI format can be concluded once aspects for DL/UL DCI formats are progressed.

One basic issue for multi-cell scheduling of PDSCHs/PUSCHs is how to indicate a set of co-scheduled cells in a DCI format for multi-cell scheduling. In 5G NR until Rel-17, a carrier indicator field (CIF) in cross-carrier scheduling configuration provides a value of {1, …,7} to indicate a scheduled cell corresponding to a scheduling cell; the CIF value 0 is reserved for self-scheduling PDSCH/PUSCH on the scheduling cell. The multi-cell scheduling operation in NR Rel-18 is an extension of the cross-carrier scheduling operation and the CIF framework can be extended to ‘set-level’ CIF configuration to indicate sets of co-scheduled cells. Accordingly, the UE can be configured one or more CIF values that correspond to one or more sets of co-scheduled cells corresponding to a scheduling cell. Such set-level CIF values can be used in a DCI format for multi-cell scheduling to indicate a set of co-scheduled cells, and can be also used in the PDCCH search space equation for monitoring DCI formats for multi-cell scheduling, as discussed in Section 5. The value range for set-level CIF can be determined based on the number/granularity of sub-sets for co-scheduled cells, for a configured set of co-scheduled cells, that can be indicated.

Proposal 2: RRC configures ‘set-level’ CIF values that correspond to sub-sets of co-scheduled cells from a set of co-scheduled cells.

# DCI fields

DCI fields in a DCI format for multi-cell scheduling can be broadly categorized into two different types:

- “Cell-common” DCI fields that provide a shared value for all co-scheduled cells. In other words, a single value for a cell-common DCI field is applicable to all serving cells in a set of co-scheduled cells. Examples can include FDRA for intra-band CA, (possibly) TDRA for intra-band CA, PUCCH resource indication (PRI), HARQ-ACK feedback timing, TPC command for PUCCH, and others. Such DCI fields (including the CRC) with a combined total number of M bits provide significant DCI overhead savings of (N-1)*M bits for N co-scheduled cells.

- “Cell-specific” DCI fields that provide an individual value for each of the co-scheduled cells. In other words, a cell-specific DCI field indicates multiple values corresponding to multiple serving cells in a set of co-scheduled cells. Examples include HARQ process number (HPN), RV, MCS, and so on, as described next. Such DCI fields increase the DCI format size and can affect the maximum number of co-scheduled cells particularly in case of spatial multiplexing (2 TBs per PDSCH).

Some fields, such as FDRA or TDRA, can be configured either as cell-common (e.g., for intra-band CA operation) or cell-specific (e.g., for inter-band CA operation).

Observation 4: The DCI fields for multi-cell scheduling can be categorized into cell-common and cell-specific, for serving cells in a set of co-scheduled cells. Some fields can be configured whether they are cell-common or cell-specific.

To avoid large code rates resulting from a large DCI format payload (considering that no more coding gains are available for aggregation levels of 8 CCEs and 16 CCEs that basically introduce repetitions) cell-specific DCI fields should have a relatively small number of bits, when possible. Configurability of cell-specific field sizes is generally possible (expect for MCS) and should be considered in the DCI format design.

Observation 5: For multi-cell scheduling of PDSCHs/PUSCHs with a single DCI, solutions minimizing the increase in DCI size without material performance penalty should be prioritized.

At least, four different non-exclusive methods can be used for indication of cell-specific DCI fields with reduced bit-width:

- Explicit separate indication with restricted value set: the DCI format includes multiple explicit DCI fields, each of which corresponds to one of the co-scheduled cells. Examples include HPN, RV, NDI, and so on. Such indication ensures high scheduling flexibility, and can result in some DCI overhead savings when the possible configuration values are restricted, such as only 2 possible RV values, or only 4 HARQ processes, for multi-cell scheduling.

- Explicit differential indication: the DCI format includes one reference value for one serving cell, and differential values for remaining serving cells from the set of co-scheduled cells. An example can be differential MCS for intra-band CA as path-loss is same and the savings can be significant especially for a large number of co-scheduled cells (e.g., 21 bits are saved for 8 co-scheduled cells when a differential MCS of 2 bits is used relative to the reference MCS of 5 bits).

- Single indication based on “multi-cell mapping”: the DCI format includes one DCI field for a scheduling parameter that corresponds to multiple values for the scheduling parameter for respective multiple co-scheduled cells. Higher layer signaling can provide a mapping for a joint indication of a parameter for multiple co-scheduled cells using a joint multi-cell configuration table or a multi-cell ‘code-point’, or a multi-cell offset. Examples include a joint TDRA table, a multi-cell trigger state for aperiodic ZP CSI-RS, or multi-cell rate matching pattern indication. Such indication may result in increased RRC overhead but reduces DCI overhead while maintaining scheduling flexibility for multi-carrier scheduling purposes.

- Omission of optional parameters from a multi-cell scheduling DCI format: the DCI format does not include indication for a scheduling parameter such as, a BWP indication field.

Observation 6: Various methods can be used for reducing the combined overhead of cell-specific DCI fields in a DCI format for multi-cell scheduling.

Considering all aspects mentioned above, maximum values for the payloads of DL/UL DCI formats for multi-cell scheduling need to be identified considering which DCI fields can be cell-common by default or by configuration, which DCI fields need to be cell-specific, and how the combined payload of cell-specific DCI fields can be reduced. Tables 1 and 2 can be starting points for such discussion.

Proposal 3: For a DCI format used for multi-cell scheduling, RAN1 to conclude on:

- which DCI fields are cell-common, by default or by configuration, and which DCI fields are cell-specific;

- indication method for each cell-specific field:

  - explicit separate indication with restricted value set

  - explicit differential indication

  - single indication based on “multi-cell mapping”

  - no indication

Tables 1 and 2 can be starting points for the RAN1 discussions.

Table 1: DCI fields for multi-cell scheduling of PDSCH receptions

| DCI field | Cell-common or Cell-specific | Note |
| --- | --- | --- |
| DL/UL indication | Cell-common | All are PDSCHs |
| CIF | Cell-common | set-level CIF |
| BWP-ID | N/A | no BWP switching via multi-cell DCI |
| FDRA, TDRA | Cell-common (for intra-band CA), Cell-specific (for inter-band CA) | Same row of single-cell TDRA, or joint TDRA table |
| VRB-to-PRB mapping, PRB bundling | N/A or cell-common | follow RRC when disabled |
| Rate Matching pattern indication, ZP CSI-RS trigger | Cell-specific | multi-cell trigger state |
| MCS | Cell-specific | Differential MCS |
| NDI | Cell-specific |  |
| RV, HPN | Cell-specific | restricted value set |
| MCS, NDI, RV for 2nd TB | Cell-specific | if configured |
| PUCCH resource indicator (PRI), PUCCH power control | Cell-common | Only one PUCCH resource |
| PDSCH-to-HARQ timing (K1) | Cell-common | Relative to the reference PDSCH |
| DAI | Cell-common | DAI counting per DCI (not per PDSCH) |
| CBGTI, CBGFI | N/A | CBG operation disabled |
| One-shot HARQ request | Cell-common | implicit ‘set-specific’ trigger |
| TCI state | Cell-specific | follow unified TCI framework |
| DMRS sequence initialization | N/A | by RRC |

Table 2: DCI fields for multi-cell scheduling of PUSCH transmissions

| DCI field | Cell-common or Cell-specific | Note |
| --- | --- | --- |
| DL/UL indication | Cell-common | All are PUSCHs |
| CIF | Cell-common | set-level CIF |
| SUL, BWP-ID | N/A | no carrier / BWP switching via multi-cell DCI |
| FDRA, TDRA | Cell-common (for intra-band CA), Cell-specific (for inter-band CA) | Same row of single-cell TDRA, or joint TDRA table |
| Frequency hopping (FH) | Cell-common |  |
| PUSCH TPC command | Cell-specific |  |
| MCS | Cell-specific | Differential MCS |
| NDI | Cell-specific |  |
| RV, HPN | Cell-specific | restricted value set |
| SRI | N/A or cell-specific | Follow unified TCI framework |
| TPMI | Cell-specific |  |
| UL-SCH, beta offset | N/A or cell-common | When enabled, only one PUSCH includes CSI/UCI |
| DMRS sequence initialization | N/A | by RRC |

# Mechanisms for joint scheduling

Several mechanisms can be considered to provide a DCI format for multi-cell scheduling depending on the corresponding payload.

In a first approach, a single ‘concatenated’ DCI format can be used for all relevant DCI fields (including cell-common and cell-specific DCI fields). This approach will require introduction of new DCI formats, such as DCI format 1_3 for multi-cell scheduling of PDSCHs and 0_3 for multi-cell scheduling of PUSCHs. The size of the concatenated DCI format can be reduced by using various techniques described in Section 3, but is still expected to be larger than that for single-cell scheduling DCI formats (unless restrictions are placed in the configuration of fields for the single-cell and the multi-cell scheduling DCI formats which is generally undesirable). Therefore, the first approach is beneficial for multi-cell scheduling of few co-scheduled cells and/or for collocated and intra-band CA scenarios with high commonalities for scheduling parameters among co-scheduled cells.

In a second approach, a DCI format for multi-cell scheduling can be include two parts where the second part is multiplexed in a PDSCH scheduled by the first part, similar to the two-stage SCI for sidelink operation. The UE can determine whether or not a PDSCH includes a second part of a multi-cell scheduling DCI format (M-DCI, for brevity) based on an indication by the DCI format that schedules the PDSCH (e.g. a dedicated 1-bit field or a separate RNTI). The UE procedure for de-multiplexing M-DCI from a PDSCH can be defined similar to the existing UE procedures for multiplexing UCI in PUSCH. The first DCI can additionally include some of the scheduling parameters for multi-cell scheduling, such as the cell-common DCI fields. Since the multi-cell scheduling DCI format is multiplexed in a PDSCH, there is little constraint in terms of the size of M-DCI. Therefore, the second approach can be beneficial especially for a large number of co-scheduled cells in various CA scenarios, including both intra-band and inter-band CA. The second approach may also avoid additional considerations on the number of DCI format sizes that a UE can decode.

In a third approach, a DCI format for multi-cell scheduling can again include two parts but instead of the second part provided by a PDSCH, it is provided by a PDCCH using linked PDCCH candidates for the first part and the second part (similar to the Rel-17 for multi-TRP operation but the PDCCHs do not need to be associated with different search space sets). For example, the first part can include the cell-common fields and the second part can include the cell-specific fields for multi-cell scheduling. To avoid increasing blind decoding requirements, RRC configuration or DCI indication can provide the UE with information of the linked PDCCHs (similar to Rel-17 M-TRP operation). The third approach is also beneficial for reducing the scheduling latency and for maintaining the same procedures for DCI format detection as in Rel-17 for the Uu link (no new UE/gNB implementations).

RAN1 needs to discuss and select one (or maybe two) mechanism(s) for multi-cell scheduling that can accommodate different CA scenarios considering various KPIs and trade-offs, as described earlier.

Proposal 4: For a multi-cell scheduling DCI format, further consider the following three mechanisms:

- single ‘concatenated’ DCI format in a PDCCH;

- DCI format for multi-cell scheduling multiplexed in a PDSCH (a.k.a., two-stage DCI with 2nd stage on a PDSCH);

- two-stage DCI on linked PDCCHs.

# PDCCH monitoring aspects

Multi-cell scheduling can impact UE procedures for PDCCH monitoring.

A first issue is how to distinguish a DCI format for single-cell scheduling from a DCI format for multi-cell scheduling. Such distinction can depend on the mechanism for multi-cell scheduling, such as those described in Section 4. If a new DCI format size for multi-cell scheduling is introduced, it can provide the differentiation; otherwise, an explicit indication/flag in the DCI format is needed.

Observation 7: Distinction of a DCI format for multi-cell scheduling from a DCI format for single-cell scheduling can be based on a new DCI format size for multi-cell scheduling, or an explicit indication/flag in the DCI formats.

A second issue is search space set configuration and search space set linking for multi-cell scheduling. In Rel-17, a search space set for cross-carrier scheduling is configured on the scheduling cell (with a full configuration) as well as on the scheduled cell (with a ‘light’ configuration). The UE determines the PDCCH candidates for a scheduled cell based on the following search space equation, where nCI is the configured CIF value for a scheduled cell.

L⋅Yp,ns,fμ+ms,nCI⋅NCCE,pL⋅Ms,max(L)+nCImodNCCE,pL+i

For Rel-18 multi-cell scheduling, a UE can be configured search space sets for PDCCH monitoring for both single-cell scheduling and multi-cell scheduling DCI formats. A search space set according to which the UE can monitor PDCCH for detection of a DCI format for multi-cell scheduling can be configured on the scheduling cell and (i) none of the scheduled cells, or (ii) some cells from a set of co-scheduled cells, or (iii) all cells in a set of co-scheduled cells. The UE can determine the PDCCH candidates based on the search space equation above by considering a set-level CIF (for a set of co-scheduled cells) or a cell-level CIF corresponding to (some of the) scheduled cell(s) on which the linked search space set(s) are configured. The former option is useful for UE implementation since the UE will know, prior to DCI decoding, which cell or which set of co-scheduled cells the DCI format can possibly schedule. The method for association between a search space set and a set-level CIF needs to be established, such as by a multi-cell extension of the search space linking procedure or by higher layer configuration.

Proposal 5: The nCI value in the search space equation for PDCCH monitoring for multi-cell scheduling corresponds to a set of co-scheduled cells. FFS on the method for associating a search space set and a set-level CIF.

A third issue is the PDCCH monitoring limits for operation with multi-cell scheduling. Unlike multi-TRP operation, multi-cell scheduling does not change/increase a UE budget for PDCCH monitoring. Rather, multi-cell scheduling is a new method for consuming the UE budget for PDCCH monitoring by using fewer PDCCHs for scheduling PDSCH/PUSCH on multiple serving cells. Therefore, there is no impact, due to multi-cell scheduling, on the UE limits for a maximum number of PDCCH candidates or non-overlapping CCEs that the UE can monitor.

For counting the PDCCH candidates and non-overlapping CCEs, a PDCCH candidate or CCEs associated with multi-cell scheduling should be inversely scaled by the number of co-scheduled cells. For example, for a set with 4 co-scheduled cells, the UE can count one PDCCH candidate as 1/4 PDCCH candidates and count L non-overlapping CCEs as L/4 non-overlapping CCEs for each of the 4 co-scheduled cells. The inverse scaling applies regardless of whether or not the UE detects a DCI format in the monitored PDCCH candidate, and regardless of whether a detected DCI format in the monitored PDCCH candidate schedules the same number or a different number of co-scheduled cells as the set of co-scheduled cells associated with the search space set.

Proposal 6: Define the counting of PDCCH candidates and non-overlapping CCEs for multi-cell scheduling.

A fourth issue is about a specifications limit on a UE budget for the number of DCI format sizes per serving cell, sometimes referred to as, the “3+1” rule. When the UE monitors a DCI format for multi-cell scheduling, the DCI pertains to multiple serving cells, then clarification is needed on how to determine a number of DCI sizes per cell. Various options can be considered to modify the “3+1” rule in the presence of multi-cell scheduling. However, it appears that the “3+1” budget for DCI sizes was initially considered in Rel-15 due to early implementation plans/restrictions for 5G devices. Maintaining such restriction seems not necessary anymore, so RAN1 can consider voiding the limit, at least for multi-cell scheduling.

Observation 8: The “3+1” rule for DCI size budget is rooted in early implementation plans for 5G devices, that do not appear to be applicable anymore.

Proposal 7: Further discuss the “3+1” limit on UE budget for DCI sizes, including voiding the limit for the case of multi-cell scheduling.

# HARQ-ACK procedures

Multi-cell scheduling can also impact UE procedures for HARQ-ACK codebook generation.

A first issue that is applicable to all HARQ-ACK codebooks is determination of a PUCCH resource/slot for transmission of HARQ-ACK information feedback corresponding to multiple PDSCHs on a set of co-scheduled cells. For DL multi-cell scheduling, the typical case is that the scheduled PDSCHs are distributed closely in time domain and it is straightforward to transmit the HARQ-ACK of all co-scheduled PDSCHs in a same PUCCH. Transmitting the HARQ-ACK of the scheduled PDSCHs in different PUCCHs only complicates the spec and implementation and increases resource utilization and control overhead without any gains and thus should not be supported.

Observation 9: For multiple PDSCHs scheduled on multiple serving cells by a same DCI format, the HARQ-ACK information of all the scheduled PDSCHs can be multiplexed in a same PUCCH.

A DCI format for multi-cell scheduling includes a single field for PDSCH-to-HARQ timing (K1), therefore, the PUCCH slot is indicated with respect to one of the co-scheduled PDSCHs. Therefore, determination of the PUCCH slot for multi-cell scheduling needs further clarification. For example, the K1 timing for the PUCCH slot can be with respect to the last co-scheduled PDSCH, or with respect to a PDSCH on a reference cell from the set of co-scheduled cells. For example, the reference cell can be based on the cell with the largest cell index as for determining the last DCI. Alternatively, the cell with the smallest cell index can be used for this case. The usage of a reference cell can have an added benefit as it can also be used for determining the last DCI format and DAI counting for Type-2 HARQ-ACK codebook as a unified solution.

Proposal 8: Define a reference PDSCH for determination of the PUCCH resource/slot with HARQ-ACK corresponding to multiple PDSCHs scheduled on multiple serving cells by a multi-cell scheduling DCI format.

For Type-1 HARQ-ACK codebook (CB), the UE procedure in Rel-15/16 is based on a pseudo-code to determine candidate PDSCH occasions corresponding to a PUCCH slot based on the K1 value indicated in the single-cell scheduling DCI format and the TDRA table configured for PDSCH reception on the cell. Enhancements to the Type-1 CB generation were defined as part of the “Beyond 52 GHz” WI for the case of multi-PDSCH scheduling to consider both K1 and relative K0 values from a joint TDRA table, so that the UE can ‘back-trace’ from a last PDSCH and identify the remaining candidate PDSCH occasions from the set of co-scheduled PDSCHs. However, the Rel-17 multi-PDSCH scheduling was restricted to a same serving cell, therefore a same SCS configuration is applicable to all the co-scheduled PDSCHs.

In Rel-18 multi-cell scheduling, it is possible that the co-scheduled cells are provided different SCS configuration on the corresponding active DL BWPs. Therefore, the UE needs to take the different SCS configurations into account when determining the candidate PDSCH occasions.

In addition, the TDRA table for multi-cell scheduling needs to be clarified. One option is to re-use the TDRA tables for single cell scheduling for each serving cell from a set of co-scheduled cells. Another option is to define a joint TDRA table similar to Rel-17 multi-PDSCH scheduling. Further discussion is needed to select one of the two options considering DCI overhead, scheduling flexibility, and specifications impact for Type-1 HARQ-ACK codebook generation.

Proposal 9: Consider requirements for supporting Type-1 HARQ-ACK codebook for co-scheduled PDSCHs on a set of co-scheduled cells with different SCS configurations and joint or separate TDRA tables.

For Type-2 HARQ-ACK CB generation, it is generally expected that UE procedures follow those for Rel-17 multi-PDSCH scheduling, such as DAI counting per DCI format (and not per PDSCH), generating a number of HARQ-ACK bits per DCI format for multi-cell scheduling equal to a maximum number of co-scheduled cells, using separate Type-2 sub-CBs for single-cell scheduling and multi-cell scheduling, and not supporting CBG operation in the case of multi-cell scheduling.

Observation 10: To minimize specifications impact, Type-2 HARQ-ACK codebook generation can aim to re-use procedures defined for Rel-17 multi-PDSCH scheduling, when possible.

However, some issues need to be clarified for multi-cell scheduling. One issue is about the definition of counter DAI which is used for a UE to determine missing DCI formats and reflect them (with DTX/NACKs) in the CB generation. The counter DAI is defined as the accumulative number of {serving cell, PDCCH monitoring occasion}-pairs in which PDSCH receptions or ‘special’ DCIs/PDCCHs requiring HARQ-ACK feedback information, is present up to the current serving cell and current PDCCH monitoring occasion. Such definition is clear for single-cell scheduling.

For multi-cell scheduling, a PDCCH monitoring occasion that includes a DCI format for multi-cell scheduling corresponds to multiple PDSCHs on multiple serving cells. Therefore, definition of counter DAI needs to be clarified. For example, the “serving cell” in the {serving cell, PDCCH monitoring occasion}-pair for a multi-cell scheduling DCI format needs to be defined.

Another issue for the Type-2 HARQ-ACK codebook for multi-cell scheduling is the ordering of HARQ-ACK information bits. In Rel-17 the ordering for co-scheduled PDSCH on a same serving cell is based on the increasing order of the PDSCH reception starting time. In Rel-18 multi-cell scheduling, the UE receives the co-scheduled PDSCHs in multiple serving cells, with potentially different TDRAs or SCS configurations. Another option is that the HARQ-ACK bits can be ordered in the ascending order of cell index for a DCI format. Corresponding ordering of HARQ-ACK information bits needs to be clarified.

Proposal 10: Determine counter DAI definition and ordering of HARQ-ACK information bits in a Type-2 HARQ-ACK codebook for multi-cell scheduling.

A further issue for Type-2 CB is whether to support HARQ bundling across co-scheduled cells. In Rel-17 multi-PDSCH scheduling, HARQ bundling of the multiple scheduled PDSCHs on a same serving cell is supported. However, for multi-cell scheduling, the channel conditions of different cells may be quite different, and the decoding relationship between different cells may be limited. Supporting HARQ bundling for multi-cell scheduling cannot provide similar gain, while the specifications impact may not be minimal.

Proposal 11: Don’t support HARQ bundling corresponding to multiple scheduled PDSCHs on a set of co-scheduled cells.

For Type-3 HARQ-ACK codebook, Rel-16 supports unrestricted generation of HARQ-ACK information corresponding to all HARQ processes and all serving cells, when the DCI format indicates the one-shot HARQ-ACK request. Rel-17 supports restricted generation of HARQ-ACK information corresponding to a configured set of HARQ processes or a configured set of serving cells based on triggered states configured by higher layers, where the DCI format indicates the trigger state using the Rel-17 Type 3 CB indicator field. The Type-3 HARQ-ACK CB should be supported for both single-cell and multi-cell scheduling for any configured/activated cell.

Proposal 12: Support generation of a Type-3 HARQ-ACK codebook corresponding to both individual cells and sets of co-scheduled cells.

# Other aspects

Multi-cell scheduling can also have an impact on timeline issues.

For example, in Rel-17, a UE does not expect to operate with out-of-order (OoO) scheduling of PDSCH/PUSCH. Multi-cell scheduling should not change such requirement. However, since a DCI format for multi-cell scheduling corresponds to multiple PDSCHs/PUSCHs on multiple serving cells, the OoO property should be with respect to each such PDSCH/PUSCH.

Proposal 13: Out-of-order (OoO) scheduling requirement for the case of multi-cell scheduling is applicable for each corresponding PDSCH/PUSCH.

# Conclusions

This contribution considered multi-cell scheduling of PDSCHs/PUSCHs using a single DCI format and proposed the following.

Proposals

Proposal 1: Prioritize the intra-band, collocated CA scenario in the multi-cell scheduling designs, while considering inter-band or non-collated CA as well.

Proposal 2: RRC configures ‘set-level’ CIF values that correspond to sub-sets of co-scheduled cells from a set of co-scheduled cells.

Proposal 3: For a DCI format used for multi-cell scheduling, RAN1 to conclude on:

- which DCI fields are cell-common, by default or by configuration, and which DCI fields are cell-specific;

- indication method for each cell-specific field:

  - explicit separate indication with restricted value set

  - explicit differential indication

  - single indication based on “multi-cell mapping”

  - no indication

Tables 1 and 2 can be starting points for the RAN1 discussions.

Proposal 4: For a multi-cell scheduling DCI format, further consider the following three mechanisms:

- single ‘concatenated’ DCI format in a PDCCH;

- DCI format for multi-cell scheduling multiplexed in a PDSCH (a.k.a., two-stage DCI with 2nd stage on a PDSCH);

- two-stage DCI on linked PDCCHs.

Proposal 5: The nCI value in the search space equation for PDCCH monitoring for multi-cell scheduling corresponds to a set of co-scheduled cells. FFS on the method for associating a search space set and a set-level CIF.

Proposal 6: Define the counting of PDCCH candidates and non-overlapping CCEs for multi-cell scheduling.

Proposal 7: Further discuss the “3+1” limit on UE budget for DCI sizes, including voiding the limit for the case of multi-cell scheduling.

Proposal 8: Define a reference PDSCH for determination of the PUCCH resource/slot with HARQ-ACK corresponding to multiple PDSCHs scheduled on multiple serving cells by a multi-cell scheduling DCI format.

Proposal 9: Consider requirements for supporting Type-1 HARQ-ACK codebook for co-scheduled PDSCHs on a set of co-scheduled cells with different SCS configurations and joint or separate TDRA tables.

Proposal 10: Determine counter DAI definition and ordering of HARQ-ACK information bits in a Type-2 HARQ-ACK codebook for multi-cell scheduling.

Proposal 11: Don’t support HARQ bundling corresponding to multiple scheduled PDSCHs on a set of co-scheduled cells.

Proposal 12: Support generation of a Type-3 HARQ-ACK codebook corresponding to both individual cells and sets of co-scheduled cells.

Proposal 13: Out-of-order (OoO) scheduling requirement for the case of multi-cell scheduling is applicable for each corresponding PDSCH/PUSCH.

In addition, the following observations are made.

Observations

Observation 1: DCI overhead savings are more pronounced for intra-band, collocated CA scenario due to increased commonalities of channel conditions and radio characteristics.

Observation 2: The maximum number of cells that can be jointly scheduled using a single DCI format depends on a trade-off among DCI format size, scheduling flexibility, RRC overhead, and the method used for multi-cell scheduling.

Observation 3: A maximum number of DL/UL co-scheduled cells using a DL/UL DCI format can be concluded once aspects for DL/UL DCI formats are progressed.

Observation 4: The DCI fields for multi-cell scheduling can be categorized into cell-common and cell-specific, for serving cells in a set of co-scheduled cells. Some fields can be configured whether they are cell-common or cell-specific.

Observation 5: For multi-cell scheduling of PDSCHs/PUSCHs with a single DCI, solutions minimizing the increase in DCI size without material performance penalty should be prioritized.

Observation 6: Various methods can be used for reducing the combined overhead of cell-specific DCI fields in a DCI format for multi-cell scheduling.

Observation 7: Distinction of a DCI format for multi-cell scheduling from a DCI format for single-cell scheduling can be based on a new DCI format size for multi-cell scheduling, or an explicit indication/flag in the DCI formats.

Observation 8: The “3+1” rule for DCI size budget is rooted in early implementation plans for 5G devices, that do not appear to be applicable anymore.

Observation 9: For multiple PDSCHs scheduled on multiple serving cells by a same DCI format, the HARQ-ACK information of all the scheduled PDSCHs can be multiplexed in a same PUCCH.

Observation 10: To minimize specifications impact, Type-2 HARQ-ACK codebook generation can aim to re-use procedures defined for Rel-17 multi-PDSCH scheduling, when possible.

# References:

- RP-213577, New WID on Multi-carrier enhancements, NTT DOCOMO, INC.

- R1-2102138, “Feature lead summary #4 on multi-cell scheduling via a single DCI”, RAN1#104-e, Moderator (Lenovo).