3GPP TSG RAN WG1 #109-e	        		             R1-2203928

e-Meeting, May 9th – 20th, 2022

Agenda item:		9.11.2

Source:			Samsung

Title:			Considerations on XR Capacity Improvements

Document for:		Discussion and decision

# Introduction

This contribution discusses aspects related to capacity improvement enhancements for XR [1].

# Discussion

Due to its high data rates requirements, XR imposes significant demands both for its own service and on the network as a whole. For example, for AR/VR (45 Mbps), ~4-6 UEs can be supported in a BW of 100 MHz [2] with SU-MIMO. Simulations for MU-MIMO showed that a few more UEs can be supported but, although MU-MIMO can provide some gains, in actual deployments that gain is smaller than the one derived by simulations. To make XR deployments attractive for NW operators, capacity enhancements are critical as otherwise it would not be reasonable to expect that a NW will deprioritize all other traffic/applications that will continue to generate most revenue in order to serve XR.

# DL Scheduling

Similar to other applications, XR has its own specific characteristics such as periodicity, multiple flows, jitter and its own requirements for reliability and latency. In the DL, the otherwise periodic XR transmissions have jitter caused by codec and networks delays. The jitter is associated with a distribution for the arrival time (modelled as Gaussian in the evaluations) that also makes the fact that XR has non-integer periodicity rather irrelevant (and, to some extent, also makes rather irrelevant the fact that XR traffic itself is periodic). More importantly, the packet size is variable and requires dynamic adjustments in resource allocation. Dynamic adjustments are also needed for other transmission aspects, such as for MCS selection and precoding as link adaptation is essential to maximize capacity, especially for the large data rates required by XR.

Considering the above, relying on SPS PDSCH even by using multiple configurations as enabled in Rel-16, is highly inappropriate for XR. SPS PDSCH is well suited for periodic, small and fixed packet sizes for which link adaptation is not particularly advantageous compared to the PDCCH overhead that has about a same order of magnitude as the TB size, and/or when a large number of UEs need to be served at a same time. Support of multiple SPS configurations to address the jitter is also highly inappropriate for XR as that would lead to substantially increased UE power consumption (much more than for PDCCH monitoring) and UE processing complexity as the resource allocations and TB sizes need to be very large, and/or to reduced network capacity. Adapting SPS configurations (e.g. for time-frequency resources and/or for MCS and/or for precoding) by DCI is rather pointless and inferior compared to using dynamic scheduling (and MAC CE would be worse in terms of overhead and latency). Even for the smaller XR data rates, such as for CG with 30 Mbps having a mean packet size of 62500 bytes (500,000 bits), a DCI overhead of ~60-80 bits is negligible (and evidently the same applies for other higher XR data rates, such as for AR/VR with 45 Mbps and mean packet size of 93750 bytes). Also, DCI-based scheduling is always needed for TB retransmissions. Therefore, DCI-based scheduling is the obvious and optimal choice for XR in the DL and avoids complicating/compromising XR deployments as it avoids additional UE complexity, power consumption, and specifications while maximizing capacity and scheduler flexibility.

Observation 1: Enhancements for adapting SPS PDSCH configurations are not necessary or beneficial for XR.

Enhancements related to SPS PDSCH deployments can be considered for capacity improvements. One such enhancement is to support multi-TRP/panel transmission for SPS PDSCH, in addition to the DG-PDSCH that was introduced in Rel-16 MIMO. Support multi-TRP/panel transmission for SPS PDSCH was also considered in the maintenance phase but due to the limited time and late stage of the WI, it was left for a later release.

Proposal 1: Consider support of multi-TRP/panel transmission for SPS PDSCH.

For dynamic scheduling, very few if any, enhancements are needed on top of the ones developed in Rel-17 (e.g. multi-slot scheduling by single DCI, or DSS) and will be developed in Rel-18 (e.g. multi-cell scheduling by single DCI). A potential enhancement can be to support CBG-based HARQ-ACK reporting which was not considered for multi-slot PDSCH scheduling due to lack of time and because, for operation above 52 GHz where it was specified, there was no particular need. XR is an ideal use-case for CBG-based HARQ-ACK reporting. Although DCI overhead is negligible for single-slot scheduling, multi-slot scheduling would also reduce UE power consumption for PDCCH monitoring. CBG-based HARQ-ACK can also be considered for SPS PDSCH.

Proposal 2: Consider support of CBG-based HARQ-ACK reports for multi-slot PDSCH and SPS PDSCH.

Another enhancement for multi-slot PDSCH scheduling can be to support multiple HARQ-ACK feedback occasions. This feature was discussed in Rel-17 for operation above 52 GHz but was not supported due to lack of time and as latency was not a metric for optimization. For XR traffic, using a same PUCCH for HARQ-ACK reporting may not satisfy will result to increased latency requirement for multi-slot PDSCH transmissions. Fast HARQ-ACK feedback would also increase capacity as more retransmissions can be enabled under the PDB.

Proposal 3: Consider support of multiple HARQ-ACK occasions for multi-slot PDSCH scheduling.

As XR includes multiple data flows, each having its own reliability requirements, CSI feedback enhancements are beneficial to improve link adaptation and increase capacity. Current specifications support CQI mapping to either 10-1 or 10-5 target BLER (for eMBB or URLLC respectively). Both of those BLER values can be inappropriate for XR (e.g. too high or too low). Introduction of an intermediate value of 10-3 target BLER can be considered because, although the network may to some extent infer that CQI from another target BLER value, such as 10-1, the actual CQI value for a given BLER is UE implementation dependent. Separate CQI values for corresponding data flaws with different target BLERs should also be supported (e.g. P-frames may be viewed as eMBB-like while I-frames may be viewed as URLLC-like in terms of reliability). CSI enhancements considered for URLLC in Rel-17, such as for new metrics or for triggering mechanisms, do not need to be considered for XR as latency is not as stringent, target BLERs are not as low, transmissions are not sparse, and XR applications target low UE speeds.

Proposal 4: Consider CSI report enhancements to address the different BLER requirements of different XR flows.

# UL Scheduling

In general, different considerations apply for UL traffic than for DL traffic – some are due to inherent differences in the link direction while some are due to different XR traffic characteristics in the DL and UL.

For typical UL-DL configurations, UL traffic has fewer opportunities for transmission than DL traffic which is a problem in case of relatively strict delay requirements. That limitation also creates delays in a UE request for scheduling. Currently, a UE needs to wait for an UL slot to transmit PUCCH with positive SR (may be generated at the UE when a slot is a DL one), then wait for a next UL slot to provide BSR, and then wait for a next UL slot to be scheduled according to the BSR. With such procedure and for a DDDDU configuration, the delay is at least 10 msec at 15 kHz SCS and at least 5 msec at 30 kHz SCS - too large for XR. Those values also assume possible for a UE to provide SR in every UL slot, immediate scheduling by the gNB, and do not consider additional delays, such as 2.5 msec at 15 kHz SCS and 1.25 msec at 30 kHz SCS, to account for the average time between the generation of the positive SR at the UE and the corresponding PUCCH transmission opportunity and duration. Therefore, the Rel-17 SR/BSR-based scheduling approach is not functional for XR.

One solution is to enhance the SR and bridge the information gap between the current request for scheduling with no other information and the detailed information provided by BSR. Instead of transmitting an unmodulated signal to indicate positive SR, a UE can apply BPSK/QPSK (as for HARQ-ACK) and provide 1-2 bits of information corresponding to BSR values (e.g. for pose/control vs. video) that can be configured by the gNB. In that manner, latency is reduced by ~50% as the initial scheduling by the gNB can be relatively accurate while subsequent scheduling can be enhanced based on BSR. There is no coverage issue by having a modulated SR for 1-2 bits as the UE is expected to have sufficient link quality to reliably provide thousands of bits of XR traffic, there is no additional resource overhead, and there is no impact on UE/gNB implementation as the corresponding transmission/reception is same as for 1-2 bits of HARQ-ACK.

Proposal 5: Apply BPSK/QPSK to SR to indicate 2-4 BSR values in order to reduce latency for UL scheduling.

Although delays associated with DCI-based scheduling in the UL may be reduced, CG-PUSCH transmissions remain superior for minimum latency and as a general solution. Compared to DL traffic, UL XR traffic is practically unaffected by jitter and having CG-PUSCH at deterministic time intervals is sufficient (there is also no impact on UE power consumption). Nevertheless, a same issue as for DL traffic exists in that the packet sizes can be variable (e.g. pose/control vs. video) and that is somewhat enhanced by the fact that the periodicity of UL slots for an UL-DL configuration (e.g. 2.5 msec or 5 msec) may not match the periodicity of UL traffic for XR (e.g. 4 msec).

One approach to address the variable packet size in the UL is to provide multiple CG PUSCH configurations for a UE to select from according to its traffic requirements. However, unlike URLLC which requires minimal resources for very small TBS, XR would require that a large number of resources is reserved for each UE (i.e. the maximum resources across all CG configurations for the UE). Such approach is infeasible in practice particularly considering existence of multiple UEs (not only XR ones) and it is not generally possible to provide orthogonal resources (and is also not possible to operate XR statistically with non-orthogonal resources as then the error probability for multiple XR UEs would be unacceptably large, in addition to interference spikes to neighboring cells). Having multiple CG-PUSCH configurations is also undesirable from a NW complexity perspective due to the number of blind decoding operations for large candidate TB sizes (similar to PDCCH blind decoding but for a much larger number of encoded bits over a large BW). It is noted that DTX detection at the gNB for unused CG-PUSCH configurations will not be generally possible because, due to at least partial overlap for the resources corresponding to different CG-PUSCH configurations for a UE, DM-RS will be present for each candidate CG-PUSCH configuration when the UE transmits a CG-PUSCH.

Proposal 6: Study mechanisms to enable CG-PUSCH transmissions from XR UEs with variable packet sizes without a-priori reservation of corresponding resources.

# Link Adaptation Enhancements

XR has substantial BW requirements that may not be possible/desirable for a network to provide in FR1, even for very few XR UEs, while also supporting other traffic types. BW is abundant in FR2, cost per Hz is much smaller than in FR1, and latency is not an issue for any UL-DL configuration in FR2. Those factors make FR2 ideal for XR (can also be via a smartphone, a CPE, etc.).

Beam management is one aspect that would require enhancement for XR operation in FR2 because the best beam for a UE can fail while the UE is in the DRX Off cycle. That is not a major problem for eMBB as latency can be tolerated and the DRX On cycle can be long and allow for inherent beam management (IIoT in FR2 is primarily for quasi-stationary machine devices and beam management is practically not needed). If XR is to operate with short DRX cycles, beam management should be enabled during the DRX Off cycle (based on gNB configuration). While UE power consumption would somewhat increase, the amount would be minimal compared to the savings from having short DRX cycles as the only requirements are the occasional SSB/CSI-RS receptions and a CSI report (no PDCCH/PDSCH/PUSCH processing). That would also enable accurate link adaptation (also in FR1) immediately when the UE enters DRX On (instead of the gNB waiting for the UE to measure and report CSI before scheduling or instead of the gNB scheduling without an up-to-date CSI report), which in turn would increase capacity and spectral efficiency and reduce latency and UE power consumption. It is noted that a UE receives SPS PDSCH and transmits CG PUSCH during the DRX Off cycle as determined by the corresponding configurations. The specification impact to support beam management and CSI reporting during the DRX Off cycle can be minimal (extend few existing operations during DRX Off) and no new UE/gNB functionality is introduced.

Proposal 7: For operation in FR2, allow a UE to perform beam management and CSI reporting during the DRX Off cycle.

# Conclusions

This contribution considered mechanisms for XR capacity enhancements and proposes the following.

Proposal 1: Consider support of multi-TRP/panel transmission for SPS PDSCH.

Proposal 2: Consider support of CBG-based HARQ-ACK reports for multi-slot PDSCH scheduling.

Proposal 3: Consider support of multiple HARQ-ACK occasions for multi-slot PDSCH scheduling.

Proposal 4: Consider CSI report enhancements to address the different BLER requirements of different XR flows.

Proposal 5: Apply BPSK/QPSK to SR to indicate 2-4 BSR values in order to reduce latency for UL scheduling.

Proposal 6: Study mechanisms to enable CG-PUSCH transmissions from XR UEs with variable packet sizes without a-priori reservation of corresponding resources.

Proposal 7: For operation in FR2, allow a UE to perform beam management and CSI reporting during the DRX Off cycle.

Observation 1: SPS PDSCH enhancements are not necessary or beneficial for XR non-periodic traffic.

# References

- RP-213587, “New SID “Study on XR Enhancements for NR”

- TR 38.838 v17.0.0, “Study on XR (Extended Reality) evaluations for NR”