3GPP TSG RAN WG1 #109-e	        		             R1-2203929

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.12.1

Source: 	Samsung

Title: 	On coverage enhancement for NR NTN

Document for:	Discussion and Decision

Introduction

In RAN#94e, a new WI for NR NTN enhancement is endorsed in [1] with a first 6-mont study phase focusing on the following in order to identify clear and limited scope.

- Evaluate the coverage performance and identify the candidate physical radio channels that have coverage issues specific to NTN with following target services taking into account the studies in TR38.830 where appropriate, as well as general coverage enhancement techniques specified in Rel-18 [RAN1,RAN2,RAN4]

  - VoIP and low-data rate services for commercial handset terminals

Therefore, in this contribution, it is discussed on how to evaluate the coverage performance and provide initial evaluation results to derive observations for NR NTN coverage.

Evaluation for NR NTN coverage performance

Candidate physical radio channels

Although NTN channel environment might be different to TN, the study of NR NTN coverage performance should start from the PUSCH which is well-known to be the bottleneck channel for FR1 as concluded in [2]. Besides, since main use cases for NR NTN enhancement are VoIP and low-data rate service for commercial handset terminals (a.k.a. smartphone), focusing on PUSCH for NR NTN coverage evaluation is reasonable. It is noted that the latency requirement of VoIP service had not been unclearly defined in [2]. Since NR NTN has long RTD (Round Trip Delay) channel characteristics, it should be discussed what latency requirement for VoIP is considered. Similarly, without having exact parameters such as packet size and latency for low-data rate service, it is difficult to check whether further coverage enhancement is necessary or not.

Proposal 1: PUSCH should be baseline physical channel for NR NTN coverage performance evaluation

Proposal 2: Discuss on the following parameters for NR NTN coverage performance evaluation

- Latency requirement for VoIP

- Packet size and latency requirement for low-data rate service

Link budget assumption and analysis

The most important service capabilities for mass market smartphones over NTN are expected to cover the following services:

- Short message service (SMS)/messaging

- Voice

- Possibly a certain level of data services.

Some concerns have been raised regarding the assumption of 0 dBi for smartphone antenna gain during the early stages of NTN link budget analysis. Specifically, in the WID for Rel-18 NR NTN Enhancements, regarding the objective of coverage enhancements it is mentioned that: “The work needs to cover the use case of voice and low-data rate services using commercial smartphones with more realistic assumptions on antenna gains instead of 0 dBi currently assumed for link budget analysis for non-terrestrial networks.”

In [2], the concept of MPL (Maximum Path Loss) is used to derive coverage performance based on certain path loss model. Similar concept can be reused with simplifying some assumptions because main assumptions/parameters/approaches for [2] and [3] are quite different. With this understating, MPL can be derived with following formula.

- MPL =

Total transmit power (dBm) – receiver noise figure (dB) – thermal noise density (dBm/Hz/K) + receiver antenna gain (dBi) + transmitter antenna gain (dBi) – shadow fading margin (dB) – other channel loss (e.g., atmospheric loss, scintillation loss, additional loss) (dB) – Required SNR (dB) – Polarization loss (dB) + HARQ gain (dB)

Here, HARQ gain is only considered if LLS results do not consider HARQ retransmission. Otherwise, it should be assumed as 0. For polarization loss, it should be considered for realistic NTN scenario although it hadn’t been considered in [2]. For deriving distance, free space path loss model is assumed as shown in [3]. For required SNR, it should be derived based on link level simulation results. It is noted that SNR and SINR would be similar because interference is negligible in case that option 2 is assumed for frequency reuse factor in [3].

Proposal 3: Consider a revised MPL(Maximum Path Loss) concept to identify NR NTN coverage performance.

In the following, we provide link level simulation results for PUSCH VoIP assuming NTN-TDL-A and NTN-TDL-B channel models. In Appendix, Table 3 is shown for detailed parameters used for LLS evaluations. A packet size of 368 bits with 20ms data arriving interval is adopted since [2] assumed VoIP packet size of 320 bits (with 64 bit overhead which yields 256 bits payload), and it is the next value is larger than 320 bits that NR supports. Regarding Doppler compensation, post-compensation is assumed at satellite to compensate satellite movement. In Table 1, Doppler means Doppler residuals caused by UE movement. Transmission of each VoIP packet is repeated 16 times since the first VoIP packet should be successfully transmitted before the next VoIP packet will be arrival with the assumption of 15kHz SCS. However, no HARQ retransmission is assumed. With these assumptions, Table 1 shows required SNR for the residual BLER target of 2%.

Table 1. Required SNR for different channel models

| Satellite antenna configuration | Channel Model | Delay spread (ns) | Required SNR (dB) |
| --- | --- | --- | --- |
| 2 Rx | NTN-TDL-A | 93 | -7.6 |
| 2 Rx | NTN-TDL-B | 32 | -7.6 |
| 16 Rx | NTN-TDL-A | 93 | -16.9 |
| 16 Rx | NTN-TDL-B | 32 | -16.9 |

Based on LLS results, Table 2 provides achievable distance for PUSCH with the assumption of Table 6.1.1.1-1 of [3] for satellite parameters (i.e., set 1 configuration) according to the number of receiver antennas and transmit antenna gain.

Table 2. Achievable distances for PUSCH for different satellite parameters

|  | 2 Rx |  |  |  | 16 Rx |  |  |  | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LEO |  | GEO |  | LEO |  | GEO |  |  |
| Total transmit power (dBm) | 23 |  | 23 |  | 23 |  | 23 |  | Table 6.1.1.1-3 in [3] |
| Receiver noise figure (dB) | 5 |  | 7.5 |  | 5 |  | 7.5 |  | Derived from assumptions in [3] |
| Temperature (K) | 270 |  | 270 |  | 270 |  | 270 |  | Specified in [3] |
| Bandwidth (MHz) | 0.72 |  | 0.72 |  | 0.72 |  | 0.72 |  | Assumption in LLS |
| Thermal noise density (dBm/Hz/K) | -115.71 |  | -115.71 |  | -115.71 |  | -115.71 |  | Derived from assumptions in [3] |
| Receiver antenna gain (dBi) | 30 |  | 51 |  | 30 |  | 51 |  | Specified in [3] |
| Transmit antenna gain (dBi) | 0 | -5 | 0 | -5 | 0 | -5 | 0 | -5 | -5 dBi is assumed as realistic UE antenna assumption |
| Shadow fading margin (dB) | 3 |  | 3 |  | 3 |  | 3 |  | Specified in [3] |
| Polarization loss (dB) | 3 |  | 3 |  | 3 |  | 3 |  | UE: linear polarization, gNB: circular polarization |
| Other channel loss (atmospheric loss, scintillation loss, additional loss) (dB) | 2.3 |  | 2.3 |  | 2.3 |  | 2.3 |  | Specified in [3] (Handheld) |
| Required SNR (dB) | -7.6 |  | -7.6 |  | -16.9 |  | -16.9 |  | Derived from LLS results |
| HARQ (repetition) gain (dB) | 0.00 |  | 0.00 |  | 0.00 |  | 0.00 |  |  |
| Maximum path loss (dB) | 163.01 | 158.01 | 181.51 | 176.51 | 172.31 | 167.31 | 190.81 | 185.81 |  |
| Carrier frequency (GHz) | 2 |  | 2 |  | 2 |  | 2 |  | Specified in [3] |
| Achievable distance (km) | 1686 | 949 | 14193 | 7981 | 4921 | 2768 | 41407 | 23285 | LEO: 2,000km @90° elevation angle LEO: 4,435km @10° elevation angle GEO: 35,786km @90° elevation angle GEO: 40,581km @10° elevation angle |

Based on our evaluation results, it is shown that current PUSCH repetition scheme can support a maximum coverage of around 1686km for LEO, and 14193km for GEO in case of 2 Rx and 4921km for LEO, and 41407km for GEO in case of 16 Rx assuming that transmit antenna gain is 0dBi. While, it is shown that current PUSCH repetition scheme can support a maximum coverage of around 949km for LEO, and 7981km for GEO in case of 2 Rx and 2768km for LEO, and 23285km for GEO in case of 16 Rx assuming that transmit antenna gain is -5dBi.

Observation 1: For LEO case, maximum achievable coverage of PUSCH for VoIP using legacy coverage enhancement schemes is 1686km for 2Rx and 4921km for 16Rx assuming that transmit antenna gain is 0dBi.

Observation 2: For LEO case, maximum achievable coverage of PUSCH for VoIP using legacy coverage enhancement schemes is 949km for 2Rx and 2768km for 16Rx assuming that transmit antenna gain is -5dBi.

Observation 3: For GEO case, maximum achievable coverage of PUSCH for VoIP using legacy coverage enhancement schemes is 14193km for 2Rx and 41407km for 16Rx assuming that transmit antenna gain is 0dBi.

Observation 4: For GEO case, maximum achievable coverage of PUSCH for VoIP using legacy coverage enhancement schemes is 7981km for 2Rx and 23285km for 16Rx assuming that transmit antenna gain is -5dBi.

It is evident that lower antenna gain would reduce maximum achievable coverage, while larger number of receiver antenna would increase maximum achievable coverage. Besides, although current satellite assumptions are based on [3], maximum achievable coverage could be increased if larger receiver antenna gain can be used. Therefore, realistic assumptions should be discussed for satellite as well as UE (i.e., handheld device) before deciding whether or not NTN specific coverage enhancement is necessary.

Proposal 4: Further discuss the following assumptions for the evaluation of coverage performance in NR NTN.

- Transmit antenna gain for handheld device

- The number of receiver antennas and receiver antenna gain for satellite

Conclusions

This contribution considered link budget analysis for NTN coverage and proposes the following.

Proposal 1: PUSCH should be baseline physical channel for NR NTN coverage performance evaluation

Proposal 2: Discuss on the following parameters for NR NTN coverage performance evaluation

- Latency requirement for VoIP

- Packet size and latency requirement for low-data rate service

Proposal 3: Consider revised MPL(Maximum Path Loss) concept to identify NR NTN coverage performance.

Proposal 4: Study on what the realistic assumptions are for both handheld device and satellite.

The following observations are also made.

Observation 1: For LEO case, maximum achievable coverage of PUSCH for VoIP using legacy coverage enhancement schemes is 1686km for 2Rx and 4921km for 16Rx assuming that transmit antenna gain is 0dBi.

Observation 2: For LEO case, maximum achievable coverage of PUSCH for VoIP using legacy coverage enhancement schemes is 949km for 2Rx and 2768km for 16Rx assuming that transmit antenna gain is -5dBi.

Observation 3: For GEO case, maximum achievable coverage of PUSCH for VoIP using legacy coverage enhancement schemes is 14193km for 2Rx and 41407km for 16Rx assuming that transmit antenna gain is 0dBi.

Observation 4: For GEO case, maximum achievable coverage of PUSCH for VoIP using legacy coverage enhancement schemes is 7981km for 2Rx and 23285km for 16Rx assuming that transmit antenna gain is -5dBi.

- References

[1] RP-213690, “New WI: NR NTN (Non-Terrestrial Networks) enhancements”

[2] TR 38.830 v17.0.0, “Study on NR coverage enhancement (Release 17)”

[3] TR 38.821 v16.1.0, “Solutions for NR to support non-terrestrial networks (NTN) (Release 16)”

[4] TR 38.811 v15.4.0, “Study on New Radio (NR) to support non-terrestrial networks (Release 15)”

- Appendix

Table 3. Link-level simulation assumptions

| Carrier frequency | 2GHz |
| --- | --- |
| Subcarrier spacing | 15kHz |
| Waveform | CP-OFDM |
| Physical channel | PUSCH |
| PUSCH Allocation | 4 RB, 14 OFDM symbol, rank 1, 16 repetitions, no reTx |
| MCS | 4 (TBS = 368bits) (64QAM table, QPSK) |
| DMRS | Type 1 single symbol with one additional |
| Channel estimation | Realistic |
| Doppler residuals | 56Hz |
| UE speed (Doppler) | 30km/h (56Hz) |
| Channel model | NTN-TDL-A, NTN-TDL-B (RMS delay spread=32ns, 93ns) |
| Satellite antenna configuration | 2, 16 RF chains (2, 16 Rx antennas) |
| Analog beamforming algorithm | ISAB (Iterative Single-connection Analog Beamforming) for 16 Rx antenna |
| UE antenna configuration | 1 Tx |
| rBLER Target | For VoIP, 2% rBLER |