3GPP TSG RAN WG1 #109-e	R1-2203931

# e-Meeting, May 9th – 20th, 2022

Agenda item:	9.12.3

Source:	Samsung

Title:	Improved GNSS operation for IOT NTN

Document for:	Discussion and decision

- Introduction

A work item on enabling NB-IoT and eMTC to support Non-Terrestrial Network (NTN) was approved in RAN#94e. As one of the objectives, possible enhancement related to RAN1 is as following:

4.1.1	IoT-NTN Performance Enhancements in Rel-18 to address remaining issues from Rel-17

This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below:

-	Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1, RAN2]

-	Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption [RAN1]

In this contribution, possible optimizations on GNSS operation are discussed.

- Discussion

In Rel-17, GNSS position fix was agreed to be acquired before initial access for sporadic short transmission. Before accessing the network, a UE acquires GNSS position fix and does not need to re-acquire a GNSS position fix for the transmission of the packets. If GNSS measurement becomes outdated, UE in RRC_CONNECTED mode should go back to idle mode and re-acquire a GNSS position fix. In Rel-18, long connection time is considered and re-acquiring a new GNSS position fix may be possible during RRC-CONNECTED mode. Evaluation captured in TR 36.763 shows that a GNSS fix before every uplink transmission consumes approximately 45% of the UE's total available energy, wherein a DRX cycle of ~10s and no any enhancement on UL synchronization are assumed [2]. Therefore, reducing the times of acquiring a new GNSS position fix during a long connection time is meaningful to improve UE battery life.

Actually, UL synchronization can be maintained based on eNB’s control, which is applied in Terrestrial Network. For example, closed loop TA command can be used to adapt timing shift due to UE velocity. If current TA command in MAC CE cannot track the timing with required accuracy due to a large propagation time and a latency of application for MAC CE, TA command can be considered to be carried by DCI. Similarly, closed loop pre-compensated frequency offset command can be introduced to maintain UL synchronization. If UL synchronization can be maintained based on closed loop control signaling, the times of acquiring a new GNSS position fix during a long connection time can be reduced.

Proposal 1: Triggering of a new GNSS position fix can be reduced by maintaining UL synchronization via closed loop control, e.g., closed loop TA command, and closed loop pre-compensated frequency offset command.

Even if UL synchronization can be maintained based on closed loop command signaling, UL synchronization loss may still happen. If UL synchronization is lost and UL data is arrived, UL resynchronization is needed. In this case, if validity timer of GNSS measurement is expired, acquiring a new GNSS position fix during a long connection time can be triggered by UE. Before validity timer of GNSS measurement is expired, a new GNSS position fix is not needed even if UL synchronization is lost.

Proposal 2: Acquiring a new GNSS position fix during a long connection time can be triggered by UE when there are UL data to transmit and the UL synchronization is lost.

On the other hand, acquiring a new GNSS position fix during a long connection time can also be triggered by eNB for DL data arriving case with lost UL synchronization. For example, if eNB does not receive the response from a UE for a period, eNB can assumes the UE’s UL synchronization is lost. In this case, if the UE’s DL data is arrived, UL resynchronization is needed. And, if eNB can know the UE’s validity timer on GNSS measurement is expired, acquiring a new GNSS position fix can be triggered by eNB for the UE.

Proposal 3: Acquiring a new GNSS position fix during a long connection time can be triggered by eNB when there are DL data to transmit and the UL synchronization is lost.

Considering simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed, it should be discussed that whether the GNSS measurement window is needed during a long connection time for acquiring a new GNSS position fix. For the case UE triggers a new GNSS position fix, GNSS measurement can be up to UE implementation and GNSS measurement window is not needed. However, for the case eNB triggers a new GNSS position fix at UE side, GNSS measurement window should be defined. Within the GNSS measurement window, eNB does not need to transmit unicast DL to a UE performing GNSS measurement, and the UE does not expect to receive DL from eNB.

Proposal 4: For the case that a new GNSS position fix is triggered by eNB, a GNSS measurement window needs to be defined.

To assist eNB on GNSS related configuration, UE can report some assistance information to eNB. For example, UE can report that its GNSS position is fixed. Thus, eNB does not need to trigger a new GNSS position fix. Or, UE can report that its GNSS position can be available in IoT application layer. Thus, GNSS measurement window may not be needed when eNB triggering a new GNSS position fix. Furthermore, UE can report its capability on GNSS measurement, e.g., the time consumed on GNSS measurement may be diverse for different UE capability. If UE reports its preferred length of GNSS measurement window, this information can assist the configuration of GNSS measurement window.

Proposal 5: Following information can be reported to eNB to assist GNSS operation:

- Whether UE’s GNSS position is fixed or not;

- Whether UE’s GNSS position is available in IoT application layer or not;

- UE capability on GNSS measurement, e.g., preferred length of GNSS measurement window;

- Conclusions

This contribution discusses possible enhancements on GNSS operations and the proposals are summarized below:

Proposal 1: Triggering of a new GNSS position fix can be reduced by maintaining UL synchronization via closed loop control, e.g., closed loop TA command, and closed loop pre-compensated frequency offset command.

Proposal 2: Acquiring a new GNSS position fix during a long connection time can be triggered by UE when there are UL data to transmit and the UL synchronization is lost.

Proposal 3: Acquiring a new GNSS position fix during a long connection time can be triggered by eNB when there are DL data to transmit and the UL synchronization is lost.

Proposal 4: For the case that a new GNSS position fix is triggered by eNB, a GNSS measurement window needs to be defined.

Proposal 5: Following information can be reported to eNB to assist GNSS operation:

- Whether UE’s GNSS position is fixed or not;

- Whether UE’s GNSS position is available in IoT application layer or not;

- UE capability on GNSS measurement, e.g., preferred length of GNSS measurement window;

References:

- RP-213596, New WID on IOT NTN enhancements, MediaTek Inc.

- TR 36.763, Study on NB-IoT/eMTC support for NTN.