3GPP TSG RAN WG1 #109	R1-2203933

e-Meeting, May 9th – May 20th, 2022

Agenda item:		9.12.3

Source:	Nordic Semiconductor ASA

Title:					Improved GNSS operation for IoT NTN

Document for:		Discussion and Decision

# Introduction

In RAN#95, work item in RP-220979 has been approved. The objectives regarding improved GNSS operation are the following

This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below: - Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1] - NOTE: The need for RAN4 Core requirements for this objective will be identified after the conclusion on the need for improvements.

# Improved GNSS operation

From UE point of view, Rel-17 IoT NTN system was designed for sporadic short transmissions in RRC Connected mode and after the short transmission UE goes back to Idle mode. From [1]:

For sporadic short transmission:

-	The idle UE wakes up from idle DRX / PSM, access the network, perform uplink and/or downlink communications for a short duration of time and go back to idle.

-	Before accessing the network, the UE acquires GNSS position fix and does not need to re-acquire a GNSS position fix for the transmission of the packets.

How long UE can stay in Connected mode depends on validity timer of UL synchronization and on validity of GNSS position fix of the UE. The NTN UE autonomously determines its GNSS validity duration X and reports information associated with this validity duration to the network via RRC signalling. The validity duration X may get the values:

  - X = {10s, 20s, 30s, 40s, 50s, 60s, 5 min, 10 min, 15 min, 20 min, 25 min, 30 min, 60 min, 90 min, 120 min, infinity}

From power saving point of view, it would be beneficial if UE could stay in RRC Connected mode as long as the active data transmission takes place as is the case in TN. Therefore, it would be good if UE could acquire a new GNSS fix while in Connected mode when GNSS validity expires, in order to be able to asses UE specific TA. Typically, an updated GNSS fix (“hot” fix) takes considerably less time than the first GNSS fix (“cold” fix) done before entering the Connected mode. However, even the acquisition of “hot” fix requires tens of radio frames and generally UE cannot resort to the “legacy” gaps in N/MPDCCH monitoring settings.

The simplest way to extend the duration of UE stay in Connected mode is to configure UE with the periodic GNSS measurement gaps via UE-specific RRC signalling, based on the GNSS validity duration X reported by the UE.

Proposal 1: In RRC connected mode, the eNB could configure UE with the periodic GNSS measurement gaps via UE-specific RRC signalling, based on the GNSS validity duration X reported by the UE.

Sometimes UE’s assessment of its mobility pattern is not quite accurate, or UE’s mobility pattern may change after UE’s determination of its GNSS validity duration, and the network may notice that UE’s uplink synchronization is slipping away. In such cases, it would be useful if the network has some signalling mechanism to request UE to take a new GNSS fix before continuing further transmissions. E.g., some unused code points in existing DCI formats, if such exist in eMTC or NB-IoT DCI formats, could be used to dynamically allocate GNSS measurement gap for the UE.

Proposal 2: RAN1should consider  supporting  dynamic GNSS measurement gaps trigger by eNB for a UE in RRC connected mode.

On the other hand, UE itself may notice the sudden change in its mobility and find it necessary to ask a new GNSS gap before the next gap according to the configuration would come. Thus, it should be considered to specify a signaling mechanism for UE to request a GNSS measurement gap whenever the need for it emerge. E.g, in NB-IoT, ordered NPRACH could be used for requesting GNSS measurement gap similar to scheduling request (SR) without HARQ feedback. Another alternative could be to employ UL MAC-CE reporting. The length of the gap and periodicity could be reported. Semi-static change of gap length and periodicity would be beneficial if GPS signal pathloss change and/or degree of mobility changes.

Proposal 3: It should be considered to specify a signaling mechanism for UE to request a GNSS measurement gap whenever the need for it emerge.

Proposal 4: RAN1 should consider supporting semi-static change of GNSS measurement gap periodicity and length triggered by UE.

# Conclusions

In this contribution we discussed issues related to improved GNSS operation for IoT NTN and made the following proposals:

Proposal 1: In RRC connected mode, the eNB could configure UE with the periodic GNSS measurement gaps via UE-specific RRC signalling, based on the GNSS validity duration X reported by the UE.

Proposal 2: It should be considered if some signalling mechanism is needed for the network to dynamically allocate GNSS measurement gap to the UE in RRC connected mode.

Proposal 3: It should be considered to specify a signaling mechanism for UE to request a GNSS measurement gap whenever the need for it emerge.

Proposal 4: RAN1 should consider supporting semi-static change of GNSS measurement gap periodicity and length triggered by UE.

# References

[1] RP-211456, 3GPP TR 36.763 v1.0.0 Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN) (Release 17)