3GPP TSG RAN WG1 #109-e	R1-2203941

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.5.1.1

Source:	NEC

Title: 	SL positioning scenarios and requirements

Document for:	Discussion and Decision

# Introduction

The work item on “Study on expanded and improved NR positioning solutions” for NR has been approved for Rel-18 and one of the objectives is to study solutions for sidelink positioning as follows in [1],

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2]

- Scenario/requirements

  - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage

  - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104

  - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104)

  - Spectrum: ITS, licensed

- Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1]

- Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1].

- Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2]

  - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1]

  - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1]

  - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1]

  - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required]

Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

This contribution will discuss SL positioning scenarios and requirements.

# Discussion

## Use cases and SL scenario

Four use cases including V2X, public safety, commercial and IIOT will be investigated and different scenarios can be defined for each use case as below.

Commercial

Three scenarios have been defined in [2] as below

- Scenario 1. Indoor Office for FR1 and FR2 (Open office)

- Scenario 2. UMi street canyon for FR1 and FR2 (ISD 200m)

- Scenario 3. UMa (ISD 500m) for FR1 only (Macro cell only deployment scenario)

All three scenarios can be considered for SL scenarios.

IIOT

In Rel-17, two scenarios have been defined in [3]

- Scenario 1. InF-SH for FR1 and FR2

- Scenario 2. InF-DH for FR1 and FR2

V2X

For V2X use case, two scenarios have been defined [4] as below.

- Scenario 1. Urban grid for FR1 and FR2

- Scenario 2. Highway for FR1 and FR2

Public Safety

Four use cases have been defined for PS (Public Safety) ProSe communication [5].

- Little or no coverage

- Fall Back

- Extra Capacity

- Local Communication Requirement

Urban macro scenario is defined with 6 layout options as below,

- Option 1: Urban macro (500m ISD) + 1 RRH/Indoor Hotzone per cell

- Option 2: Urban macro (500m ISD) + 1 Dual stripe per cell

- Option 3: Urban macro (500m ISD) (all UEs outdoor)

- Option 4: Urban macro (500m ISD) + 3 RRH/Indoor Hotzone per cell

- Option 5: Urban macro (1732m ISD)

- Option 6: Urban micro (100m ISD)

In total 13 scenarios have been defined for all use cases and these scenarios can be used as a starting point for SL positioning scenarios. In order to keep evaluation workload reasonable, some scenarios can be merged. Urban macro scenario is commonly defined in commercial, V2X and public safety use cases with additional setting for each individual use case. After merging, the following 5 scenarios can be considered.

- Scenario 1. Indoor Office for FR1 and FR2 (Open office)

- Scenario 2. UMi street canyon for FR1 and FR2 (ISD 200m)

- Scenario 3. UMa (ISD 500m) for FR1 only including Urban grid and Highway

- Scenario 4. InF-SH for FR1 and FR2

- Scenario 5. InF-DH for FR1 and FR2

In above 5 scenarios, scenario 3 should be prioritized for SL since for other scenarios, it is more likely the positioning can be done via Uu interface.

- The following 5 scenarios can be considered as baseline scenarios and scenario 3 can be prioritized.

- Scenario 1. Indoor Office for FR1 and FR2 (Open office)

- Scenario 2. UMi street canyon for FR1 and FR2 (ISD 200m)

- Scenario 3. UMa (ISD 500m) for FR1 only including Urban grid and Highway

- Scenario 4. InF-SH for FR1 and FR2

- Scenario 5. InF-DH for FR1 and FR2

In [6], three coverage scenarios are defined including in-coverage, partial-coverage and out-of-coverage. In-coverage scenario and out-of-coverage scenarios can be easily evaluated by enabling/disabling all gNBs. However, how to define partial-coverage scenario for evaluation is not straightforward. In such a case, some of the gNBs can be disabled and G-factor to indicate long term SINR can be used to determine if a particular UE is covered or not. A threshold can then be defined and if G-factor is lower than the threshold, the UE can be assumed to be out-of-coverage.

- For partial-coverage scenario, a threshold can be defined and if G-factor is lower than the threshold, the UE can be assumed to be out-of-coverage.

Further, it is not necessary to evaluate all three coverage scenarios for every scenario. For Indoor Office, UMi and InF use cases, UE is more likely to be covered by gNB and therefore only in-coverage and partial coverage should be considered. For Urban Macro use case, there could be some coverage holes due to blockage or loss of coverage due to being far away from gNB and thus out-of-coverage should be considered and evaluated.

- For all defined scenarios, all three coverage scenarios including in-coverage, partial-coverage and out-of-coverage should be considered and evaluated for scenario 3 UMa and for other scenarios, only in-coverage and partial-coverage scenarios can be considered and evaluated.

## SL Positioning Performance Requirements

The baseline metrics to be used to evaluate the performance of SL positioning can include accuracy, power, latency and integrity and other metrics can also be considered if needed.

- The baseline metrics to be used to evaluate the performance of SL positioning can include accuracy, power, latency and integrity and other metrics can also be considered if needed.

Same as scenarios, different requirements can be defined for difference use cases.

Commercial

The baseline requirements for commercial have been defined in Table 7.3.2.2-1 with 7 service levels [7]. In Rel-17 the target positioning requirements for commercial use case are defined as follows and can be used as baseline requirements for SL positioning [2]:

- Horizontal position accuracy (< 1 m) for 90% of UEs

- Vertical position accuracy (< 3 m) for 90% of UEs

- End-to-end latency for position estimation of UE (< 100 ms)

- Physical layer latency for position estimation of UE (< 10 ms)

IIoT

The baseline requirements for IIoT have been defined in Table 5.7.1-1 with 7 service levels [8]. In Rel-17 the target positioning requirements for IIoT use case are defined as follows and can be used as baseline requirements for SL positioning:

- Horizontal position accuracy (< 0.2 m) for 90% of UEs

- Vertical position accuracy (< 1 m) for 90% of UEs

- End-to-end latency for position estimation of UE (< 100ms, in the order of 10 ms is desired)

- Physical layer latency for position estimation of UE (<10ms)

V2X

The positioning requirements in V2X depend on the service the UE operates and the following requirements have been defined in [845] and can be used as baseline requirements for SL positioning:

- Set 1: 10 – 50 m with 68 – 95 % confidence level.

- Set 2: 1 – 3 m with 95 – 99 % confidence level.

- Set 3: 0.1 – 0.5 m with 95 – 99 % confidence level.

Public Safety

Public safety positioning requirements have been defined for the "1st responders" use case in Table B.1-1 in [7] as follows:

- Horizontal position accuracy (< 1 m);

- Absolute vertical position accuracy (< 2 m)

- Relative vertical position accuracy (< 0.3 m)

- 95 – 98 % positioning service availability.

- The following requirements can be used as baseline for SL positioning

- Commercial

  - Horizontal position accuracy (< 1 m) for 90% of UEs

  - Vertical position accuracy (< 3 m) for 90% of UEs

  - End-to-end latency for position estimation of UE (< 100 ms)

  - Physical layer latency for position estimation of UE (< 10 ms)

- IIoT

  - Horizontal position accuracy (< 0.2 m) for 90% of UEs

  - Vertical position accuracy (< 1 m) for 90% of UEs

  - End-to-end latency for position estimation of UE (< 100ms, in the order of 10 ms is desired)

  - Physical layer latency for position estimation of UE (<10ms)

- V2X

  - Set 1: 10 – 50 m with 68 – 95 % confidence level.

  - Set 2: 1 – 3 m with 95 – 99 % confidence level.

  - Set 3: 0.1 – 0.5 m with 95 – 99 % confidence level.

- Public Safety

  - Horizontal position accuracy (< 1 m);

  - Absolute vertical position accuracy (< 2 m)

  - Relative vertical position accuracy (< 0.3 m)

  - 95 – 98 % positioning service availability.

# Conclusion

In this contribution, we discussed SL positioning scenarios and requirements and the following observations and proposals are made:

Proposal 1	The following 5 scenarios can be considered as baseline scenarios and scenario 3 can be prioritized.

- Scenario 1. Indoor Office for FR1 and FR2 (Open office)

- Scenario 2. UMi street canyon for FR1 and FR2 (ISD 200m)

- Scenario 3. UMa (ISD 500m) for FR1 only including Urban grid and Highway

- Scenario 4. InF-SH for FR1 and FR2

- Scenario 5. InF-DH for FR1 and FR2

Proposal 2	For partial-coverage scenario, a threshold can be defined and if G-factor is lower than the threshold, the UE can be assumed to be out-of-coverage.

Proposal 3	For all defined scenarios, all three coverage scenarios including in-coverage, partial-coverage and out-of-coverage should be considered and evaluated for scenario 3 UMa and for other scenarios, only in-coverage and partial-coverage scenarios can be considered and evaluated.

Proposal 4	The baseline metrics to be used to evaluate the performance of SL positioning can include accuracy, power, latency and integrity and other metrics can also be considered if needed.

Proposal 5 	The following requirements can be used as baseline for SL positioning

- Commercial

  - Horizontal position accuracy (< 1 m) for 90% of UEs

  - Vertical position accuracy (< 3 m) for 90% of UEs

  - End-to-end latency for position estimation of UE (< 100 ms)

  - Physical layer latency for position estimation of UE (< 10 ms)

- IIoT

  - Horizontal position accuracy (< 0.2 m) for 90% of UEs

  - Vertical position accuracy (< 1 m) for 90% of UEs

  - End-to-end latency for position estimation of UE (< 100ms, in the order of 10 ms is desired)

  - Physical layer latency for position estimation of UE (<10ms)

- V2X

  - Set 1: 10 – 50 m with 68 – 95 % confidence level.

  - Set 2: 1 – 3 m with 95 – 99 % confidence level.

  - Set 3: 0.1 – 0.5 m with 95 – 99 % confidence level.

- Public Safety

  - Horizontal position accuracy (< 1 m);

  - Absolute vertical position accuracy (< 2 m)

  - Relative vertical position accuracy (< 0.3 m)

  - 95 – 98 % positioning service availability.

# References

[1] RP-213588, “Study on expanded and improved NR positioning”, RAN #94e, Dec. 2021.

[2] 3GPP TR 38.855 V16.0.0, “Study on NR positioning support,” Release 16, 2019-03.

[3] 3GPP TR 38.857 V17.0.0, “Study on NR Positioning Enhancements,” Release 17, 2021-03.

[4] 3GPP TR 37.855 V15.3.0, “Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR,” Release 15, 2019-06.

[5] 3GPP TR 36.843 V12.0.1, “Study on LTE Device to Device Proximity Services,” Release 12, 2014-03.

[6] 3GPP TR 38.845 V17.0.0, “Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases,” Release 17, 2021-09.

[7] 3GPP TS 22.261: "Service requirements for the 5G system".

[8] 3GPP TS 22.104: " Service requirements for cyber-physical control applications in vertical domains".