3GPP TSG RAN WG1 #109-e	R1-2203942

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.5.1.2

Source:	NEC

Title: 	Evaluation of SL positioning

Document for:	Discussion and Decision

# Introduction

The work item on “Study on expanded and improved NR positioning solutions” for NR has been approved for Rel-18 and one of the objectives is to study solutions for sidelink positioning as follows in [1],

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2]

- Scenario/requirements

  - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage

  - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104

  - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104)

  - Spectrum: ITS, licensed

- Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1]

- Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1].

- Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2]

  - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1]

  - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1]

  - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1]

  - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required]

Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

This contribution will discuss evaluation methodology for SL positioning.

# Discussion

In our companion contribution [2], five scenarios have been defined for four use cases including commercial, IIOT, V2X and public safety as follows,

- Scenario 1. Indoor Office for FR1 and FR2 (Open office)

- Scenario 2. UMi street canyon for FR1 and FR2 (ISD 200m)

- Scenario 3. UMa (ISD 500m) for FR1 only including Urban grid and Highway

- Scenario 4. InF-SH for FR1 and FR2

- Scenario 5. InF-DH for FR1 and FR2

The scenario parameters defined in table 6.1.1-1 [3] can be used as a starting point common to all scenarios.

Table-1

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 2GHz, 4GHz – Note 1 5.9GHz | 30 GHz – Note 1 |
| Bandwidth, MHz | 200MHz (DL+UL) 20MHz (SL) 100MHz (SL) | 200MHz (DL+UL) 200MHz (SL) |
| Subcarrier spacing, kHz | 15kHz for 5MHz and 50MHz 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU - Optional: Provided by company |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 [3] |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

In table-1, we should consider two different bandwidth requirements for DL+UL and SL cases based on the requirements in [4] and [3]. UE antenna pattern for FR2 defined in table-6.1.1-2 and gNB antenna patterns defined in table-6.1.1-5 [3] can be used as common settings for all scenarios.

- The following scenario parameters can be considered as common parameters to all scenarios

- Table-1 can be used as a starting point common to all scenarios considering different bandwidth requirements for DL+UL and SL cases.

- UE antenna pattern for FR2 defined in table-6.1.1-2 and gNB antenna patterns defined in table-6.1.1-5 [3] can be used as common settings for all scenarios.

Some specific parameters can also be applied for different scenarios

- For scenario 1 Indoor Office, the specific parameters defined in Table 6.1.1-3 in [3] should be used as a starting point;

- For scenario 2 UMi street canyon, the specific parameters defined in Table 6.1.1-4 in [3] should be used as a starting point;

- For scenario 3 UMa, the specific parameters defined in Table 6.1.1-6 in [3] should be used as a starting point. Additional parameters, e.g., antenna pattern for UE-type RSU, deployment of UE-type RSU, dropping of cellular/Pedestrian UE, etc., should be defined from Urban grid and Highway following assumptions in Clause 6.1 [4];

- For scenario 4 and 5 inF, the specific parameters defined in Table 6.1.1 in [5] should be used as a starting point.

- Some specific parameters can also be applied for different scenarios as follows,

- For scenario 1 Indoor Office, the specific parameters defined in Table 6.1.1-3 in [3] should be used as a starting point;

- For scenario 2 UMi street canyon, the specific parameters defined in Table 6.1.1-4 in [3] should be used as a starting point;

- For scenario 3 UMa, the specific parameters defined in Table 6.1.1-6 in [3] should be used as a starting point. Additional parameters, e.g., antenna pattern for UE-type RSU, deployment of UE-type RSU, dropping of cellular/Pedestrian UE, etc., should be defined from Urban grid and Highway following assumptions in Clause 6.1 [4];

- For scenario 4 and 5 inF, the specific parameters defined in Table 6.1.1 in [5] should be used as a starting point.

It has been agreed that existing reference signals, procedures, etc from sidelink communication and from positioning should be reused as much as possible. Therefore, existing PRS and SRS should be used as baseline for evaluation.

- Existing PRS and SRS should be used as baseline for evaluation.

# Conclusion

In this contribution, we discussed evaluation methodology for SL positioning and the following observations and proposals are made:

Proposal 1	The following scenario parameters can be considered as common parameters to all scenarios

- Table-1 can be used as a starting point common to all scenarios considering different bandwidth requirements for DL+UL and SL cases.

- UE antenna pattern for FR2 defined in table-6.1.1-2 and gNB antenna patterns defined in table-6.1.1-5 [3] can be used as common settings for all scenarios.

Proposal 2	Some specific parameters can also be applied for different scenarios as follows,

- For scenario 1 Indoor Office, the specific parameters defined in Table 6.1.1-3 in [3] should be used as a starting point;

- For scenario 2 UMi street canyon, the specific parameters defined in Table 6.1.1-4 in [3] should be used as a starting point;

- For scenario 3 UMa, the specific parameters defined in Table 6.1.1-6 in [3] should be used as a starting point. Additional parameters, e.g., antenna pattern for UE-type RSU, deployment of UE-type RSU, dropping of cellular/Pedestrian UE, etc., should be defined from Urban grid and Highway following assumptions in Clause 6.1 [4];

- For scenario 4 and 5 inF, the specific parameters defined in Table 6.1.1 in [5] should be used as a starting point..

Proposal 3	Existing PRS and SRS should be used as baseline for evaluation.

# References

[1] RP-213588, “Study on expanded and improved NR positioning”, RAN #94e, Dec. 2021.

[2] R1-2203941, “SL positioning scenarios and requirements,” NEC, RAN1#109e.

[3] 3GPP TR 38.855 V16.0.0, “Study on NR positioning support,” Release 16, 2019-03.

[4] 3GPP TR 37.855 V15.3.0, “Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR,” Release 15, 2019-06.

[5] 3GPP TR 38.857 V17.0.0, “Study on NR Positioning Enhancements,” Release 17, 2021-03.