3GPP TSG RAN WG1 #109-e	                           R1-2203943

e-Meeting, May 9th – 20th, 2022

Source:	NEC

Title:	Discussion on potential solutions for SL positioning

Agenda Item:	9.5.1.3

Document for:	Discussion and Decision

# Introduction

According to the revised SID on study on expanded and improved NR positioning, some objectives of solutions for sidelink positioning were captured as following [1]:

- Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

Based on maximum leveraging existing design in sidelink and positioning, this contribution shares some views on reference signal design and positioning methods for sidelink positioning.

# Potential solutions for Sidelink positioning

  - Reference signal for sidelink positioning

Although the design of CSI-RS in sidelink supports up to two ports, positioning reference signal is absent from existing sidelink system. Naturally, sidelink positioning reference signal (S-PRS) design could utilize NR DL PRS and/or UL SRS-Pos as a baseline. SRS-Pos is characteristic of low PAPR based on ZC sequence, while PRS based on Gold sequence has good autocorrelation and cross correlation. These two reference signals show similar positioning performance in legacy RAT dependent positioning methods. A reasonable choice should depend on relative evaluation corresponding to specific scenarios/use cases in sidelink, though SRS-Pos seems to be more applicable as all channels and signals in sidelink are transmitted in UL slots. Besides, SL sync signal also may be considered for comparison.

In addition to the S-PRS generation, considering the channel structure of sindelink, the S-PRS transmission pattern, including number of OFDM symbols, number of repetitions, gap between two repetitions, etc., should be evaluated and then determined. The slot format of sidelink is different from that of Uu interface. For example, for PSSCH transmission, within the slots, there can be from 7 to 14 of the symbols reserved for sidelink operation, of which PSSCH can be transmitted in 5 to 12 symbols. The remaining sidelink symbols transmit some or all of PSCCH, PSFCH, AGC symbol(s), guard symbol(s). Fig. 1 illustrates two examples for sidelink slot format with different PSCCH and PSSCH configurations [2]. S-PRS should be designed adapting to the new slot formats.

Fig. 1(a): Example slot format of 2-symbol PSCCH, 2-symbol PSSCH-DMRS, and no PSFCH.

Fig. 1(b): Example slot format of 3-symbol PSCCH, 3-symbol PSSCH-DMRS, and PSFCH.

Proposal 1: Sidelink positioning reference signal (S-PRS) should be designed with NR DL PRS and UL SRS-Pos as a baseline and the design should be adapted to sidelink slot formats

Proposal 2: S-PRS transmission pattern should be determined based on S-PRS design and related evaluation.

−    including number of OFDM symbols, number of repetitions, gap between two repetitions, etc.

  - Resources for S-PRS

In current sidelink system, all reference signals are located within the PSSCH bandwidth and transmitted along with PSSCH. Since positioning accuracy is highly related to the bandwidth of positioning reference signal, PSSCH bandwidth may not always fulfil the target accuracy based on S-PRS measurement. Hence, besides the legacy frequency resource confined within PSSCH bandwidth, a dedicated wideband resource (or resource pool) could be introduced for S-PRS transmission according to the evaluated bandwidth requirement related to the target accuracy. Moreover, if the dedicated resource is adopted, S-PRS transmission should have specific pattern and resource allocation scheme which are different from that of existing reference signal on common resource pool.

Proposal 3: Both reusing frequency resource within PSSCH bandwidth and introducing dedicated frequency resource should be considered for S-PRS transmission.

Further, as mentioned in SID, sidelink positioning shall cover in-coverage and partial-coverage scenarios, as well as out-of-coverage scenario. Similar to the mode 1 resource allocation, network based resource allocation scheduling through Uu interface for S-PRS resources and related reporting should be supported if UE locates in network coverage. On the other hand, if UEs are out of network coverage, UE based resource allocation with contention solution through PC5 interface should be supported. The latter can be treated as the legacy mode 2 resource allocation which is also applicable to in-coverage scenario.

Proposal 4: Both network based and UE based resource allocation for S-PRS resources and related reporting should be supported.

−    If the dedicated frequency resource for S-PRS is introduced, corresponding resource allocation scheme should be studied separately.

When the UE is not in the coverage of a gNB, it can only communicate with each other via PC5 interface and PSCCH/PSSCH cannot be transmitted in all RBs and slots in the NR system bandwidth, nor within the frequency span configured for sidelink. Resource pools are defined for the respective channels. When UE is out-of-coverage of any gNB, UE can apply autonomous resource allocation, which is determined through a sensing procedure conducted autonomously by the transmitting UE. In such a case, the UE selects an appropriate amount of resources randomly and the resources selected are not in general periodic. Since the allocated resources can be random and thus non-continuous in either frequency or in time domain. The resource allocation of S-PRS should match these allocated resources and aperiodic PRS should be supported for sidelink positioning.

Proposal 5: The resource allocation of S-PRS in time and frequency domain should support non-continuous resource allocation.

Different from only periodic DL PRS in sidelink, UL SRS-Pos can be configured as a periodic, aperiodic and semi-persistent signal to match different use cases, such as operation of power limited UE. At least on-demand aperiodic or SPS S-PRS transmission has an advantage in sidelink positioning with respect to out-of-coverage scenario where UE can autonomously trigger the positioning procedure involving road side unit (RSU) and/or vulnerable road user (VRU). In turn, reporting resources for S-PRS measurement also should be configured equally.

Proposal 6: Periodic, aperiodic and semi-persistent resource configuration for S-PRS and related reporting should be supported.

  - Sidelink positioning procedure

Intuitively, various traditional positioning methods, namely RTT, TDOA, AOA, AOD, etc., could be applied to sidelink positioning. However, the impact of sidelink aspects on positioning should be considered and evaluated to identify the feasibility of candidate sidelink positioning methods, e.g. the effect of synchronization errors between sidelink UEs/nodes on ranging based methods and antenna elements number on angle measuring based methods.

Proposal 7: The potential sidelink positioning methods should be evaluated based on sidelink features to identify the feasible items.

Compared with relative positioning, absolute positioning is not necessary or feasible in certain scenarios due to the different positioning requirements and obtainable measurement source for positioning, especially in V2X and public safety. In such cases, only an accurate location relationship, i.e. relative distance and angle, between one UE and the other UE/RSU is needed, and could be realized based on S-PRS measurements, on-board sensors or other methods. Obviously, the relative positioning is fully competent for the specific applications.

Proposal 8: The applicability of absolute positioning and relative positioning should be studied to meet different sidelink positioning requirements.

For in-coverage scenario, a UE can support both network based and UE based positioning by the aid of Uu and PC5 interface, even a combination of two methods is also practicable. While S-PRS and/or other signals transmission, measurement and reporting only can rely on PC5 interface for out-of-coverage scenario, therefore UE based positioning is a realistic choice. Further, considering the target of the latency, energy consumption, reliability of measurement source, etc., UE based positioning may achieve a more efficient and beneficial results as an active approach at least in V2X where RSU and VRU (with or without absolute coordinate) play an important role.

Proposal 9: Both UE assisted and UE based positioning methods should be supported.

−    Including corresponding signalling and procedure for study

In Rel-16/17 positioning, especially in DL positioning, all positioning methods including OTDOA, RTT, AOA/D, etc assume that the references points with already known location, i.e., gNB, are fixed. In V2X, when using another vehicle or road user with known location is used as reference point, it is quite likely these UEs are moving and the movement of reference points might affect the positioning accuracy. According to [3], the UE velocity up to 250 km/h needs to be supported for outdoor and tunnel areas so that the relative velocity can be up to 500 km/h between two moving vehicles. This brings some new challenges and one of them is that the location of the reference points is no longer fixed as shown in Fig. 2.

Fig. 2 Relative movement of reference points

In Fig. 2, two vehicles are moving toward each other and if location of vehicle A is assumed to be known, it can be used as a reference point in addition to the gNB along the road for positioning of vehicle B. However, unlike the fixed gNB along the road, vehicle A is moving. For example, assuming the relative velocity is 500 km/h and maximum allowed positioning latency is 100 ms, the distance between the original location of the moving reference points, i.e. vehicle A at the beginning of the positioning procedure and the location at the end of the procedure can be more than 13 meters, which might cause some issues, especially for use cases/scenarios that require sub-meter accuracy.

Proposal 10: The movement of reference points should be taken into account for the study of sidelink positioning methods including OTDOA, RTT, AOA/D, etc, especially for DL positioning.

  - Other issues

It has been agreed that the spectrum used for sidelink positioning includes ITS and licensed bands. For ITS spectrum, the ITS bandwidth in all the regions is smaller than 80MHz and in some countries only 20MHz is allocated to ITS [4]. However, the positioning accuracy is related to PRS spanned bandwidth. Normally, large bandwidth is needed to achieve high accuracy such as sub-meter accuracy. Therefore, ITS spectrum may not be able to provide sufficient accuracy when timing difference based positioning methods are used. In this regard, carrier phase based positioning method should also be considered for sidelink positioning.

Carrier phase measurements has long been used for estimating position and time in satellite navigation receivers. Carrier phase measurements are roughly 1000 times less noisy and much less sensitive to multipath than code phase measurements [5]. However, the main challenge is the reliable resolution of their integer ambiguities. Blewitt’s bootstrapping and Teunissen’s LAMBDA method have been widely used for solving the integer ambiguity and some new methods with reduced complexity have been proposed recently [6]. However, even with the new methods, the remaining complexity is still quite high, especially for those power saving users. In order further reduce the complexity, timing/power/AoA/AoD based positioning methods can be combined with carrier phase based positioning method to firstly used to reduce the searching space of the integer ambiguity and corresponding measurement should be conducted.

Proposal 11:	Timing/power/AoA/AoD based positioning methods can be combined with carrier phase based positioning method to firstly used to reduce the searching space of the integer ambiguity and corresponding measurement should be conducted.

# Conclusion

This contribution shares our views on potential solutions for sidelink positioning, and the following proposals are made:

Proposal 1: Sidelink positioning reference signal (S-PRS) should be designed with NR DL PRS and UL SRS-Pos as a baseline and the design should be adapted to sidelink slot formats.

Proposal 2: S-PRS transmission pattern should be determined based on S-PRS design and related evaluation.

−    including number of OFDM symbols, number of repetitions, gap between two repetitions, etc.

Proposal 3: Both reusing frequency resource within PSSCH bandwidth and introducing dedicated frequency resource should be considered for S-PRS transmission.

Proposal 4: Both network based and UE based resource allocation for S-PRS resources and related reporting should be supported.

−    If the dedicated frequency resource for S-PRS is introduced, corresponding resource allocation scheme should be studied separately.

Proposal 5: The resource allocation of S-PRS in time and frequency domain should support non-continuous resource allocation.

Proposal 6: Periodic, aperiodic and semi-persistent resource configuration for S-PRS and related reporting should be supported.

Proposal 7: The potential sidelink positioning methods should be evaluated based on sidelink features to identify the feasible items.

Proposal 8: The applicability of absolute positioning and relative positioning should be studied to meet different sidelink positioning requirements.

Proposal 9: Both UE assisted and UE based positioning methods should be supported.

−    Including corresponding signalling and procedure for study

Proposal 10: The movement of reference points should be taken into account for the study of sidelink positioning methods including OTDOA, RTT, AOA/D, etc, especially for DL positioning.

Proposal 11:	Timing/power/AoA/AoD based positioning methods can be combined with carrier phase based positioning method to firstly used to reduce the searching space of the integer ambiguity and corresponding measurement should be conducted.

# References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning”, Intel

- 3GPP TR 37.985 V17.1.1, “Overall description of Radio Access Network (RAN) aspects for Vehicle-to-everything (V2X) based on LTE and NR,” Release 17, 2022-03.

- 3GPP TR 38.845 V17.0.0, “Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases,” Release 17, 2021-09.

- Junsung Choi, Vuk Marojevic, Carl B. Dietrich, Jeffrey H. Reed and Seungyoung Ahn, “Survey of Spectrum Regulation for Intelligent Transportation Systems,” IEEE Access, vol. 8, pp. 140145 - 140160, Jul. 2020.

- Christoph Günther and Patrick Henkel, “Integer Ambiguity Estimation for Satellite Navigation,” IEEE Trans. on Signal Processing, vol. 60, pp. 3387-3393, Jul. 2012.

- Zhenyu Zhang, Shaoli Kang and Xiang Zhang, “Indoor Carrier Phase Positioning Technology Based on OFDM System,” Sensors, 2021.