3GPP TSG RAN WG1 #109-e			                                       R1-2203945

e-Meeting, May 9th – 20th, 2022

Source:	NEC

Title:	Discussion on subband non-overlapping full duplex

Agenda Item:	9.3.2

Document for: 	Discussion and Decision

# Introduction

In RAN #94-e meeting, a new SI on evolution of NR duplex operation was approved. The objective of the SI is listed as follows [1].

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution, we discuss and provide views for subband non-overlapping full duplex.

# Discussion

## Mechanisms for sub-band non-overlapping full duplex operation

In Rel-18, for subband non-overlapping full duplex at the gNB side within a TDD band, gNB can schedule transmissions with different UL and DL directions in non-overlapped sub-bands for different UEs in a cell at a time, which can improve UL coverage, latency reduction and system capacity.

In traditional NR TDD, the time domain resources are split between downlink and uplink for both gNB and UE either by semi-static TDD configuration (e.g., tdd-UL-DL-ConfigurationCommon/tdd-UL-DL-ConfigurationDedicated) and/or by dynamic TDD configuration (e.g., SFI in DCI format 2_0). Note that the TDD configurations can be dedicatedly provided to UE, hence, gNB can potentially schedule DL and UL for different UEs at the same time achieving functionality of full duplex. For subband non-overlapping full duplex, we need to study whether any enhancements are required to support TDD signaling or existing mechanisms are sufficient. Following points need to be considered especially for any enhancements to existing TDD signaling:

- Backward compatibility and the performance of legacy NR UEs need to be ensured for any TDD signaling enhancement

- Strive to define TDD signaling mechanism to maximize the performance of UEs supporting this feature, however it needs to be discussed whether the UEs need be aware of full duplex operation at gNB side

- Allow gNB to change the TDD configuration for full duplex dynamically to account for dynamic network requirements

The need for signaling enhancements can be realized after finalization of mechanism for subband non-overlapping full duplex operation and its impacts on different UE or network procedures.

Observation-1: Full duplex operation can be currently achieved in NR by providing different tdd-UL-DL-ConfigurationDedicated or SFI to different UEs in a cell, however, it needs to be studied whether any signaling enhancements are required for subband non-overlapping full duplex operation

Proposal 1:

- For subband non-overlapping full duplex at the gNB side, study whether enhancements are required to configure TDD configuration for UEs in non-overlapping sub-bands with different transmission directions. Consider the following points for discussion,

  - Backward compatibility and the performance of legacy NR UEs need to be ensured for any TDD signaling enhancement

  - Strive to define TDD signaling mechanism to maximize the performance of UEs supporting this feature, however it needs to be discussed whether the UEs need be aware of full duplex operation at gNB

  - Allow gNB to change the TDD configuration for full duplex dynamically to account for dynamic network requirements

On the other hand, gNB schedules PDSCH or PDSCH transmissions for TDD UE based on BWP configuration, e.g., UE determines the frequency domain resource allocation for data transmission based on the active BWP.  While for subband non-overlapping full duplex, whether a BWP only includes sub-bands with single transmission direction at a time or a BWP can include multiple non-overlapped sub-band with different transmission directions is not clear. It may impact other physical operation, e.g., BWP switching operation and so on, which should be studied with high priority. Below figure illustrates the mechanism for two possible approaches.

Fig. 1 Possible approaches for subband non-overlapping full duplex operation

In Option-1, subband non-overlapping full duplex is achieved by having UL and DL time occasions in parallel but in different BWPs. The figure shows the case where BWP1 is DL dominated and BWP2 is UL dominated, however, exact utilization and scheduling can be flexible based on network requirements. There are certain issues associated with supporting this option:

- Support of multiple active BWPs (with fixed frequency partitioning) which reduces feature effectiveness

- Its not clear how this option can be used in future releases to support full duplex using overlapped subbands

- Higher BWP switching rate is expected to fully utilize the benefits of this feature, which incurs high signaling overhead

In Option-2, gNB uses single BWP to enable subband non-overlapping full duplex operation. This approach allows higher flexibility but it also requires further discussion in relation to its impact on different physical channels and physical signals resource allocations.

Observation-2: Two options are possible to perform non-overlapping subband duplex operation:

- Single BWP supporting single transmission direction. UL and DL subbands are present in different BWPs for enabling full duplex

- Single BWP supporting UL and DL transmission direction in the same time occasion

Proposal 2:

- Discuss whether subband non-overlapping full duplex in gNB is performed per bandwidth part or across different bandwidth parts

## CLI Impacts for sub-band non-overlapping full duplex operation

In NR Rel-16, CLI handling and RIM were introduced for interference management. While for subband non-overlapping full duplex, some new types of interference problems may occur. For example, for a cell with subband non-overlapping full duplex, cross link interference between UEs in a cell may happen, which hasn’t been handled in prior studies. We need to study different mechanisms on how to address these interference issues based on frequency isolation (e.g configuring suitable guard band in frequency domain) or using enhanced interference measurement mechanism or using intelligent scheduling decisions (e.g. UL and DL UEs to be scheduled in different beams).

Further, there is an issue for self-interference at gNB arising from DL transmission from the gNB interfering with its UL receptions in adjacent subband. This is mainly arising due to significant power difference between DL and UL near gNB which is likely to degrade the gNBs receiver sensitivity. There is a need to study whether available self-interference mitigation techniques at gNB are sufficient to resolve this interference issue or any standard based changes are required for enabling efficient interference management.

In addition, in case one gNB with sub-band full duplex mode and another gNB with DL-dominant TDD configuration are adjacent deployed for different application scenarios, there maybe cross link interference from gNB #1 to gNB #2, as shown in the figure. In order to achieve the benefits of subband non-overlapping full duplex, the inter-UE CLI in a cell and inter-gNB CLI should be studied in Rel-18 and mechanisms need to be identified to mitigate the interference.

Observation-3: Following interference scenarios are possible for subband non-overlapping full duplex operation:

- CLI between UEs of same cell i.e. UL transmission of one UE interfering with the DL reception of nearby UE

- gNB experiencing interference in UL reception due to its DL transmission in adjacent subband

- gNB experiencing interference in UL reception due to DL transmission from nearby gNB

Fig.2 Example for CLI between gNBs with different duplex modes

Proposal 3:

- In Rel-18, for subband non-overlapping full duplex at the gNB side , study interference management for

  - inter-UE CLI in a cell

  - self-interference management support for gNB

  - inter-gNB CLI

# Conclusion

In this contribution, we discussed subband non-overlapping full duplex for NR duplex operation. Observation and Proposals are summarized as following:

Observation-1: Full duplex operation can be currently achieved in NR by providing different tdd-UL-DL-ConfigurationDedicated or SFI to different UEs in a cell, however, it needs to be studied whether any signaling enhancements are required for subband non-overlapping full duplex operation

Observation-2: Two options are possible to perform non-overlapping subband duplex operation:

- Single BWP supporting single transmission direction. UL and DL subbands are present in different BWPs for enabling full duplex

- Single BWP supporting UL and DL transmission direction in the same time occasion

Observation-3: Following interference scenarios are possible for subband non-overlapping full duplex operation:

- CLI between UEs of same cell i.e. UL transmission of one UE interfering with the DL reception of nearby UE

- gNB experiencing interference in UL reception due to its DL transmission in adjacent subband

- gNB experiencing interference in UL reception due to DL transmission from nearby gNB

Proposal 1:

- For subband non-overlapping full duplex at the gNB side, study whether enhancements are required to configure TDD configuration for UEs in non-overlapping sub-bands with different transmission directions. Consider the following points for discussion,

  - Backward compatibility and the performance of legacy NR UEs need to be ensured for any TDD signaling enhancement

  - Strive to define TDD signaling mechanism to maximize the performance of UEs supporting this feature, however it needs to be discussed whether the UEs need be aware of full duplex operation at gNB

  - Allow gNB to change the TDD configuration for full duplex dynamically to account for dynamic network requirements

Proposal 2:

- Discuss whether subband non-overlapping full duplex in gNB is performed per bandwidth part or across different bandwidth parts

Proposal 3:

- In Rel-18, for subband non-overlapping full duplex at the gNB side , study interference management for

  - inter-UE CLI in a cell

  - self-interference management support for gNB

  - inter-gNB CLI

# References

- RP-213591, ‘New SI: Study on evolution of NR duplex operation,’ CMCC.