3GPP TSG RAN WG1 #109-e		R1-2203951

e-Meeting, May 9th – 20th, 2022

Source:	OPPO

Title:	UE features for further enhancements on NR-MIMO

Agenda Item:	8.16.1

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the UE feature for Rel-17 eMIMO.

Discussion

  - UE feature for Multi-TRP Deployment

In RAN1#108 e-meeting, the candidate values for number of additional PCIs for Case 1 and Case 2 were discussed with two remaining issues: whether 0 is supported for X1 and whether 4,5,6 are supported for X1 and X2. For the first issue, considering there is a note saying “UE indicates a non-zero value for at least one of component 2 or component 3”, we think 0 can be included as candidates for X1. For the second issue, considering {4,5,6} neighboring cells are not typical deployment in network, {0,1,2,3,7} seems sufficient for capability reporting.

Proposal: The candidate values for both X1 and X2 can be {0,1,2,3,7}.

  - UE feature for CSI enhancements on MTRP and FR1 FDD reciprocity

In FG 23-7-1, there is a component for “CSI report mode selection of mode 1 with X=0 and/or mode 2” with candidates of {mode 1 with X=0, mode 2, both}. In FG 23-7-1a, there is additional FG for CSI report mode 1 saying “Maximum value of numberOfSingleTRP-CSI-Mode1” with candidate values of {X=1, X=2}. In our understanding, mode 1 with X=0 should be pre-requisite of mode 1 with X=1 and X=2. If UE cannot support mode 1 with X=0, it is impossible to support mode 1 with X=1 and X=2. That is, UE can report FG 23-7-1a only when it reports“mode 1 with X=0” or “both” for component 3 of FG 23-7-1.

Proposal: Add a note for FG 23-7-1a (Additional CSI report mode 1 selection): UE reports this capability only when UE reports “mode 1 with X=0” or “both” for component 3 of FG 23-7-1.

Conclusion

In this contribution, we discuss the UE feature for eMIMO enhancement in Rel-17 with the following proposals:

Proposal: The candidate values for both X1 and X2 can be {0,1,2,3,7}.

Proposal: Add a note for FG 23-7-1a (Additional CSI report mode 1 selection): UE reports this capability only when UE reports “mode 1 with X=0” or “both” for component 3 of FG 23-7-1.