3GPP TSG RAN WG1 #109-e		R1-2203959

e-Meeting, May 9th – 20th, 2022

Source:	OPPO

Title:	SRI TPMI enhancement for 8 TX UL transmission

Agenda Item:	9.1.4.2

Document for:	Discussion and Decision

Introduction

The following objectives were agreed for uplink 8 Tx transmission in Rel-18 MIMO Evolution WID[1]:

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In this contribution, we will provide some initial considerations on codebook/SRI/TPMI enhancement to support 8Tx uplink transmission.

Discussion

  - Codebook design for 8 Tx uplink

To support codebook based uplink 8 ports transmission, the current uplink codebook should be extended to 8 port. From LTE to NR, the codebook design has been optimized for some specific antenna pattern. Before the study and evaluation, RAN1 should first confirm the antenna pattern assumption used for 8Tx codebook. Considering the possible devices with 8Tx are various, e.g. CPE/FWA/vehicle/Industrial devices, it may be difficult to determine single antenna pattern for the study and evaluation. As shown in Fig.1, there are different antenna arrays that may be applied to 8 Tx. Considering the evaluation effort, companies should focus on limited antenna pattern(s) for evaluation of 8Tx uplink transmission

(a) Uniform linear array                                              (b) Uniform 2D linear array

(c) Clustered linear array                                             (d) Four-sided array

Fig.1: Typical antenna arrays that may be applied to 8 Tx

Proposal 1: Limited antenna pattern(s) should be confirmed for evaluation of 8Tx uplink transmission.

Three coherent assumptions are considered for 2Tx and 4Tx codebook in NR: fully-coherent, partial-coherent and non-coherent. For 8Tx, we have the following initial considerations for codebook for different coherent assumptions:

- For fully coherent codebook, the codebook design should consider practical antenna patterns. At least for uniform linear array similar to downlink 8Tx, DL 8Tx Type 1 CB with wideband beam and co-phasing can be considered as baseline. For the values of N1 and N2, whether both (4,1) and (2,2) are supported needs further study. To further reduce the DCI overhead, evaluation is needed to study whether the beam number can be reduced with minimized performance impact. For other antenna arrays, e.g. clustered or four-sided antenna array in Fig.1, other codebook design can also be considered. However, the number of uplink codebooks should be restricted considering the UE complexity and standardization effort.

- For partial coherent codebook, Rel-15 4Tx codebook only considers the coherence within each pair of dual-polarization antennae, e.g. between 1st and 3rd antenna ports, 2nd and 4th antenna ports. For 8Tx, considering similar dual-polarization antenna array will be applied, this rule can be reused. That is, 8Tx partial coherent codebook only considers intra-polarization coherence, e.g. the non-zero elements only occur in pairs in 1st and 5th ports, 2nd and 6th ports, 3rd and 7th ports, 4th and 8th ports.

- Whether non-coherent codebook is supported for uplink 8Tx needs further study. From UE antenna architecture, it seems a very corner case that the 8 TXRUs of one UE are fully non-coherent considering dual-polarization antenna array is very likely to be applied to limit antenna size. From signaling overhead, non-coherent codebook will significantly increase the DCI overhead and the application scenarios need further justification. Hence, we propose non-coherent 8Tx codebook as low priority.

In this section, we provide some initial evaluation results for 8Tx codebook design. The following cases are simulated:

- For uniform linear antenna array (a and b in Fig.1) and clustered linear antenna array (c in Fig.1), DL 8Tx Type 1 CB with wideband beam and co-phasing is evaluated. It is compared with another possible solution: UL 4Tx CB is applied to the first 4 ports and last 4 ports with QPSK co-phasing between the two CBs. Different number of beams or co-phasings are evaluated for DL 8Tx CB.

- For uniform linear antenna array (a in Fig.1) and rank 1, 8Tx partial coherent codebook is compared with 4Tx partial coherent codebook. Only the coherence within each pair of dual-polarization antennae is considered.

- For uniform linear antenna array (a in Fig.1) and rank 1, 8Tx non-coherent codebook is compared with 4Tx non-coherent codebook.

The detailed simulation assumptions can be found in appendix.

Fig.2: Performance of different codebooks with uniform linear array (N1, N2=4,1)

Fig.3: Performance of different codebooks with uniform linear array (N1, N2=2,2)

Fig.4: Performance of different codebooks with clustered linear array (N1, N2=4,1)

Fig.5: Performance of partial coherent codebook for 8 ports and 4 ports

Fig.6: Performance of non-coherent codebook for 8 ports and 4 ports

From the results, it can be observed that:

- The codebook with best performance may vary with different assumptions of antenna arrays.

- For different arrays, the required number of beams/co-phasings may be different.

- With the assumption of the same transmit power, 8Tx partial coherent and non-coherent codebook can provide some gain over 4Tx.

Proposal 2: Support full coherent and partial coherent codebook for 8 Tx uplink.

- For full-coherent codebook with uniform linear antenna array, DL 8Tx Type 1 CB (wideband beam and co-phasing) as baseline with possible beam reduction.

  - FFS for other antenna array, e.g. clustered or four-sided antenna array

- For partial-coherent codebook, only consider intra-polarization coherence (2Tx) similar to Rel-15.

- Non-coherent codebook has low priority considering practical antenna pattern and signaling overhead.

  - TRI/TPMI for 8 Tx uplink

For uplink 8Tx transmission, further enhancement to TRI and TPMI indication is needed. For TRI, the indication should support up to 8 layers transmission. For TPMI, the signaling should support new 8 port codebook. In current DCI format 0_1 and 0_2, the TRI and TPMI are jointly encoded for 2 ports and 4 ports transmission. For 8 ports transmission, the sum of codebook size is expected to be significantly larger than that of 4 ports (e.g. up to 256 codewords for 8Tx rank1), and the joint encoding may lead to the mapping table design very complicated. Furthermore, compared to single stage codebook for 2/4 Tx, dual-stage codebook similar to DL (e.g. DL 8Tx type 1 CB) may be introduced for 8Tx. For example, the TPMI may include separate beam information and co-phasing information. In this case, joint encoding of TRI and TPMI may not be appropriate for TPMI indication. Separate indication of TRI and TPMI for 8Tx uplink can be considered.

Proposal 3: Consider separate indication of TRI and TPMI for 8 Tx uplink.

  - SRI for 8 Tx uplink

In current codebook based uplink transmission, SRI can indicate one SRS resource from up to two SRS resources. To support 8 port codebook transmission, the number of antenna ports within one SRS resource should be extended to 8. In this case, no SRI enhancement is needed at least for non-full-power transmission. If full power transmission mode 2 is supported, one SRS resource set may include up to 4 or more SRS resources with 1, 2, 4 or 8 ports. Whether 2 bits SRI information is needed for this case needs further study.

For non-codebook based transmission, up to 8 SRS resources can be configured in one SRS resource set. In this case, the SRI information should be enhanced to indicate up to 8 SRS resources to support up to 8 layers transmission. If current indication method for 4Tx is reused, e.g. a larger table is introduced for 8 Tx to support any SRS resource combinations, the standardization complexity may be unacceptable, considering the considerable number of possible SRS resource combinations. For example, for the case with 8 SRS resources for non-codebook, there can be up to 256 combinations for Lmax=8. Therefore, a new method for SRI indication can be considered taking signaling overhead and standardization complexity into count.

Proposal 4: Introduce SRI enhancement to indicate up to 8 SRS resources for non-codebook uplink transmission, considering signaling overhead and standardization complexity.

Conclusion

In this contribution, we discuss the possible enhancement to codebook design and TRI/TPMI/SRI indication for 8 Tx uplink transmission. To summarize, we have the following proposals:

Proposal 1: Limited antenna pattern(s) should be confirmed for evaluation of 8Tx uplink transmission.

Proposal 2: Support full coherent and partial coherent codebook for 8 Tx uplink.

- For full-coherent codebook with uniform linear antenna array, DL 8Tx Type 1 CB (wideband beam and co-phasing) as baseline with possible beam reduction.

  - FFS for other antenna array, e.g. clustered or four-sided antenna array

- For partial-coherent codebook, only consider intra-polarization coherence (2Tx) similar to Rel-15.

- Non-coherent codebook has low priority considering practical antenna pattern and signaling overhead.

Proposal 3: Consider separate indication of TRI and TPMI for 8 Tx uplink.

Proposal 4: Introduce SRI enhancement to indicate up to 8 SRS resources for non-codebook uplink transmission, considering signaling overhead and standardization complexity.

References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung (Moderator), 3GPP TSG RAN Meeting #94e, Dec. 6th – 17th, 2021.

Appendix

Table 1: Simulation assumption

| Parameter | Assumption |
| --- | --- |
| Carrier frequency | 4GHz |
| SCS, | 30kHz |
| Channel model | CDL-C in TR 38.901 with 300ns delay spread |
| System BW | 20MHz |
| UE speed | 3km/h |
| PRBs | 48 |
| Number of UE antennas | 8Tx with (M, N, P) = (1,4,2), (dH, dV) = (0.5, 0.5) λ, 8Tx with (M, N, P) = (2,2,2), (dH, dV) = (0.5, 0.5) λ, |
| Number of gNB antennas | 8Rx with (M, N, P) = (1,4,2), (dH, dV) = (0.5, 0.5) λ, |
| Precoding | Wideband |
| SRS periodicity | 4 slots |
| SRS Comb | Comb 2 (FDM+CDM) |
| Rank | 1 |
| AMC | on |
| HARQ | on |
| Metric | Throughput |