3GPP TSG RAN WG1 #109-e		R1-2203963

e-Meeting, May 9th – 20th, 2022

Source:	OPPO

Title:	Discussion on “Questions concerning the implementation of RAN1 agreements in NRPPa”

Agenda Item:	5

Document for:	Discussion and Decision

Introduction

RAN1 received a LS from RAN3 with regard to R17 positioning enhancement [1]. In this contribution, we will discuss some of the questions raised by RAN3 and suggest the corresponding replies.

Discussion

- Issue 1: RAN1 to feedback if information on the Rx Diversity options needs to be signalled to LMF and to comment on the UL-SRS-RSRPP value range, if needed?

Those two are not really two different options for the TRP to choose one for implementing SRS-RSRPP or SRS-RSRP measurement. Furthermore, those two notes have been implemented in TS 38.215 of Rel-17. Them specifies the requirement when TRP measures SRS-RSRPP or SRS-RSRP when Rx diversity is implemented. As stated in those two Options in the RAN1 agreement and also the specification in TS 38.215, when Rx diversity is used for receiving SRS for positioning, the TRP shall use the same Rx branches to measure the SRS-RSRP and SRS-RSRPP. And when SRS-RSRPP of both the first path and additional paths are measured, the TRP shall use the same Rx branches to measure them. To summarize, they are not options that the TRP can choose one or the other in measuring SRS-RSRPP and SRS-RSRP and therefore, they are not needed to be signaled to LMF along with the SRS-RSRPP measurement.

Proposed reply 1:

From RAN1 perspective, there is no need to signal the information of Rx diversity options of UL SRS-RSRPP measurement.

- Issue 2: RAN1 to feedback if information on the SRS port index needs to be signalled to LMF when SRS resource for MIMO is used.

- From Rel-16, whether the MIMO SRS resource is used for positioning or not is totally transparent to UE. Thus, in Rel-17 positioning enhancement, the TEG is only introduced for SRS resources for positioning and is not applicable for MIMO SRS resources. Consequently, LMF/gNB cannot make any assumption on the following aspects:

- The timing difference between the different SRS ports of the SRS resource are within a threshold.

- The timing difference between the different transmissions of the same port of the same SRS resource are within a threshold.

- Consequently, the only meaningful use case for LMF is that LMF groups the reported measurement results based on the same instance of SRS transmission and the same SRS port. Then, LMF determines the location based on these measurements for the target UE. However, it is not clear about the possibility that TRPs measure the results on the same instance of SRS transmission and on the same SRS port.

- Based on the above discussion, we propose the following reply:

Proposed reply 2:

From RAN1 perspective, there is no need to signal the information on the SRS port index to LMF when SRS resource for MIMO is used since the benefit is not clear.

Conclusion

In this contribution, we discussed the questions of RAN3 LS and suggest replies as below:

Proposed reply 1:

From RAN1 perspective, there is no need to signal the information of Rx diversity options of UL SRS-RSRPP measurement.

Proposed reply 2:

From RAN1 perspective, there is no need to signal the information on the SRS port index to LMF when SRS resource for MIMO is used since the benefit is not clear.

References

- R1-2203040(R3-222721), Questions concerning the implementation of RAN1 agreements in NRPPa