3GPP TSG RAN WG1 #109-e		R1-220xxxx

e-Meeting, May 9th – 20th, 2022

Source:	OPPO

Title:	Discussion on Low Power High Accuracy Positioning

Agenda Item:	9.5.2.3

Document for:	Discussion and Decision

- Introduction

In R16, the positioning functionality was introduced to NR system. In R17, further enhancement for positioning was studied and specified to improve the accuracy, latency, UE power consumption and so on. In RAN#93e meeting, a new SI was approved for the expanded and improved NR positioning [1]. In addition to further enhancement on the traditional dimensions for NR positioning, the main focus of this SI is to expand the NR positioning technology to more scenarios and to new types of UE. To be specifically, The R18 positioning SI focuses on three aspects:

- Slidelink positioning

- Improved accuracy, integrity and power efficiency for NR positioning

- Positioning support for RedCap UE

To be specific, the detailed objectives for the low power high accuracy positioning (LPHAP) is defined as below [1]:

- Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1] - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study. - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state

In this contribution, we will discuss the low power high accuracy positioning from the perspective of the requirement, evaluation and potential enhancements.

Requirement and scenario

LPHAP is an integral part of a considerable number of IoT applications. In TS 22.104, nine use cases were captured for the low power high accuracy positioning [2]. According to the SID of R18 positioning [1], the study in RAN level should focus on Use case 6 that is highlighted by yellow as below.

Table 1: Low power high accuracy positioning use cases from Table A.7.2-1 of [2]

| Use Case # | Horizontal accuracy | Corresponding service level (22.261) | Positioning interval/ duty cycle | battery life time/ minimum operation time |
| --- | --- | --- | --- | --- |
| 1 | 10 m | Service Level 1 | on request | 24 months |
| 2 | 2 m to 3 m | Service Level 2 | < 4 seconds | > 6 months |
| 3 | < 1 m | Service Level 3 | no indication | 1 work shift - 8 hours (up to 3 days, 1 month for inventory purposes) |
| 4 | < 1 m | Service Level 3 | 1 second | 6 - 8 years |
| 5 | < 1 m | Service Level 3 | 5 seconds - 15 minutes | 18 months |
| 6 | < 1 m | Service Level 3 | 15 s to 30 s | 6 - 12 months |
| 7 | 30 cm | Service Level 5 | 250 ms | 18 months |
| 8 | 30 cm | Service Level 5 | 1 second | 6 - 8 years (no strong limitation in battery size) |
| 9 | 10 m | Service Level 1 | 20 minutes | 12 years (@20mJ/position fix) |

Use case #6 represents various typical use cases for flexible modular assembly area, e.g., tracking of workpiece (in- and outdoor) in assembly area and warehouse. The detailed Service level 3 in Table 1 is defined in TS 22.261 [3] as below:

Table 2: Performance requirements for Horizontal and Vertical positioning service levels from Table 7.3.2.2-1 of [3]

| Positioning service level | Absolute(A) or Relative(R) positioning | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Coverage, environment of use and UE velocity |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Horizontal Accuracy | Vertical Accuracy (note 1) |  |  | 5G positioning service area | 5G enhanced positioning service area (note 2) |  |
|  |  |  |  |  |  |  | Outdoor and tunnels | Indoor |
| 1 | A | 10 m | 3 m | 95 % | 1 s | Indoor - up to 30 km/h Outdoor (rural and urban) up to 250 km/h | NA | Indoor - up to 30 km/h |
| 2 | A | 3 m | 3 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 3 | A | 1 m | 2 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 4 | A | 1 m | 2 m | 99,9 % | 15 ms | NA | NA | Indoor - up to 30 km/h |
| 5 | A | 0,3 m | 2 m | 99 % | 1 s | Outdoor (rural) up to 250 km/h | Outdoor (dense urban) up to 60 km/h Along roads and along railways up to 250 km/h | Indoor - up to 30 km/h |
| 6 | A | 0,3 m | 2 m | 99,9 % | 10 ms | NA | Outdoor (dense urban) up to 60 km/h | Indoor - up to 30 km/h |
| 7 | R | 0,2 m | 0,2 m | 99 % | 1 s | Indoor and outdoor (rural, urban, dense urban) up to 30 km/h Relative positioning is between two UEs within 10 m of each other or between one UE and 5G positioning nodes within 10 m of each other (note 3) |  |  |

As shown in Table 2, the Positioning Service Level 3 considers velocity up to 500 Km/h for trains and up to 250 km/h for other vehicles in outdoor scenarios. On the other hand, the R18 SID limits the study to a UE in RRC_INACTIVE or RRC_IDLE state. If the velocity is high, the UE in RRC_INACTIVE or RRC_IDLE state will suffer the frequent RRC configuration/reconfiguration due to the fast change of serving cells. In this case, there may be no much difference between RRC_INACTIVE/RRC_IDLE and RRC_ACTIVE from the perspective of UE power consumption. It is not preferred to spend efforts on further enhancements for UE in RRC_INACTIVE/RRC_IDLE with high velocity. Therefore, we suggest to focus on the scenarios with low velocity (e.g., up to 3km/h).

Proposal 1: For the study/evaluation of LPHAP, support the scenario with low velocity (e.g., up to 3km/h) as first priority.

Regarding the positioning accuracy requirement, we can reuse the values recommended by SA1. On the other hand, according to the throughout study and evaluation for R17 positioning enhancement, we can see that the service availability of 99% cannot be achieved in typical commercial use cases and IIoT use cases [4]. Therefore, in the study and evaluation of LPHAP, we should consider practical/achievable requirement for typical use cases.  Following this principle, we suggest the following proposal by jointly considering requirements from SA1 and the practical design targets identified during R17 study.

Proposal 2: For the study/evaluation of LPHAP, the target positioning requirements is suggested as below

- Horizontal position accuracy (< 1 m) for 90% of UEs

- End-to-end latency for position estimation of UE (< 1 s)

- Evaluation methodology and assumption

Since Use case# 6 is mainly for flexible modular assembly area, e.g., tracking of workpiece (in- and outdoor) in assembly area and warehouse, the scenario of InF-DH can be used as a good starting point for evaluation. In order to reduce the efforts spent on the evaluation assumptions, we can reuse the FR1 InF-DH scenarios captured in TR 38.857 for potential evaluation.

Proposal 3: If RAN1 evaluation is needed for LPHAP, support to reuse the evaluation assumptions of FR1 InF-DH scenario captured in TR 38.857

During the study of Rel-16 UE power saving, RAN1 defined a good model to evaluate the UE power consumption [5]. Thus, if the evaluation of power assumption is needed for LPHAP, the UE power consumption model of [5] can be used as the baseline with some potential further refinement. Meanwhile, the R18 study item is limited to UE in RRC_INACTIVE or RRC_IDLE. From this perspective, we can simplify the UE power consumption of TR 38.840 by reducing the number of power states. Thus, we have the following proposal:

Proposal 4: If RAN1 evaluation is needed for power consumption of LPHAP, suggest to take the power consumption model of [5] as the starting point and consider the following power states

- For positioning methods based on DL PRS

  - Deep sleep

  - PRS reception and processing

  - UL transmission for positioning reporting

- For positioning methods based on UL SRS resources for positioning

  - Deep sleep

  - SRS transmission

- For positioning methods based on both DL PRS and UL SRS resources for positioning

  - Deep sleep

  - PRS reception and processing

  - UL transmission for positioning reporting

  - SRS transmission

    - Note: SRS transmission and UL transmission for positioning reporting may be merged into one state

- Potential enhancement

As SA1 recommended, the positioning interval/ duty cycle for Use case# 6 can be from 15s to 30s. According to the current version of TS 37.355, the candidate values of report interval can be 0.25, 0.5, 1, 2, 4, 8, 10, 16, 20 and 32 seconds. If a service requires a positioning interval of 30s to meet the requirements, LMF might have to set the value of report interval to be 20s according to the current specification. If so, UE would have to report the measurement results more frequent than the service expects, which will lead to additional power consumption. If UE can report the measurement result every 30s for this service, less frequent transmission of positioning reporting can reduce UE power consumption. From this perspective, we have the following proposal:

Proposal 5: Study whether or not to introduce more candidate values for the reporting interval for the UE power saving.

- Conclusions

In this contribution, we discussed different aspects of low power high accuracy positioning. Based on the discussion, we had the following proposals:

Proposal 1: For the study/evaluation of LPHAP, support the scenario with low velocity (e.g., up to 3km/h) as first priority.

Proposal 2: For the study/evaluation of LPHAP, the target positioning requirements is suggested as below

- Horizontal position accuracy (< 1 m) for 90% of UEs

- End-to-end latency for position estimation of UE (< 1 s)

Proposal 3: If RAN1 evaluation is needed for LPHAP, support to reuse the evaluation assumptions of FR1 InF-DH scenario captured in TR 38.857

Proposal 4: If RAN1 evaluation is needed for power consumption of LPHAP, suggest to take the power consumption model of [5] as the starting point and consider the following power states

- For positioning methods based on DL PRS

  - Deep sleep

  - PRS reception and processing

  - UL transmission for positioning reporting

- For positioning methods based on UL SRS resources for positioning

  - Deep sleep

  - SRS transmission

- For positioning methods based on both DL PRS and UL SRS resources for positioning

  - Deep sleep

  - PRS reception and processing

  - UL transmission for positioning reporting

  - SRS transmission

    - Note: SRS transmission and UL transmission for positioning reporting may be merged into one state

Proposal 5: Study whether or not to introduce more candidate values for the reporting interval for the UE power saving.

- Reference

- RP-213588, Revised SID on Study on expanded and improved NR positioning

- TS 22.104, Service requirements for cyber-physical control application in vertical domains

- TS 22.261, Service requirements for the 5G system

- TR 38.857, Study on NR Positioning Enhancements

- TR 38.840, Study on User Equipment (UE) power saving in NR