3GPP TSG RAN WG1 #109-e		R1-2203971

e-Meeting, 9th May – 20th May, 2022

Source:	OPPO

Title:	Remaining essential issues on power saving RA

Agenda Item:	8.11.1

Document for:	Discussion and Decision

- Introduction

During the RAN1#108-e meeting, finalization for the remaining details of power efficient RA mechanisms in NR sidelink was discussed and further progressed. Agreements and conclusion achieved in that meeting are captured in [1]. In this contribution, we will continue discussing the following aspects of remaining issues for power saving in Release 17 eSL.

- Remaining details for the following topics:

  - Conversion to logical slots

  - Re-evaluation and pre-emption checking

  - Condition for performing contiguous partial sensing

  - Necessary updates to resource exclusion in step 6)

  - Other remaining issues

- Remaining issues on power saving RA

  - Conversion to logical slots

In the last few meetings, the formula tySL and ty-k×PreserveSL  were introduced to illustrate the candidate slots and the sensing occasions determined in periodic-based partial sensing. However, there are some misalignments between these expressions and current specification. It should be noted that  (t0SL,t1SL,⋯,tTmax-1SL) denotes the slots that may belong to a sidelink resource pool and t'0SL, t'1SL, t'2SL,…  represents the slots that belong to a resource pool in TS 38.214. Obviously, UE ought to select the Y candidate slots within its resource pool and the sensing occasions in PBPS should be located within the resource pool according to the same rule of full sensing in R16. Therefore, we suggest to revise  tySL and ty-k×PreserveSL to t'ySL and ty-k×Preserve''SL in TS 38.214, where Preserve' is Preserve converted to units of logical slot due to Preserve corresponds to the periodicity in milliseconds. Furthermore, the expressions of ty0SL and tyiSL which are used to describe the PBPS and CPS sensing occasions in initial selection and re-evaluation/pre-emption should also be revised to the logical slot within sidelink resource pool.

Proposal 1: Update the expression and formula in section 8.1.4 of TS 38.214 due to the conversion of logical slots.

-------------------------------- Start of Text Proposal for TS 38.214 ---------------------------------

<Unchanged parts omitted>

8.1.4   UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

The following steps are used:

<Unchanged parts omitted>

2)	The sensing window is defined by the range of slots [n –T0,n–Tproc,0SL), when the UE performs full sensing, where T0 is defined above and Tproc,0SL is defined in slots in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP. The UE shall monitor slots which belongs to a sidelink resource pool within the sensing window except for those in which its own transmissions occur. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

When the UE performs periodic-based partial sensing, the UE shall monitor slots atty-k×PreserveSL ty-k×Preserve''SL, where tySL t'ySLis a slot of the selected candidate slots and Preserve' is Preserve converted to units of logical slot according to clause 8.1.7. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

The value of Preserve corresponds to periodicSensingOccasionReservePeriodList if configured, otherwise, the values correspond to all periodicity from sl-ResourceReservePeriodList.

The UE monitors k sensing occasions determined by additionalPeriodicSensingOccasion, as previously described, and not earlier than n –T0. For a given periodicity Preserve, the values of k correspond to the most recent sensing occasion earlier than t'y0SL-(Tproc,0SL+Tproc,1SL )  ty0SL-(Tproc,0SL+Tproc,1SL ) if additionalPeriodicSensingOccasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if additionalPeriodicSensingOccasion is (pre-)configured. t'y0SLty0SL is the first slot of the selected Y candidate slots of PBPS.

When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled, the sensing window is defined by the range of slots [n+TA, n+TB]. n+TA is M consecutive logical slots earlier than slot t'y0SLty0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than t'y0SLty0SL, where t'y0SLty0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. If Prsvp_TX≠0  the value of M is (pre-)configured with the contiguousSensingWindowPeriodic. If contiguousSensingWindowPeriodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection.

When the UE performs contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) disabled and if Prsvp_TX=0, the sensing window is defined by the range of slots [n+TA, n+TB]. TA and TB are both selected such that the UE has sensing results starting at M consecutive logical slots before t'y0SL ty0SL and ending at Tproc,0SL+Tproc,1SL slots earlier than t'y0SLty0SL. The value of M is (pre-)configured with the contiguousSensingWindowAperiodic. If contiguousSensingWindowAperiodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection.

<Unchanged parts omitted>

6)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

<Unchanged parts omitted>

c)	the SCI format received in slot t'mSL or the same SCI format which, if and only if the 'Resource reservation period' field is present in the received SCI format 1-A, is assumed to be received in slot(s) t'm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX  if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', where if the UE is configured with full sensing by its higher layer, t'n'SL = n if slot n belongs to the set t'0SL,t'1SL,...,t'T'max-1SL, otherwise slot t'n'SL is the first slot after slot n belonging to the set t'0SL,t'1SL,...,t'T'max-1SL; If UE is configured with partial sensing by its higher layer,tn''SL=tyiSL-Tproc,1SL t'n'SL=t'yiSL-Tproc,1SLif slottyiSL-Tproc,1SL t'yiSL-Tproc,1SLbelongs to the set (t0'SL, t1'SL,⋯, tTmax'-1'SL), otherwise, slot tn''SLt'n'SLis the first slot after slot tyiSL-Tproc,1SL t'yiSL-Tproc,1SLbelonging to the set (t0'SL, t1'SL,⋯, tTmax'-1'SL). Otherwise Q=1. If the UE is configured with full sensing by its higher layer, Tscal is set to selection window size T2 converted to units of msec. If UE is configured with partial sensing by its higher layer, Tscal=tyLSL-(tyiSL-Tproc,1SL)Tscal=t'yLSL-(t'yiSL-Tproc,1SL)shall be converted to milliseconds, where slot tyLSL t'yLSLis the last slot of the Y or Y' candidate slots. The slot tyiSLt'yiSLis the first slot of the selected/remaining set of Y or Y' candidate slots.

--------------------------------------------End of Text Proposal -----------------------------------------

  - Re-evaluation and pre-emption checking

- Correction of slot indices for the remaining Y candidate slots in the square bracket in periodic transmission

In the RAN1#107-e meeting, the following agreement was achieved for re-evaluation and pre-emption checking. However, the formular inside the yellow highlighted square brackets still needs to be finalized.

Agreement When UE is triggered to perform re-evaluation and pre-emption checking for periodic transmission (Prsvp_TX≠0) in slot n, - During the qth reservation period (q=0,1,2,…, Cresel-1), candidate resource set (SA) is initialized to the remaining Y candidate slots starts from slot tyiSL and ends at the last slot of the Y candidate slots, where the slot indices of the remaining Y candidate slots are equal to [q x Prsvp_Tx + tySL], where tySL is a slot index of Y candidate slots used in the initial resource (re)selection. …

As captured in the above, the agreement states that the candidate resource set (SA) is initialized to the remaining Y candidate slots during the qth reservation period when UE is triggered to perform re-evaluation or pre-emption for periodic transmission. However, the slot indices of the remaining Y candidate slots in the square brackets with yellow highlight of above agreement still need to be further discussed. In our opinion, the formula in the square brackets should be revised to t'y+q×Prsvp_TX'SL because the selected Y candidate slots in every period are within the resource pool of UE and Prsvp_TX shall be converted to the number of logical slots when it is calculated within the resource pool. In addition, some expression and formula need to be revised in re-evaluation and pre-emption part as well due to the conversion of logical slots.

Proposal 2: The slot indices of the remaining Y candidate slots in the qth reservation period in re-evaluation and pre-emption checking for periodic transmission should be updated to t'y+q×Prsvp_TX'SL.

---------------------------------- Start of Text Proposal for TS 38.214 -------------------------------

<Unchanged parts omitted>

8.1.4   UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

When the UE performs periodic-based partial sensing and contiguous partial sensing, and when the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX≠0,

-	During the qth reservation period (q=0,1,2,…, Cresel-1), candidate resource set (SA) is initialized to the remaining Y candidate slots starting from slot tyiSL t'yiSLand ending at the last slot of the Y candidate slots, where the slot indices of the remaining Y candidate slots are equal totySL t'y+q×Prsvp_TX'SL, where tySL t'ySLis a slot index of Y candidate slots used in the initial resource (re)selection and Prsvp_TX' is Prsvp_TX converted to units of logical slots according to clause 8.1.7.

-	tyiSL t'yiSL is the first candidate slot after slot n+T3.

-	The UE performs PBPS for the remaining Y candidate slots according to ty'-k×Preserve''SLty'-k×PreserveSL, where t'y'SLty'SL is a slot belonging to the remaining Y candidate slots, and k and Preserve are the same as resource (re)selection, where the values of k correspond to the most recent sensing occasion earlier than t'yiSL-(Tproc,0SL+Tproc,1SL )  [tyiSL]-(Tproc,0SL+Tproc,1SL ) if additionalPeriodicSensingOccasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if additionalPeriodicSensingOccasion is (pre-)configured.

-	The UE performs CPS starting from M logical slots earlier than tyiSLt'yiSL to Tproc,0SL+Tproc,1SL slots earlier than t'yiSLtyiSL.

-	By default, M is 31 unless (pre-)configured with another value. by contiguousSensingWindowPeriodic.

When the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX=0,

-	Candidate resource set (SA) is initialized to the remaining Y' candidate slots starting from slottyiSL t'yiSL and ending at the last slot of the Y' candidate slots, where tyiSL t'yiSL is the first candidate slot after slot n+T3.

-	It is up to UE implementation that UE may perform PBPS for periodic sensing occasions after the resource (re)selection when higher layer parameter sl-MultiReserveResource is enabled

-	UE performs CPS starting from at least M consecutive logical slots earlier than tyiSLt'yiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSLt'yiSL

-	For minimum size M of the CPS monitoring window [n+TA, n+TB], by default, M is 31 unless (pre-)configured with another value, by  contiguousSensingWindowAperiodic.

When the minimum M slots for CPS cannot be guaranteed, UE senses in all available slots starting from the resource (re)selection trigger slot of the same TB to Tproc,0SL+Tproc,1SL slots earlier than t'yiSLtyiSL. The UE re-evaluation and pre-emption checking is based on all available sensing results after n-T0.

-------------------------------------------End of Text Proposal ------------------------------------------

- Definition of slot tyiSL

When UE is triggered to perform re-evaluation or pre-emption checking, tyiSL is defined as the first candidate slot after slot n+T3 in both periodic and aperiodic case which can be found in the yellow highlighted part of following agreements.

Agreement When UE is triggered to perform re-evaluation and pre-emption checking for periodic transmission (Prsvp_TX≠0) in slot n, - During the qth reservation period (q=0,1,2,…, Cresel-1), candidate resource set (SA) is initialized to the remaining Y candidate slots starts from slot tyiSL and ends at the last slot of the Y candidate slots, where the slot indices of the remaining Y candidate slots are equal to [q x Prsvp_Tx + tySL], where tySL is a slot index of Y candidate slots used in the initial resource (re)selection. - tyiSL is the first candidate slot after slot n+T3. - FFS whether/how to handle the case when number of the remaining Y candidate slots is less than Ymin. - Scheme 1: - UE performs PBPS for the remaining Y candidate slots according to ty'-k×PreserveSL, where ty'SL is a slot belong to the remaining Y candidate slots, and k and Preserve are the same as resource (re)selection. - UE performs CPS starts from M logical slots earlier than tyiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSL. - By default, M is 31 unless (pre-)configured with another value. Agreement When UE is triggered to perform re-evaluation and pre-emption checking for aperiodic transmission (Prsvp_TX=0) in slot n, - The candidate resource set (SA) is initialized to the remaining Y’ candidate slots that starts from slot tyiSL and ends at the last slot of the Y’ candidate slots. - tyiSL is the first candidate slot after slot n+T3. - UE may perform PBPS for periodic sensing occasions after the resource (re)selection when sl-MultiReserveResource is enabled for the mode 2 Tx resource pool - It is up to UE implementation - UE performs CPS starting from at least M consecutive logical slots earlier than tyiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSL. - FFS: When the minimum M slots for CPS cannot be guaranteed, - All available sensing results not earlier than n–T0 for the resource pool indicated by higher layer are applied for re-evaluation and pre-emption checking procedures

However, the current definition of tyiSL will cause that the pre-selected or reserved resource is not included within the candidate resource set (SA) before resource exclusion. For example, a pre-selected resource is located at slot m and the re-evaluation is triggered at slot m - T3. Hence, tyiSL is the first candidate slot after slot m such that the pre-selected resource is not included in the candidate resource set and resource (re-)selection is always triggered in this case. Therefore, tyiSL should be the first candidate slot not earlier than slot n+T3. In the meanwhile, the above agreements for aperiodic transmission (Prsvp_TX=0) is designed for UE performing at least contiguous partial sensing. It is also necessary to add the corresponding condition in current specification.

Proposal 3: tyiSL should be the first candidate slot not earlier than slot n+T3.

-------------------------------- Start of Text Proposal for TS 38.214 ---------------------------------

<Unchanged parts omitted>

8.1.4   UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

When the UE performs periodic-based partial sensing and contiguous partial sensing, and when the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX≠0,

-	During the qth reservation period (q=0,1,2,…, Cresel-1), candidate resource set (SA) is initialized to the remaining Y candidate slots starting from slot tyiSL and ending at the last slot of the Y candidate slots, where the slot indices of the remaining Y candidate slots are equal to [q x Prsvp_Tx + tySL], where tySL is a slot index of Y candidate slots used in the initial resource (re)selection.

-	tyiSL is the first candidate slot not earlier than slot n+T3.

-	The UE performs PBPS for the remaining Y candidate slots according to ty'-k×PreserveSL, where ty'SL is a slot belonging to the remaining Y candidate slots, and k and Preserve are the same as resource (re)selection, where the values of k correspond to the most recent sensing occasion earlier than [tyiSL]-(Tproc,0SL+Tproc,1SL ) if additionalPeriodicSensingOccasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if additionalPeriodicSensingOccasion is (pre-)configured.

-	The UE performs CPS starting from M logical slots earlier than tyiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSL.

-	By default, M is 31 unless (pre-)configured with another value. by contiguousSensingWindowPeriodic.

When the UE performs at least contiguous partial sensing, and when the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX=0,

-	Candidate resource set (SA) is initialized to the remaining Y' candidate slots starting from slot tyiSL and ending at the last slot of the Y' candidate slots, where tyiSL is the first candidate slot not earlier than slot n+T3.

-	It is up to UE implementation that UE may perform PBPS for periodic sensing occasions after the resource (re)selection when higher layer parameter sl-MultiReserveResource is enabled

-	UE performs CPS starting from at least M consecutive logical slots earlier than tyiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSL

-	For minimum size M of the CPS monitoring window [n+TA, n+TB], by default, M is 31 unless (pre-)configured with another value, by  contiguousSensingWindowAperiodic.

When the minimum M slots for CPS cannot be guaranteed, UE senses in all available slots starting from the resource (re)selection trigger slot of the same TB to Tproc,0SL+Tproc,1SL slots earlier than tyiSL. The UE re-evaluation and pre-emption checking is based on all available sensing results after n-T0.

--------------------------------------------End of Text Proposal -----------------------------------------

  - Condition for UE performing contiguous partial sensing

In the RAN1#106-e meeting, the condition for UE performing contiguous partial sensing was discussed and endorsed as below.

Agreement Conditions in which contiguous partial sensing is performed by UE, when at least all of the followings are met: - L1 [is expected to be or] is triggered by higher layer to report resources for resource (re-)selection in a mode 2 Tx pool - FFS: When the trigger will be received by L1 - The resource pool is (pre-)configured to enable partial sensing - Partial sensing is configured by higher layer in the UE

Based on the current specification, UE may perform CPS when the above three conditions are met. In other words, it is up to UE implementation choice to determine whether to perform CPS or not. In our view, UE shall perform CPS when the three conditions of above agreement are satisfied. Technically, the resource reservation for re-transmissions can not be disabled by configuration such that UE always needs to perform CPS to monitor the reservation indicated by TRIV and FRIV of SCI format 1-A. Secondly, the above agreement states that UE performs CPS at least the three conditions are met and RAN1 has not agreed any other conditions for UE performing CPS, which implies the above three conditions become the necessary and sufficient conditions automatically in our understanding. Furthermore, it is always assumed that UE performs CPS as a pre-condition for all existing agreements and current description in TS 38.214 (e.g., for periodic/aperiodic transmission in resource (re)-selection and re-evaluation/pre-emption checking highlighted with yellow in the following part of specification). If UE may perform CPS based on implementation, another set of agreements is needed to specify the behavior for UE not performing CPS.

1) A candidate single-slot resource for transmission Rx,y is defined… or in a set of Y' candidate slots within the time interval [n+T1,n+T2] for UE performing contiguous partial sensing if Prsvp_TX=0, correspond to one candidate single-slot resource, where … - Y' is selected by UE where Y'≥Ymin'. When the UE performs contiguous partial sensing and if Prsvp_TX=0, if the number of candidate single-slot resources Y' is smaller than Ymin', it is up to UE implementation to include other candidate slots. 2) The sensing window is defined by the range…. When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled, the sensing window is defined by the range of slots [n+TA, n+TB]. n+TA is M consecutive logical slots earlier than slot ty0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than ty0SL, where ty0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. If Prsvp_TX≠0 the value of M is (pre-)configured with the contiguousSensingWindowPeriodic. If contiguousSensingWindowPeriodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection. When the UE performs contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) disabled and if Prsvp_TX=0, the sensing window is defined by the range of slots [n+TA, n+TB]. TA and TB are both selected such that the UE has sensing results starting at M consecutive logical slots before ty0SL and ending at Tproc,0SL+Tproc,1SL slots earlier than ty0SL. The value of M is (pre-)configured with the contiguousSensingWindowAperiodic. If contiguousSensingWindowAperiodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection. When the UE performs periodic-based partial sensing and contiguous partial sensing, and when the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX≠0, - During the qth reservation period (q=0,1,2,…, Cresel-1)…

Proposal 4: Conditions in which contiguous partial sensing is performed by UE, when all of the followings are met:

- L1 is triggered by higher layer to report resources for resource (re-)selection in a mode 2 Tx pool

- The resource pool is (pre-)configured to enable partial sensing

- Partial sensing is configured by higher layer in the UE

-------------------------------- Start of Text Proposal for TS 38.214 ---------------------------------

<Unchanged parts omitted>

8.1.4   UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

When the resource pool is (pre-)configured with allowedResourceSelectionConfig including full sensing, and full sensing is (pre-)configured in the UE by higher layers, the UE performs full sensing.

When periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool, the resource pool is (pre-)configured with allowedResourceSelectionConfig including partial sensing, and partial sensing is (pre-) configured in the UE by higher layer, the UE performs periodic-based partial sensing.

When a UE is triggered by higher layer to report resources for resource (re-)selection in a mode 2 Tx pool, the resource pool is (pre-)configured with allowedResourceSelectionConfig including partial sensing, and partial sensing is configured by higher layer, the UE performs contiguous partial sensing.

--------------------------------------------End of Text Proposal -----------------------------------------

  - Update of resource exclusion in step 6

During the last meeting, the update for reference point t'n'SL and scaling factor Tscal in Step 6c) was agreed and the proposal for value Q when additionalPeriodicSensingOccasion is (pre-)configured was supported by many companies and nearly endorsed.

Agreement In Step 6 c) of TS38.214 Section 8.1.4, when UE is configured with partial sensing by its higher layer, adopt the following changes: - t'n'SL=tyiSL-Tproc,1SL if slot tyiSL-Tproc,1SL belongs to the set t'0SL,t'1SL,...,t'T'max-1SL, otherwise, slot t'n'SL is the first slot after slot tyiSL-Tproc,1SL belonging to the set t'0SL,t'1SL,...,t'T'max-1SL. - Option D: Tscal=tyLSL-(tyiSL-Tproc,1SL) converted to milliseconds, where slot tyLSL is the last slot of the Y or Y’ candidate slots. - Slot tyiSL is the first slot of the selected/remaining set of Y or Y’ candidate slots. Proposal 4-2 (VI): In Step 6 c) of TS38.214 Section 8.1.4, when UE is configured with partial sensing by its higher layer, - When additionalPeriodicSensingOccasion is (pre-)configured, - Option 2: - Case 1: if Prsvp_RX< Tscal and Prsvp_RX'<n'-m≤2·Prsvp_RX'; Q=TscalPrsvp_RX +1 - Case 2: if Prsvp_RX≥ Tscal and Prsvp_RX'<n'-m≤2·Prsvp_RX'; Q=2 - Case 3: if Prsvp_RX< Tscal and n'-m≤Prsvp_RX'; Q=TscalPrsvp_RX - Case 4: Otherwise Q=1

According to the current specification, UE performs resource exclusion based on the SCI received in the most recent sensing occasion because the inequation n'-m≤Prsvp_RX' has to be satisfied when Q=TscalPrsvp_RX and Q is always equal to 1 otherwise. However, it is allowed that UE monitors the most recent PSO and the last PSO before the most recent PSO in partial sensing when higher layer parameter additionalPeriodicSensingOccasion is configured. In other words, UE ought to utilize the SCI received in the additional PSO for resource exclusion in partial sensing. This new defined behavior in partial sensing is not aligned with full sensing such that the value Q should be calculated separately when UE is configured to monitor the additional periodic sensing occasion. One example is shown in Figure 1, UE selects one candidate slot ty0 within the resource selection window. The sensing occasions determined according to ty0 and resource reservation period P1 are slot ty0-P1 and ty0-2*P1 when UE is configured to monitor the additional PSO. UE received a SCI in slot ty0-2*P1 and the “resource reservation period” field of this SCI indicates P1. Q evaluates to be 1 in this example based on the current step 6c so UE assumes that the same SCI will be received in slot  ty0-P1. Obviously, the resources indicated by the SCI received in slot ty0-2*P1 and the SCI to be received in slot ty0-P1 (i.e., resource a-d in Figure 1) cannot overlap with Rx,y+j×Prsvp_TX'. That is, UE is not able to exclude the candidate resource according to the SCI received in the additional PSO and the higher layer parameter additionalPeriodicSensingOccasion is meaningless in this case.

The main motivation to define the higher layer parameter additionalPeriodicSensingOccasion is that some simulation results show the PRR performance gain when UE monitors the additional PBPS sensing occasion. The performance gain is mainly due to UE doesn’t monitor the most recent PSO or fails to decode the SCI received in the default PSO but utilizes the sensing result in the additional PSO for resource exclusion. During the discussion of last meeting, some companies think it is a rare event that UE accidently ignores a SCI in the most recent PSO and decodes a SCI indicating the same resource reservation period in the additional PSO. In our opinion, the performance gain has proved that it is not a minor probability event. Otherwise, there could be no difference in the simulation result. In addition, if UE can exclude the candidate resource(s) from the selected Y candidate slots based on the SCI received in the additional PSO of P1 and the SCI indicates other resource reservation period (e.g. P2), the additional PSO of P1 must be the most recent PSO of P2. In this regard, the configured additionalPeriodicSensingOccasion is also not used so the above proposal 4-2 (VI) should be supported.

Figure 1

Proposal 5: In Step 6 c) of TS38.214 Section 8.1.4, when UE is configured with partial sensing by its higher layer,

- When additionalPeriodicSensingOccasion is (pre-)configured,

  - Option 2:

    - Case 1: if Prsvp_RX< Tscal and  Prsvp_RX'<n'-m≤2·Prsvp_RX'; Q=TscalPrsvp_RX +1

    - Case 2: if Prsvp_RX≥ Tscal and  Prsvp_RX'<n'-m≤2·Prsvp_RX'; Q=2

    - Case 3: if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX'; Q=TscalPrsvp_RX

    - Case 4: Otherwise Q=1

-------------------------------- Start of Text Proposal for TS 38.214 ---------------------------------

<Unchanged parts omitted>

8.1.4   UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

6)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

<Unchanged parts omitted>

c)	the SCI format received in slot t'mSL or the same SCI format which, if and only if the 'Resource reservation period' field is present in the received SCI format 1-A, is assumed to be received in slot(s) t'm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, when additionalPeriodicSensingOccasion is not configured,  Q=TscalPrsvp_RX  if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', otherwise Q=1; When additionalPeriodicSensingOccasion is configured, Q=TscalPrsvp_RX +1 if Prsvp_RX< Tscal and  Prsvp_RX'<n'-m≤2·Prsvp_RX', Q=2 if Prsvp_RX≥ Tscal and  Prsvp_RX'<n'-m≤2·Prsvp_RX', Q=TscalPrsvp_RX if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', otherwise Q=1, where if the UE is configured with full sensing by its higher layer, t'n'SL = n if slot n belongs to the set t'0SL,t'1SL,...,t'T'max-1SL, otherwise slot t'n'SL is the first slot after slot n belonging to the set t'0SL,t'1SL,...,t'T'max-1SL; If UE is configured with partial sensing by its higher layer, tn''SL=tyiSL-Tproc,1SL if slot tyiSL-Tproc,1SL belongs to the set (t0'SL, t1'SL,⋯, tTmax'-1'SL), otherwise, slot tn''SL is the first slot after slot tyiSL-Tproc,1SL belonging to the set (t0'SL, t1'SL,⋯, tTmax'-1'SL). Q=1If the UE is configured with full sensing by its higher layer, Tscal is set to selection window size T2 converted to units of msec. If UE is configured with partial sensing by its higher layer, Tscal=tyLSL-(tyiSL-Tproc,1SL) shall be converted to milliseconds, where slot tyLSL is the last slot of the Y or Y' candidate slots. The slot tyiSL is the first slot of the selected/remaining set of Y or Y' candidate slots.

--------------------------------------------End of Text Proposal -----------------------------------------

  - Other remaining issues

The higher layer parameter contiguousSensingWindowAperiodic indicates the minimum CPS window size for aperiodic transmission but the current description in TS 38.214 is not aligned with the functionality of this parameter.

Proposal 6: Revise the description of contiguousSensingWindowAperiodic in TS 38.214 section 8.1.4.

-------------------------------- Start of Text Proposal for TS 38.214 ---------------------------------

<Unchanged parts omitted>

8.1.4   UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

Optionally, indication of the minimum size in logical slots of contiguous partial sensing window as contiguousSensingWindowAperiodic.

--------------------------------------------End of Text Proposal -----------------------------------------

During the discussion of PBPS and CPS, there are two cases in which the UE behavior of partial sensing is different. One is for periodic transmission (i.e., Prsvp_TX≠0 and UE performs PBPS and CPS), another is for aperiodic transmission (i.e., Prsvp_TX=0 and UE performs at least CPS). However, the above two cases are not clearly distinguished in the current description of TS 38.214.

Proposal 7: Differentiate the case of periodic transmission and aperiodic transmission in TS 38.214.

-------------------------------- Start of Text Proposal for TS 38.214 ---------------------------------

8.1.4   UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

The following steps are used:

1)	A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot t'ySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 for UE performing periodic-based partial sensing if Prsvp_TX≠0, correspond to one candidate single-slot resource, or in a set of Y' candidate slots within the time interval [n+T1,n+T2] for UE performing at least contiguous partial sensing if Prsvp_TX=0, correspond to one candidate single-slot resource, where

-	selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL  , where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP;

-	if  T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to  T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots).

-	Y is selected by UE where Y≥Ymin.

-	Y' is selected by UE where Y'≥Ymin'. When the UE performs at least contiguous partial sensing and if Prsvp_TX=0, if the number of candidate single-slot resources Y' is smaller than Ymin', it is up to UE implementation to include other candidate slots.

The total number of candidate single-slot resources is denoted by Mtotal.

2)	The sensing window is defined by the range of slots [n –T0,n–Tproc,0SL), when the UE performs full sensing, where T0 is defined above and Tproc,0SL is defined in slots in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP. The UE shall monitor slots which belongs to a sidelink resource pool within the sensing window except for those in which its own transmissions occur. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

When the UE performs periodic-based partial sensing, the UE shall monitor slots at ty-k×PreserveSL, where tySL is a slot of the selected candidate slots. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

The value of Preserve corresponds to periodicSensingOccasionReservePeriodList if configured, otherwise, the values correspond to all periodicity from sl-ResourceReservePeriodList.

The UE monitors k sensing occasions determined by additionalPeriodicSensingOccasion, as previously described, and not earlier than n –T0. For a given periodicity Preserve, the values of k correspond to the most recent sensing occasion earlier than ty0SL-(Tproc,0SL+Tproc,1SL ) if additionalPeriodicSensingOccasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if additionalPeriodicSensingOccasion is (pre-)configured. ty0SL is the first slot of the selected Y candidate slots of PBPS.

When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled and if Prsvp_TX≠0 , the sensing window is defined by the range of slots [n+TA, n+TB]. n+TA is M consecutive logical slots earlier than slot ty0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than ty0SL, where ty0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. Prsvp_TX≠0  The value of M is (pre-)configured with the contiguousSensingWindowPeriodic. If contiguousSensingWindowPeriodic is not (pre-)configured, M equals to 31. Prsvp_TX=0

When the UE performs contiguous partial sensing and if Prsvp_TX=0, the sensing window is defined by the range of slots [n+TA, n+TB]. TA and TB are both selected such that the UE has sensing results starting at M consecutive logical slots before ty0SL and ending at Tproc,0SL+Tproc,1SL slots earlier than ty0SL, where ty0SL is the first slot of the selected Y'candidate slots. The value of M is (pre-)configured with the contiguousSensingWindowAperiodic. If contiguousSensingWindowAperiodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection.

--------------------------------------------End of Text Proposal -----------------------------------------

- Conclusion

In this contribution, we discussed the remaining issues on partial sensing mechanisms to complete the maintenance work for the power saving RA objective in the NR sidelink enhancement work item in Rel-17. Based on the discussions and analyses provided in this contribution, we give the following proposals.

Proposal 1: Update the expression and formula in section 8.1.4 of TS 38.214 due to the conversion of logical slots.

Proposal 2: The slot indices of the remaining Y candidate slots in the qth reservation period in re-evaluation and pre-emption checking for periodic transmission should be updated to t'y+q×Prsvp_TX'SL.

Proposal 3: tyiSL should be the first candidate slot not earlier than slot n+T3.

Proposal 4: Conditions in which contiguous partial sensing is performed by UE, when all of the followings are met:

- L1 is triggered by higher layer to report resources for resource (re-)selection in a mode 2 Tx pool

- The resource pool is (pre-)configured to enable partial sensing

- Partial sensing is configured by higher layer in the UE

Proposal 5: In Step 6 c) of TS38.214 Section 8.1.4, when UE is configured with partial sensing by its higher layer,

- When additionalPeriodicSensingOccasion is (pre-)configured,

  - Option 2:

    - Case 1: if Prsvp_RX< Tscal and  Prsvp_RX'<n'-m≤2·Prsvp_RX'; Q=TscalPrsvp_RX +1

    - Case 2: if Prsvp_RX≥ Tscal and  Prsvp_RX'<n'-m≤2·Prsvp_RX'; Q=2

    - Case 3: if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX'; Q=TscalPrsvp_RX

    - Case 4: Otherwise Q=1

Proposal 6: Revise the description of contiguousSensingWindowAperiodic in TS 38.214 section 8.1.4.

Proposal 7: Differentiate the case of periodic transmission and aperiodic transmission in TS 38.214.

Reference

[1] Chairman’s notes, RAN1 #108-e

[2] R1-2202565 “FL summary for AI 8.11.1.1 – NR sidelink resource allocation for power saving (EOM)”, Moderator (OPPO), RAN1#108-e

[3]	3GPP TS 38.214 “NR Physical layer procedures for data” V17.1.0, 2022-03