3GPP TSG RAN WG1 #109-e		R1-2203974

e-Meeting, May 9th – 20th, 2022

Source:	OPPO

Title:	Discussion on remaining issues of mechanism to improve reliability for RRC_CONNECTED UEs

Agenda Item:	8.12.1

Document for:	Discussion and Decision

Introduction

In RAN1#108-e meeting, the following agreements were achieved in [1] on the mechanism to improve reliability for RRC_CONNECTED UEs.

Agreement For supporting more than one NACK-only feedback in the same PUCCH transmission, define RRC configuration to configure between Alt1 and Alt4 (from previous agreements): - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits. - FFS: how to determine PUCCH resource - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission. - define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback, - FFS: The PUCCH slot for the transmission is based on the K1 in the “last DCI” scheduling multicast. - FFS: The PUCCH resource for the transmission is from PUCCH-config configured for NACK-only based feedback according to the mapping between number of TBs with PUCCH resource ID. - FFS mapping details. - How to determine the number of TBs is discussed separately, e.g., Type-1-like and/or Type-2-like codebook. - FFS: whether this applies to a single G-RNTI or multiple G-RNTIs - Alt4 is not supported for more than 4 TBs - FFS: whether RRC configuration between Alt1 and Alt4 is per G-RNTI or per CFR - FFS: UE capability Agreement When HARQ-ACK for unicast SPS PDSCHs and multicast dynamic grant PDSCHs with ACK/NACK based feedback are multiplexed on the same PUCCH for the same priority case, the following option 1 (from the previous agreement) is adopted: - Option 1: the PUCCH carrying the multiplexed HARQ-ACK is determined from the SPS-PUCCH-AN-List configured for unicast. - Option 2: the PUCCH carrying the multiplexed HARQ-ACK is determined from PUCCH-Config/PUCCH-ConfigurationList configured for multicast. Agreement When NACK-only based HARQ-ACK feedback is used for multicast SPS PDSCH without PDCCH scheduling, the UE determines a priority index from the HARQ-ACK codebook index in the configuration for SPS multicast, using the same method with the one for ACK/NACK based feedback. Agreement When UE is configured with unicast SPS and multicast SPS with NACK-only based feedback for multiplexing on the same PUCCH for the same priority case, NACK only based HARQ-ACK is transformed to ACK/NACK based HARQ-ACK. - For NACK only based HARQ-ACK transformed to ACK/NACK based HARQ-ACK, the HARQ-ACK codebook is constructed as for multiple SPS PDSCHs regardless of unicast SPS PDSCH or multicast SPS PDSCH and the PUCCH carrying the multiplexed HARQ-ACK is determined from the SPS-PUCCH-AN-List configured for unicast, as agreed for ACK/NACK based feedback. Agreement Regarding RRC configuring Alt1 or Alt4 (from the previous agreement) for multiplexing more than one NACK-only in the same PUCCH transmission, the configuration is per CFR. Agreement If Type-1 codebook is configured for both multicast and unicast, at least for single cell case for both unicast and multicast: - If the UE is configured to construct the HARQ-ACK codebooks for unicast and multicast jointly, a single UL DAI bit applies for unicast and multicast - Otherwise, 1 additional bit UL DAI is included for multicast in DCI format 0_1/0_2, in addition to the UL DAI for unicast. The 1-bit UL DAI for multicast is applied to all configured G-RNTIs. - FFS: additional restrictions Agreement When PUCCH transmission for the NACK-only based feedback for one G-RNTI collides with PUCCH transmission for ACK/NACK feedback for another G-RNTI with the same priority, support UE multiplexing the NACK-only based feedback with the ACK/NACK feedback onto the same PUCCH by transforming NACK-only into the ACK/NACK based HARQ-ACK bit. - Note: When the TB configured with NACK-only feedback is correctly decoded, the ACK will be transmitted and multiplexed with others. Agreement For multiplexing NACK-only feedback for the first G-RNTI with ACK/NACK based feedback for the second G-RNTI or for unicast, down-select from: - Alt1: the converted NACK-only bits are concatenated to the ACK/NACK feedback. - Alt2: the codebook construction/concatenation is the same as when the feedback mode is ‘ACK/NACK’ for both G-RNTIs. Agreement When UE is configured with enhanced Type-2 codebook for unicast and when the UE is scheduled to multiplex enhanced Type-2 HARQ-ACK for unicast with Type-1 or Type-2 HARQ-ACK codebook for multicast in the same PUCCH slot, - UE generates separate sub-codebooks for unicast and multicast respectively and appends the multicast HARQ-ACK sub-codebook to the unicast HARQ-ACK sub-codebook. Agreement For the following two cases, the fallback operation for the Type-1 HARQ-ACK codebook for multicast is applied to G-RNTI/G-CS-RNTI configured with HARQ-ACK enabled only, - a SPS PDSCH release indicated by DCI format 4_1 with counter DAI field value of 1, - a PDSCH reception scheduled by DCI format 4_1 with counter DAI field value of 1 on the PCell, - SPS PDSCH reception(s) associated with G-CS-RNTIs. Note: If the UE receives two SPS PDSCH releases for two respective G-CS-RNTIs with DAI=1, or two PDSCHs by DCI 4_1 with DAI=1 for two respective G-RNTIs on the PCell, there is no fallback. Agreement For Type-1 codebook generation, if all configured G-RNTIs are with HARQ-ACK disabled by RRC signalling, UE does not report any HARQ-ACK information in the PUCCH slot; - For other cases, FFS between the 3 alternatives below: - Alt1: if at least one configured G-RNTI is with HARQ-ACK enabled, UE will report NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. - Alt2: if at least one configured G-RNTI is with HARQ-ACK enabled, UE reports actual HARQ-ACK result for all G-RNTIs. - Alt3: UE is not expected to be configured with some G-RNTI with HARQ-ACK disabled by RRC signalling and some other G-RNTI with HARQ-ACK enabled by RRC signalling for all configured G-RNTI - Other alternatives are not precluded Agreement If Type-2 codebook is configured for unicast and multicast, the following option2-2 (from the previous agreement) is adopted: - Option2-2: 2-bit UL DAI(s) are included in DCI for multicast, in addition to the 2-bit UL DAI for unicast. - The 2-bit UL DAI for multicast is applied for all configured G-RNTIs. - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs. - Alt1-1: UL-DAI indicates the sum of DL-DAIs - Alt1-2: DL-DAIs have the same value. - Alt1-3: largest DL DAI value among the configured G-RNTIs. - Other alternatives are not precluded Agreement When Type-1 codebook is configured for unicast and Type-2 codebook is configured for multicast, or when Type-2 codebook is configured for unicast and Type-1 codebook is configured for multicast, the UL-DAI for multicast is included in DCI format 0_1/0_2, in addition to the UL-DAI field for unicast. - The UL-DAI for multicast is 1-bit for Type-1 codebook - The 1-bit UL-DAI for multicast is applied to all configured G-RNTIs. - The UL-DAI for multicast is 2-bit for Type-2 codebook applied for all configured G-RNTIs. - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs. - Alt1-1: UL-DAI indicates the sum of DL-DAIs - Alt1-2: DL-DAIs have the same value. - Alt1-3: largest DL DAI value among the configured G-RNTIs. - Other alternatives are not precluded - FFS: details of the UL DAI field Agreement For NACK-only feedback for PUCCH format 0 or format 1, only 1 cyclic shift is used. - The sequence cyclic shift for NACK-only is mcs=0. - For PF1 NACK-only, set b0=0. - Note: the initialCyclicShift is configured for PUCCH-format0 and PUCCH-format1 as legacy. Agreement For multiplexing NACK-only with HARQ-ACK feedback/CSI for unicast for the same priority or PUSCH transmission for the same priority or ACK/NACK based HARQ-ACK feedback for multicast with another G-RNTI for the same priority, the multiplexing does not depend on the decoding result. - Note: when multiplexing the NACK-only is transformed into ACK/NACK as agreed. Agreement For Type-2 codebook generation, UE reports HARQ-ACK bits only for TBs with enabled HARQ-ACK by RRC or DCI. Agreement For multiplexing NACK-only feedback for the first G-RNTI with ACK/NACK based feedback for the second G-RNTI or for unicast, the following Alt2 (from the previous agreement) is adopted: - Alt1: the converted NACK-only bits are concatenated to the ACK/NACK feedback. - Alt2: the codebook construction/concatenation is the same as when the feedback mode is ‘ACK/NACK’ for both G-RNTIs. Agreement If a UE supports ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, and if the UE is configured with ACK/NACK based feedback for multicast dynamic scheduling and is configured with disabled HARQ feedback for multicast SPS scheduling, for Type-1 codebook generation, - FFS between the 4 alternatives below: - Alt1: UE will report NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. - Alt2: UE will report ACK/NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. - Alt3: UE is not expected to be configured with G-RNTI with HARQ-ACK enabled by RRC signalling and G-CS-RNTI with HARQ-ACK enabled by RRC signalling. - Alt4: UE does not report any HARQ-ACK information for G-CS-RNTI with HARQ-ACK feedback disabled by RRC. - Other alternatives are not precluded.

In this contribution, we continue discussing the remaining issues on how to improve reliability for NR MBS RRC_CONNECTED UEs.

Discussion

  - More than one NACK-only based feedback bits

Agreement For supporting more than one NACK-only feedback in the same PUCCH transmission, define RRC configuration to configure between Alt1 and Alt4 (from previous agreements): - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits. - FFS: how to determine PUCCH resource - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission. - define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback, - FFS: The PUCCH slot for the transmission is based on the K1 in the “last DCI” scheduling multicast. - FFS: The PUCCH resource for the transmission is from PUCCH-config configured for NACK-only based feedback according to the mapping between number of TBs with PUCCH resource ID. - FFS mapping details. - How to determine the number of TBs is discussed separately, e.g., Type-1-like and/or Type-2-like codebook. - FFS: whether this applies to a single G-RNTI or multiple G-RNTIs - Alt4 is not supported for more than 4 TBs - FFS: whether RRC configuration between Alt1 and Alt4 is per G-RNTI or per CFR - FFS: UE capability Agreement Regarding RRC configuring Alt1 or Alt4 (from the previous agreement) for multiplexing more than one NACK-only in the same PUCCH transmission, the configuration is per CFR.

For more than one NACK-only feedback in the same PUCCH transmission, either Alt 1 or Alt 4 can be configured for UL feedback report. For Alt 1, one legacy issue is how to determine PUCCH resource. One possible way is to share the PUCCH resources of unicast PDSCH’s UL feedback. gNB can process proper scheduling to guarantee that multiple NACK-only feedback bits can always transmitted with unicast UL by scheduling them in one overlapped slot. Furthermore, it was proposed that dedicated UL resource can be configured for NACK-only feedbacks, but it may lead to resource waste when there is no negative feedback. Therefore, PUCCH resource can be determined by gNB’s implementation with proper scheduling.

- For more than one NACK-only feedback in the same PUCCH transmission with Alt 1, the PUCCH resource can be up to gNB implementation with proper scheduling.

For Alt 4, while there are multiple DCIs scheduling group-common PDSCHs, the actual PUCCH slot is determined by the last DCI and the K1 indications in previous DCIs can be considered as invalid. On how to determine the number of TBs, Type-1-like codebook design can be considered. Because semi-static codebook can help to differentiate the feedbacks among different TBs, while dynamic codebook cannot. About G-RNTI, a single G-RNTI is supported by considering about the complexity.

- For more than one NACK-only feedback in the same PUCCH transmission with Alt 4, Type-1-like codebook design can be considered to determine the number of TBs.

- Alt 4 is applied to a single G-RNTI.

For configuration, Alt 1 and Alt 4 can be configured per CFR rather than per G-RNTI. If it is configured according to each G-RNTI, multiple MBS sessions with different G-RNTIs may occurs with TDMed GC-PDSCHs to the group of UEs but with different UL feedback alternatives. From a UE’s perspective, UE has to process different multiplexing scheme based on different G-RNTIs. From network’s perspective, gNB may have to guarantee and optimize the transmissions between unicast PDSCH and group-common PDSCH, which dramatically increase the system’s complexity and burden.

- Alt 1 and Alt 4 should be configured by RRC configuration per CFR.

  - Codebook generation

Agreement If a UE supports ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, and if the UE is configured with ACK/NACK based feedback for multicast dynamic scheduling and is configured with disabled HARQ feedback for multicast SPS scheduling, for Type-1 codebook generation, - FFS between the 4 alternatives below: - Alt1: UE will report NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. - Alt2: UE will report ACK/NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. - Alt3: UE is not expected to be configured with G-RNTI with HARQ-ACK enabled by RRC signalling and G-CS-RNTI with HARQ-ACK enabled by RRC signalling. - Alt4: UE does not report any HARQ-ACK information for G-CS-RNTI with HARQ-ACK feedback disabled by RRC. - Other alternatives are not precluded.

One legacy issue on the Type-1 codebook generation is how to report ACK/NACK when HARQ is enabled for dynamic scheduling while it is disabled for SPS, and 4 alternatives were left for down-selection. Semi-static codebook should be fixed size in order to solve the issue of DCI missing. If UE anyway needs to report ACK or NACK to gNB, reporting NACK seems having potential benefit to improving network decoding UCI accuracy. For NR MBS, the codebook design should also consider about complexity and impact on unicast services. Therefore, Alt 4 is slightly preferred that UE report nothing for this case.

- For Type-1 codebook generation, Alt 4 is supported that UE does not report any HARQ-ACK information for G-CS-RNTI with HARQ-ACK feedback disabled by RRC.

Conclusion

In this contribution, the remaining issues of UL feedback on reliability improvement mechanism are discussed for RRC_CONNECTED UEs, and the proposals are as follows:

- For more than one NACK-only feedback in the same PUCCH transmission with Alt 1, the PUCCH resource can be up to gNB implementation with proper scheduling.

- For more than one NACK-only feedback in the same PUCCH transmission with Alt 4, Type-1-like codebook design can be considered to determine the number of TBs.

- Alt 4 is applied to a single G-RNTI.

- Alt 1 and Alt 4 should be configured by RRC configuration per CFR.

- For Type-1 codebook generation, Alt 4 is supported that UE does not report any HARQ-ACK information for G-CS-RNTI with HARQ-ACK feedback disabled by RRC.

Reference

- RAN1 Chairman’s Notes, RAN1#108-e, e-Meeting, February 21st – March 3rd, 2022.