3GPP TSG RAN WG1 #109-e		R1-2203975

e-Meeting, May 9th – 20th, 2022

Source:	OPPO

Title:	Discussion on UE features for NR MBS

Agenda Item:	8.16.12

Document for:	Discussion and Decision

Introduction

In RAN1#108-e meeting, Rel-17 UE features for NR MBS were discussed and the agreements [1] on the corresponding FGs were reached. In this contribution, some following analysis and views on the remaining issues are discussed.

Discussion

  - Discussion on FG 33-1

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. - support of higher layer configured slot-level repetition up to 8 for MTCH DCI indicated slot-level repetition up to 16 for MTCH is defined as another FG, FFS whether to merge with FG 33-3-1 |  | Up to RAN2 |  |  | Up to RAN2 | Up to RAN2 | Up to RAN2 |  |  | Up to RAN2 |

Agreement - The capability for support of dynamic slot-level repetition for MTCH is separated from FG 33-1. - maximum number of dynamic slot-level repetitions is 16

In RAN1 #108-e meeting, it was agreed that dynamic slot-level repetition for MTCH is separated from FG 33-1, and the maximum number of dynamic slot-level repetitions is 16. In FG 33-3-1, it is about dynamic slot-level repletion for group-common PDSCH in multicast which is focused on UEs in RRC_CONNECTED states, while this separated component above is for PDSCH (conveying MTCH) for broadcast. Both components are dynamic slot-level repetition, but they are related to different cast-type. To differentiate them associated with multicast and broadcast, respectively, DCI indication on slot-level repetition up to 16 for MTCH is not merged with FG 33-3-1.

- “DCI indicated slot-level repetition up to 16 for MTCH” is defined as an independent FG which is not merged with FG 33-3-1.

  - Discussion on FG 33-2

| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast for PCell | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI for PCell. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support {2, 4, 8} times semi-static slot-level repetition for group-common PDSCH for multicast | FFS | Yes |  |  | [Per band or per FSPC] | [N/A] | [N/A] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-2a | Support of ACK/NACK based HARQ-ACK feedback andRRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | 1) Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling 2) Support of PTM retransmission for multicast 3) support of Type-1 and Type-2 HARQ-ACK CB for multicast feedback only 4) Support of shared PUCCH resource configurations with unicast FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-2a | 33-2 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2b | DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling | 33-2a | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2d | PTP retransmission for multicast | Support of PTP retransmission for multicast | 33-2a | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2e | Multiple G-RNTIs for group-common PDSCHs | Capability on number of G-RNTI for groupcast FFS details. | [33-2] | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2f | Dynamic multicast with DCI format 4_2 | Support of DCI format 4_2 with CRC scrambled with G-RNTI for multicast | 33-2 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2g | MIMO layers for multicast PDSCH | Supported maximal number of MIMO layers for multicast PDSCH | 33-2 | Yes |  | UE supports 1 MIMO layer only for multicast PDSCH | Per FSPC | N/A | N/A |  | Candidate values: {2,4,8} Note: If UE supports up to 8 layers, the UE supports TB2 | Optional with capability signalling |
| 33. NR_MBS | 33-2h | Dynamic scheduling for multicast for SCell | Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI for SCell. | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |

Agreement - Add a component for supporting Type-1 and Type-2 HARQ-ACK CB for multicast feedback only in FG 33-2a Agreement - FG 33-2g is updated as follows 33. NR_MBS; 33-2g; MIMO layers for multicast PDSCH; Supported maximal number of MIMO layers for multicast PDSCH; 33-2; Yes; UE supports 1 MIMO layer only for multicast PDSCH; Per FSPC; N/A; N/A; Candidate values: {2,4,8} Note: If UE supports up to 8 layers, the UE supports TB2; Optional with capability signalling. Agreement - Shared PUCCH resource configurations with unicast is included as a component of FGs 33-2a/33-4 - FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FGs 33-2a/33-4 Agreement - FG 33-2 is separated to two FGs: One for PCell case, the other for SCell case - Type of FG for PCell case is [per band or per FSPC] - Type of FG for SCell case is per FSPC - FG for PCell is added as a Prerequisite FG for FG for SCell

In FG 33-2a, one legacy issue for ACK/NACK-based feedback is whether to have a separate FG on PUCCH resource configurations from unicast. It is agreed that when PUCCH resources for multicast is not configured, the PUCCH resources configured for unicast can be used/shared by MBS multicast ACK/NACK feedback transmission. If a separate FG on PUCCH resource configuration is defined, some UEs may not support such FG while other UEs in the same multicast group can support it, the only result is that all UEs in the group have to share PUCCH resources with unicast, which may lead to increase UL feedback burden and complexity, and even impact on unicast UL feedback performance. It is suggested that separate PUCCH resource configurations from unicast should be merged as one of the components in FG 33-2a.

- For FG 33-2a, define “separate PUCCH resource configurations from unicast” should be merged as one of the components in FG 33-2a.

For FG 33-2e, firstly it should be clarified about the use of groupcast rather than multicast. Multicast is used for MBS reception by UEs in RRC_CONNECTED states, and broadcast is used for MBS reception by UEs in RRC_IDLE/RRC_INACTIVE and RRC_CONNECTED states. FG 33-2 is FG related to multicast, while FG 33-1 is related to broadcast, so FG 33-2e should use multicast instead of groupcast. On the other hand, FG 33-2e is to define the capability on the number of G-RNTIs, meanwhile both broadcast and multicast transmission use G-RNTI(s). If FG 33-2e defines the number of G-RNTI(s) including both broadcast and multicast, the component description should be explicitly mentioned, i.e. “Capability on number of G-RNTI for broadcast and multicast”. Therefore, groupcast should be replaced by the exact intention terminology, e.g. “multicast” or “multicast and broadcast”. Secondly, about the details of this FG, the baseline is that UE implementation complexity and requirement should not be increased by additionally supporting G-RNTI(s) used for MBS. It was agreed that each G-RNTI is mapped with one MBS session, and Rel-17 NR MBS supports multiple sessions transmission TDM/FDM. In latest version of TS 38.331 (v17.0.0), the maximum number of G-RNTI is defined as 16. When a UE is interested in multiple MBS sessions, it can receive MBS sessions as much as it can as long as its capability allows. Furthermore, the reception of MBS cannot impact on the normal unicast reception or degrade the unicast reception performance. Only if gNB can properly schedule unicast and MBS, the system can work efficiently and UE can support up to the maximum number of G-RNTIs, i.e. 16.

- For FG 33-2e, the terminology “groupcast” should be replaced by either “multicast” or “multicast and broadcast”.

- For FG 33-2e, the maximum number of G-RNTI can be supported by a UE is 16.

  - Discussion on FG 33-3

| 33. NR_MBS | 33-3-1 | Dynamic Slot-level repetition for group-common PDSCH | - Support up to X times dynamic slot-level repetition for group-common PDSCH for multicast. | 33-2 | Yes |  |  | [Per UE] | [No] | [No] |  | Candidate values for X is: {8, 16} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. | [33-1, 33-2] | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). | [33-1, 33-2] | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-3a | FDM-ed Type-1 and Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast | Support of FDM-ed Type-1 and Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast FFS value of X G-RNTIs | [TBD] | Yes |  |  | [Per UE] | [No] | [No] |  | Note1: FDM-ed Type-1 HARQ-ACK codebook is generated by concatenating the Type-1 sub-codebook for unicast and the Type-1 sub-codebook for multicast. Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. | Optional with capability signalling |
| 33. NR_MBS | 33-3-3b | Mode 2 TDM-ed Type-1 and Type-2 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast | Support of Mode 2 TDM-ed Type-1 and Type-2 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast FFS value of X G-RNTIs | [TBD] | Yes |  |  | [Per UE] | [No] | [No] |  | Note1: Mode 2 TDM-ed Type-1 HARQ-ACK codebook is generated based on the union TDRA tables from unicast and multicast and the union of k1 sets from unicast and multicast. Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. | Optional with capability signalling |
| 33. NR_MBS | 33-3-4 | Mode 1 for type1 codebook generation | Supports type1-Codebook-Generation-Mode configured as mode 1 | [TBD], 33-3-3b | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-5 | Feedback multiplexing for unicast PDSCH and group-common PDSCH for multicast with same priority and different codebook type | Support of multiplexing HARQ-ACK for unicast and multicast with the same priority and different HARQ-ACK codebook types in the same PUCCH slot | [33-2b] | Yes |  |  | [Per FSPC] | [No] | [No] |  |  | Optional with capability signalling |

Agreement - Add an FG 33-3-3a for FDM-ed Type-1 and Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast - Note1: FDM-ed Type-1 HARQ-ACK codebook is generated by concatenating the Type-1 sub-codebook for unicast and the Type-1 sub-codebook for multicast. - Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. - FFS value of X G-RNTIs - Add an FG 33-3-3b for Mode 2 TDM-ed Type-1 and Type-2 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast - Note: Mode 2 TDM-ed Type-1 HARQ-ACK codebook is generated based on the union TDRA tables from unicast and multicast and the union of k1 sets from unicast and multicast. - Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. - FFS value of X G-RNTIs - FG 33-3-3b is added as a prerequisite FG for FG 33-3-4

For FG 33-3-1, the upper bound of dynamic slot-level repetition for group-common PDSCH for multicast has two candidates for further down selection, i.e. 8 or 16. In this contribution about FG 33-1, it is proposed not to merge the “DCI indicated slot-level repetition up to 16 for MTCH” with FG 33-3-1, but the number of repetitions can be aligned since they are both related to DCI-based indication on the repetition. Therefore, FG 33-3-1 should also support up to 16 times dynamic repetition for multicast.

- For FG 33-3-1, support up to 16 times dynamic slot-level repetition for group-common PDSCH for multicast.

For FG 33-3-3a and 33-3-3b, there is a common legacy issue about the value of X G-RNTIs. One baseline is that here the value of X should not exceed the G-RNTI number defined in FG 33-2e which is the total number of G-RNTIs supported by a UE. Following this baseline, the value of X should be any integer number, e.g. 1, 2, 3 and etc.

- For FG 33-3-3a and 33-3-3b, the value of X G-RNTIs should be no larger than the number of G-RNTI defined in FG 33-2e.

  - Discussion on FG 33-4

| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast | - [Support NACK-only based HARQ-ACK feedback.] - Support of shared PUCCH resource configurations with unicast FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-4 | [33-2] | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-4-1 | DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling | [33-4] | Yes |  |  | [Per UE] | FFS | FFS |  |  | Optional with capability signalling |

Agreement - Shared PUCCH resource configurations with unicast is included as a component of FGs 33-2a/33-4 - FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FGs 33-2a/33-4

For FG 33-4, similar with ACK/NACK based feedback in FG 33-2a, enabling/disabling of NACK-only based feedback by RRC signaling can be within the same FG. Supporting NACK-only based HARQ-ACK feedback requires enabling/disabling mechanism, while DCI-based mechanism is not the only one. Furthermore, using RRC-based signaling to enable/disable NACK-only based HARQ-ACK feedback is considered as a default method. Therefore, the bracket should be removed from the first component, and RRC signaling configuration on enabling/disabling should also be added.

- For FG 33-4, the bracket should be removed.

- For FG 33-4, “and support enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling” should be added to the component.

For FG 33-4, one legacy issue similar with FG 33-2a is that whether to define separate PUCCH resource configurations from unicast as separated FG. NACK-only based feedback has different characteristic from that of ACK/NACK based feedback. Always sharing PUCCH resource between multicast with NACK-only and unicast with ACK/NACK based feedback mechanisms may have impact or degradation on unicast reliability performance. A third component “Support of separated PUCCH resource configurations with unicast” should be added to FG 33-4.

- For FG 33-4, adding another component that “Support of separated PUCCH resource configurations with unicast”.

  - Discussion on FG 33-5

| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | - Support one SPS group-common PDSCH configuration for multicast - Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH FFS whether to separate the capability for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling | 33-2 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-5-1a | Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, SPS group-common PDSCH activation, and SPS release PDCCH | 33-5-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1b | DCI-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC signaling | 33-5-1a | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1d | PTP retransmission for SPS group-common PDSCH for multicast | Support of PTP retransmission for SPS multicast FFS whether to merge with 33-5-1a | 33-5-1a | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1e | Dynamic Slot-level repetition for SPS group-common PDSCH for multicast | Support up to X times dynamic slot-level repetition for SPS group-common PDSCH for multicast. | 33-5-1 | Yes |  |  | [Per UE] | [No] | [No] |  | Candidate values for X is: {8, 16} | Optional with capability signalling |
| 33. NR_MBS | 33-5-1f | NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast | 1) Support NACK-only based HARQ-ACK feedback, and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling 2) Support of PTM retransmission associated with G-CS-RNTI for SPS multicast | 33-5-1 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1g | DCI-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast | Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC signaling | 33-5-1f | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1h | Multiple G-CS-RNTIs for SPS group-common PDSCHs | Max number of G-CS-RNTIs for SPS multicast [per slot per CC] | 33-5-1 | Yes |  |  | [Per UE] | [No] | [No] |  | FFS max number | Optional with capability signalling |
| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration | - Support [N>1] SPS group-common PDSCH configuration per CFR for multicast, and up to K SPS group-common PDSCH configuration in a cell group - Support M>=1 activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast | 33-2 | Yes |  |  | [Per UE] | [No] | [No] |  | FFS the value range for K FFS: Candidate values for M | Optional with capability signalling |

Agreement - Component 1 in FG 33-5-2 is updated as follows - Support [N>1] SPS group-common PDSCH configuration per CFR for multicast, and up to K SPS group-common PDSCH configuration in a cell group - FFS the value range for K

For FG 33-5-1, one legacy issue is whether to separate the capability for support of DCI format 4_2 for multicast SPS scheduling. At least DCI format 4_1 can be used for multicast SPS scheduling, therefore, DCI format 4_2 can be a separated FG from FG 33-5-1.

- For FG 33-5-1, separate the capability for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling.

For FG 33-5-1d, support of PTP retransmission for SPS multicast can be kept in a separate FG, while PTM can be used as the default retransmission scheme in multicast SPS.

- For FG 33-5-1d, “Support of PTP retransmission for SPS multicast” can be a separate FG and not merged with FG 33-5-1a.

Conclusion

In this contribution, the analysis and corresponding views are provided on the UE features for Rel-17 NR MBS with the following proposals:

- “DCI indicated slot-level repetition up to 16 for MTCH” is defined as an independent FG which is not merged with FG 33-3-1.

- For FG 33-2a, define “separate PUCCH resource configurations from unicast” should be merged as one of the components in FG 33-2a.

- For FG 33-2e, the terminology “groupcast” should be replaced by either “multicast” or “multicast and broadcast”.

- For FG 33-2e, the maximum number of G-RNTI can be supported by a UE is 16.

- For FG 33-3-1, support up to 16 times dynamic slot-level repetition for group-common PDSCH for multicast.

- For FG 33-3-3a and 33-3-3b, the value of X G-RNTIs should be no larger than the number of G-RNTI defined in FG 33-2e.

- For FG 33-4, the bracket should be removed.

- For FG 33-4, “and support enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling” should be added to the component.

- For FG 33-4, adding another component that “Support of separated PUCCH resource configurations with unicast”.

- For FG 33-5-1, separate the capability for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling.

- For FG 33-5-1d, “Support of PTP retransmission for SPS multicast” can be a separate FG and not merged with FG 33-5-1a.

Reference

- R1-2201520, Summary on UE features for NR MBS, Moderator (NTT DOCOMO, INC.), RAN1#108-e, e-Meeting, February 21st – March 3rd, 2022.

- R1-2202928, Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e, Moderator (AT&T, NTT DOCOMO, INC.), RAN1#108-e, e-Meeting, February 21st – March 3rd, 2022.