3GPP TSG RAN WG1 #109-e		R1-2203978

e-Meeting, May 9th – 20th, 2022

Source:	OPPO

Title:	Discussion on SL positioning scenarios and requirements

Agenda Item:	9.5.1.1

Document for:	Discussion and Decision

Introduction

In RAN#94-e, the study on expanded and improved NR positioning was endorsed with following justification and objectives on SL positioning [1]:

Justification: In Release 17, 3GPP RAN conducted studies on "NR Positioning Enhancements" and "Scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases". The study on "Scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases" focussed on V2X and public safety use cases with the outcome being captured in TR38.845. Additionally, SA1 has developed requirements in TS22.261 for "Ranging based services", and has developed positioning accuracy requirements in TS22.104 for IIoT uses cases in out-of-coverage scenarios. There is a need for 3GPP to study and develop sidelink positioning solutions that can support the use cases, scenarios and requirements identified during these activities. Objective: - Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

In this contribution, the scenarios of V2X, Public safety, commercial and IIoT as well as the corresponding requirements on Sidelink positioning will be discussed.

Discussion

  - V2X

Based on the scenarios and requirements descriptions in TR38.845 [2], in terms of the horizontal or lateral/longitudinal accuracy, the requirements for the absolute position or relative position can be categorized into three sets as follows by incorporating the requirements:

- Set 1: 10 – 50 m with 68 – 95 % confidence level. This includes Group 1 in [5] and Service level 1 in [3].

- Set 2: 1 – 3 m with 95 – 99 % confidence level. This includes Group 2 in [5], Service level 2, 3, 4 in [3].

- Set 3: 0.1 – 0.5 m with 95 – 99 % confidence level. This includes Group 3 in [5], Service level 5, 6, 7 in [3], the requirements in [4].

Besides the above three sets, some other requirements can also be found in TR38.845 [2]:

- For other performance metrics, the corresponding requirements are also defined in a range depending on the positioning service level in TS 22.261 [3]; 2 – 3m (absolute) or 0.2m (relative) vertical accuracy, 95 – 99.9% positioning service availability, 10ms – 1s positioning service latency.

- Positioning service should be provided in indoor, outdoor, tunnel areas.

- The UE velocity up to 250 km/h needs to be supported for outdoor and tunnel areas.

- The requirements should be also fulfilled when the GNSS-based positioning is not available or not accurate enough.

Based on the requirements identified in TS 22.186 [4], two requirements on position accuracy are provided as follows:

- [R.5.1-007] The 3GPP system shall support relative lateral position accuracy of 0.1 m between UEs supporting V2X application (General requirements).

- [R.5.2-003] The 3GPP system shall support relative longitudinal position accuracy of less than 0.5 m for UEs supporting V2X application for platooning in proximity (Requirements to support Vehicles Platooning).

For V2X scenario in Rel-18 SL positioning, the requirements can be 0.1m for horizontal positioning accuracy, 2m (absolute) or 0.2m (relative) for vertical positioning accuracy; physical layer latency for positioning estimation of UE should less than 10ms.

- In Rel-18 SL positioning for V2X scenario, the requirements can be:

- 0.1m for horizontal positioning accuracy;

- 2m (absolute) or 0.2m (relative) for vertical positioning accuracy;

- Less than 10ms for physical layer latency of positioning estimation of UE.

  - Public safety

Based on the scenarios and requirements descriptions in TR38.845 [2], Public safety positioning requirements are defined in TS 22.261 [3] and TS 22.280 [6]. Mission Critical Organizations require mission critical services to have accurate positioning such that first responders can be located at all times during normal and critical operations, indoors as well as outdoors. In [3], 1m horizontal accuracy, 2m (absolute) or 0.3m (relative) vertical accuracy, 95 – 98% positioning service availability. In [6], it also specifies some qualitative positioning requirements which are applicable to both relative and absolute positioning. The level of positioning accuracy required is much more stringent than that required by local and regional regulatory requirements for commercial 5G users. For public safety scenarios in Rel-18 SL positioning, the requirements can be 1m for horizontal positioning accuracy, 2m (absolute) or 0.3m (relative) for vertical positioning accuracy.

- In Rel-18 SL positioning for Public safety scenario, the requirements can be:

- 1m for horizontal positioning accuracy;

- 2m (absolute) or 0.3m (relative) for vertical positioning accuracy.

  - Commercial and IIoT

The scenarios and requirements descriptions for commercial services and IIoT are identified in TS 22.261 [3] and TS 22.104 [9], respectively. Commercial services include industrial automation, UAV control, AR, and etc. For example, UAV control will require more precise positioning information that includes altitude, speed, and direction, in addition to horizontal coordinates. For IIoT, positioning is particularly important for cyber-physical control applications in vertical domains like factories. The requirements for positioning vary widely between different use cases. In Rel-16/Rel-17 NR positioning, the corresponding requirements are identified in TR 38.857 [7] and TR 38.855 [8]. For Rel-17 NR positioning, the following target requirements are defined for commercial use cases and IIoT use cases:

- Rel-17 target positioning requirements for commercial use cases:

  - Horizontal position accuracy (< 1 m) for 90% of UEs

  - Vertical position accuracy (< 3 m) for 90% of UEs

  - End-to-end latency for position estimation of UE (< 100 ms)

  - Physical layer latency for position estimation of UE (< 10 ms)

- Rel-17 target positioning requirements for IIoT use cases:

  - Horizontal position accuracy (< 0.2 m) for 90% of UEs

  - Vertical position accuracy (< 1 m) for 90% of UEs

  - End-to-end latency for position estimation of UE (< 100ms, in the order of 10 ms is desired)

  - Physical layer latency for position estimation of UE (<10ms)

In Rel-18 SL positioning for commercial and IIoT, the target requirement defined in TR 38.857 [7] and TR 38.855 [8] can be reused, because these requirements still fitful by considering the use cases in the new release.

- In Rel-18 SL positioning for commercial and IIoT, the target requirements defined in TR 38.857 / TR 38.855 can be reused.

Conclusion

In this contribution, the scenarios of V2X, Public safety, commercial and IIoT, and the corresponding requirements on Sidelink positioning are discussed. For summary, the following proposals can be found:

- In Rel-18 SL positioning for V2X scenario, the requirements can be:

- 0.1m for horizontal positioning accuracy;

- 2m (absolute) or 0.2m (relative) for vertical positioning accuracy;

- Less than 10ms for physical layer latency of positioning estimation of UE.

- In Rel-18 SL positioning for Public safety scenario, the requirements can be:

- 1m for horizontal positioning accuracy;

- 2m (absolute) or 0.3m (relative) for vertical positioning accuracy.

- In Rel-18 SL positioning for commercial and IIoT, the target requirements defined in TR 38.857 / TR 38.855 can be reused.

Reference

- RP-213588, Revised SID on Study on expanded and improved NR positioning, Intel (Email discussion moderator).

- 3GPP TR 38.845: “Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases”.

- 3GPP TS 22.261: “Service requirements for the 5G system, Stage 1”.

- 3GPP TS 22.186: “Enhancement of 3GPP support for V2X scenarios, Stage 1”.

- 3GPP RP-210040: “Reply LS to RP-201390 on requirements of in-coverage, partial coverage, and out-of-coverage positioning use cases” (source: 5GAA).

- 3GPP TS 22.280: “Mission Critical Services Common Requirements (MCCoRe)”.

- 3GPP TR 38.857: “Study on NR Positioning Enhancements”.

- 3GPP TR 38.855: “Study on NR Positioning support”.

- 3GPP TS 22.104: “Service requirements for cyber-physical control applications in vertical domains, Stage 1”.