3GPP TSG RAN WG1 #109-e		R1-2203980

e-Meeting, May 9th – 20th, 2022

Source:	OPPO

Title:	Discussion on potential solutions for SL positioning

Agenda Item:	9.5.1.3

Document for:	Discussion and Decision

Introduction

In RAN#94-e expanded and improved NR positioning was endorsed as a SI of Rel-18 with following objectives on SL positioning[1]:

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2]

- Scenario/requirements

  - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage

  - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104

  - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104)

  - Spectrum: ITS, licensed

- Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1]

- Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1].

- Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2]

  - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1]

  - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1]

  - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1]

  - Study of positioning architecture and signaling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required]

In this paper we will discuss the potential solutions for SL positioning.

Discussion

  - Positioning methods

In Rel-16 a number of positioning methods were supported in NR-Uu based positioning, including ECID, OTDOA based positioning (UL/DL OTDOA), RTT based positioning and angle-based positioning (DL AoD/UL-AoA)[2]. For sidelink based positioning, ECID cannot be used as there is no concept of cell in sidelink，but the rest of methods supported in NR-Uu positioning could be considered.

More specifically, for UE-based or UE-assisted absolute positioning, some RSUs could be deployed as anchor nodes and play a role that is similar as the TRP in Uu-based positioning, such that the absolute position of a target UE could be estimated based on the measurements (RSTD, RTT or angles) relative to the anchor nodes. One example in V2X scenario is shown in Figure 1.

Figure 1. Absolute positioning and relative positioning in V2X scenario

However, sidelink based positioning may be used for ranging or relative positioning between 2 UEs, like Target UE and Reference UE in Figure 1, which is a functionality but not supported in NR-Uu based positioning. In this use case, it is possible that only 2 UEs are involved, OTDOA based positioning method may not be used naturally. In this case, RTT-based method could be used to estimate the relative distance between the 2 UEs, and AoA/AoD based method could be used for the relative direction estimation.

In general, we have following proposal on positioning methods for sidelink based positioning.

- OTDOA, RTT, and AoA/AoD based positioning methods should be studied for UE-based or UE-assisted absolute positioning, and RTT based and AoA/AoD based positioning methods should be studied for ranging.

  - SL-PRS Signal

In NR-Uu based positioning, different sequences are used for DL and UL. DL-PRS is QPSK modulated Gold sequence, while low-PAPR sequence is used as UL-PRS to guarantee the coverage. DL-PRS sequence is basically the same as DMRS sequence of PSCCH and PSSCH, which has already been supported for any sidelink device, so it is beneficial to use DL-PRS sequence on sidelink from complexity reduction perspective. Furthermore, from the use cases that sidelink positioning supposed to be supported, it seems like that the coverage requirement of SL-PRS is typically small. For example, in V2X use cases, the target of sidelink positioning is to cover the area lack of GNSS and cellular signals, e.g., tunnel, indoor parking lot or city canyon, the size of which is usually rather small. For IIoT, public safety and commercial use cases, sidelink positioning is also intended to be applied within a limited area. In a word, there seems no motivation to use low-PAPR sequence for sidelink positioning.

- DMRS sequence of sidelink PSCCH/PSSCH should be used as SL-PRS, and low-PAPR sequence can be considered if larger coverage needs to be supported in sidelink positioning.

Comb-like (Comb=>2) PRS pattern is used in both DL and UL in NR-Uu based positioning for PRS multiplexing between different TRPs and also PRS power boosting from a single TRP perspective. To cover all REs within the PRS bandwidth, staggered mapping pattern is used in different OFDM symbols, as shown in Figure 2. For sidelink positioning, due to the existence of in-band emission, RE-level PRS multiplexing between different UEs could be challenging. As already mentioned above, there seems no large coverage requirement for SL-PRS, so the motivation of always using comb-like pattern for power boosting is also not clear.

In NR sidelink communication, Comb-like pattern is also used for PSCCH and PSSCH DMRS to support multiplexing between DMRS RE and data RE from a single UE perspective. However, SL-PRS may not always be transmitted together with PSCCH/PSSCH in one slot. For example, for the RSUs shown in Figure 1, it could transmit SL-PRS only in a slot for support of absolute positioning, the configuration of the SL-PRS (e.g., the periodicity, number of repetition, bandwidth, etc.) and other information than needed for positioning (e.g., time difference w.r.t. the reference node, geo-location of the node, etc.) could be indicated in separate slot through PSCCH/PSSCH. For the ranging operation in the same figure, if the configuration of the SL-PRS for ranging is also (pre-)configured, the target UE or the reference UE can also transmit SL-PRS only in a slot. Even in some cases UE needs to transmit PSCCH/PSSCH within a single slot, it is hardly to ensure their bandwidth are always the same. The bandwidth of SL-PRS is expected to be always large to satisfy the stringent accuracy requirement, but the bandwidth of PSCCH/PSSCH is variable to adapt the changing of data size.

- In sidelink positioning SL-PRS may not be transmitted always with PSCCH/PSSCH from the same UE within a single slot, and even they are, it is hardly to ensure the bandwidth of SL-PRS and PSCCH/PSSCH are always the same.

- Standalone SL-PRS transmission within a slot from a single UE perspective should be considered for SL-PRS design in sidelink positioning.

Figure 2. Comb-like pattern and full RE mapping pattern

Therefore, the benefit of comb-like pattern in sidelink positioning is not so clear as NR-Uu based positioning. If the comb-like pattern is used, more OFDM symbols are needed to cover all REs within the bandwidth (i.e., for staggered mapping). In SI phase of sidelink positioning, it is necessary to further study the performance of full RE mapping pattern in addition to comb-like pattern.

- Both full RE mapping pattern and comb-like pattern should be further studied for SL-PRS design.

  - Sidelink positioning procedures

As already mentioned above both absolute positioning and ranging are supposed to be supported by sidelink positioning, the UEs that transmitting SL-PRS could be stable or moving. For the stable UEs that providing SL-PRS for absolute positioning, e.g., the RSUs in Figure 1, they are usually deployed within areas that needs constant positioning signals, they could transmit SL-PRS periodically, this is particularly beneficial when sidelink positioning is used to complement Uu based positioning. On the contrary for ranging in V2X scenario, when vehicle can already achieve accurate geo-location through GNSS or cellular coverage, the basic safety message periodically transmitted by the vehicles can be used to determine relative distance/direction, SL-PRS is necessary only when the mechanism above cannot work. In the sense, on-demand SL-PRS transmission seems also necessary for sidelink positioning.

- Both periodic SL-PRS transmission and on-demand SL-PRS transmission should be considered in sidelink positioning.

In NR sidelink communication all sidelink channels are subject to DL pathloss based power control, in unicast, PSCCH and PSSCH are also subject to sidelink based power control [3]. As SL-PRS can also use licensed carrier, at least DL pathloss based power control should be used to determine the transmission power of SL-PRS.

- In sidelink positioning SL-PRS is at least subject to DL pathloss based power control.

  - Resource allocation

In NR sidelink communication all resource allocation operations are performed within a specific resource pool, SL-PRS as a new candidate SL signal should also follow this basic principle of sidelink. In general, there are 2 options for SL-PRS resource pool, that is to configure dedicated resource pool for SL-PRS or to share resource pool with SL communication.

The first option could be considered for periodically transmitted SL-PRS, as these SL-PRS would occupy constant resources. However, for the on-demand transmitted SL-PRS, dedicate resource pool may degrade the resource efficiency of the system, as the resources within the resource pool may not be used sometimes, to share the resource pool with sidelink communication is beneficial. If SL-PRS shares a mode 2 resource pool with sidelink communication, compatibility with legacy UEs should be guaranteed.

- Following 2 options should be considered for SL-PRS resource pool:

  - Option 1: Dedicated resource pool;

  - Option 2: Shared resource pool with sidelink communication.

Similar as NR sidelink communication both network centric (mode 1 like) and UE autonomous resource allocation (mode 2 like) should be considered for SL-PRS. If network centric resource allocation is used, SL-PRS can share a mode 1 resource pool with sidelink communication. Current mode 2 resource allocation can be the baseline for autonomous resource allocation of SL-PRS, but should be adapted if there is standalone SL-PRS.

- Both network centric (mode 1 like) and UE autonomous resource allocation (mode 2 like) should be considered for SL-PRS.

  - Measurements

Depending on the positioning methods (i.e., TDOA, RTT or AoA/D) to be used in SL positioning, physical layer measurements, including RSTD, Rx-Tx time difference, SL-PRS-RSRP, SL AoA/AoD, etc., could be supported.

- Measurements including RSTD, Rx-Tx time difference, SL-PRS RSRP, SL AoA/AoD, etc., could be supported depending on the positioning methods to be used in sidelink positioning.

Conclusion

In this contribution, we discussed the potential solutions for sidelink positioning, we have following observations and proposals:

- In sidelink positioning SL-PRS may not be transmitted always with PSCCH/PSSCH from the same UE within a single slot, and even they are, it is hardly to ensure the bandwidth of SL-PRS and PSCCH/PSSCH are always the same.

- Measurements including RSTD, Rx-Tx time difference, SL-PRS RSRP, SL AoA/AoD, etc., could be supported depending on the positioning methods to be used in sidelink positioning.

- OTDOA, RTT, and AoA/AoD based positioning methods should be studied for UE-based or UE-assisted absolute positioning, and RTT based and AoA/AoD based positioning methods should be studied for ranging.

- DMRS sequence of sidelink PSCCH/PSSCH should be used as SL-PRS, and low-PAPR sequence can be considered if larger coverage needs to be supported in sidelink positioning.

- Standalone SL-PRS transmission within a slot from a single UE perspective should be considered for SL-PRS design in sidelink positioning.

- Both full RE mapping pattern and comb-like pattern should be further studied for SL-PRS design.

- Both periodic SL-PRS transmission and on-demand SL-PRS transmission should be considered in sidelink positioning.

- In sidelink positioning SL-PRS is at least subject to DL pathloss based power control.

- Following 2 options should be considered for SL-PRS resource pool:

  - Option 1: Dedicated resource pool;

  - Option 2: Shared resource pool with sidelink communication.

- Both network centric (mode 1 like) and UE autonomous resource allocation (mode 2 like) should be considered for SL-PRS.

References

- RP-213588, Revised SID on Study on expanded and improved NR positioning, Intel (Email discussion moderator);

- 3GPP TR TS 38.305, Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN

- 3GPP TS 38.213 V17.1.0 (2022-03) 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Physical layer procedures for control (Release 17);