3GPP TSG-RAN WG1 Meeting #109-e			R1-2203982

e-Meeting, 09 – 20 May, 2022

Agenda item:	9.4.1.1

Title:	Access mechanisms of NR sidelink in unlicensed channel

Source:	OPPO

Document for:	Discussion and Decision

1. Introduction

In RAN#94e, a new Release 18 NR sidelink WI, an evolution towards 5G Advanced, was approved. The latest corresponding WID can be found in [1]. As one major evolution step for NR sidelink to better support mostly commercial use cases, the WID includes an objective to study and specify the support of operating NR sidelink in unlicensed spectrum (SL-U) I.e., not the 5.9GHz frequency band dedicated for ITS that is also commonly known as an unlicensed band.

According to the objective for SL-U in the WID (copied below for convenience), one major work topic is on defining mechanism(s) for unlicensed channel access that are reused from the existing ones supported in NR-U, and assess the usage of existing NR-SL resource allocation schemes (yellow highlights).

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

In addition to the above-described technical areas that are to be handled as part of the work for SL-U, any necessary updates on SL evaluation methodology for the unlicensed channel access is also captured in the meeting agenda for this e-meeting.

In this contribution, we will provide discussions and our views on the following technical topics:

- Essential updates of evaluation methodology for SL-U

  - Evaluation scenarios and layouts

  - Traffic model

  - Performance metric

  - In-band emission model

- Reusing the existing NR-U channel access mechanisms and features for SL-U

  - Type 1 and Type 2 (2A/2B/2C) LBT

  - Channel access priority class

  - Contention window adjustment

  - COT sharing

  - CP extension (CPE)

  - Short control signaling transmission (SCSt) for PSFCH and S-SSB

- Applicability of R16/R17 SL resource allocation schemes in SL-U operation

  - Mode 1 and Mode 2 RA schemes

  - Partial sensing and random resource selection

  - Inter-UE coordination

2. Discussion

2.1 Update of evaluation methodology for NR sidelink in unlicensed spectrum

In this section, we discuss on the evaluation methodology for SL-U channel access.

From a high level, according to the Rel-18 Work item on NR sidelink evolution [1], it is not one objective of this WID to study/update the evaluation methodology for unlicensed sidelink. On the other hand, it is necessary for companies to align with the evaluation assumptions before starting the study on NR sidelink evolution item. Based on the purpose of implementing sidelink technology over unlicensed spectrum is to expand the applicability of NR sidelink to commercial cases, it is a more appropriate way to reuse (with some essential updating) the evaluation methodology updated for commercial use cases in Rel-17 sidelink as the baseline for evaluation on SL-U channel access.

Proposal 1: The evaluation methodology updated for commercial use cases in Rel-17 sidelink should be used as the baseline for evaluation on SL-U channel access.

In sub7GHz bands, different RATs maybe coexist over unlicensed spectrum, for example, IEEE 802.11ac (i.e. Wi-Fi) is widely deployed in 5GHz bands. In Rel-16 NR-U, the evaluation methodology and assumptions for the coexistence of NR-U and Wi-Fi has be study and discussed[2], which can be reused with necessary modification for SL-U evaluation.  Based on the above principles, in the following we give our views on some essential updating for evaluation methodology.

- Evaluation scenarios

As mentioned in the above, SL-U technology is expected to support commercial use cases. These potential commercial cases cover both indoor and outdoor scenarios such as better vehicle communication (outdoor), smart home network (indoor), smart factories network (indoor), etc. Referring to the deployment scenarios discussed in NR-U, SL-U should also be evaluated in both indoor and outdoor scenarios.

For indoor scenario, the layout is derived from NR-U coexistence evaluation scenario with proper modifications to introduce SL-U communication. Two operators each with 3 gNBs/APs are also deployed in a room of size 120m x 80m and the number of UE dropping is also defined for each gNB/AP. However, for SL-U communication, only the UE-to-UE direct communication over unlicensed spectrum will be considered with unicast, groupcast or broadcast traffic in SL-U evaluation.

For outdoor scenario, the following agreement was reached in RAN1 #103-e[3] and RAN1 #104-e[4]:

Agreements: - For the layout for public safety and commercial use cases, support “7 macro sites with 3 cells per site in the layout” Agreements: - For commercial use case, at least following layout options are supported: - Option 3 of TR 36.843: Urban macro (500m ISD) (all UEs outdoor) - UE dropping as in Table A.2.1.1-1 - All UEs are outdoors UEs - Option 1: Urban macro (500m ISD) + 1 RRH/Indoor Hotzone per cell for optional - UE dropping as in Table A.2.1.1-1 - Mix of outdoor and indoor UEs - Option 5 of TR 36.843: Urban macro (1732m ISD) for optional - UE dropping as in Table A.2.1.1-1 - All UEs are outdoors UEs - Mix of outdoor and indoor UEs

It was agreed that the urban macro layout with 7 macro sites and 3 cells per site was supported for commercial cases. In our view, the first option in the agreements (i.e. Option 3 of TR 36.843) can be reused for SL-U evaluation. Since only sidelink operation is considered in commercial cases, UE dropping associated with different operators should refer to NR-U outdoor deployment.

Proposal 2: The indoor scenario used in NR-U and the outdoor scenario used for commercial cases in Rel-17 sidelink should be reused for SL-U evaluation with essential modifications.

- Traffic model

In NR-U study phase, the same traffic model was used for evaluation on coexistence of NR-U and Wi-Fi. In SL-U, considering that sidelink technology has different performance requirements and resource allocation modes and Rel-18 sidelink item is aim to apply sidelink for commercial cases, different traffic models should be used for SL-U and Wi-Fi. Aligning with NR-U simulation, FTP model 3 can be reused for Wi-Fi traffic model. For SL-U traffic model, periodic model 2 and aperiodic model 1 defined in TR 37.885 can be used as the baseline.

Proposal 3: FTP model 3 can be used for Wi-Fi traffic, and periodic model 2 and aperiodic model 1 defined in TR 37.885 can be used as the baseline for SL-U traffic.

- Performance metrics

Considering the point to evaluate the coexistence performance of SL-U and Wi-Fi, the basic performance metrics used for SL-U evaluation should be the same as in NR-U evaluation in TR 38.889, including user perceived throughput (UPT) and latency. In addition, Packet Reception Ratio (PRR) and Packet Inter-reception Ratio (PIR) defined in TR 37.885 are not precluded to evaluate SL-U performance [5].

Proposal 4: The basic performance metrics used for SL-U should be the same as in NR-U, namely user perceived throughput (UPT) and latency, while PRR and PIR defined in 37.885 are not precluded.

- In-band emission model

In RAN1 #103-e, the following agreement on in-band emission model was reached:

Agreements: - For public safety and commercial use cases, reuse in-band emission model used for NR V2X specified in section 6.4E.2.4 in TS 38.101

It was agreed that there is no update for in-band emission model used for commercial cases. In our view, SL-U evaluation can also reuse the in-band emission model used for NR V2X.

Proposal 5: SL-U evaluation should reuse the in-band emission model used for NR-V2X.

The other detail evaluation parameters and assumptions are proposed in the following tables

Table 1: Evaluation parameters for sub7GHz indoor scenario

| Parameter | Value |
| --- | --- |
| Layout deployment | a=20 meters, b=40 meters, c=20 meters, and d=40 meters. |
| UE/STA dropping | Random dropping within coverage of the room |
| UE/STA speed | 3 km/h |
| Number of users | 5 UEs associated with each gNB per 20MHz |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz |
| Number of carriers | 1 |
| SCS | 30KHz |
| UE/STA to UE/STA link channel model | Reuse the parameters of “Channel models” for commercial cases in Rel-17 sidelink enhancement |
| UE/STA to BS/AP link channel Model | NR InH Mixed Office model |
| UE/STA Tx Power | 18dBm |
| UE/STA Antenna gain | 0dBi |
| UE/STA Antenna height | 1.5m |
| UE/STA Receiver Noise Figure | 9dB |
| UE receiver | MMSE-IRC as the baseline receiver |
| UE/STA antenna Array configuration | Baseline Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ Optional Tx/Rx: (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| SL-U traffic model | Periodic traffic model 2 and aperiodic traffic model 1 defined in TR 37.885 |
| Wi-Fi Traffic model | FTP Model 3: Based on FTP model 2 as in TR 36.814 with the exception that packets for the same UE arrive according to a Poisson process with arrival rate 𝜆 and the transmission time of a packet is counted from the time instance it arrives in the queue. FTP model file size: 0.5 Mbytes. |

Table 2: Evaluation parameters for sub7GHz outdoor scenario

| Parameter | Value |
| --- | --- |
| Layout deployment | 1 micro TRP per operator per macro cell |
| ISD | 500m |
| UE/STA dropping | Random dropping within all coverage of the layout |
| UE/STA speed | 3 km/h |
| Number of users | 5 UEs associated with each gNB per 20MHz |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz |
| Number of carriers | 1 |
| SCS | 30KHz |
| UE/STA to UE/STA link channel model | Reuse the parameters of “Channel models” for commercial cases in Rel-17 sidelink enhancement |
| UE/STA to BS/AP link channel Model | NR UMi street canyon |
| UE/STA Tx Power | 18dBm |
| UE/STA Antenna gain | 0dBi |
| UE/STA Antenna height | 1.5m |
| UE/STA Receiver Noise Figure | 9dB |
| UE receiver | MMSE-IRC as the baseline receiver |
| UE/STA antenna Array configuration | Baseline Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ Optional Tx/Rx: (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| SL-U traffic model | Periodic traffic model 2 and aperiodic traffic model 1 defined in TR 37.885 |
| Wi-Fi Traffic model | FTP Model 3: Based on FTP model 2 as in TR 36.814 with the exception that packets for the same UE arrive according to a Poisson process with arrival rate 𝜆 and the transmission time of a packet is counted from the time instance it arrives in the queue. FTP model file size: 0.5 Mbytes. |

2.2 Update of evaluation methodology for NR sidelink in unlicensed spectrum

- Type 1 and Type 2 (2A/2B/2C) channel access

According to the WID[1], it says to reuse channel access mechanism of NR-U for SL-U. The channel mechanisms have been extensively discussed in NR-U and can be applied to SL-U. The channel access mechanisms include LBE (Load based equipment) and FBE (Frame based equipment). And furthermore, LBE includes Type 1 channel access, Type 2A channel access, Type 2B channel access and Type 2C channel access.

Proposal 6: The channel access mechanisms in NR-U can be reused for SL-U, such as Type 1 CA, Type 2A CA, Type 2B CA, Type 2C CA and FBE.

In NR-U, Type 1 CA is used as baseline channel access mechanism, while Type 2 CA is generally used in COT sharing case. If an equipment (gNB or UE) can share the COT from another equipment (UE or gNB), it can use Type 2 CA to access to the channel, otherwise, Type 1 CA should be used. Type 2A CA is also applied to gNB for discovery burst transmission. This principle can also be applied to SL-U.

Proposal 7: Type 1 channel access is default channel access mechanism. Type 2A/2B/2C channel access is applied in COT sharing case.

For Type 1 CA, it is counter based random back-off channel access mechanism. When will the counter equal to 0 is not predictable, and depends on the congestion level of shared spectrum. In NR-U Type 1 CA, if the counter is equal to 0 and UE/gNB does not perform transmission, an additional short LBT (corresponding length is Td+Tsl) is performed before the transmission to sense whether the channel is idle or not. The motivation of the short LBT is to avoid potential collision with other RATs since other RAT have opportunity to access to the channel during the gap between the time when counter is equal to 0 and slot boundary for transmission.

The description of Type 1 CA for UE is copied below [6]

“If a UE has not transmitted a UL transmission on a channel on which UL transmission(s) are performed after step 4 in the procedure above, the UE may transmit a transmission on the channel, if the channel is sensed to be idle at least in a sensing slot duration Tsl when the UE is ready to transmit the transmission and if the channel has been sensed to be idle during all the slot durations of a defer duration Td immediately before the transmission. If the channel has not been sensed to be idle in a sensing slot duration Tsl when the UE first senses the channel after it is ready to transmit, or if the channel has not been sensed to be idle during any of the sensing slot durations of a defer duration Td immediately before the intended transmission, the UE proceeds to step 1 after sensing the channel to be idle during the slot durations of a defer duration Td. ”

Observation 1: For Type 1 CA in NR-U, an additional short LBT is performed before transmission if there is gap between the time when counter is equal to 0 and slot boundary of the transmission.

For Type 2 CA, there are specific gap restriction between two adjacent transmissions. For Type 2C CA, the gap is less than or equal to 16us. For Type 2B CA, the gap is equal to 16us. While for Type 2A CA, whether the gap is equal to 25us, or equal to and larger than 25 us, different companies may have different understanding. According to the Type 2A UL CA procedure description “The UE may transmit the transmission immediately after sensing the channel to be idle for at least a sensing interval Tshort_ul=25us.”, it is possible that UE can sense the channel to be idle more than 25us. Therefore, we think Type 2A CA is applicable if the gap between two transmission is equal to or larger than 25us.

Observation 2: For Type 2 CA, the gap restriction for each channel access mechanism is as follows:

    - Type 2A CA, gap ≥ 25us；

    - Type 2B CA, gap = 16us;

    - Type 2C CA, gap ≤ 16us;

- Channel access priority class

For Type 1 CA, the channel access parameters are determined by corresponding channel access priority class (CAPC). In NR-U, two separate channel access parameter tables are defined for gNB and UE respectively [6], which follows the regulation that one table is for supervising device and another is for supervised device [7]. For SL-U, it does not support one SL-U UE to schedule resource for another SL-U UE. Therefore, we think it is reasonable to reuse the CAPC parameter table of NR-U UL, as shown in Table 3.

Table 3: Channel Access Priority Class (CAPC) for UL

| Channel Access Priority Class (p) | mp | CWmin,p | CWmax,p | Tulmcot,p | allowed CWp sizes |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 7 | 2 ms | {3,7} |
| 2 | 2 | 7 | 15 | 4 ms | {7,15} |
| 3 | 3 | 15 | 1023 | 6ms or 10 ms | {15,31,63,127,255,511,1023} |
| 4 | 7 | 15 | 1023 | 6ms or 10 ms | {15,31,63,127,255,511,1023} |
| NOTE1: For p=3,4, Tulmcot,p=10ms if the higher layer parameter absenceOfAnyOtherTechnology-r14 or absenceOfAnyOtherTechnology-r16 is provided , otherwise, Tulmcot,p=6ms. NOTE 2: When Tulmcot,p=6ms it may be increased to 8ms by inserting one or more gaps. The minimum duration of a gap shall be 100us. The maximum duration before including any such gap shall be 6ms. |  |  |  |  |  |

Proposal 8: For Type 1 CA in SL-U, the CAPC table for UL in NR-U can be reused.

In NR-U UL, the CAPC is determined based on channel type. For example, for SRS and PUCCH transmission, specific CAPC is applied. For PUSCH transmission, the CAPC is determined by 5QI [8]. The mapping between CAPC and 5QI is shown in Table 4.

# Table 4: Mapping between Channel Access Priority Classes and 5QI

| CAPC | 5QI |
| --- | --- |
| 1 | 1, 3, 5, 65, 66, 67, 69, 70, 79, 80, 82, 83, 84, 85 |
| 2 | 2, 7, 71 |
| 3 | 4, 6, 8, 9, 72, 73, 74, 76 |
| 4 | - |
| NOTE: lower CAPC value means higher priority - |  |

In SL-U, the same mechanism can be applied to PSCCH/PSSCH, i.e., the CAPC for PSCCH/PSSCH is based on PQI.

Proposal 9: It supports to determine CAPC based on PQI

If SL feedback is enabled, when UE1 performs PSCCH/PSSCH transmission, the corresponding receiver UE2 will perform PSFCH transmission. If UE1 can access the channel by Type 1 CA, it can share the COT to UE2 so that UE2 can use Type 2 CA when it performs SL transmission within the COT duration, instead of Type 1 CA. That can improve PSFCH performance. Therefore, when UE1 determines CAPC for PSCCH/PSSCH, it can also consider the maximal COT duration of shared COT so that it can cover the corresponding PSFCH slot.

Proposal 10:  When UE determines CAPC for PSCCH/PSSCH, the corresponding PSFCH slot to be covered by the shared COT from the UE can be supported.

- Contention window adjustment

In NR-U, it supports to adjust contention window based on HARQ feedback. We think this mechanism can also be applied to SL-U, especially for the case of unicast and groupcast with SL feedback enabled. While SL-U also support broadcast which does not support SL feedback. Even for unicast and groupcast, the SL feedback can be disabled. In those cases, the adjustment of contention window based on HARQ feedback cannot be applied. Some other mechanisms should be studied.

Proposal 11:

    - For the case of unicast and groupcast with SL feedback enabled, it supports to adjust the contention window based on HARQ feedback.

    - For the case of broadcast, or unicast and groupcast with SL feedback disabled, how to adjust contention window needs FFS.

- COT sharing

In NR-U, it supports COT sharing between UE and gNB. If gNB/UE uses the shared COT from UE/gNB, it can perform Type 2 CA within the shared COT duration. Such kind of COT sharing mechanism can also be applied to SL-U. If a SL-U UE access to the channel successfully, it can share the COT to other UEs. If a UE’s transmission resource is within the shared COT duration, it can use Type 2 CA, instead of Type 1 CA, which can improve LBT performance.

Proposal 12:

    - COT sharing among SL-U UEs is supported.

    - Within the shared COT, Type 2 CA can be applied.

In NR-U, when UE can access to the channel through Type 1 CA, it can perform UL transmission to gNB and share the COT to gNB. gNB can use the shared COT from the UE to perform DL transmission. While when gNB shares the COT from the UE, it can only perform the DL transmission that contains transmission to the UE that initiated the channel occupancy and can include non-unicast and/or unicast transmissions where any unicast transmission that includes user plane data is only transmitted to the UE that initiated the channel occupancy. In that case, when gNB shares the COT from the UE, it can perform DL non-unicast and/or unicast transmissions only when the UE is target receiver. Otherwise, gNB cannot use the shared COT.

Observation 3: In NR-U, only when the UE that initiated the COT is a target receiver, gNB can use the shared COT to perform DL transmission which includes non-unicast and/or unicast transmissions.

The above restriction of COT sharing should be also applied in SL-U. When UE2 receives a shared COT from UE1, it can use the shared COT to perform SL transmission only when UE1 is a target receiver of the SL transmission. It is straightforward for unicast. For groupcast, that means UE1 and UE2 belongs to the same group. For broadcast, SL transmission corresponds to service type. If UE1 is interested in the service type to be transmitted by UE2, UE2 can use the shared COT to perform SL transmission in broadcast mode. For broadcast, the service type can be determined based on destination ID carried in SCI.

Proposal 13: In SL-U, if UE2 receives shared COT from UE1, UE2 can use the shared COT to perform SL transmission only when UE1 is a target receiver.

For the COT sharing among UEs in SL-U, the following open issues can be further studied.

- How to initiate a COT

- How to share a COT to other UEs

- Which UE can use the shared COT

- Whether/how to handle the case if there is no SL TX within some slots within COT

Proposal 14: For COT sharing in SL-U, the following open issues need FFS:

    - How to initiate a COT

    - How to share a COT to other UEs

    - Which UE can use the shared COT

    - Whether/how to handle the case if there is no SL TX within some slots within COT

- CP extension (CPE)

CP extension is supported in NR-U. in our view, there are two benefits for CPE in NR-U. Firstly, it can avoid other RAT device to occupy the channel. When the LBT success, UE/gNB can occupy the channel by transmitting CPE. Secondly, it can guarantee the required gap for a specific Type 2 channel access mechanism.

Observation 4: CPE is beneficial for channel access.

Whether and how CPE mechanism can be applied to SL-U needs FFS. The key difference between SL-U and NR-U is that NR-U is gNB scheduling based, while SL-U is distributed system. In NR-U, all UL transmission is controlled by gNB.  gNB can determine whether or not CPE is used by UE, and determine the starting position of the CPE per UE. While for SL-U, there is no central scheduler in mode 2, if CPE is used and in case the starting position of CPE per UE can be different, there will be inter-UE blocking issue. One illustration is shown below. In the figure, UE1 access the channel through Type 1 CA, and perform SL transmission in slot n+1. UE1 shares the COT to other UE2. In slot n+2, UE2 can use the shared COT while UE3 not. UE3 have to perform Type 1 CA. If there is gap between the time when the counter equal to 0 and slot boundary of slot n+2, UE3 needs to perform additional LBT (Td+Tsl) before the slot boundary. If UE2 performs Type 2A CA in the GP symbol of slot n+1 and perform CPE transmission in case sensing the channel is idle, it will block the additional LBT procedure of UE3 so that UE3 cannot access the channel. Similarly, in slot n+3, both UE4 and UE5 can use the shared COT from UE1, and Type 2A CA are performed. While if the starting positions of CPE from UE4 and UE5 are different, such as CPE from UE4 is earlier than UE5, it will block Type 2A CA procedure of UE5.

Observation 5: in SL-U, if starting position of CPE is different, it will result in inter-UE blocking.

Proposal 15: whether and how to apply CPE in SL-U needs FFS.

Figure 1

- Short control signaling transmission (SCSt) for PSFCH and S-SSB

Short control signaling transmission mechanism is supported in unlicensed band [enXXX]. If SCSt is applied, no LBT is needed before transmission. While there are two limitations to apply SCSt method:

- Limit 1: within an observation period of 50 ms, the number of Short Control Signalling Transmissions by the equipment shall be equal to or less than 50;

- Limit 2: the total duration of the equipment's Short Control Signalling Transmissions shall be less than 2 500 μs within said observation period

If SCSt method can be applied in SL-U, there can be the following benefits:

- SCSt can simplify the physical layer procedure design especially for the case of LBT failure.

In SL-U, UE should perform LBT before each SL transmission occasions. If LBT failure, UE cannot perform SL transmission. That will be not critical for PSCCH/PSSCH since there could be retransmission occasions. While for PSFCH, according to legacy NR SL mechanism, there is only one PSFCH transmission occasion for one PSSCH transmission. If LBT failure, UE cannot transmit PSFCH and TX UE will detect PSFCH DTX and perform PSSCH retransmission, which will result in lower spectrum efficiency. Enhancement of PSFCH due to LBT failure should be studied, such as configure multiple PSFCH transmission occasions for one PSSCH transmission. If SCSt can be applied, UE can transmit PSFCH without LBT, no further enhancement for PSFCH is necessary.

- NACK-only feedback is applicable in SL-U with SCSt method.

NACK-only feedback is supported in NR SL. It is reasonable to support NACK-only feedback in SL-U. While one drawback of NACK-only is it is hardly to differentiate DTX and ACK. If it is applied in SL-U, it is also possible that UE cannot transmit NACK feedback in case of LBT failure. Even if multiple PSFCH transmission occasions are configured, it cannot avoid LBT failure. Therefore, if PSSCH TX UE cannot detect PSFCH, it cannot differentiate whether the RX UE does not transmit PSFCH (if ACK) or it is because of LBT failure. In this case, we think SCSt is beneficial since UE perform PSFCH transmission without LBT which can guarantee the PSFCH can be transmitted if needed.

Observation 6: SCSt is beneficial for:

    - Simplify physical layer procedure due to LBT failure

    - NACK-only PSFCH mechanism applied in SL-U

The following two kinds of transmission can be considered to apply SCSt in SL-U.

- S-SSB transmission

For S-SSB, the periodicity is 160ms. Within a S-SSB period, the number of S-SSB resources for 15/30/60 kHz are {1}/{1,2}/{1,2,4} respectively. Two or three sets of synchronization resources can be configured while only one is used for S-SSB transmission, the others for S-SSB reception. One S-SSB transmission occupies one slot, and corresponds to 1ms/0.5ms/0.25ms for 15/30/60 kHz respectively. It can fulfill both limit 1 and limit 2 of SCSt.

Observation 7: S-SSB transmission can fulfill the limitations of SCSt.

- PSFCH transmission

A PSFCH occupies 2 OFDM symbols (1/7 of a slot duration). The periodicity of PSFCH can be 1/2/4 slots. The following table shows whether the limitations of SCSt can be fulfilled. For P=4, both limitations of SCSt can be fulfilled, while not for P=1 and P=2. But even in case of P=1 and P=2, that does not mean a UE will transmit PSFCH every PSFCH occasion, especially considering the minimum gap between two adjacent PSSCH transmissions is z=a+b, where a is gap between PSSCH and associated PSFCH (provided by sl-MinTimeGapPSFCH) and b is gap between PSFCH and PSSCH retransmission. Therefore, in most cases, the limitations of SCSt can be fulfilled for PSFCH transmission.

Observation 8: PSFCH transmission can fulfill the limitations of SCSt in most cases.

Table 5.

| SL SCS | SCSt limitations | P=1 | P=2 | P=4 |
| --- | --- | --- | --- | --- |
| 15 kHz | Limit1 | √ | √ | √ |
|  | Limit2 | X | X | √ |
| 30 kHz | Limit1 | X | √ | √ |
|  | Limit2 | X | X | √ |
| 60 kHz | Limit1 | X | √ | √ |
|  | Limit2 | X | X | √ |

If SCSt is applied to PSFCH or S-SSB transmission, that will cause potential collision with other RATs since there is no LBT is performed. How the collision will impact the performance of SL transmission and other RATs transmission needs FFS.

2.3 Applicability of R16 and R17 SL resource allocation schemes in SL-U operation

- Mode 1 and Mode 2 RA schemes

In NR sidelink Mode 1, the sidelink BWP, resource pools and resource grants for transmission (including dynamic scheduling and SL configured grant resources) are fully managed, configured and scheduled by either a NR gNB or LTE eNB. It is the same as UL transmissions in NR-U, the base station essentially has the full control of when and where the UE can perform its sidelink transmission. Therefore, it is straight forward to see that when a UE is operating in Mode 1, where SL transmission resources are fully configured and scheduled by LTE eNB or NR gNB, Tx collision with either LTE-LAA or NR-U uplink and downlink operations on the unlicensed carrier can be fully avoided by the base station.

For NR sidelink operation in unlicensed spectrum, as stipulated by the WI objective the “Uu operation for Mode 1 is limited to licensed spectrum only”. By this, it means the sidelink BWP, resource pools, sidelink configured grant resources, DCI for scheduling SL transmissions and PUCCH for reporting SL HARQ-ACK information are all to be configured/provided over a licensed carrier. It is only the SL transmission is performed over the unlicensed channel(s). As such, as long as UE performs LBT prior to a SL transmission to assess whether the channel is clear (LBT success = transmit; LBT failure = no transmit), the remaining Mode 1 operation remains the same as in Rel-16.

However, it remains unclear and further discussion is needed on whether the scheduling NR gNB would still be able to indicate the type of LBT for the UE to perform in the SL scheduling DCI (just as in NR-U for UL transmission), as this depends on whether the gNB is also operating NR-U on the same unlicensed channel.

In summary, when a UE is operating in Mode 1 where the SL transmission timing and resources are fully configured and scheduled by the connected base station, any Tx collision/conflict with LTE-LAA, NR-U operations and other SL Mode 1 UEs on the unlicensed spectrum can be fully controlled and avoided by proper base station scheduling. In order to avoid Tx collision/conflict with other RATs operating in the unlicensed spectrum (e.g., Wi-Fi access points and stations), just as what LTE-LAA and NR-U have adopted for their unlicensed channel access, sidelink UE would perform a LBT procedure just before its actual transmission to ensure the unlicensed channel is clear for access.

Observation 9: When a UE is operating in Mode 1 where the SL transmission timing and resources are fully configured and scheduled by the connected base station, any Tx collision/conflict with LTE-LAA, NR-U operations and other SL Mode 1 UEs on the unlicensed spectrum can be fully controlled and avoided by proper base station scheduling. In order to avoid Tx collision/conflict with other RATs operating in the unlicensed spectrum (e.g., Wi-Fi access points and stations), just as what LTE-LAA and NR-U have adopted for their unlicensed channel access, sidelink UE would perform a LBT procedure just before its actual transmission to ensure the unlicensed channel is clear for access.

Proposal 16: In sidelink Mode 1, whether the scheduling NR gNB would still be able to indicate the type of LBT for the UE to perform in the SL scheduling DCI (just as for UL transmission in NR-U) should be FFS.

When a SL UE operates in Mode 2, on the other hand, SL transmission resources are fully selected by the Tx-UE on its own based on a “sensing and reservation” operation to avoid collision/conflict with other SL transmitting UEs. Different from the channel sensing in LBT (also often known as clear channel assessment CCA), which is performed just before the actual transmission of a signal/channel on an unlicensed channel, the SL sensing and reservation requires the UE to “sense” the radio channel by performing blind decoding of PSCCH/SCI transmitted by other SL UEs in every slot during a sensing window (e.g., 1100ms). When a PSCCH is detected and multiple SL resources are indicated in the SCI (including future resources for SL transmissions), the sensing UE will exclude these reserved future resources as part of a resource selection procedure (subject to the measured SL-RSRP level) to avoid collision / minimize interference among SL transmissions.

This “sensing and reservation” operation has worked very well for sidelink since Rel-14 LTE-V2X to avoid Tx collision and it continued to be adopted for NR sidelink in Rel-16 (full sensing) and Rel-17 (partial sensing). Beside the collision avoidance, the sensing and reservation mechanism allows multiple transmission of SL signals and channels from different UEs to be multiplexed in the frequency domain (FDM) in the same slot to improve spectral efficiency. That is, if a UE reserved only a portion of the channel in the frequency domain (e.g., just one sub-channel with 10 PRBs for small data transmission), based on the SL sensing operation it allows other UE(s) to select other sub-channel(s) in the same slot for their SL transmissions without incurring collision or creating interference to each other. Similar to LTE-LAA, NR-U and SL Mode 1 operation, where UL and SL resources for different UEs are frequency multiplexed in the same slot/subframe as scheduled by the base station, the only difference in Mode 2 is that the resources are selected by SL UEs autonomously, without a central scheduling node.

Then based on the above explanation, it is clear that since the resource allocation mechanism in SL Mode 2 is based on a “sensing and reservation” operation to avoid intra-RAT collision/conflict among the SL UEs, a Mode 2 UE should additionally perform LBT to ensure the unlicensed channel is clear before the actual transmission of SL signals/channels to avoid inter-RAT collision/conflict (just as in Mode 1) with Uu and WiFi operations.

In summary, sidelink Mode 2 “sensing and reservation” operation avoids/minimizes collisions among SL UEs while allowing their SL transmissions to be multiplexed in the frequency domain (FDM) to achieve high spectral efficiency. As a result, this will help reducing the COT (channel occupancy time) in the unlicensed channel taken up by SL transmissions. In order to avoid Tx collision/conflict with other RATs operating in the unlicensed spectrum (e.g., Wi-Fi access points and stations), just as described in the above for SL Mode 1 operation in the unlicensed channel, Mode 2 UE would perform a LBT procedure just before its actual transmission to ensure the unlicensed channel is clear for access.

Observation 10: Sidelink Mode 2 “sensing and reservation” operation avoids/minimizes collisions among SL UEs while allowing their SL transmissions to be multiplexed in the frequency domain (FDM) to achieve high spectral efficiency. Subsequently, this will help reducing the COT (channel occupancy time) in the unlicensed channel taken up by SL transmissions. In order to avoid Tx collision/conflict with other RATs operating in the unlicensed spectrum (e.g., Wi-Fi access points and stations), just as described in the above for SL Mode 1 operation in the unlicensed channel, Mode 2 UE would perform a LBT procedure just before its actual transmission to ensure the unlicensed channel is clear for access.

- Partial sensing and random resource selection

In Rel-17, power efficient resource allocation schemes based on UE performing PSCCH (SCI) decoding and SL-RSRP measurement in only subsets of sidelink slots (a.k.a. partial sensing) was introduced to avoid collisions with periodic and aperiodic transmissions from other sidelink UEs in the same resource pool. Similar to the Rel-16 Mode 2 RA scheme, this partial sensing mechanism (intended to be used by battery/power limited UEs) can effectively mitigate transmission conflicts with other SL UEs by monitoring the usage/reservation status of the radio channel resources within a set of candidate slots and selecting available ones that have not been reserved.

For NR sidelink operation in unlicensed channels, similar to the Mode 1 and Mode 2 RA schemes, the same partial sensing and resource selection operation can be adopted by battery/power limited UEs to avoid intra-RAT conflicts in the same resource pool and combine this operation with LBT just before the SL transmission to avoid inter-RAT collision (e.g., with LTE-LAA, NR-U and WiFi signals).

Observation 11: Similar to the Rel-16 Mode 2 full sensing and reservation RA, the partial sensing RA mechanism introduced in Rel-17 for battery/power limited UEs can be also used together with the LBT schemes to gain access to the unlicensed channel just before the actual SL transmission. If the partial sensing does not detect any PSCCH/SCI transmission is some sensing slots (e.g., due to no SL transmission from other UEs or the channel is occupied by other RATs), it just means no SL resource is reserved from those slots and less SL resources need to be excluded from the candidate resource set. The partial sensing operation itself would still work with the LBT schemes.

On the other hand, if a SL transmitter UE performs random resource selection without any receiver / channel sensing capability/hardware such that it is not possible to perform LBT before a SL transmission, this type of UE should not be allowed to operate/transmit in an unlicensed spectrum. However, if a SL transmitter UE is capable of performing LBT in an unlicensed channel, the UE should still be allowed to perform random selection of SL resources (e.g., due to short PDB or the minimum M monitoring slots for CPS cannot be guaranteed) as long as LBT is performed just before the SL transmission to ensure the unlicensed channel is clear.

Observation 12: If a SL transmitter UE is capable of performing LBT in an unlicensed channel, the UE should still be allowed to perform random selection of SL resources as long as LBT is performed just before the SL transmission to ensure the unlicensed channel is clear.

- Inter-UE coordination

The main purpose of the inter-UE coordination (IUC) schemes introduced in Rel-17 is to improve the reliability of SL transmission (hence the communication) by indicating a set of preferred, non-preferred channel resources or a future collision to another NR sidelink UE to avoid collision/conflict. Due to the hidden node issue and/or the half-duplex problem, a sidelink UE-A may be explicitly or implicitly (based on a condition) triggered to inform UE-B which SL resources in the future are safe or not safe to use as they may collide with another UE’s SL transmission. But since these IUC indications are carried in either PSCCH/PSSCH or PSFCH, similar to the previous discussions, as long as the sidelink UE performs LBT before the actual SL PHY channel transmission on an unlicensed channel, the IUC schemes specified in Rel-17 would still be applicable and useful in SL-U to ensure high transmission/communication reliability (by reducing intra-RAT collisions).

Observation 13: It is beneficial and feasible to continue supporting the R17 inter-UE coordination schemes for NR sidelink operation also in unlicensed spectrum to ensure high reliability of SL transmissions by indicating preferred/non-preferred resources or potential collision to another UE for avoiding collisions / interference, as long as the UE adheres to the channel access rule (i.e., LBT) before sending the IUC information on the licensed channel.

3. Conclusion

The channel access related topics are discussed in this paper. The following proposals and observations are given to summarize our views.

Proposal 1: The evaluation methodology updated for commercial use cases in Rel-17 sidelink should be used as the baseline for evaluation on SL-U channel access.

Proposal 2: The indoor scenario used in NR-U and the outdoor scenario used for commercial cases in Rel-17 sidelink should be reused for SL-U evaluation with essential modifications.

Proposal 3: FTP model 3 can be used for Wi-Fi traffic, and periodic model 2 and aperiodic model 1 defined in TR 37.885 can be used as the baseline for SL-U traffic.

Proposal 4: The basic performance metrics used for SL-U should be the same as in NR-U, namely user perceived throughput (UPT) and latency, while PRR and PIR defined in 37.885 are not precluded.

Proposal 5: SL-U evaluation should reuse the in-band emission model used for NR-V2X.

Proposal 6: The channel access mechanisms in NR-U can be reused for SL-U, such as Type 1 CA, Type 2A CA, Type 2B CA, Type 2C CA and FBE.

Proposal 7: Type 1 channel access is default channel access mechanism. Type 2A/2B/2C channel access is applied in COT sharing case.

Observation 1: For Type 1 CA in NR-U, an additional short LBT is performed before transmission if there is gap between the time when counter is equal to 0 and slot boundary of the transmission.

Observation 2: For Type 2 CA, the gap restriction for each channel access mechanism is as follows:

    - Type 2A CA, gap ≥ 25us；

    - Type 2B CA, gap = 16us;

    - Type 2C CA, gap ≤ 16us;

Proposal 8: For Type 1 CA in SL-U, the CAPC table for UL in NR-U can be reused.

Proposal 9: It supports to determine CAPC based on PQI

Proposal 10:  When UE determines CAPC for PSCCH/PSSCH, the corresponding PSFCH slot to be covered by the shared COT from the UE can be supported.

Proposal 11:

    - For the case of unicast and groupcast with SL feedback enabled, it supports to adjust the contention window based on HARQ feedback.

    - For the case of broadcast, or unicast and groupcast with SL feedback disabled, how to adjust contention window needs FFS.

Proposal 12:

    - COT sharing among SL-U UEs is supported.

    - Within the shared COT, Type 2 CA can be applied.

Observation 3: In NR-U, only when the UE that initiated the COT is a target receiver, gNB can use the shared COT to perform DL transmission which includes non-unicast and/or unicast transmissions.

Proposal 13: In SL-U, if UE2 receives shared COT from UE1, UE2 can use the shared COT to perform SL transmission only when UE1 is a target receiver.

Proposal 14: For COT sharing in SL-U, the following open issues need FFS:

    - How to initiate a COT

    - How to share a COT to other UEs

    - Which UE can use the shared COT

    - Whether/how to handle the case if there is no SL TX within some slots within COT

Observation 4: CPE is beneficial for channel access.

Observation 5: in SL-U, if starting position of CPE is different, it will result in inter-UE blocking.

Proposal 15: whether and how to apply CPE in SL-U needs FFS.

Observation 6: SCSt is beneficial for:

    - Simplify physical layer procedure due to LBT failure

    - NACK-only PSFCH mechanism applied in SL-U

Observation 7: S-SSB transmission can fulfill the limitations of SCSt.

Observation 8: PSFCH transmission can fulfill the limitations of SCSt.

Observation 9: When a UE is operating in Mode 1 where the SL transmission timing and resources are fully configured and scheduled by the connected base station, any Tx collision/conflict with LTE-LAA, NR-U operations and other SL Mode 1 UEs on the unlicensed spectrum can be fully controlled and avoided by proper base station scheduling. In order to avoid Tx collision/conflict with other RATs operating in the unlicensed spectrum (e.g., Wi-Fi access points and stations), just as what LTE-LAA and NR-U have adopted for their unlicensed channel access, sidelink UE would perform a LBT procedure just before its actual transmission to ensure the unlicensed channel is clear for access.

Proposal 16: In sidelink Mode 1, whether the scheduling NR gNB would still be able to indicate the type of LBT for the UE to perform in the SL scheduling DCI (just as for UL transmission in NR-U) should be FFS.

Observation 10: Sidelink Mode 2 “sensing and reservation” operation avoids/minimizes collisions among SL UEs while allowing their SL transmissions to be multiplexed in the frequency domain (FDM) to achieve high spectral efficiency. Subsequently, this will help reducing the COT (channel occupancy time) in the unlicensed channel taken up by SL transmissions. In order to avoid Tx collision/conflict with other RATs operating in the unlicensed spectrum (e.g., Wi-Fi access points and stations), just as described in the above for SL Mode 1 operation in the unlicensed channel, Mode 2 UE would perform a LBT procedure just before its actual transmission to ensure the unlicensed channel is clear for access.

Observation 11: Similar to the Rel-16 Mode 2 full sensing and reservation RA, the partial sensing RA mechanism introduced in Rel-17 for battery/power limited UEs can be also used together with the LBT schemes to gain access to the unlicensed channel just before the actual SL transmission. If the partial sensing does not detect any PSCCH/SCI transmission is some sensing slots (e.g., due to no SL transmission from other UEs or the channel is occupied by other RATs), it just means no SL resource is reserved from those slots and less SL resources need to be excluded from the candidate resource set. The partial sensing operation itself would still work with the LBT schemes.

Observation 12: If a SL transmitter UE is capable of performing LBT in an unlicensed channel, the UE should still be allowed to perform random selection of SL resources as long as LBT is performed just before the SL transmission to ensure the unlicensed channel is clear.

Observation 13: It is beneficial and feasible to continue supporting the R17 inter-UE coordination schemes for NR sidelink operation also in unlicensed spectrum to ensure high reliability of SL transmissions by indicating preferred/non-preferred resources or potential collision to another UE for avoiding collisions / interference, as long as the UE adheres to the channel access rule (i.e., LBT) before sending the IUC information on the licensed channel.

4. References

[1] RP-220300, “WID revision: NR sidelink evolution”, RAN #95e

[2] 3GPP TR 38.889 V16.0.0, “Study on NR-based access to unlicensed spectrum (Release 16)”

[3] RAN1 Chairman’s Notes, RAN1 #103-e

[4] RAN1 Chairman’s Notes, RAN1 #104-e

[5] 3GPP TR 37.885 V15.3.0, “Everything (V2X) use cases for LTE and NR (Release 15)”

[6] 3GPP TS37.213 V17.0.0,  “Physical layer procedures for shared spectrum channel access”.

[7] ETSI EN 301 893 V2.1.1, “5 GHz RLAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU”

[8] 3GPP TS38.300 V16.8.0,  “NR and NG-RAN Overall Description”.