3GPP TSG RAN WG1 #109-e		R1-2203984

e-Meeting, May 9th – 20th, 2022

Source:	OPPO

Title:	Discussion on co-channel coexistence for LTE and NR V2X

Agenda Item:	9.4.2

Document for:	Discussion and Decision

Introduction

According to the WID on NR SL evolution revised in RAN#95-e with the following objective [1]:

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4]

- Reuse the in-device coexistence framework defined in Rel-16 as much as possible

In this paper, we discuss the motivation, feasibility and potential solutions for co-channel coexistence and also provide the corresponding system level simulation result.

Discussion

  - Motivation for co-channel coexistence

The main motivation for proposing the co-channel coexistence work was due to a few factors: scarcity of the existing frequency spectrum dedicated to the Intelligent Transportation System (ITS), the amount of available bandwidth is barely sufficient, and there are already different types/versions of communication technologies that can be used for V2X services. Depending on regional regulation and ITS operation requirements, currently there can be more than one V2X communication systems deployed in the same geographical area. For example, since there could be no state regulation on which specific type or version of V2X communication system that must be used in a region and it is left to be determined by market demand, where a group of car companies could use IEEE 802.11p based V2X system while another group uses cellular-V2X (C-V2X) system. Another possible scenario could be both LTE and NR based C-V2X systems are deployed in the field concurrently to provide basic safety services and advanced V2X use cases, respectively. In this scenario, it is very possible that the limited allocated spectrum (at least until more spectrum is assigned for ITS) needs to be shared among the different types/versions of V2X systems. And therefore, it is essential that different V2X communication technologies should co-exist in the same spectrum or even in the same frequency channel with minimal and controllable interference and interruption to each other’s operation.

Observation 1: The co-channel coexistence for NR V2X and LTE V2X is essential due to the limited ITS spectrum and different types/versions of V2X systems.

Another main motivation for the coexistence work is to facilitate an anticipated technology transition / migration in C-V2X communication from LTE-based to NR-based sidelink system. During the initial deployment, it is likely only the LTE-based C-V2X is rolled out for trials and early market adoption of V2X communications between vehicles with less complex system. As the NR-based C-V2X system is becoming more mature and fully tested, it is expected that there will be a transition / migration of C-V2X technology since NR-V2X can be also used to provide basic safety services, and hence, replacing the LTE-V2X counterpart. Without assuming more dedicated ITS spectrum will be allocated for V2X in the future, at least the LTE-based and NR-based V2X systems will need to coexist in the same frequency channel due to the limited available frequency spectrum during the technology transition period.

Observation 2: It is expected that the NR-based and LTE-based V2X systems will coexist during the technology transition period.

  - Potential solutions for co-channel coexistence

To reduce the interference between LTE V2X and NR V2X when both LTE V2X and NR V2X are deployed in the same frequency channel, there are two potential solutions:

- Semi-static partitioning of resources in the same frequency channel between LTE V2X and NR V2X operation with no overlapping resource pools. For this, resource partitioning in the time division multiplexing (TDM) fashion would be better than the frequency division multiplexing (FDM) approach, since the overall bandwidth for sidelink transmission is not sacrificed especially for large data transmissions. The resource pool partitioning is the simplest way to solve the co-channel coexistence issue. However, this could potentially cause a transmission delay issue to latency sensitive services due to not all available slots are allocated for NR SL operation. In the meanwhile, the semi-static configuration or pre-configuration cannot adapt flexibly to the changes in V2X traffic demand between the two RATs which can result in high congestion for one RAT but underutilization for the other RAT. Furthermore, if PSFCH is configured for NR SL, the FDM approach may not be feasible due to the AGC issue.

- Dynamic sharing of resources for coexistence of LTE V2X and NR V2X technologies in the same frequency channel by configuring overlapping resource pools between the two RATs. Potentially, the resource pool for each RAT covers the entire channel bandwidth and all sidelink subframes/slots (except for slots with PSFCH symbols). This, however, raises the question of how to mitigate interference / transmission collision issue between the two RATs and a certain mechanism is definitely needed to resolve this issue due to the high reliability requirements from both basic and advanced V2X use cases. In this case, some information and configurations need to be exchanged between LTE module and NR module in the same UE. For example, the LTE module forwards the sensing result or the candidate resource set after resource exclusion to the NR module and the NR module performs resource exclusion/(re-)selection based on the input from LTE module. By adopting this mechanism, the major advantage is full utilization of all available radio resources allocated to either LTE or NR-based SL communications while transmission collisions can be largely avoided. However, this would require some updates and changes to NR specification. Meanwhile, the system performance ought to be evaluated if the system only relies on the NR SL counterpart to do the active conflict avoidance.

Proposal 1: Both semi-static partitioning of resources and dynamic sharing of resources between LTE V2X and NR V2X should be considered.

The in-device coexistence between LTE SL and NR SL in different frequency channels was studied and discussed in Rel-16. And it was mainly handled by dropping the transmission of LTE or NR SL based on the priority comparison. which may cause the performance loss of the RAT with lower priority. In the co-channel coexistence work, the preferred approach is to avoid the in-device collision instead of dropping transmissions. For example, the LTE module forwards the selected SL grant to NR module such that NR module can exclude the candidate resource overlapping with the SL grant in time domain.

Proposal 2: NR module performs resource exclusion based on the SL grant determined by LTE module to address the in-device coexistence issue in the same frequency channel.

  - System level simulation result

In this section, we provide the system level simulation results of both LTE and NR V2X UE deployed in the same frequency channel to evaluate the performance of reliability when different collision avoidance mechanisms (i.e., dynamic sharing of resources and semi-static partitioning) are used. As for the dynamic sharing mechanism, LTE and NR UE are configured with overlapped resource pool and NR UE can active avoid the collision with LTE UE based on the sensing result of LTE module. For semi-static partitioning, the TDM resource pools are configured for LTE and NR UE. In our simulation, three typical scenarios are simulated and the corresponding simulation results are shown in Figure 1 to Figure 3, respectively. Furthermore, other simulation assumptions can be found in the appendix.

Scenario 1：NR UE with periodic traffic + LTE UE with periodic traffic + Freeway layout

Scenario 2：NR UE with aperiodic traffic + LTE UE with periodic traffic + Freeway layout

Scenario 3：NR UE with periodic traffic + LTE UE with periodic traffic + Urban layout

Figure 1 and Figure 2 illustrate the PRR of LTE and NR UE in scenario 1 and scenario 2, it is observed that there is no much difference for the performance of LTE UE or NR UE when the dynamic sharing mechanism and the semi-static approach are simulated. This is mainly because a relatively small number of UEs in freeway and the ratio of the slots included in NR resource pool to the subframes included in LTE resource pool is set according to the traffic of NR and LTE UE. It should be noted that the around 2% performance loss for the semi-static case of NR UE in scenario 2 is mainly due to the mismatch between the aperiodic traffic of NR UE and the periodic configured TDM resource pool.

Observation 3: There is no much difference for the PRR of LTE UE or NR UE when dynamic sharing and semi-static configuration are applied in freeway scenario.

However, although we still try to adjust the ratio of slots/subframes between NR and LTE resource pool in the semi-static case of scenario 3, the simulation results always show the performance loss of one RAT and the performance gain for another RAT compared with the dynamic sharing approach. In other words, it is hard to find a proper TDM ratio between the NR and LTE resource pool in urban to avoid the performance loss of NR-based or LTE-based V2X RAT.

Observation 4: It is hard to find a proper TDM ratio between NR and LTE resource pool in urban scenario.

Figure 1 PRR of LTE and NR UE in scenario 1

Figure 2 PRR of LTE and NR UE in scenario 2

Figure 3 PRR of LTE and NR UE in scenario 3

Conclusion

In this contribution, we discussed the motivation and solutions of co-channel coexistence for LTE V2X and NR V2X, we have following observations and proposals:

Observation 1: The co-channel coexistence for NR V2X and LTE V2X is essential due to the limited ITS spectrum and different types/versions of V2X systems.

Observation 2: It is expected that the NR-based and LTE-based V2X systems will coexist during the technology transition period.

Observation 3: There is no much difference for the PRR of LTE UE or NR UE when dynamic sharing and semi-static configuration are applied in freeway scenario.

Observation 4: It is hard to find a proper TDM ratio between NR and LTE resource pool in urban scenario.

Proposal 1: Both semi-static partitioning of resources and dynamic sharing of resources between LTE V2X and NR V2X should be considered.

Proposal 2: NR module performs resource exclusion based on the SL grant determined by LTE module to address the in-device coexistence issue in the same frequency channel.

References

- RP-220300, “WID revision: NR sidelink evolution”, RAN #95e

Appendix

Table 1 Simulation assumption for scenario 1

|  | NR UE | LTE UE |
| --- | --- | --- |
| Layout and UE dropping | Freeway Option A LTE UE:50% NR UE:50% |  |
| Solution for Inter-RAT interference | Case A: NR and LTE UE co-exist in a resource pool dynamically Case B: NR and LTE UE are configured with TDM resource pool separately (slots for NR: slots for LTE = 4:1). |  |
| Traffic model | Periodic traffic Model 2 specified in 37.885 with following changes: -Inter-packet arrival time: 50 ms -Packet size: 1200 bytes with probability of 0.2 and 800 bytes with probability of 0.8 -Latency requirement: 50 ms | Periodic traffic Model 1 specified in 37.885: -Inter-packet arrival time: 100 ms -Packet size: Pattern of {300 bytes, 190 bytes, 190 bytes, 190 bytes, 190 bytes} with random starting point for each UE -Latency requirement: 100 ms |
| Cast type | Broadcast (to NR UE only) | Broadcast (to LTE and NR UE) |
| Resource selection | R16 mode 2 full sensing (includes re-evaluation) | R14 mode 4 full sensing (includes backward indication and RSSI ranking) |
| Subchannel size | 20PRB |  |
| SCS | 15KHz |  |
| Bandwidth | 20MHz |  |
| Carrier frequency | 6GHz |  |
| Channel Model | TR 37.885 V2V Channel Model |  |
| Retransmission | 1 |  |

Table 2 Simulation assumption for scenario 2

|  | NR UE | LTE UE |
| --- | --- | --- |
| Layout and UE dropping | Freeway Option A LTE UE:50% NR UE:50% |  |
| Solution for Inter-RAT interference | Case A: NR and LTE UE co-exist in a resource pool dynamically Case B: NR and LTE UE are configured with TDM resource pool separately (slots for NR: slots for LTE = 4:1). |  |
| Traffic model | Aperiodic traffic Model 1 specified in 37.885 with following changes: -Inter-packet arrival time: 15 ms + an exponential random variable with the mean of 15 ms -Packet size: Uniformly random in the range between 200 bytes and 2000 bytes with the quantization step of 200 bytes -Latency requirement: 15 ms | Periodic traffic Model 1 specified in 37.885: -Inter-packet arrival time: 100 ms -Packet size: Pattern of {300 bytes, 190 bytes, 190 bytes, 190 bytes, 190 bytes} with random starting point for each UE -Latency requirement: 100 ms |
| Cast type | Broadcast (to NR UE only) | Broadcast (to LTE and NR UE) |
| Resource selection | R16 mode 2 full sensing (includes re-evaluation) | R14 mode 4 full sensing (includes backward indication and RSSI ranking) |
| Subchannel size | 20PRB |  |
| SCS | 15KHz |  |
| Bandwidth | 20MHz |  |
| Carrier frequency | 6GHz |  |
| Channel Model | TR 37.885 V2V Channel Model |  |
| Retransmission | 1 |  |

Table 3 Simulation assumption for scenario 3

|  | NR UE | LTE UE |
| --- | --- | --- |
| Layout and UE dropping | Urban Option A LTE UE:50% NR UE:50% |  |
| Solution for Inter-RAT interference | Case A: NR and LTE UE co-exist in a resource pool dynamically Case B: NR and LTE UE are configured with TDM resource pool separately (slots for NR: slots for LTE = 9:1 or 8:2 or 7:3 or 6:4 or 5:5). |  |
| Traffic model | Periodic traffic Model 2 specified in 37.885 with following changes: -Inter-packet arrival time: 50 ms -Packet size: 1200 bytes with probability of 0.2 and 800 bytes with probability of 0.8 -Latency requirement: 50 ms | Periodic traffic Model 1 specified in 37.885: -Inter-packet arrival time: 100 ms -Packet size: Pattern of {300 bytes, 190 bytes, 190 bytes, 190 bytes, 190 bytes} with random starting point for each UE -Latency requirement: 100 ms |
| Cast type | Broadcast (to NR UE only) | Broadcast (to LTE and NR UE) |
| Resource selection | R16 mode 2 full sensing (includes re-evaluation) | R14 mode 4 full sensing (includes backward indication and RSSI ranking) |
| Subchannel size | 20PRB |  |
| SCS | 15KHz |  |
| Bandwidth | 20MHz |  |
| Carrier frequency | 6GHz |  |
| Channel Model | TR 37.885 V2V Channel Model |  |
| Retransmission | 1 |  |