3GPP TSG RAN WG1 #109-e		                                             R1-2203992

e-Meeting, May 9th – 20th, 2022

Item:	8.6.2

Source: 	OPPO

Title:	Other remaining issues for Reduced Capability NR Devices

Document for:	Discussion & Decision

Introduction

In the previous discussions, it considers RedCap UE should support coverage enhancement basic functionality.

To make those function complete, RAN1 has made the following agreements related to HD-FDD UE with repetition in coverage enhancement agenda.

Agreement - For HD-FDD RedCap UEs configured with DMRS bundling, an event is constituted for a case where a dropping or cancellation of a PUSCH transmission according dropping rules in [17.2, TS 38.213]. Agreement - For HD-FDD RedCap UEs configured with DMRS bundling, an event is constituted for a case where the gap between two consecutive PUSCH transmissions overlaps with any symbol of downlink reception or downlink monitoring even if neither of the repetitions overlaps with it. Agreement The Rel-17 collision rules among SSB and Msg3 PUSCH transmission for HD-FDD UEs, which is subject to the discussion on collision handling for Msg3 transmission in AI 8.6.1.2, can be reused for Msg3 PUSCH repetition for HD-FDD UEs.

We propose to fully cover HD-FDD RedCap UE to operate in different cases. In this contribution, we further analyze related issues and propose text change on the specifications.

Supporting HD-FDD in initial access

RAN1 had the specification change of 17.2, 38.213 in the last meeting:

“If a HD-UE would transmit a PRACH based on a detected DCI format, or PUSCH, or PUCCH, or PRACH, or SRS based on a detected DCI format and the HD-UE is indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon in a set of symbols, the HD-UE does not transmit PUSCH or PUCCH or PRACH if a transmission would overlap with any symbol from the set of symbols and the HD-UE does not transmit SRS in the set of symbols.”

The reason for making the change is to apply collision handling for PUSCH/PUCCH/SRS triggered by either DCI or higher layer configuration. In general, it is OK. However, the text also covers the case of HD-FDD RedCap UE with the Msg3 repetition, which will mean the HD-FDD UE should avoid transmission of in a repetition if there is overlapping with the SSB.

We have raised the related issues in the previous discussions that the gNB may assume the initially accessed RedCap UE (by Msg1 identification) will always be the HD-FDD, to ensure the same understanding of exact slots be used for repetition in gNB and UE.

However, the current specification goes to different direction, e.g. HD-FDD UE should assume all the slot are available in the initial access.

That direction already been reflected in the 6.1.2.1, 38.214:

“ For paired spectrum and SUL band:

-	The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2.

-	For the case of a reduced capability half-duplex UE, and when AvailableSlotCounting is enabled, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

-	The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, based on the TDRA information field value in the RAR UL grant.

-	The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, based on the TDRA information field value in the DCI scheduling the PUSCH.

”

We are also fine with the direction. This scheme is feasible since HD-FDD UE can only transmit PUSCH repetition regardless of other downlink channel overlapped, during the initial access stage. But the text in 17.2, 38.213 have to be updated to avoid the conflict of behaviors. The different is due to the cited text in 38.213 also cover the Msg3 PUSCH, and then it means the Msg3 also need to not transmit when overlapped with SSB.

Note, the subclause 17.2 of 213 have to be determined before 6.1.2.1 of 214 as specified. One straightforward change is following. Without the change, the 17.2 will let the collided slot punctured, which is inconsistent to the text of 38.214.

------------------------------ TP#1: TS 38.213 ----------------------------------- ## 17.2 Half-Duplex UE in paired spectrum *** Unchanged text is omitted *** If a HD-UE would transmit a PRACH based on a detected DCI format, or PUSCH except scheduled by RAR UL grant, or PUCCH, or SRS and the HD-UE is indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon in a set of symbols, the HD-UE does not transmit PUSCH or PUCCH or PRACH if a transmission would overlap with any symbol from the set of symbols and the HD-UE does not transmit SRS in the set of symbols. *** Unchanged text is omitted ***

HD-FDD RedCap supporting repetition.

We have agreed that the collision handling for Msg3 transmission in AI 8.6.1.2 can be generally applied for the HD-FDD. This decision was made in the coverage enhancement session. In the specification, it reflected as in case of “AvailableSlotCounting is enabled”. There is no decision on how to handle in the case “AvailableSlotCounting is disabled”, e.g. Rel-16 Type A repetition is activated. To make the specification complete, 2 possible options can be considered.

Option 1, UE capability of 28-3 HD-FDD can only support Rel-17 Type A PUSCH repetition based on available slot.

Option 2, HD-FDD UE can drop the slot which colliding with SSB and still count for PUSCH repetition.

Considering Rel-16/15 normal capability should be allowed for HD-FDD UE, we prefer the Option2 and suggest following TP.

It is not clear either agenda of RedCap or coverage enhancement should treat the issue. But since this is not related to the Rel-17 coverage enhancement, we prefer to discuss the issue in RedCap.

------------------------------ TP#2: TS 38.214 ----------------------------------- - 6.1.2.1 Resource allocation in time domain *** Unchanged text is omitted *** For paired spectrum and SUL band: - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. - For the case of a reduced capability half-duplex UE, and when AvailableSlotCounting is enabled, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. If AvailableSlotCounting is enabled, a A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. Otherwise, the slot is dropped but counted. *** Unchanged text is omitted ***

Conclusion

In this contribution, we provide our views on open issues for HD-FDD operation and other cross function. We make the following proposals.

Proposal 1: HD-FDD UE should assume all the slot are available for Msg3 repetition in the initial access.

Proposal 2: HD-FDD UE can drop the slot which colliding with SSB and still count for PUSCH repetition.

The text proposals are given in above sections.

Reference

[1] Chairman’s notes for RAN1#108-e

[2] 3GPP TS 38.213, v17.1.0, ‘Physical layer procedures for control’

[2] 3GPP TS 38.214, v17.1.0, ‘Physical layer procedures for data’