3GPP TSG RAN WG1 #109-e		R1-2203996

e-Meeting, May 9th – May 20th, 2022

Agenda item:	9.6.2

Source: 	OPPO

Title:	Simulation and evaluation for RedCap enhancement

Document for:	Discussion

Introduction

In RAN #94 meeting, a new SID on Study on further NR RedCap UE complexity reduction has been approved [1]. The objective is to further reduce the complexity of RedCap devices:

- Study further UE complexity reduction techniques based on Rel-17 evaluation methodology in TR 38.875 [RAN1] - Consider network impact, coexistence of Rel-17 and Rel-18 RedCap and non-RedCap UEs in a cell, UE impact, specification impact - Potential solutions, which may complement each other, for reducing device complexity are focusing on: - UE bandwidth reduction to 5MHz in FR1, - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - reduced UE peak data rate in FR1, - Possibly including restricted bandwidth for PDSCH and/or PUSCH - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - Notes: - Rel-15 SSB should be reused and L1 changes minimized. - Operation in BWP with/without SSB and without/with RF retuning should be considered. - It is not precluded that some solutions for FR1 can be applied to FR2 in WI stage. - Aim to define a single Rel-18 RedCap UE type for further UE complexity reduction.

In this contribution, simulation scenarios and assumptions are discussed in detail.

Discussions

In the earlier RedCap study, several enhancement areas are identified to be beneficial for lower capability NR devices.  For maximum UE bandwidth, only one bandwidth per FR was accepted for the first release of RedCap devices. Maximum bandwidth of an FR1 RedCap UE during and after initial access is 20MHz. Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100MHz.

In Rel-18, the target of bandwidth reduction of lower capability UE is 5MHz. For the Rel-17 and non-RedCap UEs, the CORESET#0 support flexible configuration by SIB1, it may wider than 5MHz. When the subcarrier spacing is 30kHz, the bandwidth of SSB is also wider than 5MHz.

Simulation scenarios

For Rel-18 RedCap UE, there would be 2 possible subcarrier spacing configuration to be supported, i.e., 15kHz and 30kHz.

15kHz subcarrier spacing.

For FR1, the SSB bandwidth is 20PRBs and 3.6MHz for 15kHz SCS. The target of Rel-18 RedCap UE bandwidth could cover the SSB during the initial access. The CORESET#0 support some flexible configuration by SIB1. The maximum bandwidth of Rel-18 RedCap UE is 5MHz, it will limit CORESET#0 with the minimal 24 PRBs for Rel-18 RedCap UE.

Figure1: 15kHz subcarrier spacing for operating in 5MHz bandwidth - CORESET#0 with 24PRBs

Observation 1: If subcarrier spacing is 15kHz, it will limit CORESET#0 with the minimal 24 PRBs for Rel-18 RedCap UE.

For the CORESET#0, bandwidth of 5 MHz can provide maximum control resource as 12 CCEs, if the 24 PRBs and 3 symbols are used. It is not sufficient for supporting AL 16 for that CORESET#0 configuration. Even for AL 8, it is also difficult to select proper resources since the CORESET occupied all the carrier bandwidth. Then, the AL4 generally means the coverage loss will be 3.0-6.0 dB. For the NR network camping RedCap and non-RedCap UE, the optimal design would be a CORESET size of >= 48 PRB @ 15 kHz SCS.

Observation 2: If subcarrier spacing is 15kHz, the coverage performance of Rel-17 RedCap, Rel-18 RedCap and non-RedCap UEs is difference. The coverage performance needs to be evaluated for the Rel-18 RedCap with minimal 24 PRBs CORESET#0.

A network should consider coexistence among non-RedCap, Rel-17 RedCap and Rel-18 RedCap UEs. For the carrier without coverage issue, it can configure narrow CORESET#0 with 24PRBs, which also allow RedCap and non-RedCap UE to access. For carriers need larger CORESET#0 only can be accessed by non-RedCap or Rel-17 RedCap UE, a dedicated CORESET#0 can be configured for 5MHz RedCap UE. That dedicated CORESET#0 will have lower overhead as the bandwidth is much smaller than legacy one.

Observation 3: If a dedicated CORESET#0 can be configured for 5MHz RedCap UE. That dedicated CORESET#0 will have lower overhead as the bandwidth is much smaller than legacy one.

30kHz subcarrier spacing.

For FR1, the SSB bandwidth is 20PRBs and 7.2MHz for 30kHz SCS, it is wider than the maximum bandwidth of Rel-18 RedCap UE. The subcarrier number of PSS and SSS is 127, thus bandwidth of 5 MHz could only cover the PSS and SSS while not cover the whole SSB during the initial access. If Rel-18 RedCap UE could support 30kHz SCS, PBCH in SSB need to be punctured from 20 to 12 PRBs. At the same time, SIB1 also need to be punctured to 12PRBs.

Figure2: 30kHz subcarrier spacing for operating in 5MHz bandwidth – PBCH and SIB1 are punctured

Observation 4: If subcarrier spacing is 30kHz, limited by the bandwidth of 5MHz, PBCH in SSB and SIB1 need to be punctured to 12 PRBs for Rel-18 RedCap UE.

Considering the different solutions between 15kHz and 30kHz subcarrier spacing, we suggest evaluating coverage performance of the following scenarios:

Scenario 1: Legacy CORESET#0 with 24PRBs for Rel-18 RedCap UEs in 5MHz bandwidth, 15kHz subcarrier spacing;

Scenario 2: Dedicated CORESET#0 with 24PRBs for Rel-18 RedCap UEs in 5MHz bandwidth, 15kHz subcarrier spacing;

Scenario 3: PBCH punctured to 12 PRBs for Rel-18 RedCap UEs in 5MHz bandwidth, 30kHz subcarrier spacing;

Scenario 4: Legacy CORESET#0 punctured to 12 PRBs for Rel-18 RedCap UEs in 5MHz bandwidth, 30kHz subcarrier spacing;

Scenario 5: Dedicated CORESET#0 with 12PRBs for Rel-18 RedCap UEs in 5MHz bandwidth, 30kHz subcarrier spacing;

Proposal 1: suggest evaluating coverage performance of the following scenarios:

- Scenario 1: Legacy CORESET#0 with 24PRBs for Rel-18 RedCap UEs in 5MHz bandwidth, 15kHz subcarrier spacing;

- Scenario 2: Dedicated CORESET#0 with 24PRBs for Rel-18 RedCap UEs in 5MHz bandwidth, 15kHz subcarrier spacing;

- Scenario 3: PBCH punctured to 12 PRBs for Rel-18 RedCap UEs in 5MHz bandwidth, 30kHz subcarrier spacing;

- Scenario 4: Legacy CORESET#0 punctured to 12 PRBs for Rel-18 RedCap UEs in 5MHz bandwidth, 30kHz subcarrier spacing;

- Scenario 5: Dedicated CORESET#0 with 12PRBs for Rel-18 RedCap UEs in 5MHz bandwidth, 30kHz subcarrier spacing;

Simulation assumptions

In this section, we suggest RAN1 to consider the detailed link level simulation assumptions as in the Table1.

Table 1: Simulation assumption for PBCH

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 700MHz/2.6GHz/4GHz |
| UE speed | 3 km/h |
| UE bandwidth | 5MHz |
| # gNB Tx chains | 2 |
| # UE Rx chains | 1,2 |
| Subcarrier spacing | 15kHz / 30kHz |
| PBCH payload (excluding 24bits CRC) | 32 bits |
| Number of symbols for CORESET | 2 |
| Modulation | QPSK |
| Channel coding | Polar code (DCI) |
| Transmission scheme | 1-port precoder cycling |
| Channel model | TDL-C (delay spread: 300ns) |
| Channel estimation | Practical |
| Receiver type | MMSE |
| BLER target | 1% |
| Number of interfering TRPs | 0 TRP |
| Combined number | 4 SSBs |
| Frequency Offset | Initial acquisition TRP: uniform distribution +/- 0.05 ppm UE: uniform distribution +/- 5, 10, 20 ppm Non-initial acquisition TRP: uniform distribution +/- 0.05 ppm UE: uniform distribution +/- 0.1 ppm |

Table 2: Simulation assumption for PDCCH

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 700MHz/2.6GHz/4GHz |
| UE speed | 3 km/h |
| UE bandwidth | 5MHz |
| # gNB Tx chains | 2 |
| # UE Rx chains | 1,2 |
| Subcarrier spacing | 15kHz / 30kHz |
| Number of RBs | 24 for 15kHz / 12 for 30kHz |
| Payload size | 24 bits |
| Number of symbols | 2 |
| Modulation | QPSK |
| Channel coding | Polar code |
| Aggregation level | 4 |
| Channel model | TDL-C (delay spread: 300ns) |
| BLER target | 1% |

Table 3: Simulation assumption for PDSCH (SIB1)

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 700MHz/2.6GHz/4GHz |
| UE speed | 3 km/h |
| UE bandwidth | 5MHz |
| # gNB Tx chains | 2 |
| # UE Rx chains | 1,2 |
| Subcarrier spacing | 15kHz / 30kHz |
| PRBs | 24 for 15kHz / 12 for 30kHz |
| MCS | 0 |
| Duration | 12 OS |
| Channel model | TDL-C (delay spread: 300ns) |
| Channel estimation | Practical |
| Receiver type | MMSE |
| BLER target | 10% |
| DMRS configuration | Type I, 2 DMRS symbol, no multiplexing with data |

Conclusions

In this contribution, simulation scenarios and assumptions are discussed. As summary, we have the following proposals.

Observation 1: If subcarrier spacing is 15kHz, it will limit CORESET#0 with the minimal 24 PRBs for Rel-18 RedCap UE.

Observation 2: If subcarrier spacing is 15kHz, the coverage performance of Rel-17 RedCap, Rel-18 RedCap and non-RedCap UEs is difference. The coverage performance needs to be evaluated for the Rel-18 RedCap with minimal 24 PRBs CORESET#0.

Observation 3: If a dedicated CORESET#0 can be configured for 5MHz RedCap UE. That dedicated CORESET#0 will have lower overhead as the bandwidth is much smaller than legacy one.

Observation 4: If subcarrier spacing is 30kHz, limited by the bandwidth of 5MHz, PBCH in SSB and SIB1 need to be punctured to 12 PRBs for Rel-18 RedCap UE.

Proposal 1: Suggest evaluating coverage performance of the following scenarios:

- Scenario 1: Legacy CORESET#0 with 24PRBs for Rel-18 RedCap UEs in 5MHz bandwidth, 15kHz subcarrier spacing;

- Scenario 2: Dedicated CORESET#0 with 24PRBs for Rel-18 RedCap UEs in 5MHz bandwidth, 15kHz subcarrier spacing;

- Scenario 3: PBCH punctured to 12 PRBs for Rel-18 RedCap UEs in 5MHz bandwidth, 30kHz subcarrier spacing;

- Scenario 4: Legacy CORESET#0 punctured to 12 PRBs for Rel-18 RedCap UEs in 5MHz bandwidth, 30kHz subcarrier spacing;

- Scenario 5: Dedicated CORESET#0 with 12PRBs for Rel-18 RedCap UEs in 5MHz bandwidth, 30kHz subcarrier spacing;

Reference

- RP-213661, “New SID on Study on further NR RedCap UE complexity reduction”, Ericsson.