3GPP TSG RAN WG1 #108-e							R1-2203997

e-Meeting, May 9th – May 20th, 2022

Agenda item:	8.16.7

Source: 	OPPO

Title:	Rel-17 UE Power Saving features

Document for:	Discussion & Decision

Introduction

In RAN1#108-e, the UE feature of UE power saving enhancements for NR was discussed. Including Feature Groups 29-1 Paging Enhancement, 29-2 TRS resources for IDLE/Inactive UEs and 29-3 PDCCH monitoring adaptation within an active BWP and the following agreement was made [1].

Agreement

- FG 29-2 is updated as follows

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability | FFS | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode UE cannot receive TRS resources for idle/inactive mode | Per UE N/A | N/A | N/A | N/A | Receiving L1 indication via DCI format 2_7 is supported only if the UE supports receiving DCI format 2_7 | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- The type of FG 29-1 is per band

Agreement

- The type of FGs 29-3a to 29-3d is per band

Agreement

FG 29-1 is updated as follows

| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support receiving paging early indication in DCI format 2_7 2. Support receiving UE subgroup indication in DCI format 2_7 [3. For type 2A CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 or 2A CSS configurations within a single span of three consecutive OFDM symbols within a slot] |  | Yes |  | UE does not support paging enhancement | Per band | N/A | N/A | N/A | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication Leave RAN2 to decide whether ‘optional with capability signalling’ or ‘optional without capability signalling’ Leave RAN2 to decide whether Need for the gNB to know if the feature is supported is Yes or No | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- Revise component of 29-3a as: Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured

- Revise component of FG 29-3b as: Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based SSSG switching, without PDCCH skipping if PDCCHSkippingDurationList is not configured

- Revise component of FG 29-3c as: Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based SSSG switching, without PDCCH skipping if PDCCHSkippingDurationList is not configured

- Revise component of FG 29-3d as: Support of 2-bit indication of SSSG switching between 2 SSSGs, with PDCCH skipping by scheduling DCI, and timer based SSSG switching

In this contribution, we share our views on NR Rel-17 UE Power Saving features.

UE Features of UE Power Saving Enhancements for NR

- UE feature for paging enhancement (UE feature 29-1)

FG 29-1 is captured as below.

| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support receiving paging early indication in DCI format 2_7 2. Support receiving UE subgroup indication in DCI format 2_7 [3. For type 2A CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 or 2A CSS configurations within a single span of three consecutive OFDM symbols within a slot] |  | Yes |  | UE does not support paging enhancement | Per band | N/A | N/A | N/A | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication Leave RAN2 to decide whether ‘optional with capability signalling’ or ‘optional without capability signalling’ Leave RAN2 to decide whether Need for the gNB to know if the feature is supported is Yes or No | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

During last meeting, component 3 was discussed and kept with square brackets. In our views, it seems no difference with existing monitoring behavior defined by FG3-1 for CSS. We don’t see the need of including component 3 in the description.

Proposal 1: Prefer not including component 3 in 29-1.

- TRS resources for idle/inactive UEs (UE feature 29-2)

FG 29-2 is captured as below.

| Features | Index | Feature group | Components |  |  |  | Consequence if the feature is not supported by the UE | Type |  |  |  | Note |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability |  | N |  | UE cannot receive TRS resources for idle/inactive mode | N/A | N/A | N/A | N/A | Receiving L1 indication via DCI format 2_7 is supported only if the UE supports receiving DCI format 2_7 | Optional without capability signalling |

If the UE supports 29-2 but not 29-1, then UE can only read TRS availability from paging DCI. If UE supports 29-2 and 29-1 then UE can used PEI based indication for TRS availability. We are fine with current text of component 2 and the note. Component 2 will be ambiguous if without the note.

Proposal 2: Fine with the current components. Note in FG 29-2 is necessary.

Conclusions

As summary, we have the following proposals:

Proposal 1: Prefer not including component 3 in 29-1.

Proposal 2: Fine with the current components. Note in FG 29-2 is necessary.

Reference

- R1-2201516 Summary on UE features for UE power saving enhancements	Moderator