3GPP TSG RAN WG1 #109-e	R1-2204001

e-Meeting, May 9th – 20th, 2022

Source:	OPPO

Title:	Clarification on collision handling for overlapping PUCCHs with repetitions

Agenda Item:	7.2.5

Document for:	Discussion and Decision

Introduction

In RAN1 #108e, the ambiguity issue when resolving collision of overlapping PUCCHs with repetitions was raised, and in this contribution, we elaborate our views on the above issue.

Discussion

In last RAN1 meeting, Qualcomm brought the case in Figure 1 [1] to clarify handling order for resolving collision of overlapping PUCCHs with repetitions. Based on the following specification text in TS 38.213 [2], it can be understood that for resolving collision of overlapping PUCCHs of same priority, UE first resolve collision of PUCCHs with repetitions and after that, UE resolves collision of PUCCHs without repetitions. When handling collision of overlapping PUCCHs with repetitions, UE would regard the PUCCH with repetitions as the “first PUCCH” and other PUCCH(s) overlapping with the “first PUCCH” can be classified as “at least a second PUCCH”. Then according to the above understanding, in the case show in Figure 1, in slot 0, SR 1 can be regarded as the “first PUCCH”, and CSI/SR 2/A/N can be regarded as “at least a second PUCCH” since they are all overlapping with SR 1, then UE will transmit A/N since it is the highest priority (with the assumption of the starting slot of SR 1 is earlier than that of SR 2). So to our understanding, there is no ambiguity in the case shown in Figure 1.

Figure 1: An example of overlapping PUCCHs with repetitions

Observation 1: When resolving collision of overlapping PUCCHs with repetitions, the PUCCH with repetitions is regarded as the “first PUCCH”, the PUCCHs overlapping with the “first PUCCH” are regarded as “at least a second PUCCH”. Among the “first PUCCH” and “at least a second PUCCH”, the PUCCH with highest UCI priority is transmitted.

9.2.5 UE procedure for reporting multiple UCI types This clause is applicable to the case that a UE has resources for PUCCH transmissions or for PUCCH and PUSCH transmissions that overlap in time and each PUCCH transmission is over a single slot without repetitions. Any case that a PUCCH transmission is with repetitions over multiple slots is described in clause 9.2.6. …… 9.2.6 PUCCH repetition procedure A UE does not multiplex different UCI types in a PUCCH transmission with repetitions over NPUCCHrepeat>1 slots. If a UE would transmit a first PUCCH over more than one slot and at least a second PUCCH over one or more slots, and the transmissions of the first PUCCH and the second PUCCH would overlap in a number of slots then, for each slot of the number of slots and with UCI type priority of HARQ-ACK > SR > CSI with higher priority > CSI with lower priority - the UE does not expect the first PUCCH and any of the second PUCCHs to start at a same slot and include a UCI type with same priority - if the first PUCCH and any of the second PUCCHs include a UCI type with same priority, the UE transmits the PUCCH starting at an earlier slot and does not transmit the PUCCH starting at a later slot - if the first PUCCH and any of the second PUCCHs do not include a UCI type with same priority, the UE transmits the PUCCH that includes the UCI type with higher priority and does not transmit the PUCCH that include the UCI type with lower priority

Although the current specification works well when there is only one PUCCH with repetitions in a slot, there exists ambiguity if there are more than one PUCCH with repetitions in a slot. Take Figure 2 as an example, there are 3 overlapping PUCCHs in slot 1, A/N 1 and A/N 2 are with repetitions while SR or CSI is without repetitions, we see two possible interpretations of UE behavior in slot 1 (the staring slot of A/N 1 is slot 1 while the starting slot of A/N 2 is slot 0):

- Interpretation 1: If UE firstly regards A/N 2 with repetitions as the “first PUCCH”, then only A/N 1 would be the “at least a second PUCCH”, since the staring slot of A/N 2 is earlier than that of A/N 1, then A/N 1 is dropped. After that, there is no collision between SR or CSI and A/N 2 with repetitions, so both SR or CSI and A/N 2 would be transmitted by UE.

- Interpretation 2: If UE firstly regards A/N 1 with repetitions as the “first PUCCH”, then both CSI or SR and A/N 2 would be the “at least a second PUCCH”, in such a case, both SR or CSI and A/N 1 are dropped. UE would only transmit A/N 2.

Figure 2: Example of overlapping PUCCHs with repetitions in a slot with more than one PUCCH with repetitions

Figure 2 is just an example case where ambiguity exits, but not the only example. The ambiguity is enlarged when repetition of PUCCH format 0/2 is introduced in Rel-17 URLLC/IIoT. As discussed in our companion document [3], the ambiguity also exists in the case shown in Figure 3 and more details can be found in [3]. Therefore, we think it is needed for RAN1 to clarify the intended UE behavior for resolving collision of PUCCHs with repetitions in a slot with more than one PUCCH with repetitions.

Figure 3: Another example of overlapping PUCCHs with repetitions in a slot with more than one PUCCH with repetitions

Proposal 1: RAN1 to clarify the intended UE behavior for resolving collision of PUCCHs with repetitions in a slot with more than one PUCCH with repetitions.

Conclusion

In this contribution, we discuss the ambiguity issue when resolving collision of overlapping PUCCHs with repetitions, and the following observation and proposal are made:

Observation 1: When resolving collision of overlapping PUCCHs with repetitions, the PUCCH with repetitions is regarded as the “first PUCCH”, the PUCCHs overlapping with the “first PUCCH” are regarded as “at least a second PUCCH”. Among the “first PUCCH” and “at least a second PUCCH”, the PUCCH with highest UCI priority is transmitted.

Proposal 1: RAN1 to clarify the intended UE behavior for resolving collision of PUCCHs with repetitions in a slot with more than one PUCCH with repetitions.

Reference

- R1-2202770 Summary #3 of email thread [108-e-R17-IIoT-URLLC-03] Moderator (CATT)

- TS 38.213 v16.9.0 Physical layer procedures for control (Release 16)

- R1-2204002 Remaining issues on HARQ-ACK enhancements OPPO