3GPP TSG RAN WG1 #109-e 	                                                         	   R1-2204003

e-Meeting, May 9th – 20th, 2022

Source:	OPPO

Title:	Enhancements on intra-UE multiplexing/prioritization

Agenda Item:	8.3.2

Document for:	Discussion and Decision

Introduction

In RAN1 #107b-e, intra-UE multiplexing/prioritization for Rel-17 URLLC was agreed with following agreements:

Agreement

When a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 2/3/4 overlaps with a PUCCH carrying LP HARQ-ACK, information bits for K HP SRs are appended to HP HARQ-ACK bits, and treat them as HP UCI, where K (K≥1) PUCCHs semi-statically configured for K HP SRs overlap with the original PUCCH carrying the HP HARQ-ACK.

- The number of HP UCI bits is , same as Rel-15;

  - FFS: PF0, PF1

- Reuse other procedures for multiplexing of LP HARQ-ACK and HP HARQ-ACK on PUCCH resource with PF 2/3/4, i.e. separate coding, PRB determination, rate matching and power control.

- If the HP HARQ-ACK is a dynamic HARQ-ACK, a PUCCH resource indicated by PRI is used for multiplexing.

- If the HP HARQ-ACK is a SPS HARQ-ACK, a PUCCH resource determined from the PUCCH resource(s) provided by sps-PUCCH-AN-List is used for multiplexing.

In this contribution, we show our views on the remaining issues of Rel-17 intra-UE multiplexing/prioritization.

Discussion

The priority of the resultant PUCCH/PUSCH

In Rel-16, if PHY priority is configured for a UE, a PUSCH or a PUCCH is either of priority index 0 or of priority index 1. The priority of uplink channel is not only used for Rel-16 intra-UE prioritization, but also used for:

  - Inter-UE uplink cancellation;

  - Prioritizations for transmission power reductions; and

  - Prioritization between SL HARQ-ACK information in a PUCCH and DL HARQ-ACK or SR or CSI in a PUCCH.

In order to keep minimize impact on other procedures, we support that the priority of the resultant PUCCH/PUSCH for Rel-17 intra-UE multiplexing should be specified.

Proposal 1: The priority of the resultant PUCCH/PUSCH for Rel-17 intra-UE multiplexing should be specified.

Multiplexing of HP SR, HP HARQ-ACK with PF 0/1 and LP HARQ-ACK

If multiple HP SRs overlap with HP HARQ-ACK with PF 0/1, based on Rel-15 rules, at most 1 positive HP SR can be multiplexed with HP HARQ-ACK with PF0/1. If the resulted PUCCH after step 1 overlaps with PUCCH carrying LP HARQ-ACK, the following two options were proposed for intra-UE multiplexing step 2 in RAN1 #108e:

- Option 1: LP HARQ-ACK is dropped.

- Option 2: 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1.

We support Option 2, which aligns with previous agreement for PF2/3/4 case and reduces LP PUCCH dropping.

Proposal 2: When a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs, in step 2, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1.

Multiplexing of LP HARQ-ACK in PUSCH

For the scenarios where multiplexing low-priority HARQ-ACK onto high-priority PUSCH, the following three options are to be down-selected to clarify procedure on insufficient resource:

  - Option 1: In case of insufficient resource for LP HARQ-ACK, LP HARQ-ACK is entirely dropped

  - Option 2: LP HARQ-ACKs are mapped to the rest REs of the PUSCH based on the rate matching equation, if HP HARQ-ACK and/or HP CSI have been mapped in prior on the PUSCH.

  - Option 3: UE does not expect insufficient resource for multiplexing low-priority HARQ-ACK.

Among three options, we prefer to option 1 due to it is more aligned with Rel-15 rule that UCI is dropped when resource for UCI in PUSCH is insufficient. Option 2 sacrifices HP PUSCH reliability for invalid LP HARQ-ACK transmission. Option 3 leads scheduling complexity, especially when different scheduling timelines are applied for information with different priorities.

Proposal 3: In case of insufficient resource for LP HARQ-ACK, LP HARQ-ACK is entirely dropped.

Conclusions

In this contribution, we show our views on the enhancements on intra-UE multiplexing/prioritization with following proposals:

Proposal 1: The priority of the resultant PUCCH/PUSCH for Rel-17 intra-UE multiplexing should be specified.

Proposal 2: When a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs, in step 2, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1.

Proposal 3: In case of insufficient resource for LP HARQ-ACK, LP HARQ-ACK is entirely dropped.

References

- R1-2202770, “Summary #3 of email thread [108-e-R17-IIoT-URLLC-03]”, Moderator (CATT)

- R1-2202830, “Summary#3 of email thread [108-e-R17-IIoT-URLLC-04]”, Moderator (OPPO)