3GPP TSG RAN WG1 #109-e		R1-2204004

e-Meeting, May 9th – 20th, 2022

Source:	OPPO

Title:	Remaining issues of URLLC PDC

Agenda Item:	8.3.3

Document for:	Discussion and Decision

# Introduction

This contribution discusses one remaining issue in support of time synchronization by PDC:

- TP handling relating to QCL source of PDC-PRS.

# Discussions

RAN1 #107bis-e had the following conclusion:

Conclusion There is no consensus to introduce new RRC parameter “DL-PRS-PDC-QCL-Info” to specify the QCL indication with other DL reference signals, for DL PRS configuration for PDC.

There were two interpretations for the above conclusion:

- Interpretation #1: PDC-PRS has no (regardless of new or legacy) QCL source configuration at all.

- Interpretation #2: PDC-PRS has legacy (not new) QCL source configuration inherited from positioning PRS.

Our understanding for the RAN1 #107b-e discussion context is the Interpretation #2. Accordingly, the corresponding 38.214 TP should be updated as following:

## 9.1 PRS reception procedure for RTT-based propagation delay compensation <Unchanged part omitted> A DL PRS resource is defined by: - nr-DL-PRS-ResourceID determines the DL PRS resource configuration identity. All DL PRS resource IDs are locally defined within the DL PRS resource set. - dl-PRS-SequenceID is used to initialize cinit value used in pseudo random generator as described in Clause 7.4.1.7.2 of [4, TS 38.211] for generation of DL PRS sequence for a given DL PRS resource. - dl-PRS-CombSizeN-AndReOffset defines the comb size of a DL PRS resource, where the allowable values are given in Clause 7.4.1.7.3 of [TS38.211], and the starting RE offset of the first symbol within a DL PRS resource in frequency. The UE may expect the same comb size to be configured for all DL PRS resources of the PRS resource set. The relative RE offsets of the remaining symbols within a DL PRS resource are defined based on the initial offset and the rule described in Clause 7.4.1.7.3 of [4, TS 38.211]. - dl-PRS-ResourceSlotOffset determines the starting slot of the DL PRS resource with respect to corresponding DL PRS resource set slot offset. - dl-PRS-ResourceSymbolOffset determines the starting symbol of a slot configured with the DL PRS resource. - dl-PRS-QCL-Info defines any quasi co-location information of the DL PRS resource with other reference signals. The DL PRS may be configured with QCL 'typeD' with a DL PRS in the same PRS resource set, or with rs-Type set to 'typeC', 'typeD', or 'typeC-plus-typeD' with a SS/PBCH Block from a serving cell. <Unchanged part omitted>

There was comment in the last RAN1 meeting, suggesting to modify the wording of “with a SS/PBCH Block from a serving cell” to “with a SS/PBCH Block from PCell”. However, the dl-PRS-QCL-Info is defined to include cell-ID for SSB as QCL source, as in following RRC IE. Therefore, UE behavior should be defined to anyway read this cell-ID, instead of assuming PCell without reading the cell-ID field.  On the other hand, we are open to adding a sentence saying, e.g., “UE does not expect the serving cell is a cell other than PCell”.

DL-PRS-QCL-Info-r16 ::= CHOICE { ssb-r16 SEQUENCE { pci-r16 NR-PhysCellID-r16, ssb-Index-r16 INTEGER (0..63), rs-Type-r16 ENUMERATED {typeC, typeD, typeC-plus-typeD} }, dl-PRS-r16 SEQUENCE { qcl-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16, qcl-DL-PRS-ResourceSetID-r16 NR-DL-PRS-ResourceSetID-r16 } }

# Conclusions

Proposal 1: To adopt following 38.214 TP.

## 9.1 PRS reception procedure for RTT-based propagation delay compensation <Unchanged part omitted> A DL PRS resource is defined by: - nr-DL-PRS-ResourceID determines the DL PRS resource configuration identity. All DL PRS resource IDs are locally defined within the DL PRS resource set. - dl-PRS-SequenceID is used to initialize cinit value used in pseudo random generator as described in Clause 7.4.1.7.2 of [4, TS 38.211] for generation of DL PRS sequence for a given DL PRS resource. - dl-PRS-CombSizeN-AndReOffset defines the comb size of a DL PRS resource, where the allowable values are given in Clause 7.4.1.7.3 of [TS38.211], and the starting RE offset of the first symbol within a DL PRS resource in frequency. The UE may expect the same comb size to be configured for all DL PRS resources of the PRS resource set. The relative RE offsets of the remaining symbols within a DL PRS resource are defined based on the initial offset and the rule described in Clause 7.4.1.7.3 of [4, TS 38.211]. - dl-PRS-ResourceSlotOffset determines the starting slot of the DL PRS resource with respect to corresponding DL PRS resource set slot offset. - dl-PRS-ResourceSymbolOffset determines the starting symbol of a slot configured with the DL PRS resource. - dl-PRS-QCL-Info defines any quasi co-location information of the DL PRS resource with other reference signals. The DL PRS may be configured with QCL 'typeD' with a DL PRS in the same PRS resource set, or with rs-Type set to 'typeC', 'typeD', or 'typeC-plus-typeD' with a SS/PBCH Block from a serving cell. <Unchanged part omitted>

# References

- R1-2202961, 38.214 CR for Rel-17 URLLC/IIoT, Nokia