3GPP TSG RAN WG1 #109-e                                                                                             R1-2204006

e-Meeting, May 9th – 20th, 2022

Source:	OPPO

Title:	   Discussion on UE features for IIoT and URLLC

Agenda Item:	8.16.3

Document for:	Discussion and Decision

Introduction

After RAN1 #108-e meeting, the updated RAN1 UE features list for Rel-17 NR was provided in [1]. In this contribution, we further elaborate our views on UE features for Rel-17 IIoT and URLLC.

Discussion

FG 25-1: SPS HARQ-ACK deferral in case of TDD collision

Prerequisite feature groups: Regarding the prerequisite feature groups for FG 25-1, from our point of view, FG 5-18 (downlinkSPS) and FG 12-6 (SPS periodicity shorter than 10 ms) should be included since short SPS periodicity is the main motivation of SPS HARQ-ACK deferral. Otherwise, if the minimum SPS periodicity is 10 ms, gNB can indicate the HARQ-ACK transmission in the UL slot that well match TDD pattern. However, adding FG 12-2 (Multiple SPS configurations) is unnecessary since UE may choose to implement SPS HARQ-ACK defer feature without support of multiple SPS configurations.

Type: To align with the prerequisite feature groups, per UE is fine for FG 25-1.

Proposal 1: The type of FG 25-1 is per UE.

Proposal 2: The prerequisite feature groups for FG 25-1 include FG 5-18 (downlinkSPS) and FG 12-6 (SPS periodicity shorter than 10 ms).

FG 25-2 and FG 25-3/3a/3b: PUCCH Repetition enhancements

Prerequisite feature groups:

For FG 25-2, our preference is not to include FG 4-23 since the two feature groups are not relevant.

For FG 25-3a, it is preferred to remove FG 30-5 from the prerequisite feature groups since there is no dependency for these two features although the mechanism of dynamic indication of repetition number is the same.

For FG 25-3b, the prerequisite feature groups should include FG 25-3 for support of sub-slot PUCCH repetition.

Type: To align with their prerequisite feature groups, we support per UE for FG 25-2, per FS for FG 25-3/3a/3b.

Proposal 3: Remove FG 4-23 from the prerequisite feature groups for FG 25-2.

Proposal 4: Remove FG 30-5 from the prerequisite feature groups for FG 25-3a.

Proposal 5: The prerequisite feature groups for FG 25-3b includes FG 25-3.

Proposal 6: The type of FG 25-2 is per UE. The type of FG 25-3/3a/3b is per FS.

FG 25-4~25-7: HARQ-ACK retransmission

Type: To align with their prerequisite feature groups, we support per band for FG 25-4~25-6. For FG 25-7, we do not have strong views on its type.

Proposal 7: The type of FG 25-4~25-6 is per band.

FG 25-8: Semi-static HARQ-ACK codebook for sub-slot PUCCH

Type: To align with their prerequisite feature groups, we support per FS for FG 25-8.

Proposal 8: The type of FG 25-8 is per FS.

FG 25-9 and FG 25-10/10a: PUCCH cell switching

There is an FFS for FG 25-9 and 25-10/10a on “whether/how to indicate the capability for support of PUCCH cell switch in two PUCCH groups”. From our understanding, it is preferred not to support PUCCH cell switch in two PUCCH groups since this would introduce additional complexity for UE implementation.

Proposal 9: Do not support PUCCH cell switch in two PUCCH groups.

FG 25-16: Intra-UE multiplexing with different priorities

Regarding to the prerequisite feature groups for FGs 25-16, FG 11-3 should be changed to FG 11-4. In addition, we prefer to remove FG 12-1 since it is unnecessary to couple Rel-16 intra-UE prioritization capability with Rel-17 intra-UE multiplexing capability. UE may choose to implement FG 25-16, while not to implement 12-1. This is because for a UE capable of intra-UE multiplexing almost no longer needs to perform cancelation between UL channels with different priorities.

To align with the prerequisite feature groups, the type of FG 25-16 should be per FS.

In previous RAN1 meeting, the multiplexing mechanisms for the following scenarios were agreed: 1) LP HARQ-ACK and HP SR as component 2 in FG 25-16 described; 2) HP HARQ-ACK and HP SR and LP HARQ-ACK as component 3 in FG 25-16 described. So the square brackets of component 2 and component 3 can be removed.

Agreement When a PUCCH carrying HP SR with PF0/1 overlaps with a PUCCH carrying LP HARQ-ACK with PF2/3/4: - For positive SR, transmit SR on the SR PUCCH resource and drop HARQ-ACK. - For negative SR, transmit HARQ-ACK only on the HARQ-ACK PUCCH resource. Note: It was agreed to support multiplexing a LP HARQ-ACK and a HP SR into a PUCCH for some HARQ-ACK/SR PF combinations in Rel-17. Agreement When a PUCCH carrying HP SR with PF0/1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0/1, the LP HARQ ACK is dropped when colliding with positive SR Agreement When a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 2/3/4 overlaps with a PUCCH carrying LP HARQ-ACK, information bits for K HP SRs are appended to HP HARQ-ACK bits, and treat them as HP UCI, where K (K≥1) PUCCHs semi-statically configured for K HP SRs overlap with the original PUCCH carrying the HP HARQ-ACK. - The number of HP UCI bits is , same as Rel-15; - FFS: PF0, PF1 - Reuse other procedures for multiplexing of LP HARQ-ACK and HP HARQ-ACK on PUCCH resource with PF 2/3/4, i.e. separate coding, PRB determination, rate matching and power control. - If the HP HARQ-ACK is a dynamic HARQ-ACK, a PUCCH resource indicated by PRI is used for multiplexing. - If the HP HARQ-ACK is a SPS HARQ-ACK, a PUCCH resource determined from the PUCCH resource(s) provided by sps-PUCCH-AN-List is used for multiplexing.

Proposal 10: For the prerequisite feature groups of FG 25-16: change FG 11-3 to FG 11-4 and remove 12-1.

Proposal 11: The type of FG 25-16 is per FS.

Proposal 12: Remove the [] of component 2 and component 3 of FG 25-16.

FG 25-19a: RTT-based Propagation delay compensation based on DL PRS and SRS

This paper focuses on two UE feature issues related to PDC based on PRS and SRS. One issue is left from following RAN1 #108e agreement (with related part underlined), regarding to taking PRS as spatial relation RS of SRS:

Agreement

The following working assumption made in RAN1#107b-e is confirmed with modification in BLUE/RED:

Working Assumption Alt.1: Add new “spatialRelationInfo-PDC-r17” field to SRS-Resource to indicate the spatial relation between a reference RS and the target SRS, with spatialRelationInfo-PDC-r17 as below: spatialRelationInfo-PDC-r17 ::= SEQUENCE { referenceSignal CHOICE { ssb-Index SSB-Index, csi-RS-Index NZP-CSI-RS-ResourceId, dl-PRS-PDC nr-DL-PRS-ResourceID-r16 srs SEQUENCE { resourceId SRS-ResourceId, uplinkBWP BWP-Id } } } …… Note 4: Whether/how to update FG 25-19a can be further discussed in UE feature session, e.g. add either a new component under FG 25-19a or a new FG to enable UE capability reporting for the support of DL PRS for PDC as the spatial relation reference signal for periodic SRS for PDC.

It should be noted that the positioning framework listed the PRS-based spatial relation RS of SRS as a separate FG, i.e., FG 13-10b as defined in TS 38.822 (coped below):

| Features | Index | Feature group | Components | Prerequisite feature groups | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13-10 | Spatial relation for SRS for positioning based on SSB from the serving cell | 1. Spatial relation for SRS for positioning based on SSB from the serving cell in the same band | 13-8 | n/a | n/a (FR2 only) | Need for location server to know if the feature is supported. | Optional with capability signaling |
|  | 13-10a | Spatial relation for SRS for positioning based on CSI-RS from the serving cell | 1. Spatial relation for SRS for positioning based on CSI-RS from the serving cell in the same band | 13-10 | n/a | n/a (FR2 only) | Need for location server to know if the feature is supported. | Optional with capability signaling |
|  | 13-10b | Spatial relation for SRS for positioning based on PRS from the serving cell | 1. Spatial relation for SRS for positioning based on PRS from the serving cell in the same band | One of {13-2, 13-3, 13-4} and13-8 | n/a | n/a (FR2 only) | Need for location server to know if the feature is supported. | Optional with capability signaling |
|  | 13-10c | Spatial relation for SRS for positioning based on SRS | 1. Spatial relation for SRS for positioning based on SRS in the same band | 13-8, | n/a | n/a (FR2 only) | Need for location server to know if the feature is supported. | Optional with capability signaling |
|  | 13-10d | Spatial relation for SRS for positioning based on SSB from the neighbouring cell | 1. Spatial relation for SRS for positioning based on SSB from the neighbouring cell in the same band | 13-10 | n/a | n/a (FR2 only) | Need for location server to know if the feature is supported. | Optional with capability signaling |
|  | 13-10e | Spatial relation for SRS for positioning based on PRS from the neighbouring cell | 1. Spatial relation for SRS for positioning based on PRS from the neighbouring cell in the same band | 13-10b | n/a | n/a (FR2 only) | Need for location server to know if the feature is supported. | Optional with capability signaling |

If the PDC case does not list PRS-based spatial relation RS as either a separate FG or a feature component of FG 25-19a, there would be certain inconsistent implementation between PDC case and positioning case, which is not the design assumption in PDC discussion:

- In PDC case: UE supporting {PRS+SRS}-based RTT-PDC must support PRS as spatial relation RS of SRS;

- In positioning case: UE supporting {PRS+SRS}-based RTT-positioning may or may not support PRS as spatial relation RS of SRS.

In order to keep the consistent UE feature design logic for using PRS as spatial relation RS of SRS in positioning, we prefer to add a new FG instead of adding a feature component.

In addition, as pointed in existing FG 13-10 series, the FG of spatial relation RS of SRS is applicable to FR2 only. This should be also reflected in PDC case.

Proposal 13: To add a new FG to reflect support of PRS as spatial relation RS of SRS.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 25-21 | Spatial relation for SRS for PDC based on PRS from the serving cell | 1. Spatial relation for SRS for PDC based on PRS from the serving cell | 25-19a | n/a | n/a (FR2 only) |  | Optional with capability signaling |

Note that the cases with spatial relation RS based on SSB/CSI-RS/SRS can simply follow the existing specification outside of positioning framework. The granularity of new FG25-21 can be the same as that of FG 25-19a.

The second issue under FG25-19a is also a leftover from previous meetings, regarding to whether to add necessary components and corresponding notes from FG 13-1 to FG 25-19a or to add FG 13-1 as a prerequisite FG for FG 25-19a (requiring FG 13-1 to be reported to gNB). In our view, because some of feature components (such as buffering) is not needed for PDC, it is more reasonable to pick some of FG 13-1 components and add them to FG 25-19a, instead of making the whole FG 13-1 as the prerequisite of FG 25-19a.

Proposal 14: To add necessary components and corresponding notes from FG 13-1 to FG 25-19a.

Conclusion

In this contribution, we further discuss the UE features for Rel-17 IIoT and URLLC and the following proposals are made:

Proposal 1: The type of FG 25-1 is per UE.

Proposal 2: The prerequisite feature groups for FG 25-1 include FG 5-18 (downlinkSPS) and FG 12-6 (SPS periodicity shorter than 10 ms).

Proposal 3: Remove FG 4-23 from the prerequisite feature groups for FG 25-2.

Proposal 4: Remove FG 30-5 from the prerequisite feature groups for FG 25-3a.

Proposal 5: The prerequisite feature groups for FG 25-3b includes FG 25-3.

Proposal 6: The type of FG 25-2 is per UE. The type of FG 25-3/3a/3b is per FS.

Proposal 7: The type of FG 25-4~25-6 is per band.

Proposal 8: The type of FG 25-8 is per FS.

Proposal 9: Do not support PUCCH cell switch in two PUCCH groups.

Proposal 10: For the prerequisite feature groups of FG 25-16: change FG 11-3 to FG 11-4 and remove 12-1.

Proposal 11: The type of FG 25-16 is per FS.

Proposal 12: Remove the [] of component 2 and component 3 of FG 25-16.

Proposal 13: To add a new FG to reflect support of PRS as spatial relation RS of SRS.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 25-21 | Spatial relation for SRS for PDC based on PRS from the serving cell | 1. Spatial relation for SRS for PDC based on PRS from the serving cell | 25-19a | n/a | n/a (FR2 only) |  | Optional with capability signaling |

Proposal 14: To add necessary components and corresponding notes from FG 13-1 to FG 25-19a.

Reference

- R1-2202928 Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e Moderators (AT&T, NTT DOCOMO, INC.)

Appendix

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-1 | SPS HARQ-ACK deferral in case of TDD collision | 1. Idenfify HARQ-ACK bits of active SPS configurations for deferral in the initial PUCCH slot 2. Determination of the target PUCCH slot for SPS HARQ-ACK deferral 3. Multiplexing and transmission of deferred SPS HARQ-ACK information in the target PUCCH slot 4. Handling of the collision for the same HARQ process due to deferred SPS HARQ-ACK | [5-18] | Yes | N/A |  | [Per UE] | [No] (TDD only) | [No] | [N/A] |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-2 | Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8 | Repetitions for PUCCH format 0 and 2 over multiple slots with K = 2, 4, 8 | [4-23] | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-3 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with RRC configured repetition factor K = 2, 4, 8 Note: The support of FG 25-3 doesn’t imply an increase of the maximum number of PUCCHs per slot that supported by the UE | [4-23] [11-3] | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-3a | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication. Note: Dynamic PUCCH repetition factor indication is only supported for HARQ-ACK | [25-3 30-5] | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-3b | Inter-subslot frequency hopping for PUCCH repetitions | 1. Support inter-subslot frequency hopping for PUCCH repetition operation of PUCCH Formats 0, 1, 2, 3 and 4 for 7OS slot-based PUCCH configurations. 2. Support inter-subslot frequency hopping for PUCCH repetition operation of PUCCH Format 0 and Format 2 for 2OS slot-based PUCCH configurations | [TBD] | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-4 | One-shot HARQ ACK feedback triggered by DCI format 1_2 | [1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 scheduling a PDSCH] [2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 without scheduling a PDSCH using a reserved FDRA value] | [10-16[ [11-1] | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-5 | PHY priority handling for one-shot HARQ ACK feedback | [Support transmission of type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI] | [10-16] [11-4] | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-6 | Enhanced type 3 HARQ-ACK codebook feedback | 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) 5. Supported maximum number of actual PUCCH transmissions for [type 3 or] enhanced type 3 HARQ-ACK codebook feedback within a slot | 10-16 | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] | For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1, 2, 4, 8} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured Candidate values for component 5 is: {1, 2, 3, 4, 5, 6, 7}. [The values higher than 1 can be applied to sub-slot based configuration only.] | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-7 | Triggered HARQ-ACK codebook re-transmission | 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) 3. Supported minimum value M for the HARQ re-tx offset 4. Supported maximum value N for the HARQ re-tx offset | FFS | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] | Candidate values for component 3 is: M = {-7, -5, …, 1} Candidate values for component 4 is: N= {4, 6, …, 24} | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-8 | Semi-static HARQ-ACK codebook for sub-slot PUCCH | Semi-static (Type 1) HARQ-ACK codebook for sub-slot based PUCCH configuration | [4-11] [11-3] | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-9 | Semi-static PUCCH cell switching | Semi-static PUCCH cell switching using configured time-domain domain pattern of applicable PUCCH cell / carrier FFS whether/how to indicate the capability for support of PUCCH cell switch in two PUCCH groups |  | Yes | N/A |  | Per BC | N/A (TDD only) | N/A | N/A | Note: this feature applies to cells in the same TAG only If UE supporting this FG also supports both FGs 6-9 and 6-9a or both FGs 22-7b and 22-7c [or FGs 22-6 or 22-6a], the UE supports the cases of both same and different numerologies between switchable cells. Otherwise, the UE supports the case of same numerology between switchable cells | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-10 | PUCCH cell switching based on dynamic indication for same length of overlapping PUCCH slots/sub-slots | PUCCH cell switching based on dynamic indication in the DCI scheduling the PUCCH for same length (in physical time) of overlapping PUCCH slots/sub-slots FFS whether/how to indicate the capability for support of PUCCH cell switch in two PUCCH groups |  | Yes | N/A |  | Per BC | N/A(TDD only) | N/A | N/A | Note: this feature applies to cells in the same TAG only If UE supporting this FG also supports both FGs 6-9 and 6-9a or both FGs 22-7b and 22-7c [or FGs 22-6 or 22-6a], the UE supports the cases of both same and different numerologies between switchable cells. Otherwise, the UE supports the case of same numerology between switchable cells | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-10a | PUCCH cell switching based on dynamic indication for different length of overlapping PUCCH slots/sub-slots | PUCCH cell switching based on dynamic indication in the DCI scheduling the PUCCH for different length (in physical time) of overlapping PUCCH slots/sub-slots FFS whether/how to indicate the capability for support of PUCCH cell switch in two PUCCH groups |  | Yes | N/A |  | Per BC | N/A (TDD only) | N/A | [N/A] | Note: this feature applies to cells in the same TAG only If UE supporting this FG also supports both FGs 6-9 and 6-9a or both FGs 22-7b and 22-7c [or FGs 22-6 or 22-6a], the UE supports the cases of both same and different numerologies between switchable cells. Otherwise, the UE supports the case of same numerology between switchable cells | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-11 | 4-bits subband CQI | Subband CQI reporting with 4 bits per subband | FFS | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-12 | UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations | [Support initiating a semi-static channel access occupancy by the UE where the corresponding period is the same as, integer multiple of, or inter-factor of the period configured for a semi-static channel occupancy that can be initiated by gNB. ] | 10-1a | Yes | N/A |  | Per band | N/A | N/A | N/A | The signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-13 | UE initiating a semi-static channel occupancy with independent configurations from gNB semi-static channel access configurations | Support initiating a semi-static channel access occupancy by the UE where the corresponding period is independently configured from the period configured for a semi-static channel occupancy that can be initiated by gNB. | 10-1a, 25-12 | Yes | N/A |  | Per band | N/A | N/A | N/A | The signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-14 | PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH | Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH | [12-1] | Yes | N/A |  | [Per band] | [N/A] | [N/A] | [N/A] |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-15 | PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH | 1. Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell 2. Additional number of symbols (d3) needed on top of Rel-16 cancellation time (which results N2+d1+d3 in total cancellation time). | [12-1] | Yes | N/A |  | [Per band] | [N/A] | [N/A] | [N/A] | Candidate value set for component 2: d3 = {0, 1, …, 2μ+1} symbol(s) upon UE capability report, where μ=0,1,2,3 for SCS=15/30/60/120kHz, respectively. | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-16 | HARQ-ACK with different priorities multiplexing on a PUCCH/PUSCH | 1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] 4. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 5. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 6. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 7. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. | [11-3, 12-1] | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions on different cells [at least] for inter-band CA. | FFS | Yes | N/A |  | Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-19 | RTT-based Propagation delay compensation based on CSI-RS for tracking and SRS | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on CSI-RS for tracking and SRS | [2-51, 2-53] | Yes | N/A |  | [FS] | [N/A] | [N/A] | [N/A] |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-19a | RTT-based Propagation delay compensation based on DL PRS and SRS | 1. Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on DL PRS and SRS 2. Max number of DL PRS Resources in DL PRS Resource Set for PDC Values = {1, 2, 4, 8, 16, 32, 64} Note: 16, 32, 64 are only applicable to FR2 bands FFS whether to add necessary components and corresponding notes in FG 13-1 to FG 25-19a or to add FG 13-1 as a prerequisite FG for FG 25-19a - With potential different value ranges from FG 13-1 | [25-19, 13-1, 2-53] | Yes | N/A |  | [FS] | [N/A] | [N/A] | [N/A] | [Note: FG 13-1 is now only reported to LMF. If UE reports the support of this FG, it needs to report FG 13-1 to gNB also.] | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-20 | Propagation delay compensation based on legacy TA procedure | Support propagation delay compensation based on legacy TA procedure | FFS | Yes | N/A |  | [Per UE] | [no] | [no] | [N/A] |  | Optional with capability signaling |