3GPP TSG RAN WG1 #109-e		R1-2204009

e-Meeting, May 9th – 20th, 2022

Source:	OPPO

Title:	Discussion on UE features for NTN-IoT

Agenda Item:	8.16.14

Document for:	Discussion and Decision

Introduction

In this contribution, we continue providing our views on UE features for NTN-IoT.

Discussion

| 2. LTE_NBIOT_eMTC_NTN | 2-1 | Basic IoT over NTN support for eMTC | 1. UE derives its position based on its GNSS measurements [1-1. UE reports the information associated with the validity duration of GNSS] 2. UE receives serving satellite ephemeris in either state vector format or orbital element format 4. UE calculates UE specific TA in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris 6. UE calculates common TA in RRC_IDLE and RRC_CONNECTED according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) 7. For TA update in RRC_CONNECTED state, UE uses a combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops 8. In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link [in DL] [and] [in UL] 10. UE supports a validity timer of UL synchronization that is configured by the network 13. UE applies cell specific K_offset in timing relationship enhancements for Msg 3 PUSCH and PUCCH before dedicated PUCCH resource configuration 15. UE estimates UE-gNB RTT and delays the starts of ra-ResponseWindow by UE-gNB RTT 16. Delay the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated 17. In RRC_IDLE state and RRC_CONNECTED state, UE pre-compensates the calculated frequency offset and TA in uplink transmissions 18. UE receives cell-specific K_offset/K_mac in system information |
| --- | --- | --- | --- |

| 2. LTE_NBIOT_eMTC_NTN | 2-2 | Enhancing timing relationships using a time offset for eMTC | 1. UE receives and applies UE specific K_offset, K_mac in timing relationship enhancements 2. UE applies cell specific K_offset in timing relationship enhancements for uplink transmissions except for Msg 3 PUSCH and PUCCH before dedicated PUCCH resource configuration |
| --- | --- | --- | --- |

For FG2-1 component 13, it should be applied only for Msg3 transmission and PUCCH transmission. While for the rest of the uplink transmission, the UE capability with signaling should be allowed. Thus, we suggest that in the FG2-1 component 13 only takes UE applies cell specific K offset in timing relationship enhancement for PUCCH and PUSCH transmission. Then for other uplink transmission where UE applies cell specific K offset, for which UE may report its capability can be captured in the FG2-2, as mark in yellow above.

Proposal 1: for FG2-1 component 13, only keep that UE applies cell specific K offset in timing relationship enhancement for Msg 3 PUSCH and PUCCH before dedicated PUCCH resource configuration.

Proposal 2: other uplink transmission than Msg 3 PUSCH and PUCCH before dedicated PUCCH resource configuration where UE applies cell specific K offset can be captured in FG2-2, for which UE may report its capability.

Conclusion

In this document, we provide our views on UE features for NTN-IoT. Our proposal is made as follows:

Proposal 1: for FG2-1 component 13, only keep that UE applies cell specific K offset in timing relationship enhancement for Msg 3 PUSCH and PUCCH before dedicated PUCCH resource configuration.

Proposal 2: other uplink transmission than Msg 3 PUSCH and PUCCH before dedicated PUCCH resource configuration where UE applies cell specific K offset can be captured in FG2-2, for which UE may report its capability.

Reference

- R1-2202856, Summary of UE features for IoT over NTN, Moderator (AT&T).