3GPP TSG RAN WG1 #109-e		R1-2204012

e-Meeting, May 9th -May 20th, 2022

Source:	OPPO

Title:	Discussion on disabling of HARQ feedback for IoT NTN

Agenda Item:	9.12.2

Document for:	Discussion and Decision

Introduction

In RAN#95-e meeting, the disabling of HARQ feedback mechanism is considered for IoT-NTN performance enhancements in Rel-18 as follows:

4.1.1 IoT-NTN Performance Enhancements in Rel-18 to address remaining issues from Rel-17 This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below: - Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2] - Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1] - NOTE: The need for RAN4 Core requirements for this objective will be identified after the conclusion on the need for improvements.

In this contribution, we provide our views on introducing disabling of HARQ feedback in IoT NTN.

Disabling of HARQ feedback

In NTN, the propagation delays are much longer than TN, ranging from tens of milliseconds to hundreds of milliseconds depending on the satellite scenario. The HARQ RTT is much longer in NTN, so throughput degradation will happen due to HARQ stalling. With disabling HARQ feedback mechanism, the HARQ process does not need to wait for the feedback before a new data transmission, so the UE data rate can be enhanced in the large propagation delay scenario with limited spec impact and UE complexity.

In IoT NTN, the UE can be configured with one HARQ process or two (or more) HARQ processes. For the UE with one HARQ process, if the disabling of HARQ feedback is introduced, some mechanisms depending on the HARQ feedback in current spec can not work, e.g., delivering MAC CE command. In this case, the corresponding enhancement should be further discussed, e.g., disabling HARQ feedback can be configured semi-statically or dynamically. In our opinion, RAN1 should firstly discuss whether to introduce disabling HARQ feedback for the UE configured with one HARQ process. For the UE with two (or more) HARQ process, disabling HARQ feedback can be configurable per HARQ process semi-statically or dynamically to ensure the UE to always find an available HARQ process for data transmission.

Proposal 1: RAN1 should firstly discuss whether to introduce disabling HARQ feedback for the UE configured with one HARQ process.

Proposal 2: For the UE with two (or more) HARQ process, disabling HARQ feedback can be configurable per HARQ process.

Conclusion

In this contribution, we provide our views on introducing disabling of HARQ feedback for the UE configured with one HARQ process or two (or more) HARQ processes in IoT NTN. The following proposals are made:

Proposal 1: RAN1 should firstly discuss whether to introduce disabling HARQ feedback for the UE configured with one HARQ process.

Proposal 2: For the UE with two (or more) HARQ process, disabling HARQ feedback can be configurable per HARQ process.

Reference

[1]        RP-220979, “WID on IoT NTN enhancements”, MediaTek Inc, RAN#95-e, March 2022

[2]    3GPP TR 36.763: Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)