3GPP TSG RAN WG1 #109-e		R1-2204013

e-Meeting, May 9th – 20th, 2022

Source:	OPPO

Title:	Discussion on improved GNSS operations for IoT NTN

Agenda Item:	9.12.3

Document for:	Discussion and Decision

Introduction

In RAN #95e meeting, a new WI for study on IoT NTN enhancements was approved [1]. The objectives of the study include:

This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below:

-	Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2]

-	Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1]

- NOTE: The need for RAN4 Core requirements for this objective will be identified after the conclusion on the need for improvements.

In this contribution, we discuss our considerations on GNSS operation enhancements.

Discussion

GNSS position time to first fix (TTFF)

As studied in the SI phase of R17 IoT NTN and summarized in [2], there are three kinds of GNSS Position Time To First Fix (TTFF): cold start, warm start, and hot start. For cold start, no almanac information is stored in the receiver. The UE have to search signals from all the possible satellites and at least 4 satellites are needed for the positioning. Therefore, the duration will be affected by the rate of GNSS signal transmission and quality of reception. The time duration of cold start can range from tens of seconds to more than ten minutes. The typical values of cold start is 30 s if the GNSS signal is received with not much interruption. For warm start, it is based on the assumption that the some ephemeris information and clock correction data is already obtained. With some available information, the positioning time will be reduced to several seconds. For hot start, it is based on the assumption that GNSS receiver has valid ephemeris, clock correction and GNSS time reference with time for positioning can be as low as 1~2s.

Observation 1: For GNSS position TTFF, hot start requires about 1~2 seconds, warm start requires several seconds, and cold start requires about 30 seconds.

GNSS operation in R17

In R17 IoT NTN, it mainly considers sporadic short transmission. For idle UE wakes up from idle DRX/PSM, the UE acquires GNSS position fix, accesses the network, performs uplink and/or downlink communications for a short duration of time and goes back to idle. During this procedure, the UE does not need to re-acquire a GNSS position fix.

In RAN1 #107e meeting, the following agreement regarding GNSS is achieved.

Agreement Send LS to RAN2 to take the following RAN1 agreements into consideration to specify the aspects related to GNSS position validity: - For sporadic short transmission, UE in RRC_CONNECTED should go back to idle mode and re-acquire a GNSS position fix if GNSS becomes outdated - The UE autonomously determines its GNSS validity duration X and reports information associated with this valid duration to the network via RRC signalling. - X = {10s, 20s, 30s, 40s, 50s, 60s, 5 min, 10 min, 15 min, 20 min, 25 min, 30 min, 60 min, 90 min, 120 min, infinity} - Note: The duration of the short transmission is not longer than the “validity timer for UL synchronization” referred to in the WID objective (but which still needs further discussion for specifying further details)

In RAN2 #116bis-e meeting, the following agreements regarding GNSS are achieved.

UE need to have a valid GNSS fix before going to connected. RAN2 assumes that the UE may need to re-aquire the GNSS fix right before establishing the connection (regardless if previously valid or not), if needed to avoid interruption during the connection. When the GNSS fix becomes outdated in RRC_CONNECTED mode, the UE goes to IDLE mode.

From the above agreements, it can be observed that UE need to have a valid GNSS fix before going to connected mode. If GNSS becomes outdated, the UE in connected mode should go back to idle mode. If UE still needs to perform uplink and/or downlink communications, then the UE should re-acquire a GNSS position fix and access the network again.

Observation 2: In R17 IoT NTN, UE in connected mode should go back to idle mode if GNSS becomes outdated.

Potential enhancements on GNSS operation in R18

The potential enhancements on GNSS operation in R18 may change UE behavior when GNSS becomes outdated, in order to support long-term connection. For example, if the GNSS becomes outdated, the UE may declare loss of UL synchronization, switch to GNSS module to re-acquire a GNSS position fix, come back to communication module to recover UL synchronization with re-acquisition of satellite ephemeris and common TA parameters, perform RACH procedure, and then perform uplink and/or downlink communications, instead of declaring go back to idle mode.

However, the above procedure may require UE to re-acquire a GNSS position fix at a short time. If UE takes a long time to re-acquire a GNSS position fix, i.e., using cold start as discussed in section 2.1, then it is more reasonable that the UE should go back to idle mode if GNSS becomes outdated. Therefore, enhancements on defining new UE behaviors for GNSS outdated should focus on hot start and warm start.

Proposal 1: For re-acquiring GNSS position fix with cold start, UE in connected mode should go back to idle mode if GNSS becomes outdated.

Proposal 2: For re-acquiring GNSS position fix with hot start and warm start, enhancements on UE behaviors can be considered when GNSS becomes outdated.

Conclusion

This contribution discusses our considerations on GNSS operation enhancements and provides the following observations and proposals.

Observation 1: For GNSS position TTFF, hot start requires about 1~2 seconds, warm start requires several seconds, and cold start requires about 30 seconds.

Observation 2: In R17 IoT NTN, UE in connected mode should go back to idle mode if GNSS becomes outdated.

Proposal 1: For re-acquiring GNSS position fix with cold start, UE in connected mode should go back to idle mode if GNSS becomes outdated.

Proposal 2: For re-acquiring GNSS position fix with hot start and warm start, enhancements on UE behaviors can be considered when GNSS becomes outdated.

Reference

[1] RP-220979, Revised WID on IoT NTN enhancements, MediaTek Inc.

[2] 3GPP TR36.763, Study on Narrow-Band Internet of Things (NB-IoT)/enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN).