3GPP TSG-RAN WG1 109-e                                                    	 R1-2204015

Electronic Meeting, May 9th – 20th, 2022

Source:	OPPO

Title:	Evaluation methodology and preliminary results on AI/ML for CSI feedback enhancement

Agenda Item:	9.2.2.1

Document for:	Discussion

Introduction

The SID [1] for artificial intelligent (AI)/machine learning (ML) for NR air interface was agreed in RAN#94e meeting. The evaluation on AI/ML for CSI feedback enhancement was considered as following:

- Evaluation on AI/ML for CSI feedback enhancement [RAN1]

  - Evaluation methodology

  - Key performance indicator (KPI)

  - Performance evaluation results

In this contribution, the evaluation methodology on AI/ML for CSI feedback enhancement is discussed, and the some preliminary evaluation results are provided.

Discussion

Evaluation methodology and KPI

For AI/ML-based CSI feedback enhancement, several sub use cases and collaboration levels were addressed in [1]. And in our companion contribution [2], an improved categorization for the collaboration levels is proposed. And we propose to first focus on the evaluation of CSI feedback compression. And among CSI feedback compression schemes, we suggest to prioritize encoderdecoder-based algorithms, which requires signaling exchange before and during the AI/ML-based CSI encoding/decoding procedure. If the model downloading/uploading procedure needs to be specified in the specifications, CSI feedback compression is with Collaboration level 3b. In the model downloading/uploading procedure does not need to be specified in the specifications (e.g. assuming the model has been preloaded with gNB/UE implementation), CSI feedback compression is with Collaboration level 1b.

Proposal 1: For CSI feedback compression, the evaluations first focus on following collaboration levels:

- Level 1b: Signaling exchange for AI/ML inference without model exchange before and during AI/ML inference.

- Level 3b: Signaling exchange for AI/ML inference before and during AI/ML inference, and model exchange for AI/ML inference before AI/ML inference.

As the starting point of evaluation, the KPIs used for evaluating traditional solutions can be reused for the SI. For comparison, the Rel-16 Type I and eType II codebook based mechanisms and corresponding evaluation assumptions can be used as the baseline. Specifically, two KPIs can be considered to evaluate the performance of AI/ML for CSI feedback enhancements from the following aspects:

- Intermediate AI/ML results

  - CSI feedback accuracy, such as square of generalized cosine similarity (SGCS) over different CSI feedback overhead

    - SGCS on sub-band level

    - SGCS on resource block (RB) level

- System-level results

  - Throughput over different CSI feedback overhead

We propose that the system-level throughput results over different CSI feedback overhead should be used to evaluate the final performance of AI/ML for CSI feedback. In the initial stage of the study, we propose also to evaluate the intermediate AI/ML results, e.g., the CSI feedback accuracy, in terms of generalized cosine similarity (SGCS). Furthermore, both SGCS on sub-band level and RB level can be utilized as the evaluation metric. Firstly, when using SGCS on sub-band level as the evaluation metric for CSI feedback accuracy, the SGCS on the k-th sub-band, denoted as ρk2 can be expressed as:

ρk2=wkHwk'wkwk'2

where wk is the real channel eigenvector for the k-th sub-band and wk' is the recovered channel eigenvector for the k-th sub-band. The averaged SGCS ρ2 on the whole band for all samples in testing set can be further denoted as

ρ2= 1Nspnsp=0Nsp-11Nsbk=0Nsb-1ρk2

where Nsp is the number of samples in testing set, Nsb is the number of sub-band. Besides the SGCS per sub-band metric, the SGCS per RB can be used as well. The SGCS on the j-th RB in the k-th sub-band, denoted as νk,j2 can be expressed as:

νk,j2=wk,jHwk'wk,jwk'2

where wk,j denotes the real channel eigenvector for the j-th RB in the k-th sub-band. The averaged SGCS ν2 on the whole band for all samples in testing set can be further denoted as:

ν2= 1Nspnsp=0Nsp-11Nsbk=0Nsb-11NRBj=0NRB-1νk,j2

where NRB is the number of RB in one sub-band. Generally, larger averaged SGCS ρ2 or ν2 indicate better CSI feedback performance with higher CSI recovery accuracy. Considering that the SGCS on sub-band level is directly corresponding to the CSI feedback accuracy, which is beneficial for AI/ML model and dataset calibration in current stage between companies, we prefer to use SGCS on sub-band level as the referable intermediate AI/ML results for CSI feedback enhancement. Based on the above discussions, we have the following proposals:

Proposal 2: For evaluating AI/ML-based CSI feedback compression, use the Rel-16/17 evaluation assumptions as the baseline, i.e.:

- Carrier frequency: 4GHz

- Bandwidth: 10MHz

- Subcarrier spacing: 15KHz

- Resource block (RB) number: 52

- Sub-band number: 13

- Antenna configuration: 32 Tx ports with (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ and 4 Rx ports with (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ

Proposal 3: For evaluating AI/ML-based CSI feedback compression, use SGCS on sub-band level as an intermediate KPI for calibration.

Proposal 4: For evaluating AI/ML-based CSI feedback compression, use system-level throughput as the final KPI for evaluation.

Dataset

We have analyzed different candidate channels defined in TR 38.901 during pre-studies, including a system-level UMa channel, a link-level CDL-C channel, a link-level CDL-A channel and a field data in real-world environments. Among these channels, clearly the UMa channel embodies rich multipath characteristics and is the most complex channel. The CDL-C channel is with a moderate level of multipath fading. The CDL-A channel has been biased towards a relatively simple single-path environment. The channel extracted from field data, as a comparative case, is with a very low level multipath fading, thus even simpler than CDL-A.

Based on the above observations, channels defined in TR 38.901 standing for different levels of multi-path fading can be used to construct the dataset and then train an AI/ML model. Considering that system-level UMa channels have been widely used for system-level evaluations from Rel-15 to Rel-17, we prefer that the system-level UMa channel is used to construct the basic dataset for AI/ML based CSI feedback evaluation. Moreover, to calibrate the intermediate AI/ML results on CSI feedback accuracy with different AI models, the link-level channels including CDL-A and CDL-C can also be used as complementary. Finally, the performance on field test channel can also be evaluated if possible. Therefore, we have the following proposal:

Proposal 5: For evaluating AI/ML-based CSI feedback compression, use system-level channels (e.g. UMa) as the baseline dataset.

- Link-level channels (including CDL-A and CDL-C) can be used as the complementary dataset.

- Field test-based dataset can also be considered after its quality is examined.

Besides the dataset with single type channel, the mixed dataset constructed using more than two kinds of channels should also be considered to evaluate the generalization performance of AI model on CSI feedback. For example, the system-level UMa and UMi mixed channel dataset should be considered firstly to evaluate the AI/ML based CSI feedback performance on different channel scenarios. As complementary, the link-level mixed channel dataset, e.g. CDL-A and CDL-C mixed channel, can also be constructed. Other possible mixed channel dataset type and mixing proportion can be remained for further study. Therefore, we have the proposal:

Proposal 6: For evaluating AI/ML-based CSI feedback compression, the mixed channel datasets can be used to evaluate the generalization performance of the AI/ML models, e.g.:

- UMa + UMi system-level channel dataset.

- CDL-A + CDL-C link-level channel dataset.

- FFS other potential mixed channel datasets.

Moreover, the dataset construction method should also be calibrated between companies so that the datasets are basically consistent and the further evaluations for can be conducted

AI model

In this contribution, we use the encoder at UE side and decoder at gNB side with Transformer backbone about 40MFLOPs.  We used a model named EVCsiNet-T (as shown in Figure 2), in which each vector of sub-band in the input CSI matrix is firstly processed by an embedding layer and then 10 self-attention based blocks are sequentially introduced before a uniform quantization layer. As for the decoder part, after dequantization layer, a dense layer with 512 nodes is employed, followed by 10 self-attention based blocks as well. Finally, a reshape layer is implemented to obtain the output with the shape of the original CSI. The reference model for AI/ML based CSI feedback enhancement can be find in https://wireless-intelligence.com/#/download.

Figure 1: Illustration of EVCsiNet-T model for CSI feedback compression

Performance evaluation results

In this section, we give some preliminary performance evaluation results. The SGCS on sub-band level for different channel datasets are provided in Table 1. The AI model architecture with Transformer backbone and 64bits CSI feedback overhead is utilized, and the Rel-16 eType II is used as the comparable baseline.

Table 1. Comparison of SGCS for AI based CSI feedback

| SGCS |  | Testing set (5k for each channel) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | CDL-A300 | CDL-C30 | CDL-C300 | UMa | UMi |
| Training set | CDLA300 (100k) | 0.925 | 0.722 | 0.657 | - | - |
|  | CDLC30 (100k) | 0.818 | 0.904 | 0.673 | - | - |
|  | CDLC300 (100k) | 0.773 | 0.748 | 0.840 | - | - |
|  | CDLA300 (100k) + CDLC300 (100k) + CDLC30 (100k) | 0.928 | 0.898 | 0.841 | - | - |
|  | UMa (600k) | 0.897 | 0.819 | 0.742 | 0.772 | 0.754 |
|  | UMi (600k) | 0.905 | 0.834 | 0.771 | 0.766 | 0.773 |
|  | UMa (600k) + UMi (600k) | 0.909 | 0.837 | 0.772 | 0.776 | 0.773 |
|  | All Mixed | 0.927 | 0.883 | 0.815 | 0.766 | 0.762 |
|  | Rel-16 eType II baseline | 0.817 | 0.765 | 0.668 | 0.720 | 0.713 |

Observed from Table 1, AI based CSI feedback with Transformer backbone can achieve higher SGCS performance compared with Rel-16 eType II baseline on both link-level channels including CDL-A300, CDL-C30 and CDLC300, and system-level channels including UMa and UMi channels. Here, sufficient channel datasets are utilized so that AI model can reach stable SGCS performance. Specifically, 100k channel samples are used for each link-level channel dataset and 600k channel samples are used for each system-level channel dataset. Based on the evaluation results, we have the following observation:

Observation 1: The AI based method can achieve better SGCS performance than Rel-16 eType II.

Moreover, the AI models trained on mixed channel dataset and tested on single channel dataset should be evaluated from the perspective of the intermediate AI/ML results with CSI feedback accuracy SGCS (at least) and the system-level throughput results.  We can find that AI shows comparable SGCS performance compared with the results training on corresponding single channel dataset, which indicates the good generalization performance and hence the same AI model may be utilized for different channels. Therefore, we have the following observation and proposal:

Observation 2: The SGCS performance of AI model trained on the mixed channel dataset with sufficient samples is comparable to that trained on the single channel dataset.

Conclusion

In this contribution, we provide some discussions and preliminary results about the evaluation on AI/ML for CSI feedback enhancement. Based on the discussions and evaluations, we have following observations and proposals:

Observation 1: The AI based method may achieve better SGCS performance than Rel-16 eType II.

Observation 2: The SGCS performance of AI model trained on the mixed channel dataset with sufficient samples is comparable to that trained on the single channel dataset.

Proposal 1: For CSI feedback compression, the evaluations first focus on following collaboration levels:

- Level 1b: Signaling exchange for AI/ML inference without model exchange before and during AI/ML inference.

- Level 3b: Signaling exchange for AI/ML inference before and during AI/ML inference, and model exchange for AI/ML inference before AI/ML inference.

Proposal 2: For evaluating AI/ML-based CSI feedback compression, use the Rel-16/17 evaluation assumptions as the baseline, i.e.:

- Carrier frequency: 4GHz

- Bandwidth: 10MHz

- Subcarrier spacing: 15KHz

- Resource block (RB) number: 52

- Sub-band number: 13

- Antenna configuration: 32 Tx ports with (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ and 4 Rx ports with (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ

Proposal 3: For evaluating AI/ML-based CSI feedback compression, use SGCS on sub-band level as an intermediate KPI for calibration.

Proposal 4: For evaluating AI/ML-based CSI feedback compression, use system-level throughput as the final KPI for evaluation.

Proposal 5: For evaluating AI/ML-based CSI feedback compression, use system-level channels (e.g. UMa) as the baseline dataset.

- Link-level channels (including CDL-A and CDL-C) can be used as the complementary dataset.

- Field test-based dataset can also be considered after its quality is examined.

Proposal 6: For evaluating AI/ML-based CSI feedback compression, the mixed channel datasets can be used to evaluate the generalization performance of the AI/ML models, e.g.:

- UMa + UMi system-level channel dataset.

- CDL-A + CDL-C link-level channel dataset.

- FFS other potential mixed channel datasets.

References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#94e, Dec. 2021

- R1-2204014, On general aspects of AI/ML framework, OPPO, RAN1#109-e, May. 2022

- R1-2204016, On sub use cases and other aspects of AI/ML for CSI feedback enhancement, OPPO, RAN1#109-e, May. 2022

Appendix

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 4GHz |
| Bandwidth | 10MHz |
| Subcarrier spacing | 15KHz |
| RB number | 52 |
| Sub-band number | 13 |
| Antenna configuration | 32 Tx ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ 4 Rx ports: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Channel Model | CDL-A300,CDL-C30,CDL-C300, UMa, UMi (80% indoor & 20% outdoor for UMa/UMi) |
| UE speed | 3km/h for indoor, 30km/h for outdoor |
| eType II, {L,pv,β} | {2,0.25,0.25} |
| AI model type | Transformer based model |
| AI model complexity | ~40MFLOPs |
| CSI feedback overhead | 64bits |