3GPP TSG RAN WG1 #109-e                                                                                R1-2204017

e-Meeting, May 9th-20th, 2022

Source:	OPPO

Title:	Evaluation methodology and preliminary results on AI/ML for beam management

Agenda Item:	9.2.3.1

Document for:	Discussion

# Introduction

The SID [1] of artificial intelligent (AI) and machine learning (ML) for NR air interface was agreed in RAN#94e meeting. The initial set of use cases including beam management was selected as followings

- CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

- Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

- Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

In this contribution, we present our initial evaluation on beam prediction in spatial domain.

# Discussion

## Use case of beam management

In our view, the use cases of beam management can be categorized into sub use cases, but not limited to

- Beam prediction in spatial domain for overhead/latency reduction

- Beam prediction in time domain for overhead/latency reduction

- Beam selection accuracy improvement

In our initial evaluation, we focused on the sub use case of beam prediction in spatial domain for overhead and latency reduction. Next, we will present more details on how to achieve the overhead/latency reduction with the aid from AI/ML. Conventionally, NW broadcasts up to 64 SSBs periodically at FR2 for DL beam sweeping. Deployed with either single panel or multiple panels, UEs sweep all its Rx beams (typically 4 Rx beams) per panel to search the best DL beam pair(s). To fulfill the P1 procedure which was specified from Rel.15 NR, large DL overhead and latency would be expected.

Intuitively, both NW and UE apply the Tx and Rx beams respectively with certain pattern. For instance, NW carries out analog beamforming in both vertical and horizontal domain, e.g. 16 beams in horizontal domain and 4 beams in vertical domain. The combinations of vertical beam and horizontal beam can be seen by UE as an actual beam (totally 64 DL Tx beams in above example).

Assuming the beamforming pattern can be somehow learnt by neural network (NN), then only a limited number of DL measurement would be enough for NN to infer the quality of all DL beam pair links. The benefit comes from that a large portion of DL measurement of beam pairs can be replaced by beam prediction in spatial domain. This can be called as super-resolution problem.

The illustration on traditional beam selection and AI/ML beam prediction in spatial domain can be found in Figure 1. For traditional beam sweeping, there are 64 Tx beams, 4 Rx beams, and then the total number of beam pairs for UE to measure can be up to 256. For AI/ML beam prediction, only a subset of all beam pairs (e.g. 8 Tx beams highlighted as yellow) are actually measured by UE with all Rx beams. Then the measurement number can be calculated as 32 beam pairs (8 Tx beam * 4 Rx beam). Correspondingly, the overhead and latency can be significantly compressed as 1/8 (32 beam pairs / 256 beam pairs). More evaluation assumption can be found in Appendix.

One may wonder how the Tx beams for UE to measure are selected. We show our selection in Figure 2 where the highlighted circles represent the most “welcomed” Tx beams from UEs perspective. These welcomed Tx beams are widely selected by UE when the traditional beam sweeping procedure showed us this hint. Intuitively, the selected Tx beams represent a spatial pattern of beamforming which can be useful for UE to access the NW. Surely, other Tx beams selection can be carried out with certain rules and probably with even better performance. We didn’t try all the possibilities, and the intention is just to show that this typical selection is workable.

- : DL beam sweeping procedure where for traditional beam sweeping 64 Tx beams at NW and 4 Rx beams per panel and for AI/ML beam prediction 8 Tx beams and 4 Rx beams per panel

- : Only a subset of DL beams measured among all DL beams

- For overhead and latency reduction, study and evaluate the sub use case of AI/ML-based beam prediction in spatial domain.

## Performance metrics

In legacy (prior to Rel.18) beam management, L1-RSRP has been widely-used as beam selection metric. With its features of simplicity and computation-friendly, it would be proper to serve as the metric for AI/ML-based beam prediction as well. From the TS 38.215 [2], L1-RSRP is defined based on two types of DL RS.

- SS reference signal received power (SS-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals. The measurement time resource(s) for SS-RSRP are confined within SS/PBCH Block Measurement Time Configuration (SMTC) window duration. If SS-RSRP is used for L1-RSRP as configured by reporting configurations as defined in TS 38.214 [6], the measurement time resources(s) restriction by SMTC window duration is not applicable.

- CSI reference signal received power (CSI-RSRP), is defined as the linear average over the power contributions (in [W]) of the resource elements of the antenna port(s) that carry CSI reference signals configured for RSRP measurements within the considered measurement frequency bandwidth in the configured CSI-RS occasions.

One may argue that to verify the performance eventually, the classic metric of throughput should be verified by SLS. But at the early stage of this SI, we would not suggest to go that far at current stage. In Section 2.4, we will show the performance gaps between actual L1-RSRP and predicted L1-RSRP. We think this performance comparison on L1-RSRP can clearly capture the advantage of adopting AI/ML beam prediction in spatial domain over traditional beam sweep scheme.

- Adopt L1-RSRP as performance metric for AI/ML beam prediction as a starting point.

Along with L1-RSRP that measures the beam predication accuracy, another key performance metric is the correct beam prediction rate. It is defined as the ratio between the number of correct beam prediction and total number of beam prediction. Literally, it evaluates how much portion of beam prediction can find the best beam(s) among all candidate beam pairs. Apparently, the higher correct beam prediction rate, the better performance of AI/ML algorithm can achieve. Additionally, we also observe in our following evaluations that the correct beam prediction come along with smaller L1-RSRP prediction gap (around 1dB), whereas the incorrect beam prediction normally results in larger L1-RSRP prediction gap (around 2.5dB). Therefore, we have

- Study and evaluate the correct beam prediction rate for AI/ML beam prediction in spatial and time domain as one of the key performance metrics.

## Data set and neural network models

In our evaluation, the data set are generated by SLS as two parts. The 1st part is a subset of measured beam pairs (highlighted as in Figure 1) and the corresponding L1-RSRP. This can be used as input of the NN for training purpose. The 2nd part is the best (with highest L1-RSRP) beam pair index among all the beam pairs and its L1-RSRP, which can be seen as the label to the NN.

With the labelled data, NN can see which beam pair(s) is/are the best one(s) and then learn the relation or pattern between the limited measurements (as input) and the best one(s) (as output). In other words, the Tx and Rx beam pattern can be somehow educated to the NN. Then even with a small portion of actual measurement, NN is able to infer which beam pair(s) are most likely to be optimal with the highest L1-RSRP.

In terms of the NN models for this evaluation, we adopt two separate DNN (Deep Neural Network) models which are depicted in Figure 3. The inputs to both AI/ML models are the subset of DL measurements, i.e. L1-RSRP values of measured beam pairs. Since the beam pair indexes of the measured beam pairs can be sequentially coupled with the NN as inputs, the beam pair indexes can be implicitly captured by the NN models.

Specifically, the AI/ML model 1 is to predict the best beam pair index(es) and it is depicted in Figure 4 whereas the AI/ML model 2 is to predict the best L1-RSRP(s). Since these two AI/ML models are trained separately, the prediction of both models may not be perfectly aligned. Note that the best L1-RSRP predicted by AI/ML model 2 may not always correspond to the best beam pair predicted by AI/ML model 1. There could be a case that the predicted beam pair index is not correct, but the predicted L1-RSRP is quite close to the actual L1-RSRP of the best beam pair.

- : NN models to predict best beam pair index and the highest L1-RSRP separately

- : Beam pair prediction (model 1) implementation by using DNN model

## Evaluation results

Given the previously introduced data set and NN model setting, we would like to share our initial evaluation results in Table 1. In this table, we compare the AI/ML based prediction with traditional beam selection, where the latter applies no interpolation, but just select the best measured beam pair with highest L1-RSRP among the actual measurements.

In the 2nd column of Table 1, we compare the best beam selection/prediction rate for both AI/ML beam prediction and traditional beam selection. One may see that the correct best beam pair prediction rate of AI/ML could be around 80% and the traditional beam selection has a smaller chance, i.e. round 24% to find the best beam pair.

In the 3rd and 4th column of Table 1, two performance metrics on L1-RSRP error are calculated. L1-RSRP prediction error is the difference between actual (real) L1-RSRP of the best beam pair and the predicted L1-RSRP. L1-RSRP selection error is the difference between actual (real) L1-RSRP of the best beam pair and the actual (real) RSRP of the selected beam pair.

When the best beam pair is predicted right by the NN model, its L1-RSRP prediction error is 1.02dB. That is very close to the quantization step for L1-RSRP beam reporting (e.g. 1dB step size) in TS 38.214 [3]. Note that when the best beam pair is correctly selected, then the L1-RSRP selection error (between the actual L1-RSRP of the best beam pair and the actual R1-RSRP of selected best beam) is 0dB, since in this case, they are of the same beam pair.

The false beam pair selection rate of AI/ML based prediction is round 20%, and the L1-RSRP prediction error is around 2.5dB. Intuitively, if the NN cannot predict the right beam pair, then it would be harder to predict the best L1-RSRP within very small error. Moreover, in the cases of false beam prediction, the L1-RSRP selection error is more than 5dB, which reflects the difference between the actual (real) L1-RSRP of best beam pair and actual (real) L1-RSRP of the predicted beam pair.

As for traditional beam selection, since there is no interpolation/prediction, the L1-RSRP prediction error is not applicable. Hence, we only average the L1-RSRP selection error of traditional beam selection which could be up to 11dB. The reason of such huge gap lies in the fact that UE can only pick up the best beam within 1/8 beam pair of all candidate beam pairs.

- : performance based on L1-RSRP

|  | Correct beam prediction rate | Mean of L1-RSRP prediction error | Mean of L1-RSRP selection error |
| --- | --- | --- | --- |
| AI/ML-Right selection | 80.35% | 1.02dB | 0dB |
| AI/ML-False selection | 19.65% | 2.47dB | 5.49dB |
| Traditional-Right selection | 23.94% | / | 0dB |
| Traditional-False selection | 76.06% | / | 11.16dB |

With above analysis on performance, we would like to share our observations.

- AI/ML beam prediction in spatial domain can yield relatively high correct prediction rate (around 80%) while using only a small portion (1/8) of DL measurement.

- When AI/ML beam prediction is correct, the L1-RSRP prediction error is acceptably small (around 1dB); otherwise (AI/ML beam prediction incorrect), the L1-RSRP prediction error would increase slightly (up to 2.5dB).

In Figure 5, we show the CDF of both predicted L1-RSRP and actual (real) L1-RSRP when the AI/ML beam prediction is correct. Statistically speaking, thanks to the technique of AI/ML, these CDFs are closed to each other. It aligns well with average L1-RSRP prediction error (1.02dB as shown in Table 1).

In Figure 6, we show the CDFs when false beam pair selection/prediction happens. When AI/ML model 1 (introduced in Figure 3) predicts an incorrect best beam pair, the CDF of its predicted L1-RSRP (generated by AI/ML model 2 separately) stays not far away from that of actual (real) L1-RSRP of the best beam pair. This curve is marked as “AI-predicted L1-RSRP”.

As for the curve marked as “AI-selected L1-RSRP”, it refers to the actual (real) L1-RSRP of the wrongly-predicted best beam pair. This gap between actual L1-RSRP of AI/ML prediction and actual L1-RSRP of the best beam pair is around 5dB. This performance also matters, since it reflects actual L1-RSRP performance gap of the best beam predicted wrongly by NN.

As we explained previously, the traditional sub-set beam selection scheme has a large gap when compared with the best beam. The reason lies in the fact that the traditional beam selection only has partial beam pair candidates. Note that in this contribution, we only select the very basic traditional beam selection scheme. To make more meaningful comparison, we may need to study more advanced traditional scheme, e.g. beam selection with linear interpolation.

- : CDF of actual (real) L1-RSRP and predicted L1-RSRP when the beam prediction is correct

- : CDF of actual (real)/selected/predicted L1-RSRP when the beam prediction is incorrect

- To make more meaningful comparison between traditional beam selection scheme and AI/ML beam prediction, study and decide the EVM and benchmark for comparison.

# Conclusion

In this section, allow us to repeat our observations and proposals

- AI/ML beam prediction in spatial domain can yield relatively high correct prediction rate (around 80%) while using only a small portion (1/8) of DL measurement.

- When AI/ML beam prediction is correct, the L1-RSRP prediction error is acceptably small (around 1dB); otherwise (AI/ML beam prediction incorrect), the L1-RSRP prediction error would increase slightly (up to 2.5dB).

- For overhead and latency reduction, study and evaluate the sub use case of AI/ML-based beam prediction in spatial domain.

- Adopt L1-RSRP as performance metric for AI/ML beam prediction as a starting point.

- Study and evaluate the correct beam prediction rate for AI/ML beam prediction in spatial and time domain as one of the key performance metrics.

- To make more meaningful comparison between traditional beam selection scheme and AI/ML beam prediction, study and decide the EVM and benchmark for comparison.

# References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#94e, Dec. 2021

- TS 38.215 v17.1.0 Physical layer measurements

- TS 38.214 v17.1.0 Physical layer procedures for data

# Appendix: evaluation assumptions

| Parameter | Value |
| --- | --- |
| Scenario | TR38.901: Indoor hotspot, 12 sites, 3 cells per site |
| SCS | 120kHz |
| Bandwidth | 40MHz |
| Carrier Frequency | 30GHz |
| Tx power | 20dBm |
| BS antenna config | [Mg Ng M N P] = [1 1 4 16 2] |
| UE antenna config | [Mg Ng M N P] = [1 1 1 4 2] |
| BS Tx beam pattern | 64 Tx = 16 horizontal * 4 vertical Azimuth angle = [-16*pi/32, -14*pi/32, …, 0, …, 12*pi/32, 14*pi/32] Zenith angle = [2*pi/8, 3*pi/8, 4*pi/8, 5*pi/8] (azimuth, zenith) = (0, pi/2) is the direction perpendicular to the array |
| BS mech. tilting | 20 degree |
| BS height | 3m |
| UE Rx beam pattern | 4 Rx per panel = 4 horizontal * 1 vertical UE panels deployment [-90, 0, 90], assumed the best panel is selected Azimuth angle = [-3*pi/8, 1*pi/8, pi/8, 3*pi/8] Zenith angle = [pi/2] |
| UE velocity | 3km/h |
| UE height | 1.5m |
| Beam selection method | L1-RSRP |
| Training set | 39600 samples = 12 sites * 3 cells per site * 100 UE per cell * 11 drops |