3GPP TSG RAN WG1 #109-e		R1-2204019

e-Meeting, May 9th – 20th, 2022

Source:	OPPO

Title:	Evaluation methodology and preliminary results on AI/ML for positioning accuracy enhancement

Agenda Item:	9.2.4.1

Document for:	Discussion and Decision

- Introduction

As a promising technology, AI/ML based solutions have been widely used and has shown impressive performance in various areas, e.g., computer vision, speech recognition, natural language processing (NLP), language translation and so on.

Motivated by the great success of AI/ML in the afore-mentioned areas, 3GPP also started study on AL/ML for wireless communications in SA2 and RAN3. On the basis of the existing study in 3GPP, RAN#94e meeting decided to start a new study item (SI) on AI/ML for NR air interface in Rel-18 [1]. According to the new SID, this study will focus on three typical use cases of physical layer in the first step:

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

In this contribution, we will discuss the use case of AI-based positioning accuracy enhancement, including the typical scenarios, evaluation assumptions, performance metric and so on. Moreover, we will present our initial evaluation results for some AI/ML-based schemes.

Discussion

AI-based schemes

During the evolution of R16 NR, 3GPP developed multiple RAT-based positioning methods to provide high accuracy positioning for NR cellular system:

- DL-TDOA positioning

- UL-TDOA positioning

- DL-AoD positioning

- UL-AoA positioning

- Multi-RTT positioning

Theoretically, AI/ML can be introduced to work along with each of the above positioning schemes. In our initial evaluation, we focus on the combination / integration of DL-TDOA positioning and AI/ML. The same approach and evaluation methodologies can be directly extended to other NR positioning methods.

For the existing NR mechanism of DL-TDOA positioning method, UE will measure DL RSTD or estimate the location based on the reception of DL PRS.  Then, UE will report the DL RSTD measurements, optionally along with DL PRS RSRP, or the estimated location to LMF. For UE-assisted DL-TDOA positioning method, LMF will further calculate the location of the target UE based on the reported measurement results, by using traditional algorithm (e.g., CHAN algorithm). We refer to the traditional NR DL-TDOA as “Non-AI-based DL-TDOA”.

In order to evaluate the potential advantages of AI/ML over traditional positioning algorithm, we consider the following AI/ML-based schemes, which are based on the assumptions of different types of measurement results and will have different impacts on the system design and specifications.

The first AI-based scheme is based on LMF’s implementation:

- UE measures and reports the DL RSTD results via the existing mechanism.

- Then, LMF uses a Neural Network (NN) to estimate the location.

For this scheme, the whole air interface is the same as R16/17 positioning mechanism and it is totally up to LMF implementation.  In other words, the AI/ML operations are transparent to UE. Accordingly, the advantages of this scheme are no impact on specification and easier for the deployment of AI/ML. We refer to this scheme as “AI + DL-TDOA” in subsequent discussions.

In order to further improve the performance, we can consider other types of measurement results to exploit more information from the reception of PRS. One way is to use TOA rather than TDOA. In our initial evaluation, we assume the TRPs and UEs are perfectly synchronized. In this ideal assumption, UE can estimate the TOA directly based on DL PRS. LMF uses a NN to estimate the location. We refer to this scheme as “AI + TOA”. Obviously, this scheme is too ideal for practical deployment. Our main intentions of this scheme are as below

- It can provide an upper bound for the performance of AI-based DL-TDOA

- It can provide an upper bound for the performance of Multi-RTT. By using the multi-RTT method, LMF can get an estimation of TOA. However, due to the measurements at two sides and the potential imperfect matching of the measurements, the TOA value estimated by LMF will has less accuracy compared to the one used in this scheme.

No matter TOA or TDOA, only partial information can be attracted from the reception of DL PRS. That is to say, some information, more or less, can not be exploited at LMF side. Thus, another way is to measure and report the channel information to LMF. In our evaluation, UE estimates the channel impulse response (CIR) and reports its normalized versions without quantization to LMF. LMF uses a NN to estimate the location. We refer to this method as “AI + Normalized CIR”. For this scheme, a new type of measurements and reporting format are needed.

Since the reported CIR is normalized, information regarding the received power of DL PRS is missing. UE can also report this information via DL PRS RSRP. We refer to it as “AI + Normalized CIR + RSRP”.  Similarly, RSRP measurements can also be used for other AI-based schemes.

In summary, the following schemes are used in our initial evaluation:

- Non-AI-based DL-TDOA: CHAN algorithm is used to estimate the location based on measurement results of DL RSTD

- AI + DL-TDOA: A trained NN is used to estimate the location based on measurement results of DL RSTD

- AI + DL-TDOA + RSRP: A trained NN is used to estimate the location based on measurement results of DL RSTD and associated RSRP

- AI + TOA: A trained NN is used to estimate the location based on measurement results of TOA

- AI + TOA + RSRP: A trained NN is used to estimate the location based on measurement results of TOA and associated RSRP

- AI + Normalized CIR: A trained NN is used to estimate the location based on measurement results of normalized CIR

- AI + Normalized CIR + RSRP: A trained NN is used to estimate the location based on measurement results of normalized CIR and associated RSRP

Scenarios and performance metrics

In Rel-17 NR positioning enhancement, two scenarios, namely InF-SH and InF-DH, are used for evaluation [2]. Based on the R17 study, the traditional positioning method can achieve quite good performance for LOS-dominant scenarios, but will suffer worse positioning accuracy for the scenarios with heavy NLOS conditions. In other words, the NLOS issue is the main bottleneck for traditional positioning methods. AI/ML may have potential advantage to better address the NLOS issue since it can learn more information from the training data. Thus, we have the following proposal for the evaluation:

Proposal 1: For the evaluation of AI/ML-based positioning accuracy improvement, support the InF-DH scenario with the high clutter density set as 60%.

For the performance metrics, we can also reuse the same as R16 and R17 study, e.g., CDF of the positioning accuracy, achieved positioning accuracy at 90%.

Proposal 2: Regarding the positioning accuracy, reuse the following metrics for the evaluation:

- CDF of the positioning accuracy

- Achieved accuracy at 90%

Compared to the traditional positioning algorithms, one potential issue of AI/ML-based method is the high complexity. Due to the different frameworks of deep learning and different implementation of products (e.g., LMF/UE), it is quite challenging to find a unified metric for various products to accurately estimate the complexity from the aspects including resource utilization, power consumption, interference latency and so on. As a rough tool, floating point operations (FLOPs) is widely used to describe a frozen neural network in other areas. Thus, we suggest to reuse FLOPs as the basic metric to evaluate the complexity of AI/ML-based method at the initial stage. Whether to introduce new metric(s) or not can be determined based on further study.

Proposal 3: Regarding the complexity of AI/ML-based method, use FLOPs as the metric at the initial stage

Generation of data sets

In our evaluation, the data sets are generated by system-level simulations. The measurement samples are based on the reception of DL PRS within one slot. For each reception of DL PRS, UE will do the measurement corresponding to each scheme, e.g., DL-RSTD, DL TOA, CIR and RSRP. The label associated with each sample is the known location information of the target UE.

For the evaluation of each AI/ML-based scheme, 80,000 samples are generated with associated labels, 98% of which are used for the training of NN model and the other 2% are used as testing data. The samples of test data set are selected randomly out of 80,000 samples.

In our evaluations, there are two different alternatives to general the 80,000 samples for each scheme:

- Alt.1: We simulate 80,000 drops, and only 1 UE is randomly dropped per drop. Compared to the second alternative, the data of UEs have less correlation among each other.

- Alt.2: We simulate 1 drop and 80,000 UEs are dropped in this drop. As the number of UEs in a drop increases, the UEs are closed to each other. Thus, the data for close UEs are with higher correlations.

Alt.2 can be regarded as a snapshot of a fixed scenario, while Alt.1 is reflecting a set of many similar scenarios with the same TRP deployments. Thus, in some sense, Alt.2 can be used to show some upper bound of AI-based solutions, but the corresponding trained AI modes may suffer the generalization issue. Some other alternative(s) to generate the data sets are in the middle ground between Alt.1 and Alt.2.

NN models

For each sample of CIR, we use 256 time-domain complex values in our evaluation.  We use the three-dimensional matrix with the size of [M, 256, 2], which are generated from the CIRs corresponding to M TRPs, as the input of neural network.  Since the input size are relatively large, we can consider the input as an “image”. With this intuitive, we choose ResNet for “AI + Normalized CIR” and “AI + Normalized CIR + RSRP” as ResNet is widely used for image classification tasks. Fig.1 illustrates the basic structure of our AI model. To be specific, based on ResNet backbone, the input is directly added to the output of a residual block (consisting of 9 Conv2D layers with 64 filters and 1 Conv2D layer with 2 filters, 3×3 kernels are used for these filters), then followed by 1 dense layer with 2 nodes to provide the NN output. The NN outputs are the estimated values of the location (x, y). The AI mode used in our evaluation can be downloaded from the website of https://wireless-intelligence.com/#/download.

Fig.1: Illustration of the AI model for “AI + Normalized CIR” and “AI + Normalized CIR + RSRP”

For the other AI/ML-based scheme, since the size of the input data size is much smaller, we choose a simple neural network mode. The AI models for “AI + DL-TDOA” and “AI + TOA” are based on fully connected neural network, and 5 layers with {64,128,256,128,2} nodes are cascaded to obtain the output, which is illustrated in Fig.2.

Fig.2: Illustration of the AI model for “AI + DL-TDOA”, “AI + DL-TDOA+RSRP”, “AI + TOA” and “AI + TOA + RSRP”

In summary, the AI modes used in our evaluation are as below:

- AI + DL-TDOA, AI + DL-TDOA + RSRP: fully connected neural network (DNN)

- AI + TOA, AI + TOA + RSRP: fully connected neural network (DNN)

- AI + Normalized CIR, AI + Normalized CIR + RSRP: ResNet

Initial evaluation results

Based on the above-mentioned data sets and AI models, we present some initial evaluation results in this section to show the positioning accuracy of each AI-based or non-AI-based scheme. We use InF-DH scenario in the simulations, where {clutter density, height, size} is set to be {0.6,6m,2m}. In order to reduce the workload for the initial evaluation, we assume no timing error and don’t implement in the simulator the spatial consistency of small scale parameters that are defined in Section 7.6.3.1 of TR 38.901. More details for the evaluation assumption can be found in Appendix.

In Table 1, the evaluation results of positioning accuracy for non-AI-based and AI-based schemes are shown for InF-DH scenario. We generated the data sets by Alt.2. That is to say, we simulated 1 drop and 80,000 UEs are dropped in this drop. From Table 1, we can get the following observation:

Observation 1: For InF-DH scenario with clutter density of 0.6, if the data sets are generated by Alt.2 (i.e., 1 drop and 80,000 UEs per drop), the positioning accuracy achieved at 90% for different methods are as below

- Non-AI-based DL-TDOA: 12.35m

- AI + DL-TDOA + RSRP: 0.61m

- AI + Normalized CIR + RSRP: 0.42m

Table 1: Positioning accuracy achieved for 90% UEs

|  | Non-AI-based DL-TDOA | AI + DL-TDOA + RSRP | AI + Normalized CIR + RSRP |
| --- | --- | --- | --- |
| Accuracy achieved @90% | 12.35m | 0.61m | 0.42m |
| Note: Data sets are generated by Alt.2: 1 drop, 80,000 UEs per drop |  |  |  |

It is well-known that different data sets may have significant impacts on the performance of AL/ML-based methods. In order to investigate the impact of different data set generation alternatives, we also evaluated the performance of the same AL/ML-based schemes with different data sets. The corresponding evaluation results are shown in Table 2 as below:

Table 2: Positioning accuracy achieved for 90% UEs with different data sets

| Accuracy achieved @90% | AI + DL-TDOA | AI + DL-TDOA + RSRP | AI + TOA | AI + TOA + RSRP | AI + Normalized CIR | AI + Normalized CIR + RSRP |
| --- | --- | --- | --- | --- | --- | --- |
| Alt.1: 80,000 drops, 1 UE per drop | 6.45m | 5.11m | 5.39m | 4.52m | 5.23m | 4.35m |
| Alt.2: 1 drop, 80,000 UEs per drop | 0.73m | 0.61m | 0.69m | 0.59m | 0.46m | 0.42m |

From Table 2, we can see that for each AI-based scheme, the positioning error with data set generated by Alt.1 is about 10 times of that with data set generated by Alt.2.

Observation 2: For the same AI-based method, there is larger performance difference, in terms of positioning error at 90%, between the AI modes trained via the different data sets generated by different alternatives.

Therefore, in order to keep the consistency among companies and ensure the results are comparable, we have the following proposal:

Proposal 4: For the evaluation of AI/ML-based positioning accuracy improvement, the data sets of companies should be generated with the same values of the following parameters:

- X drops

- Y UEs per drop

- FFS: values of X, Y

Regarding the performance of different AI-based schemes, we can see that the performance of different AI-based schemes are comparable, no matter the data sets are generated by Alt.1 or Alt.2.  To be specific, the scheme “AI+DL-TDOA + RSRP” has be worst performance, but with very limited performance gap with the best one “AI+ Normalized CIR + RSRP”, e.g., 5.11m vs 4.23m or 0.61m vs 0.42m.

Observation 3: No matter the data sets are generated by Alt.1 or Alt.2, the schemes “AI+DL-TDOA+RSRP”, “AI + TOA+RSRP” and “AI+ Normalized CIR + RSRP” have comparable performance in terms of positioning accuracy.

From the above results, we can also see the impact of RSRP measurements on the positioning accuracy. For each type of AI-based method, jointly utilizing RSRP can always offer some performance improvement, e.g., the method “AI + Normalized CIR + RSRP” has 0.88m or 0.04m performance gain over “AI + Normalized CIR” for the different data sets. The reason is that RSRP measurement results can provide additional information, which can be exploited by the AI models to further improve the positioning accuracy.

Observation 4: For each type of AI-based method, joint utilization of RSRP can offer some performance improvement.

The CDF curves for the positioning accuracy performance of AI-based schemes are shown in Fig.3 and Fig.4. The results in Fig. 3 are corresponding to the case where the date sets are generated by 80,000 drops and 1 UE per drop, whereas those in Fig.4 are corresponding to the case where the date sets are generated by 1 drop and 80,000 UEs in this drop.  From Fig.3 and Fig.4, we can see that Observation 3 and Observation 4 are both applicable as well.

Fig.3: CDF of Positioning Accuracy for AI-based methods (Alt.1: 80,000 drops, 1 UE per drop)

Fig.4: CDF of Positioning Accuracy for AI-based methods (Alt.2: 1 drop, 80,000 UEs per drop)

Complexity of AI models

In our initial assessment on the complexity of AI models, FLOP is used as a rough metric. The corresponding complexity of the AI modes used in our evaluations are summarized in Table 3.

Table 3: Complexity of AI modes for different AI-based schemes

|  | AI + DL-TDOA | AI + DL-TDOA + RSRP | AI + TOA | AI + TOA + RSRP | AI + Normalized CIR | AI + Normalized CIR + RSRP |
| --- | --- | --- | --- | --- | --- | --- |
| MFLOPs | ~ 0.15 | ~ 0.15 | ~ 0.15 | ~ 0.15 | ~ 2.96 | ~ 2.96 |

For the first four AI models are based on fully connected neural network and only have very small differences, e.g., the input size. Therefore, the numbers of FLOP are almost the same for them. In contrast, the last two AI modes are based on ResNet and their complexity are much larger than that of the first four AI modes.

- Conclusions

In this contribution, we discussed AI based positioning accuracy enhancement from the different aspects including target scenario(s), AI-based schemes, generation of data sets, AI modes used in evaluation. We also present our initial evaluations on the positioning accuracy of AI-based methods and complexity of different AI modes as well. Based on the discussion and evaluation results, we have the following observations and proposals.

Observation 1: For InF-DH scenario with clutter density of 0.6, if the data sets are generated by Alt.2 (i.e., 1 drop and 80,000 UEs per drop), the positioning accuracy achieved at 90% for different methods are as below

- Non-AI-based DL-TDOA: 12.35m

- AI + DL-TDOA + RSRP: 0.61m

- AI + Normalized CIR + RSRP: 0.42m

Observation 2: For the same AI-based method, there is larger performance difference, in terms of positioning error at 90%, between the AI modes trained via the different data sets generated by different alternatives.

Observation 3: No matter the data sets are generated by Alt.1 or Alt.2, the schemes “AI+DL-TDOA+RSRP”, “AI + TOA+RSRP” and “AI+ Normalized CIR + RSRP” have comparable performance in terms of positioning accuracy.

Observation 4: For each type of AI-based method, joint utilization of RSRP can offer some performance improvement.

Proposal 1: For the evaluation of AI/ML-based positioning accuracy improvement, support the InF-DH scenario with the high clutter density set as 60%.

Proposal 2: Regarding the positioning accuracy, reuse the following metrics for the evaluation:

- CDF of the positioning accuracy

- Achieved accuracy at 90%

Proposal 3: Regarding the complexity of AI/ML-based method, use FLOPs as the metric at the initial stage

Proposal 4: For the evaluation of AI/ML-based positioning accuracy improvement, the data sets of companies should be generated with the same values of the following parameters:

- X drops

- Y UEs per drop

- FFS: values of X, Y

- Reference

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- TR 38.857, Study on NR Positioning Enhancements

Appendix: Evaluation assumptions

| Parameter | Value |
| --- | --- |
| Scenario | 1nF-DH 120m * 60m |
| {clutter density, height, size} | {0.6,6m,2m} |
| Bandwidth | 100M |
| Total gNB TX power, dBm | 24dBm |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ |
| UE antenna height | Baseline: 1.5m |
| Carrier frequency, GHz | 3.5GHz |
| Subcarrier spacing, kHz | 30kHz for 100MHz |
| CIR | 256 complex values |
| Data sets | Total: 80,000 Training: 98% of 80,000 Testing: 2% of 80,000 |
| Timing error | No timing error (e.g,, Rx/Tx timing group delay) Perfect synchronization |
| Spatial consistency for small scale parameters | NO |