3GPP TSG RAN WG1 #109-e	R1-2204023

e-Meeting, May 9th – May 20th, 2022

Source:	OPPO

Title:	Discussion on potential enhancements on dynamic/flexible TDD

Agenda Item:	9.3.3

Document for:	Discussion and Decision

# Introduction

In RAN 94e meeting, study on evolution of NR duplex operation was approved [1] and the objective of SI is shown in the following:

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

This paper discusses potential enhancements on dynamic/flexible TDD, especially the inter-gNB and inter-UE CLI issues.

# Discussion

In Rel-14 NR SI, duplexing flexibility with cross-link interference mitigation shows better user throughput compared to static UL/DL operation or dynamic UL/DL operation without interference mitigation in indoor hotspot (4GHz and 30GHz) and urban macro scenarios (4GHz and 2GHz). During Rel-15 NR WI, enablers for basic support of cross-link interference mitigation schemes to support duplexing flexibility for paired and unpaired spectrum were discussed but could not specified as the work has been deprioritized. Rel-15 NR specification supports mechanisms to allow dynamic DL/UL assignments. Yet, any cross-link interference mitigation techniques and coexistence requirements have not been specified and thus the use of dynamic DL/UL assignment operation is considerably restricted.

In R16 WID on Cross Link Interference handling and Remote Interference Management for NR (NR_CLI_RIM), some inter-UE CLI handling was introduced [2]

- gNBs can exchange and coordinate their intended TDD DL-UL configurations over Xn and F1 interfaces;

- The victim UEs can be configured to perform CLI measurements. There are two types of CLI measurements:

  - SRS-RSRP measurement in which the UE measures SRS-RSRP over SRS resources of aggressor UE(s);

  - CLI-RSSI measurement in which the UE measures the total received power observed over RSSI resources.

- Layer 3 filtering applies to CLI measurement results and both events triggered and periodic reporting are supported.

Observation 1: R16 inter-UE CLI handling remains applicable.

Inter-remote gNB CLI handling were also discussed in NR_CLI_RIM, however, this item tries to solve interference from remote gNB caused by atmospheric ducting phenomenon. RIM framework, including RIM- RS and related procedure, was introduced for coordination between victim and aggressor gNBs.  This framework is not applicable for inter-adjacent gNB CLI.

For inter-adjacent gNB CLI, the following mitigation techniques can be considered: coordinated scheduling/beamforming and power control. For efficient coordination, interference measurement and resource configuration exchange are prerequisite, especially for inter-operator gNB CLI. So, mechanism on interference measurement and resource configuration exchange can be studied. Note that intended TDD DL-UL configurations exchange over Xn and F1 interfaces has been introduced in R16 inter-UE CLI, which can be a starting point.

Proposal 1: For inter-gNB CLI handling, especially for inter-operator gNB CLI, mechanism on interference measurement and resource configuration exchange should be studied.

Dynamic/flexible TDD could provide flexible resource allocation in time domain to adapt with flexible uplink and downlink traffic. And dynamic/flexible TDD could allocate more uplink or downlink resource to improve uplink or downlink coverage. However, when more uplink symbol/slot is allocated for uplink coverage improvement, downlink coverage and throughput are impacted due to downlink resource with whole bandwidth reduces. Subband non-overlapping full duplex have similar benefit as dynamic/flexible TDD and be more applicable for coverage improvement due to downlink/uplink resource configuration is more delicate in frequency domain. So for these two functions to solve the same issue, the SI should clarify whether to support dynamic/flexible TDD and subband non-overlapping full duplex jointly.

- (b)

Figure 1 CLI for dynamic/flexible TDD without (a)/with(b) subband non-overlapping full duplex

Figure 1(a) shows interference in case that dynamic/flexible TDD runs alone in two neighboring cells. Figure 1 (b) shows interference in case that dynamic/flexible TDD and subband non-overlapping full duplex runs in two neighboring cells. Comparing to the cells in Figure 1(a), where the CLI handling could be unified across the whole bandwidth, the cell running full-band UL with dynamic TDD in Figure 1(b) may observe different inter-subband CLI statistics, such as strength, across different subbands. So, if dynamic/flexible TDD can run together with subband non-overlapping full duplex, interference mitigation on inter-gNB CLI from adjacent sub-band and inter-UE CLI from adjacent subband need to be studied.

Whether dynamic/flexible TDD runs together with subband non-overlapping full duplex may also impact the design of inter-gNB coordination. Assume a cell running dynamic/flexible TDD is the aggressor cell and a neighboring cell running full-duplex is the victim cell, the question is whether the gNB of aggressor cell should be aware that the victim cell runs full-duplex, for example, whether the X2 signaling coming from the victim cell gNB could include both information for DL impacts and information for UL impacts (comparing to information not distinguishing between DL and UL). In other words, the question is whether the full duplex should be made transparent to the Rel-18 study of gNB-side enhancement for legacy dynamic/flexible TDD. Note that full duplex should be transparent to UE within any Rel-18 enhancement study.

Proposal 2: It should be clarified whether the dynamic/flexible TDD (from UE/gNB perspective) and subband non-overlapping full duplex (from gNB-only perspective) should be assumed to operate together.

- If yes, interference mitigation to handle inter-gNB/inter-UE CLI from adjacent full-duplex sub-band need to be studied.

- If yes, it should be further determined whether Rel-18 study of dynamic/flexible TDD should assume full-duplex being transparent to gNB-side enhancement (if any) for dynamic/flexible TDD.

# Conclusions

In this contribution, we discuss cross link interference handling for dynamic/flexible TDD with the following observations and proposals:

Observation 1: R16 inter-UE CLI handling remains applicable.

Proposal 1: For inter-gNB CLI handling, especially for inter-operator gNB CLI, mechanism on interference measurement and resource configuration exchange should be studied.

Proposal 2: It should be clarified whether the dynamic/flexible TDD (from UE/gNB perspective) and subband non-overlapping full duplex (from gNB-only perspective) should be assumed to operate together.

- If yes, interference mitigation to handle inter-gNB/inter-UE CLI from adjacent full-duplex sub-band need to be studied.

- If yes, it should be further determined whether Rel-18 study of dynamic/flexible TDD should assume full-duplex being transparent to gNB-side enhancement (if any) for dynamic/flexible TDD.

# Reference

# [1] RP-213591 Study on evolution of NR duplex operation CMCC (Moderator)

# [2] 3GPP TS 38.300 NR and NG-RAN Overall Description Stage 2 v17.0.0