3GPP TSG RAN WG1 #109-e		R1-2204028

e-Meeting, May 9th – 20th, 2022

Source:	OPPO

Title:	Discussion on XR specific power saving techniques

Agenda Item:	9.11.1

Document for:	Discussion and Decision

# Introduction

A study item of study on XR enhancements for NR was approved in RAN#94e with the following objectives [1]:

Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement.

In this contribution, we discuss XR specific power saving techniques.

# Discussion

## XR traffic characteristics

In 3GPP, there have been lots of work on XR and CG on various aspects, e.g., use case, service architectures, service flow, traffic characteristics, performance requirement, gap analysis and so on. TR 26.928 collects information on XR in the context of 5G radio and network services, where use cases and device types are classified, and processing and media centric architectures are introduced.  And in Rel-17 XR SI, the traffic model of XR and CG was widely discussed. From the agreed traffic model of XR and CG, we could see that the XR and CG has the following characters:

- High data rate, up to 60Mbps with limited latency, around 10-30ms

- Non-integer period with jitter

- Large and various packet size

Observation 1: XR and CG have the following characters:

- High data rate, up to 60Mbps with limited latency, around 10-30ms

- Non-integer period with jitter

- Large and various packet size

## C-DRX enhancements

- Non-integer period

As agreed in Rel-17 XR SI, the packet arrival rate of XR are {30, 60, 90, 120} fps, so the corresponding traffic periodicities of XR are {33.33, 16.67, 11.11, 8.33}ms. For convenience, the DRX cycle can be configured as {10ms, 20ms, 32ms, s40ms, 60ms, 64ms…} if long DRX cycle is applied while the candidate cycle values for short DRX cycle are {2ms, 3ms, 4ms,5ms, 6ms…}. Obviously, there exists mismatch between the traffic arrival interval and the DRX cycle. Even if the DRX cycle is configured with finer granularity and more candidate cycle values are provided, the mismatch problem cannot be completely solved since the arrival interval is not divisible by the time duration of radio resource allocation unit, e.g. symbol. The mismatch would lead to larger latency for data scheduling and further reduce the XR/CG capacity. Figure 1 illustrates the mismatch issue between DL video with 60bps and 16ms DRX cycle with 2ms on duration timer. It can be observed that with the following DRX configuration, the mismatch issue appears every three traffic arrival intervals. This means, the mismatch problem occurs with a certain pattern which relates to the fps of DL video and DRX configuration including DRX cycle, length of on duration timer.

Figure 1: Mismatch between DL video @60bps and 16ms DRX cycle with 2ms on duration timer

Observation 2: There exists periodicity mismatch issue between XR traffic arrival and DRX cycle, which may lead to larger scheduling latency.

One solution is to config two DRX cycles in one DRX configuration and the two cycles are applied in a semi-static pattern. Take Figure 2 as an example, the traffic arrival interval and DRX config is the same as above, except that two DRX cycles are configured: 17ms and 16ms. Then the two DRX cycles are applied in a pattern of {17ms 17ms 16ms}, in this way, the issue of periodicity mismatch can be solved.

Figure 2: Two DRX cycles are applied in a semi-static pattern {17ms, 17ms, 16ms}

Similar mechanism can be designed for drx-startoffset, e.g. change drx-startoffset periodically. As shown in Figure 3, the traffic arrival interval and DRX config is the same as above, the periodicity mismatch would appear after three cycles, then the drx-startoffset can be changed every 3 cycles to keep aligned with traffic pattern.

Figure 3: drx-startoffset change periodically

Proposal 1: The following DRX configuration enhancements can be further studied to solve periodicity mismatch issue:

- Configure a DRX cycle pattern with two cycle values;

- Change drx-startoffset periodically.

- Jitter handling

There exists jitter characteristic for XR traffic arrival. According to the previous RAN1 agreements, the jitter can be modeled as truncated Gaussian distribution with varying range of [-4,4] ms (baseline) or [-5,5] ms (optional). If the mismatch between arrival interval and DRX cycle can be regarded as some kind of semi-static mismatch, the jitter impact would be lead to dynamic mismatch between data arrival and UE on duration.

One way to solve the jitter problem is to config the DRX on duration timer long enough to cover the jitter range. Then no matter when the packet is arrived, UE maintains awake for data reception. However, considering the jitter range is relatively large, the DRX on duration timer needs to be configured much longer, which is unfavorable for UE power saving. In Rel-17 power saving WI, layer-1 based dynamic PDCCH monitoring reduction schemes, i.e. the PDCCH skipping and search space group switching are being discussed and these schemes can be applied if long DRX on duration timer is configured.

Proposal 2: Layer-1 based power saving schemes discussed in Rel-17 power saving WI should be the starting point for further Rel-18 XR study.

Another way to solve jitter problem is to config DRX on duration timer with a small value. However, with smaller on duration timer, the packet would be possible to arrive too early or too late. If the packet arrives too early, that is, the UE is still outside active time when packet arrives, then the packet can not be scheduled until UE wakes up. This would bring additional latency for the packet and may result in packet loss within the required time budget. While if the packet arrives too late, that is, UE has been complete its on duration time and no longer stay awake for PDCCH monitoring. In such a case, not only UE would perform unnecessary PDCCH monitoring during on duration timer, but also the late arrived packet cannot get timely scheduling and need to wait for the next on duration timer. This increases the end to end latency more significantly compared to early packet arrival because a reasonable DRX configuration is to configure the short DRX on duration timer to cover the most central area of the jitter range. This makes the delay between packet arrival time and next on duration timer caused by late arrival (delay 2 in Figure 4) much longer than that of early arrival (delay 1 in Figure 4), so late arrival issue results in more serious problem.

Figure 4: jitter issue when short DRX on duration timer is configured

Observation 3: If DRX on duration timer is configured not long enough to cover jitter range, there would exist early/late packet arrival issue, and late packet arrival would lead to longer scheduling latency and additional power consumption.

One solution to cope with late packet arrival issue is to trigger an additional on duration time or extending the current on duration time dynamically. As shown in Figure 5, there is no XR data arriving in the original on duration time, gNB can send a dynamic signaling to UE to indicate another on duration time, in which the UE can perform PDCCH monitoring for the XR traffic. By the way, gNB can also indicate the proper SSSG to monitor.

Figure 5 Additional on duration time duration DRX off

Proposal 3: An additional DRX-on duration time can be dynamically indicated to UE to solve jitter issue.

## Discussion on PDCCH monitoring enhancements

Although XR and CG traffic is quasi-periodic, the packet arrival time and packet size are various to some extent. So dynamic scheduling could also be considered to support XR for more efficient transmission. For dynamic scheduling, resource allocation is very flexible, however, the scheduling opportunity is restricted by PDCCH monitoring occasions which are determined by the configured monitoringSlotPeriodicityAndOffset, as well as duration for the search space sets.

- Non-integer period

Similar with DRX, there also exists mismatch issue between the periodicity of search space sets and that of XR/CG packet. So a unified mechanism for both DRX and search space set is preferred.

Proposal 4: The following search space set configuration enhancements can be further studied to solve periodicity mismatch between XR traffic and search space monitoring:

- Configure a monitoring periodicity pattern with two periodicity values;

- Change monitoring offset periodically.

- Jitter and various packet size

Similar with DRX, there also exists jitter issue for search space set configuration. To be specific, the packet would be possible to arrive out of the duration of the search space set, then the same early arrival and late arrival may happen, leading to additional scheduling delay or unnecessary power consumption.

In addition, as discussed above, the XR packet size is not fixed. Taking AR/VR 30Mbps as example, maximum packet size is 93750 bytes but minimum packet size is 31250 bytes. Obviously, there exists large gap between maximum packet size and minimum packet size. So different XR packets may need different number of slots for transmission and hence different time length of PDCCH monitoring is needed. In such a case, a fixed “duration” value for the search space set cannot work well since it fails to match the various packet size.

One solution to cope with both jitter problem and various packet size issue is to change the value of monitoringSlotPeriodicityAndOffset and duration on the fly. Take Figure 6 as an example, search space set #1 is configured with 16 slots periodicity, 1 slot offset and 2 slot duration, search space set #2 is configured with 10 slots periodicity, 12 slot offset and 2 slot duration. If the packet size is very large and 2 slots are not enough for the scheduling, gNB can indicate UE to extend the duration range via dynamic signaling. If the packet arrives out of the duration time, gNB can also dynamically indicate UE to adjust the monitoring offset. Details can be further studied such as the DCI format used for dynamic indication, the effective time of the dynamic indication and so on.

Figure 6: dynamic change of monitoringSlotPeriodicityAndOffset and duration

Proposal 5: The value of monitoringSlotPeriodicityAndOffset and duration for a search space set can be changed dynamically by gNB to cope with jitter and packet size variation.

# Conclusions

In this contribution, we discuss XR specific power saving techniques and following observation and proposals are made:

Observation 1: XR and CG have the following characters:

- High data rate, up to 60Mbps with limited latency, around 10-30ms

- Non-integer period with jitter

- Large and various packet size

Observation 2: There exists periodicity mismatch issue between traffic arrival and DRX cycle, which may lead to larger scheduling latency.

Observation 3: If DRX on duration timer is configured not long enough to cover jitter range, there would exist early/late packet arrival issue, and late packet arrival would lead to longer scheduling latency and additional power consumption.

Proposal 1: The following DRX configuration enhancements can be further studied to solve periodicity mismatch issue:

- Configure a DRX cycle pattern with two cycle values;

- Change drx-startoffset periodically.

Proposal 2: Layer-1 based power saving schemes discussed in Rel-17 power saving WI should be the starting point for further Rel-18 XR study.

Proposal 3: An additional DRX-on duration time can be dynamically indicated to UE to solve jitter issue.

Proposal 4: The following search space set configuration enhancements can be further studied to solve periodicity mismatch between XR traffic and search space monitoring:

- Configure a monitoring periodicity pattern with two periodicity values;

- Change monitoring offset periodically.

Proposal 5: The value of monitoringSlotPeriodicityAndOffset and duration for a search space set can be changed dynamically by gNB to cope with jitter and packet size variation.

# References

- RP-213587 New SID “Study on XR Enhancements for NR” Nokia