3GPP TSG-RAN WG1 Meeting #109-e	R1-2204030

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.1.3

Source:	Ericsson

Title:	Remaining issues on CSI enhancements for Multi-TRP

Document for:	Discussion, Decision

# 1	Introduction

In this contribution we discuss remaining issues on CSI enhancements for Multi-TRP.

# 2	Discussion

There are still a few issues on multi-TRP CSI, which have ASN.1 impact. These should be settled as soon as possible.

In an incoming RAN2 LS, RAN2 asked the following question:

Issue 3: CSI-mTRP

RAN2 introduced 2 types of RI restrictions and two codebook subset restrictions (CBSR) per CodebookConfig. However, it is not clear how those features are enabled: Currently, same as in previous releases, RAN2 signalling assumes both RI restrictions and CBSR are configured simultaneously, but RAN2 would like to verify this is the correct assumption for the signallling.

Question 4: Which of the following assumptions are correct?

- If two RI restrictions are configured, two CBSRs are configured and if two CBSRs are configured two CBSRs are configured (i.e. when two are configured for either RI restriction or CBSR, two are also configured for the other).

- UE can be configured with either RI restriction for sTRP or RI restriction for NCJT, but not both at the same time.

- If two CBSRs are configured, two CMR groups are configured and if two CMR groups are configured, two CBSRs are configured (i.e. when two are configured for either CBSR or CMR groups, two are also configured for the other and there cannot be configuration of e.g. one CBSR but two CMR groups)

With regards to the first question, it is not necessary that two RI restrictions are configured if two CBSRs are configured.  For instance, when CSI report mode 1 with X=0 is configured in a CSI Report Config, then the UE only reports NC-JT CSI.  In this case, it is sufficient that only the RI restriction corresponding to NC-JT (i.e., typeI-SinglePanel-ri-RestrictionSDM-r17) is configured while two CBSRs are configured.  When CSI report mode 1 with X = 1 or 2, or in the case of CSI report mode 2, configuration of two RI restrictions with two CBSRs is a valid configuration.

Proposed Answer 4-1: It is not necessary that two RI restrictions are configured if two CBSRs are configured.  For instance, when CSI report mode 1 with X=0 is configured in a CSI Report Config, then the UE only reports NC-JT CSI. In this case, it is sufficient that only the RI restriction corresponding to NC-JT (i.e., typeI-SinglePanel-ri-RestrictionSDM-r17) is configured while two CBSRs are configured.  When CSI report mode 1 with X = 1 or 2, or in the case of CSI report mode 2, configuration of two RI restrictions with two CBSRs is a valid configuration.

Regarding the second question, it is possible that the UE can be configured with any of the following: (1) RI restriction for sTRP, (2) RI restriction for NCJT, (3) RI restrictions for both sTRP and NCJT.  For instance, when CSI report mode 1 with X = 0 is configured, then it is enough to configure the RI restriction for NCJT.  When CSI report mode 1 with X = 1 or 2 or when CSI report mode 2 is configured, then RI restrictions for both sTRP and NCJT can be configured.

Proposed Answer 4-2:  It is possible that the UE can be configured with any of the following: (1) RI restriction for sTRP, (2) RI restriction for NCJT, (3) RI restrictions for both sTRP and NCJT.  For instance, when CSI report mode 1 with X = 0 is configured, then it is enough to configure the RI restriction for NCJT.  When CSI report mode 1 with X = 1 or 2 or when CSI report mode 2 is configured, then RI restrictions for both sTRP and NCJT can be configured.

Regarding the third question, the following are valid:  (1) if two CBSRs are configured, two CMR groups are configured, and (2) if two CMR groups are configured, two CBSRs are configured.

Proposed Answer 4-3:  the following are valid:  (1) if two CBSRs are configured, two CMR groups are configured, and (2) if two CMR groups are configured, two CBSRs are configured.

# 3	Conclusion

Regarding question 4 on the incoming LS from RAN2 regarding ASN.1 issues, we propose the following responses:

Proposed Answer 4-1: It is not necessary that two RI restrictions are configured if two CBSRs are configured.  For instance, when CSI report mode 1 with X=0 is configured in a CSI Report Config, then the UE only reports NC-JT CSI. In this case, it is sufficient that only the RI restriction corresponding to NC-JT (i.e., typeI-SinglePanel-ri-RestrictionSDM-r17) is configured while two CBSRs are configured.  When CSI report mode 1 with X = 1 or 2, or in the case of CSI report mode 2, configuration of two RI restrictions with two CBSRs is a valid configuration.

Proposed Answer 4-2:  It is possible that the UE can be configured with any of the following: (1) RI restriction for sTRP, (2) RI restriction for NCJT, (3) RI restrictions for both sTRP and NCJT.  For instance, when CSI report mode 1 with X = 0 is configured, then it is enough to configure the RI restriction for NCJT.  When CSI report mode 1 with X = 1 or 2 or when CSI report mode 2 is configured, then RI restrictions for both sTRP and NCJT can be configured.

Proposed Answer 4-3:  the following are valid:  (1) if two CBSRs are configured, two CMR groups are configured, and (2) if two CMR groups are configured, two CBSRs are configured.