3GPP TSG-RAN WG1 Meeting #109-e	Tdoc R1-2204034

e-Meeting, May 9th–20th, 2022

Agenda Item:	9.1.1.2

Source:	Ericsson

Title:	Two TAs for multi-DCI

Document for:	Discussion

# 1	Introduction

During RAN#94e, a new WID for Rel-18 MIMO evolution for DL and UL was agreed [1].  Part of objective 7 is relevant for this AI:

- Study, and if justified, specify the following

  - Two TAs for UL multi-DCI for multi-TRP operation

  - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed.

For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

For completeness, the full objective is included. The part that is not applicable for this AI is written in italics.

In this contribution, we discuss how to support two TAs for multi-DCI based multi-TRP operation.

# 2	Discussion

In this contribution, we discuss how to specify two TAs for multi-DCI multi-TRP operation.

The focus of the mTRP improvements in Rel-18 is on extension of the Rel-17 unified TCI framework to mTRP operation. Interestingly, the only sub-objective where this is not mentioned is for two TAs for UL multi-DCI multi-TRP operation. In our understanding, this is an oversight, and we propose to clarify this:

Two TAs for UL multi-DCI for multi-TRP operation is only supported for the extension of the Rel-17 unified TCI framework.

We should note that enhancements to TA handling is also part of the WI Further NR Mobility Enhancements [2]. It would be beneficial if the solutions agreed in the two Wis are aligned:

Strive for aligned solutions for TA adjustment in the Rel-18 MIMO and mobility WIs.

## 2.1	UL Tx timing adjustment in NR

Different UEs in a same serving cell may be located at different locations within the cell and thus, have different distances to the base station. A UE acquires DL slot and symbol timing based on a SSB during cell search and transmits in the UL a PRACH preamble towards the base station using the DL timing as a reference.  Due to round trip propagation delay, the PRACH will be received at the base station with a time offset with respect to the expected UL timing at the base station. A timing correction is then sent from the base station to the UE in a RACH response message (RAR) for the UE. The timing correction is referred to as a timing advance (TA), which is used to compensate the round-trip propagation delay such that the subsequent UL channels or signals can reach the base station at the desired UL slot or symbol time.

The timing advance command included in RAR indicates NTA values by index values of TA = 0, 1, 2, ..., 3846. For UL transmissions following the reception of the RAR, the UE applies the timing advance , where ,  Δfmax=480∙103 Hz, , and is provided using higher layer signaling..

An example is shown in Figure 1, where to achieve UL time alignment the base station a time advance of NTA=2τ is needed for UL transmissions at the UE with respect to the DL timing at the UE to compensate the UL time offset due to the propagation delay τ.

…

Slot n+0

Slot n+1

…

Slot n+k

Slot n+0

Slot n+1

…

Slot n+k

Slot n+0

Slot n+1

…

Slot n+k

Slot n+k

τ

NTA=2τ

DL:

UL:

…

…

DL:

UL:

At base station:

At the UE:

time

…

τ

Propagation delay

UE

Base station

Slot n+0

Slot n+1

…

Figure 1 Time alignment of uplink transmissions with timing advance.

In addition to the initial TA adjustment carried in the RAR, regular TA adjustment is performed using a time advance command (TAC) MAC CE as shown in Figure 2 where it consists of

- TAG Identity (TAG ID): This field indicates the TAG Identity of the addressed TAG. The TAG containing the SpCell has the TAG Identity 0. The length of the field is 2 bits;

- Timing Advance Command: This field indicates the index value TA (0, 1, 2… 63) used to control the amount of timing adjustment that the MAC entity has to apply. The length of the field is 6 bits.

Figure 2 Timing Advance Command MAC CE [3]

At the reception of the TAC MAC CE, the UE performs a relative adjustment of the current NTA value, NTA_old, to the new NTA value, NTA_new, by index values of TA = 0, 1, 2,..., 63, where NTA_new=NTA_old+TA-31∙16∙642μ.

The TAC MAC CE contains a reference to a timing advance group (TAG), and the UE maintains one NTA value for each TAG.  Each serving cell is configured with a TAG, and all UL transmissions in that serving cell use the timing advance corresponding to that TAG

## 2.2	Associating different TAs with different UL transmissions

In Rel-17, all UL transmissions in one serving cell use the same TA: since only one TAG is configured for each serving cell, any update of the TA using the MAC CE TAC will affect all transmissions in that serving cell. This restriction must be relaxed to support two TAs for multi-TRP, since all the multi-TRP transmissions are still associated with the same serving cell, even when the different PCIs are associated with the different TRPs:

Even in “inter-cell” mTRP, all the UL transmissions use the same TA.

In objective 2, the target is to remove the above restriction, and allow for different TAs for transmissions to different TRPs in mDCI mTRP operation. We note that every UL transmission is associated with a joint UL/DL or UL TCI state, and that transmissions to different TRPs must be associated with different joint UL/DL or UL TCI states:

UL transmissions to different TRPs will be associated with different joint UL/DL or UL TCI states.

One natural solution is to include a TAG in each joint UL/DL or UL TCI state. Any UL transmission that relies on a TCI state would apply the TA associated with the TAG included in the TCI state.

Include a TAG in each joint or UL TCI state.

Moreover, the TCI state will contain a DL reference signal, either the RS that defines the spatial UL TX filter, or the PL RS. This DL RS could be used to define the timing reference for the corresponding UL transmission.

The timing reference for any UL transmission is the reference signal providing the UL TX spatial filter. If the RS providing the UL spatial TX spatial filter is an SRS, the PL RS for the UL TCI state provides the timing reference.

In other words, the UE would apply the TA associated with the TAG in a TCI state to the receive timing of the DL RS in the same TCI state. The resulting Tx timing is used for the PUCCH, PUSCH and SRS for which the corresponding TCI state is indicated.

One potential issue is that in Rel-17, UL transmissions may not be associated with TCI states in FR1. However, there is nothing that prevents that UL transmissions are associated with TCI states also in FR1: for example, power control parameters can still be associated with TCI states in FR1 for Rel-17. In fact, as shown in [4], it becomes necessary to associate UL transmissions with TCI states as soon as we want to support UL power control for mTRP using the unified TCI framework. Hence, we observe

UL transmissions can be associated with TCI states also in FR1.

We note that the Rel-17 TCI states are a part of the solutions for L1/L2 mobility in Rel-17, and it is quite likely that the Rel-18 mobility enhancements will be based on the Rel-17 TCI states. Including the TA in a Rel-17 TCI state makes it likely that the method can be reused also for the mobility enhancements:

A solution that integrates the TA with the Rel-17 TCI framework can be reused also for mobility enhancements.

In contrast, a solution based on CORESET pool index could not be reused in the mobility work, since mobility must work also without NC-JT.

## 2.3	TA update

We recall that the legacy mechanism to update the TA is related to the TAG. The MAC CE shown in Figure 2 include the TAG, and the corresponding MAC CE updates the TA associated with the TAG. Since the TAG concept is fully reuse, we realize that we can reuse the legacy mechanism for TA update:

As a starting point, reuse the legacy MAC CE to update the TA per timing advance group.

As described in the background, the TA remains valid until the TA timer associated with the TAG expires. When the corresponding TA timer expires, the UE is not allowed to transmit PUCCH, PUSCH or SRS for the serving cell configured with the corresponding TAG. When the timer expires, the UE is only allowed to transmit PRACH on the serving cell.

This concept can easily be extended to the case where the TAG is included in a TCI state: transmission of PUCCH, PUSCH and SRS is allowed only if the timer associated with the TAG in the corresponding TCI state has not expired:

Transmission of PUCCH, PUSCH and SRS is only allowed if the timer associated with the TAG in the corresponding TCI state has not expired.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Strive for aligned solutions for TA adjustment in the Rel-18 MIMO and mobility WIs.

Observation 2	Even in “inter-cell” mTRP, all the UL transmissions use the same TA.

Observation 3	UL transmissions to different TRPs will be associated with different joint UL/DL or UL TCI states.

Observation 4	UL transmissions can be associated with TCI states also in FR1.

Observation 5	A solution that integrates the TA with the Rel-17 TCI framework can be reused also for mobility enhancements.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Two TAs for UL multi-DCI for multi-TRP operation is only supported for the extension of the Rel-17 unified TCI framework.

Proposal 2	Include a TAG in each joint or UL TCI state.

Proposal 3	The timing reference for any UL transmission is the reference signal providing the UL TX spatial filter. If the RS providing the UL spatial TX spatial filter is an SRS, the PL RS for the UL TCI state provides the timing reference.

Proposal 4	As a starting point, reuse the legacy MAC CE to update the TA per timing advance group.

Proposal 5	Transmission of PUCCH, PUSCH and SRS is only allowed if the timer associated with the TAG in the corresponding TCI state has not expired.

# References

RP-213598, Revised WID: MIMO evolution for downlink and uplink, Samsung, RAN#94-e, Dec 2021

RP-213565, New WID on Further NR mobility enhancements, MediaTek, RAN#94-e, Dec 2021

TS 38.321, NR; Medium Access Control (MAC) protocol specification v 17.0.0, RAN#95-e, March 2022

R1-2204033, Unified TCI framework extension for multiple TCI states, Ericsson, RAN1#109-e, May 2022