3GPP TSG-RAN WG1 Meeting #109-e	Tdoc R1-2204035

e-Meeting, May 9th – 20th, 2022

Agenda Item:	5

Source:	Ericsson

Title:	Discussion of BWP operation without bandwidth restriction

Document for:	Discussion

# 1	Introduction

In [1], RAN1 received an LS from RAN2 regarding BWP operation without bandwidth restriction. RAN2 asks the following questions:

Question 1:

Whether it is a valid scenario in the standard to support the operation of BWP without SSB where the UE does not perform BM/RLM/BFD due to the lack of necessary reference signal (SSB and CSI-RS) in the active BWP.

Question 2:

If the answer to question 1 is that this is not valid, how should the UE perform BM/RLM/BFD when the active BWP does not contain SSB.

In this contribution, we discuss the questions from RAN2.

# 2	Discussion

The UE may indicate support of BWP operation without restriction using FG6-1a (field name: bwp-WithoutRestriction). In this case, the UE should be able to operate in a BWP that does not contain CORESET#0 or SSB.

For BM/RLM/BFD operation on DL BWPs NOT containing the SSB associated to the initial DL BWP, the following text in TS38.300 suggests that CSI-RS based measurements are used.

9.2.3.1 Overview […] SSB-based Beam Level Mobility is based on the SSB associated to the initial DL BWP and can only be configured for the initial DL BWPs and for DL BWPs containing the SSB associated to the initial DL BWP. For other DL BWPs, Beam Level Mobility can only be performed based on CSI-RS. 9.2.7 Radio Link Failure […] SSB-based RLM is based on the SSB associated to the initial DL BWP and can only be configured for the initial DL BWP and for DL BWPs containing the SSB associated to the initial DL BWP. For other DL BWPs, RLM can only be performed based on CSI-RS. 9.2.8 Beam failure detection and recovery […] SSB-based Beam Failure Detection is based on the SSB associated to the initial DL BWP and can only be configured for the initial DL BWPs and for DL BWPs containing the SSB associated to the initial DL BWP. For other DL BWPs, Beam Failure Detection can only be performed based on CSI-RS.

This is also in accordance with RAN1 specifications. We will look at RLM, BFD and BM separately.

Excerpt from 38.213, Clause 5:

The downlink radio link quality of the primary cell is monitored by a UE for the purpose of indicating out-of-sync/in-sync status to higher layers. The UE is not required to monitor the downlink radio link quality in DL BWPs other than the active DL BWP, as described in clause 12, on the primary cell. If the active DL BWP is the initial DL BWP and for SS/PBCH block and CORESET multiplexing pattern 2 or 3, as described in clause 13, the UE is expected to perform RLM using the associated SS/PBCH block when the associated SS/PBCH block index is provided by RadioLinkMonitoringRS.

<omitted>

If the UE is configured with a SCG, as described in [12, TS 38.331], and the parameter rlf-TimersAndConstants is provided by higher layers and is not set to release, the downlink radio link quality of the PSCell of the SCG is monitored by the UE for the purpose of indicating out-of-sync/in-sync status to higher layers. The UE is not required to monitor the downlink radio link quality in DL BWPs other than the active DL BWP on the PSCell.

Clearly, the UE is not required to monitor an SSB that is not in the active DL BWP:

The UE is not required to monitor an SSB that is not in the active DL BWP.

We also note that if the UE is not explicitly configured with a RadioLinkMonitoringRS, the UE is required to monitor the RS provided for the active TCI state for PDCCH reception. The only RS that is  a valid QCL source for PDCCH reception is the CSI-RS.

All of this indicates that if the UE does not support CSI-RS-based RLM, the UE would not perform RLM.

For BFD, the following text is applicable:

Excerpt from 38.213, Clause 6:

A UE can be provided, for each BWP of a serving cell, a set q0 of periodic CSI-RS resource configuration indexes by failureDetectionResourcesToAddModList and a set q1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList or candidateBeamRSListExt or candidateBeamRSSCellList for radio link quality measurements on the BWP of the serving cell.

<omitted>

If the UE is not provided q0 by failureDetectionResourcesToAddModList for a BWP of the serving cell, the UE determines the set q0 to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-State for respective CORESETs that the UE uses for monitoring PDCCH.

Hence, the RAN1 specifications do not even allow the use of SSB as a failure detection RS – under any circumstances: the only option is CSI-RS.

RAN1 specifications only allow CSI-RS as beam failure detection reference signal.

Beam management procedures were designed from the start to use only CSI-RS. They can operate in a BWP without SSB.

Beam management procedures can operate without SSB measurements.

Based on these observations, we can conclude that it is indeed a valid scenario in the standard to support operation in a BWP without SSB where the UE does not perform BM/RLM/BFD due to the lack of necessary reference signal.

# 3	Draft LS reply to RAN2

RAN1 thanks RAN2 for the LS on BWP operation without bandwidth restriction, and would like to provide the following answer to the question raised in the LS:

Question 1:

Whether it is a valid scenario in the standard to support the operation of BWP without SSB where the UE does not perform BM/RLM/BFD due to the lack of necessary reference signal (SSB and CSI-RS) in the active BWP.

## Answer:

According to RAN1 specifications, it is a valid scenario in the standard to support the operation of BWP without SSB where the UE does not perform BM/RLM/BFD due to the lack of necessary reference signal in the active BWP.

# Conclusion

In this contribution, we have discussed the RAN2 LS on BWP operation without bandwidth restriction. We propose to adopt the answer in section 3.

# References

R1-2203043, LS on BWP operation without bandwidth restriction, RAN2