3GPP TSG RAN WG1 Meeting #109-e	R1-2204037

e-Meeting, May 9th – 20th, 2022

Agenda item:		8.6.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Other Remaining Issues in RedCap Support

Document for:		Discussion and Decision

# Introduction

Under the Rel-17 RedCap WI, the following issues were discussed but remain unresolved –

- RAN1 specification update due to the RAN2 agreement regarding RSRP measurement before Msg1/MsgA retransmission

In this contribution, we address remaining issues related to RedCap support.

# PRACH Transmission Timeline

In RAN1#108-e, the issue of PRACH transmission timeline was discussed based on the following RAN2 agreement –

- From RAN2 perspective, if a RedCap UE in idle/inactive mode is configured with a separate initial BWP associated with no SSB (CD or NCD) for RACH, it is up to UE implementation to perform new RSRP measurement in a DL BWP associated with CD-SSB before Msg1/A retransmission.

The issue is whether the RedCap UE will need to follow the timeline requirement in 38.213 in case the UE would need to perform new RSRP measurement in a DL BWP associated with CD-SSB before Msg1/MsgA retransmission. The current timeline is stated as follows –

- UE is expected to transmit a PRACH no later than  msec after the last symbol of the window, or the last symbol of the PDSCH reception, where  is a time duration of  symbols corresponding to a PDSCH processing time for UE processing capability 1 assuming  corresponds to the smallest SCS configuration among the SCS configurations for the PDCCH carrying the DCI format 1_0, the corresponding PDSCH when additional PDSCH DM-RS is configured, and the corresponding PRACH. For , the UE assumes  [6, TS 38.214].

As discussed in RAN1#108-e, the UE may not be able to re-tune to a DL BWP with CD-SSB to perform RSRP measurement before Msg1/MsgA retransmission. However, in the RAN2 agreement, the decision to perform new RSRP measurement is up to the UE. In addition, our understanding is that the NT,1 + 0.75 msec is the earliest the UE can transmit Msg1/MsgA (as there may not be a random access occasion before then). Alternately, in case new RSRP measurement is required, the higher layer can wait and not trigger this until the measurement is peformed. Therefore, our view is that there is no change needed in RAN1 specification with respect to RAN2 agreement.

Observation 1: There is no need for a RAN1 specification update due to the RAN2 agreement regarding RSRP measurement before Msg1/MsgA retransmission.

# Conclusions

In this contribution, we consider the issue related to PRACH transmission timeline and make the following observation –

Observation 1: There is no need for a RAN1 specification update due to the RAN2 agreement regarding RSRP measurement before Msg1/MsgA retransmission

# References

- RP-210918, “Revised WID on support of reduced capability NR devices,” Nokia, Ericsson, RAN#91-e, electronic meeting.

- R1-2202532, “FL summary #5 on reduced maximum UE bandwidth for RedCap,” Moderator (Ericsson), RAN1#108-e, electronic meeting.

- R1-2202668, “FL summary #2 on other aspects of UE complexity reduction for RedCap,” Moderator (Qualcomm), RAN1#108-e, electronic meeting.