3GPP TSG RAN WG1 #109									    R1-2204048

e-Meeting, May 9th – May 20th, 2022

Agenda Item:	8.11.3

Source:	InterDigital, Inc.

Title:	Additional discussions on conflict indication timeline

Document for:	Discussion and Decision

# Introduction

The PSFCH timeline has been discussed in RAN1 #108e meeting and the following conclusion was agreed.

Conclusion There is no consensus in RAN1 to further introduce enhancement in Rel-17 on Mode 2 resource selection procedure to ensure the timeline (i.e., minimum time gap between PSFCH and a slot where a SCI is transmitted of sl-MinTimeGapPSsFCH, minimum time gap between PSFCH and a slot where expected/potential resource conflict occurs on PSSCH resource indicated by a SCI of T_3) for a conflict indication.

In this contribution, we discuss additional consideration for the PSFCH timeline.

# Discussion

It is recognized that the PSFCH timeline should ensure sufficient time for UE-A to perform a conflict detection and PSFCH transmission for a reserved resource. This imposes a time constraint, i.e. a minimum time gap, between the PSFCH occasion and SCI(s) scheduling the resource in conflict, when PSFCH occasion is determined based on the slot of the reserved resource. For this scenario, the following agreement was reached in RAN1 #107bis-e meeting [2].

Agreement - When PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI, time gap between the PSFCH and SCI(s) scheduling conflicting TBs is larger than or equal to X value - X = sl-MinTimeGapPSFCH - UE does not transmit the conflict indicator or receive the conflict indicator if the timeline is not satisfied

It is presumed that UE-B may not take this PSFCH timeline consideration into resource selection. Thus, it was agreed that no PSFCH transmission will be performed if the determined PSFCH occasion is too close to the SCI scheduling the resource in conflict.

Moreover, we think the PSFCH timeline should also be evaluated when PSFCH occasion is determined based on a slot in which the SCI scheduling the resource in conflict is received. This is a scenario indicated in the RAN #107-e agreement [3] below where a resource pool can be (pre)configured with the option of deriving a PSFCH occasion by a slot in which UE-B’s SCI is transmitted.

It is stated in the agreement that “Time gap between the PSFCH and a slot where expected/potential resource conflict occurs is larger than or equal to T_3”. However, in our view, it is not clear how this minimum time gap can be always ensured. For example, when there is only one PSFCH occasion between the slot in which UE-B’s SCI is transmitted and the slot in which the expected/potential resource conflict occurs, it is not guaranteed that the time gap between the PSFCH occasion and the slot in which the expected/potential resource conflict occurs will be larger than or equal to T_3.

Agreement - A resource pool level (pre-)configuration uses either of the following options - Option 1: PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted - Reuse PSSCH-to-PSFCH timing as specified in TS 38.213 Section 16.3 to determine the PSFCH occasion for resource conflict indication - Time gap between the PSFCH and a slot where expected/potential resource conflict occurs is larger than or equal to T_3 - Option 2: PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI - UE-A transmits the PSFCH in a latest slot that includes PSFCH resources for inter-UE coordination information and is at least T_3 slots of the resource pool before the PSSCH resource indicated by UE-B’s SCI in which expected/potential resource conflict occurs - FFS: How to account for processing timeline - Note that it is possible not to configure either option1 or option 2.

One option is UE-B always ensure a minimum time gap in resource selection between two selected resources when Scheme 2 is enabled and PSFCH occasion derived by a slot where UE-B’s SCI is transmitted is (pre)configured for the resource pool. The minimum time gap can e.g., be the sum of PSFCH periodicity of the resource pool and T_3. However, this imposes a limit on UE-B’s resource selection.

Another option is to specify a UE behavior when the agreed PSFCH timeline is not satisfied in this scenario. For example, when the time gap between the PSFCH and a slot where expected/potential resource conflict occurs is smaller than T_3, no PSFCH transmission should be performed. This is also consistent with the abovementioned RAN1 #107bis-e agreement for the other option of PSFCH occasion based on the slot where expected/potential resource conflict occurs.

Proposal 1:  UE does not transmit the conflict indicator or receive the conflict indicator if time gap between a PSFCH occasion determined based on a slot where UE-B’s SCI is transmitted and resource in conflict is smaller than T_3.

# Conclusion

In this contribution, we discussed additional consideration for the PSFCH timeline. We propose the following:

Proposal 1:  UE does not transmit the conflict indicator or receive the conflict indicator if time gap between a PSFCH occasion determined based on a slot where UE-B’s SCI is transmitted and resource in conflict is smaller than T_3.

# References

- RAN1 #108-e Chairman’s Notes, February 2022

- RAN1 #107bis-e Chairman’s Notes, February 2022

- RAN1 #107-e Chairman’s Notes, February 2022